## Mobile Software Programming

Due to this separation of common mobile software and language package, it is possible to fulfill the demands of the market requiring customization and flexibility.

As a consequence the software programming process in the LSO is divided into two different steps as followed:

Software update to latest version of signaling software

Programming of customer specific language package

STANDARD

SIEMENS

CHARGER

SIEMENS DONGLE

PC

XELIBRI X1

SIEMENS CABLE

(BA 2000 – SLIM)

SERIAL

CABLE

RS 232

FLASH  ADAPTER

SFA-110

xelibri x1 software programming setup

### Mobile Software Updating

Update the latest signalling software and language package with flash programmer. The software of the Xelibri X1 is loaded from a PC directly. Hardware interconnection between the mobile and the PC is shown in the figure above.

Flashing is done to the phone via the system connector. Flashing adapter SFA-110 for Xelibri is needed.

Flashing requires that the battery of the phone is charged.

### Flashing tools

Flash programmer is used for flashing. The latest software release and language package can be chosen. A Siemens dongle must be connected.

Flashing the product is done via the phone's system connector. In order to have a connection with a PC, the following equipment are needed: Siemens dongle (part no.x) adapter in PC's printer port, a Siemens connecting cable BA 2000-SLIM, a flash adapter SFA-110 (programming interface) connected with a standard RS 232 cable and a standard Siemens charger (US/China (part no.x), Euro (part no.x) or UK (part no.x).

The RS 232 cable is connected between the PC's serial port and the flashing adapter. A charger needs to be plugged to the flash adapter. Finally, the Siemens connecting cable needs to be connected between the flash adapter and the phone's system connector.

Use the flashing tools according to the instructions given in the Flash Programmer.

## Language Groups

### EU		xx-1 (xx= software release number)

- T9-English (EN)

- T9-French (FR)

- T9-Spanish (ES)

- T9-German (DE)

- T9-Italian (IT)

### China		xx-21 (xx= software release number)

- T9-Chinese (Simplified) (ZS)

- T9-Chinese Traditional (ZT)

- T9-English (EN)



Latest software release can be found from communication market

## Software programming process

This information is subject to change!



Contact your Service Manager for the order number of the right version of mobile software for your market.

No

Yes

Yes

No

No

Yes

Yes

No

Yes

No

Plug in the Flash Adapter to the PC and Mobile

Connect the Charger to the Flash Adapter

Power  up Flash Adapter & Check  Green LED

Check Charger

Faulty Charger

Faulty Flash Adapter

Power up the PC in Windows environment

Execute the Flash Programmer

Check settings and correct COM port from Flash Programmer

Update the SW according instructions

Correct settings and COM port

Test Mobile

Take note of error and repeat process again

Feedback Error to ICM CD ASC

END

Error ?

Error ?

Ok ?

Ok?

Ok?

Test with other mobile phone

No

Repair mobile phone

Yes

software programming process

## Flash Programmer

Open the Flash Programmer and the following window will appear.

Flash

### Flashing process

The flash button is disabled if the Flash Programmer options are not correctly set. (see page 37, Options)

- Select the correct hardware version from the list.

- Select the software release from the list. Latest is the biggest number of the software versions.

- Select the language package from the list. None means that no language package is selected.

- Press “Flash”. (Window shown below appears)

Preparing to flash

Reset the phone (turn phone off and on) and the flashing will start automatically.

The window seen below appears when flashing is done.

Flashing completed

Flashing is completed. Press “Close” to exit the flashing.

### Recovery Flash

If flashing of the phone fails you can use Recovery Flash. Select “Recovery Flash…” from the Phone menu as seen below. After using Recovery Flash you can try to flash the phone again.

Phone Menu

The window seen below will appear.

Recovery Flash

Reset the phone (turn phone off and on) and the flashing will start automatically.

The window seen below appears when flashing is done.

Recovery Flash complete

Flashing is completed. Press “Close” to exit the flashing.

### Copy User Data

Select “User Data Copy” from the Phone menu or press the  button to open the User Data Copying Wizard (see fig. below). It is used for copying all user data from one phone to another phone.

Phone Menu

Following window will appear.

Wizard

Start wizard by pressing the “Next>” button. The “Help” button opens Help for flash programmer. “Cancel” takes you back to main menu.

If “Delete copied user data from the source phone” is selected, source phone’s data will be deleted. (to use this function SIM-card must be placed to the phone)

Connect the source phone to the communication cable and press “Next>” to continue.

The following window will appear when copying user data.

Wizard copying user data

When the user data is copied, the following window appears.

User data is copied

Connect the target phone to the communication cable and press the “Next>” button to continue.

The following window appears after copying is successfully completed.

User data is copied to another phone

Press “Close” to exit.

### Options

To change Flash Programmer setups, select “Settings” from the Options menu (see figure below).

Options Menu

The following window will appear.

Settings

- Set baud rate to 115200 bit/s or higher depending on a serial port speed.

- Define the correct Root folder, where the software releases have been saved in, by pressing  button.

The following window will appear.

Browse for folder

- Select the correct root folder and press OK.

- Define the correct COM port(s) by selecting the “Ports” button.

The following window will appear.

Configure Ports

- Select the correct COM port(s) and press “OK”. (Multi-flashing can be done if the computer supports it.)