C35(i), M35(i), S35i

Level 2.5e

Repair Documentation

Annex 2

Winsui - Siemens User Interface

V1.0

How to install and use Winsui

Create a new directory named C:\Winsui_P35. Unzip the file Winsui_P35.zip into this directory C:\Winsui_P35.

Ensure that all files, shown in the picture above, are inside the directory.

Connect the mobile via Bootadapter to the PC on Com Port 1. Now start Winsui.exe.

Winsui is switching the mobile into Service Mode automatically.

The following setting are recommended for RX GSM900/GSM1800 measurements:

!!!! For GSM1800 change the channel only !!!!

!!!! All Pictures which are shown in the Annex 1 are based on this settings!!!!

External RF generator setting :

- RF channel: 124 - frequency 959,8 MHz

- RF level : -50dBm

- Frequency offset: 5kHz

Winsui setting:

GSM900

- Set the synthesizer to static channel mode. Use for RX/TX and MON channel 124.

- Set the AFC Value of the 13MHz oscillator to 16384.

- Select RxCont

- Set the IF amplifier to 58dB and the LNA  to High.

- Press Send all

Start RX measurements.

The following setting are recommended for TX GSM900 measurements

!!!! For GSM1800 change the channel only !!!!

Winsui:

- Set the synthesizer to static channel mode. Use for RX/TX and MON channel 124.

- Set the AFC Value of the 13MHz oscillator to 16384.

- Select NORM  for Rx/Tx Power

- Set Power Class to “5” and read out the Ramp.

- press the button Read TXPWM

- Select  Random With TSC and press the button read to get the DC Offset.

- Press Send all

Start TX measurements.

Ringer test

Winsui:

- Switch Mobile into Normal Mode

- Select Tools Menu

- Start Ringer measurement