A35/A36/A40

Level 2.5e

Repair Documentation

V 1.0

Table of Contents:

1	List of level 2,5e  parts A35/36	4

2	Required Equipment	5

3	Required Software	5

4	Radio Part	6

4.1  Power supply for RF part	6

4.2  Frequency generation	7

4.2.1  discrete 13MHz VCXO	7

4.2.2  VCO LO1	8

4.2.3  VCO LO2	9

4.2.4  PLL IC  part of SMARTi	10

4.3  Antenna switch	11

4.4  Receiver	12

4.4.1  receiver gsm 900/1800	12

4.4.2  Mixer, IF-Amplifier and demodulator 	13

4.4.3  SMARTI IC 	14

4.5  Transmitter	15

4.5.1 Moulator and up-conversion Loop	15

4.5.2 Power Amplifier and Antenna Switch	16

5     Power Supply	17

5.1 Overview and voltages	17

5.2 Pover supply ASIC	18

6     Battery and Charging	24

6.1 battery	24

6.2 Charging Concept	24

7     Logic part	26

7.1  Overview  Logic	26

7.2  Overview   EGOLD+	28

7.3  Overview   EGAIM inside Egold+	29

8     MMI Functions	32

8.1 Acoustics	32

8.1.1 Vibra	32

8.1.2 Microphone and Loudspeaker (Ringer)	33

8.2 Light (keypad and Backlight)	35

8.3 SIM-Card and Display	36

8.4 Connectors	37

8.4.1 I/O Connector	37

8.4.2 Battery connector	38

# 1. List of available level 2,5e parts  A3x

ID-No		Type		Name(function)/Location	Rep-Code	Order  No.

| V100 | Transistor | TXon1/Logic | 4SWT | L36840-C4009-D670 |
| --- | --- | --- | --- | --- |
| D100 | IC | Egold+/Logic | 4EGO | L36810-G6103-D670 |
| Z100 | Quartz | OSC_Egold+/Logic | 4OSC | L36145-F102-Y10 |
| V200 | Transistor | Charge/Logic | 4CHT | L36830-C1064-D670 |
| D200 | IC | ASIC/Pow.Supply | 4SPA | L36145-J4682-Y23 |
| N201 | IC | Volt.Reg./Logic | 4REG | L36810-C6062-D670 |
| V193 | Transistor | Light/Logic | 4LIT | L36830-C1096-D670 |
| V501 | Transistor | TX_PA/RF | 4PAT | L36840-C4013-D670 |
| V502 | Transistor | TX_PA/RF | 4PAT | L36840-C4013-D670 |
| V508 | Transistor | TX_PA/RF | 4PAT | L36840-C4014-D670 |
| N501 | IC | Op.Amp/RF | 4OPA | L36810-C6053-D670 |
| V435 | Transistor | Volt_SW/RF | 4SWT | L36840-C4009-D670 |
| V509 | Transistor | Diplex_SW/RF | 4TDI | L36840-C4035-D670 |
| V510 | Transistor | Diplex_SW/RF | 4TDI | L36840-C4010-D670 |
| V506 | Transistor | Diplex_SW/RF | 4TDI | L36840-C4009-D670 |
| V507 | Transistor | Diplex_SW/RF | 4TDI | L36840-C4009-D670 |
| N430 | IC | Volt.Reg./RF | 4REG | L36820-C6070-D670 |
| N431 | IC | Volt.Reg./RF | 4REG | L36820-C6147-D670 |
| Z502 | IC | PA/RF | 4PAM | L36851-Z2002-A40 |
| D400 | IC | Demod/Mod/RF | 4DEM | L36820-L6102-D670 |
| Z500 | Filter | IF_360/RF | 4IFF | L36145-K280-Y127 |
| Z555 | Filter | RX_PCN/RF | 4FI1 | L36145-K280-Y165 |
| Z556 | Filter | RX_GSM/RF | 4FI3 | L36145-K280-Y160 |
| D550 | IC | LNA/RF | 4LNA | L36820-L6047-D670 |
| Z600 | Quartz | 13MHz/RF | 4VCX | L36145-F220-Y6 |
| Z530 | VCO | 1LO_VCO | 4VC1 | L36145-G100-Y50 |
| Z570 | VCO | TX/RF | 4VCT | L36145-G100-Y32 |
| V530 | Transistor | 1LO/RF | 4TLO | L36820-C6047-D670 |
| V601 | Transistor | 13_AMP/RF | 4T13 | L36840-C4039-D670 |
| V230 | Transistor | Vibra/Logic | 4VIT | L36702-C1340-S67 |

- Required Equipment for Level 2,5e A35/36/40

- GSM-Tester		(CMU200 or 4400S incl. Options)

- PC-incl. Monitor, Keyboard and Mouse

- Bootadapter 2000  (L36880-N9241-A200)

- Troubleshooting Frame A35/36 (F30032-P81-A2)

- Power Supply

- Spectrum Analyser  (Advantest 3221)

- RF-Probe incl. Power Supply  (e.g. from Agilent)

- Oscilloscope incl. Probe

- RF-Connector  (N<>SMA(f))

- Power Supply Cables

- Dongle (F30032-P28-A1)

- BGA Soldering equipment

Reference: Equipment recommendation Level 2,5e

3. Required Software for Level 2,5e A35/36/40

- Windows NT Version4

- Winsui version1.22 or higher

- Winswup

- Windows software for GSM-Tester ( Cats or CMU-GO)

- Software for 13MHz adjustment

- Internet unblocking solution for “Service Mode”

4.  Radio Part

The radio part converts the I/Q base band signals supplied, by the logic (EGOLD+)	Notes

into RF-signals with characteristics as per the GSM recommendation

(transmission) which are radiated by the antenna.

Or the radio part converts the received GMSK signal, supplied by the antenna

into I/Q base band signals, which can be further processed by the logic (EGOLD+).

The radio part is designed for Dual Band operation and can therefore serve the

frequency bands EGSM900 and GSM1800. The radio part can never transmit and

receive in both bands simultaneously. However, the monitor time lot can be selected

independently of the frequency band.

Transmitter and receiver are of course never operated simultaneously.

The radio part consists of the following blocks:

- Power supply (RF-Voltage regulators)

- Synthesizer (partly located in SMARTI)

- Receiver (partly located in SMARTI)

- Transmitter (Up conversion loop partly located in SMARTI)

- Transmitter (Power amplifier)

- Antenna Switch

### 4.1 Power Supply RF-Part

Two voltage regulators (N430/431) with a nominal output voltage of 2.8V are

used, to perform the required “RF-Voltages”.

The voltage regulator N430 is activated via SLEEPQ_SMARTI provided by the

EGOLD+ (TDMA-Timer H12).

The voltage regulator N431 is activated via VCXOEN provided by

the EGOLD+  (Functional P7).

The name of the voltages are: a) VCC_SYN activated by VCXOEN

and  b) VCC2_8    activated by SLEEPQ_SMARTI

For both voltages BATT+ is required.

4.2 Frequency generation

4.2.1 Synthesizer : The discrete VCXO (13MHz)

The Axx 13MHz signal is generated by a discrete VCXO. consisting of a colpitts 		Notes

oscillator with a crystal Z600 and a post-switched buffer stage as oscillator switch.

The subsequent oscillating circuit (C611,C619, L600) and the resistor R611

create a de-coupling of the synthesiser from interference signals, coming from the

logic (SIN13M (functional F13M)).

The oscillator frequency is controlled by the EGOLD+ (functional R3) generated

AFC_PNM signal, and the capacity diode V600, to ensure that the circuit is

working is stable conditions < 7,7ms after switching on the oscillator.

To reduce the charging time of the low pass (R615, C616) the resistor R615 is

bridged by the diode V602.

For temperature control a temperature-dependent resistor R613 is placed near the

VCXO.

The required voltage VCC_SYN is provided by the N431.

The picture 4211 shows the signal at the collector of transistor V601_2.

4211

to SMARTI

to EGOLD+

4.2.2  Synthesizer : LO1

The first local oscillator (LO1) consists of the D400 PLL part, a loop filter			Notes

and a VCO (Z530) module. This LO1 circuit generates frequencies from 1445MHz

to 1520MHz for GSM 1800 RX-operation, and from 1285MHz to 1361MHz

for the other operations. The Z530 is switchable to select the channels in stages of

200kHz. The VCO module is activated  by the EGOLD+  signal

PLLON (TDMA-Timer J12) via V530. The switching between GSM900 and GSM1800 is

realised by the OSW signal from the SMARTI (D400 pin 21),

The channel programming happens via the EGOLD+. signals SYGCCL, SYGCDT,

SYNSTR.

The VCO output signal fulfils three functions:

- It enables the SMARTI IC to mix the RXIF-Frequency (360 MHz)

- It enables the SMARTI IC to mix the TXIF-Frequency (424 MHz)

- It ensures with the help of the 13MHz signal and the SMARTIs PLL part the

frequency stability by generating a control voltage at (SMARTI pin 20)

The required voltages are: VCC2_8 for D400  provided by N430.

VCC_SYN for the VCO provided by N431

The picture 4221 shows the VCO output signal

The picture 4222 shows the control voltage

The picture 4223 shows the programming signals for the PLL

4221

4223

4222

4.2.3 Synthesizer : LO2

The second local oscillator (LO2) consists of:						Notes

- the SMARTIs (D400) PLL part

- the 13MHz reference signal

- a switchable VCO (inside the SMARTi)

- an external filter  (C509,C510,R501).

The VCO generates 2 frequencies (1440MHz for RX-Mode and 1696 MHz for

TX-Mode).

Both frequencies are divided by 4, to get the 360MHz demodulator frequency and

the 424MHz modulator frequency.

The RX/TX switching is done internally,  initiated by the EGOLD+ through the

SYGCCL, SYGCDT,SYNSTR signals.

Using the EGOLD+ signal PUPLO2 (TDMA-Timer L11) at pin 16, the second local

oscillator is switched “ON and OFF”

The required voltage VCC2_8 for D400 is provided by N430

PUPLO2

External filter LO2

4.2.4 Synthesizer : PLL

## PLL as a part of the PMB6250 (SMARTI) IC D400

## Blockdiagramm

4.3  Antenna switch (electrical)

Internal > Receiver/Transmitter <> GSM900/1800

The Axx mobile has in opposite to P35 only one antenna switch (electrical).		Notes

The electrical antenna switch (Diplexer Z503) is used for the differentiation

between the receiving and transmitting signals, just like the differentiation

between GSM900 and GSM1800.

To do so the transistors V506,V507,V509 and V510 are used to switch the

diplexer signals VC1–VC4 as required.

from PA

to PCN receiver

to GSM receiver

Please note, that Axx mobiles have no external antenna connector.

The matrix below shows the different conditions at the diplexer and the

accompanying signals.

4.4  Receivers

4.4.1 Receiver: GSM900/1800 Diplexer > Filter > LNA > Mixer

From the antenna switch up to the IF-Mixer, the GSM1800 receiver circuit 		Notes

consists of:

- a ceramic front end filter (Z555)

- a LNA (Low Noise Amplifier D550)

- and a subsequent discrete distortion LC-high-pass-filter.

For GSM900 the signal flow is as follows: From the antenna switch, via a SAW filter

(Z556), through the LNA (D550) and a discrete distortion LC-low-pass-filter to the

IF-Mixer..

The amplification of both LNA´s  is approx. 20dB with a matched 50 ohm output.

The LNA is  switched “On” with the signal LNAON (SMARTI pin 36).

For switching between GSM1800 and GSM900 the signal BSW is generated from

the (SMARTI pin 37).

The signal LNAG (SMARTI pin 33) is in use, if the received signal level is too high.

with LNAG “active”, the LNA reduces the incoming signal in one step by 20dB

The non-symmetrical LNA output  is connected to the IF mixer via a discrete

balancing and adaptation circuit. This circuit converts the asymmetrical

signal into a symmetrical signal.

The required voltage VCC2_8  is provided by N430

4411

4413

4412

4414

4.4.2	Receiver : Mixer, IF Amplifier and Demodulator

The symmetrical LNA amplified EGSM900 and GSM1800 signals arrive at the		Notes

SMARTI pins 31,32,34,35, to be mixed down to the IF frequency of 360MHz..

To do so, the LO1 is guided via the SMARTI pins 22,23 to the different mixers.

Frequency calculation of the LO1:   (Freq.GSM = RF+IF and Freq.PCN = RF-IF)

After mixing, the IF is filtered by an external SAW-Filter (Z500) to arrive again at the

SMARTI  (input of the PGC-Amplifier).

The programmable PGC-Amplifier ensures an equalized level of the IF-Signal at the

input of the I/Q demodulator. To perform this, the PGC-Amplifier is switched

in 2 dB steps using the EGOLD+ signals (SYNSTR;SYGCDT;SYGCCL).

The demodulation is realised by the I/Q demodulator. To mix out the IF, the

2nd local oscillator frequency (1440MHz) divided by 4, is used.

Finally the “I” and “Q”  signals are amplified and ready for further operations

through the GAIM part of the EGOLD+ (A/D conversion) as RX_I,RX_IX-RX_Q,RX_QX

at the SMARTI pins 1,3,4,48

The required voltage VCC2_8  is provided by N430

4433

4434

to

baseband

from LNA

4.4.3  SMARTI IC

Not active

4.5  Transmitter

### 4.5.1 Transmitter: Modulator and Up-conversion Loop

The Axx modulation is based on the principle of the up-conversion modulation		Notes

phase locked loop and is accomplished via the SMARTI(D400).

The 2nd local oszillator provides the quadratic modulator with the TX IF signals

(1696MHz divided by 4 = 424MHz).

This resulting GMSK RF signal is compared in a phase detector with the down mixed

GMSK RF signal from the TX-VCO to get a control voltage for the TX-VCO.

Down mixing,  is done in an internal mixer.

To close the loop, the control voltage (output of phase detector) is connected to the

switchable TX-VCO (Z570) after passing  a low pass filter.

The TX-VCO, a “two in one” VCO  is switched with the EGOLD+ signals

GSM_TX_VCOENQ(TDMA-Timer J13) and PCN_TX_VCOENQ(TDMA-Timer K12) into

the different modes.

The required voltage VCC_2,8  is provided by N430

4512

4511

: 4

Phase

Detector

I Q

Mod

ulator

2.LO

1696 MHz

1.LO

1285-1361 MHz

424 MHz

424 MHz

424 MHz

TX-VCO

TP

880-915 MHz

1710-1785 MHz

Mod Signal

4513

4.5.2 Transmitter : Power Amplifer Amplifier and Antenna Switch

Splited by a discrete circuit into EGSM900 and GSM1800, the TXVCO output signal 	Notes

arrives at the power amplifier. The dual band power amplifier module (Z502)

is assembled on a ceramic substrate in one housing. The module amplifies the

output signal of the TXVCO to the required PCL (controlled by the feedback circuit

according to settings from the logic) .The different amplifiers are switched on by

TXONPCN/TXONGSM via the transistor (V508). The signal PA_Comp is required for

the operation point setting of low GSM PCLs. The power amplifier is supplied directly

from the battery (BATT+).

After amplification the signal passes on the way to the antenna the diplexer (Z503).

A part of the TX output signal is decoupled via a directional coupler (realised by

conductive tracks), is equalised and temperature compensated by a detector

diode (V505).

This so gained voltage, is compared through an operation amplifier (N501) with the

PA_RAMP signal provided from EGOLD+(Baseband H2). The resulting voltage is

switched with the signals TXONPCN and TXONGSM (V509) to the control ports of

the PA.

The required voltage BATT+ is provided by the battery.

The required voltage VCC2.8SW is provided by transistor V435 by using TXON1

from the EGOLD+(TDMA-Timer H14) and V100.

4522

4521

4523

4524

### 5. Power Supply

Ri

Linear-

regulator

[ASIC]

Linear-

[ASIC]

1,6 A

TX

110 mA max.

2,9V +-2%

2,65V +-3%

140mA max.

220mA max.

2,8V +- 3,5%

2,8V +- 3,5%

75 mA max.

45 mA max.

EGOLD+ Core

Flash

Display

Illumination, Display

+ Keypad, Vibra (optional)

Flash 2) + SIM

EGOLD+ Portpin

RX, TX

RF Synth / VCXO

NiMH 600mAh

PA

+Control

Linear-

regulator

[ASIC]

2,07V +-3%

VDDRTC

Linear-

regulator

[ASIC]

2,0V +-5%

100mA max.

10mA max.

EGOLD+ Ringing

(16Ohm HK+GAIM)

193mA + 20mA.

55mA+6mA

22 µF

(at PA)

Display

20mA max.

1mA max.

2) Intel 55mA max.

ST 20mA max.

100mA max.

100mA max.

regulator

Linear-

N430

regulator

Linear-

N431

regulator

5.1  Overview and Voltages							 Notes

The following restrictions must be considered:

- The phone cannot be operated without battery.

- The phone will be damaged if the battery is inserted the wrong way round

(the mechanics of the phone prevent the battery from being put in the

wrong way round). The electric system assumes that the battery as been

inserted correctly.

#### Voltages overview

5.2  STV-ASIC

The power supply ASIC contains the following functions:					Notes

- Control of “switching on” the mobile phone via

1.The On/Off Key. (ON_OFF)

2.The bottom connector with (External Power)

3.The Real Time Clock RTC (ON_OFF2)

- Watchdog monitoring

1.Control of “switching off” the mobile phone via WATCHDOG_P.

2. Watchdog observation

- “Switching off” of mobile phone in the case of overvoltage at battery

connectors.

- Generation of RESET signal for EGOLD+, Flash and MMI components

- Generation of 2.90 V via linear controller for the logic IC.

- Generation of 2.65 V via linear controller for the logic IC.

- Generation of 2.00 V via linear controller for the logic IC.

- Battery charge support:

1.Normal charging

2.Trickle charging, if the battery voltage is below 3,2 V

- Timer generation:

All internal timers and pulses are derived from a 900 kHz  10% internal oscillator.

Responsible for the frequency stability is an external resistor (R213) (1%) at the

RREF pin

5.2.1 Switch “ON” sequence

- Generation of the “2,0; 2,65; 2,9” voltages

- Generation of the “RESET_2,0V and RESET_2,65V”

- Generation of the “POWER_ON” through N201

- Generation of the “Watch Dog” signal through the EGOLD+

after “POWER_ON”

Power supply ASIC

Pin Configuration in accordance with Component Specifications:

| Functions | Pin Requirements | Implementation / Explanation |
| --- | --- | --- |
| “Switching on” the mobile phone | ON_OFF EXT_POWER ON_OFF2 | The following 3 “switch on” conditions will be accepted: - Falling slope to ON_OFF - Rising slope to EXT_POWER - Rising slope to ON_OFF2 If one of this 3 conditions is recognised, the ASIC switches into the POWER-Up state and the internal oscillator starts. After T1 (approx. 60ms) the source of the “switch on” signal is checked again. If the required level is no longer present, the mobile phone will not switch on and the ASIC switches to the POWER-DOWN mode. If the required level is still there, the mobile phone will “switch on”. To do so VCXOEN (internal SLEEP) will go to HIGH and the 3 voltage regulators VREG1 (2,9V), VREG2 (2,0V)and VREG3 (2,65V) are active. After T2 (approx. 60ms) counted from “switch on” (i.e. 120ms from initial recognition of the “switch on” condition) the supply voltages for 1V9, 2V65 and 2V9 will be checked. If the 1V9 and 2V9 voltages are in order, timer T3 will start and RESET will be enabled after approx. 60ms. To ascertain by what signal the phone was switched on, the E-Gold checks the following signals. KB7 >>>>>>>>>>> to check the “On-Off Keypad button” RTC_INT>>>>>>>> to check the timer “on-off” EXT_Power_c>>> to check ON-OFF via accessory |

| Functions | Pin Requirements | Implementation / Explanation |
| --- | --- | --- |
| Watchdog monitoring | WATCHDOG_UP | The first Watchdog Impulse of the EGOLD+ - must be operated at the latest 800ms after the rising edge of the “Reset” signal and - the WD-Signal must have a rising edge. If one of these conditions is not fulfilled, the mobile phone will switch off. If this conditions are fulfilled, rising and falling edges are evaluated alternately and used to reset the WD-Timer. With each edge at the WATCHDOG_UP pin the WD-Timer will be reseted. Conditions: The next (inverted to the previous one) edge have to occur within a time of 0.4s...2.6s. If the next edge is recognised before expiry of 0.4s or after an expiry of 2.6s or if the next recognised edge is not inverted, the mobile phone will switch off. |
| Regular “switch off” of the mobile phone | WATCHDOG_UP | Switching off of the phone is possible, as described in the watchdog monitoring control, if the EGOLD+ is not longer serving the WATCHDOG_UP pin. “Switching off” means: RESET to LOW POWER_ON to LOW CHARGE to HIGH-Z. Regulator 1V9, 2V65 and 2V9 are “OFF” The ASIC goes into the Power-Down mode. |
| Low Voltage Detector | LOWVDD | Not used |
| Reset Signal | RESET | Power-Up: During the entire “switch on” procedure the RESET signal is on “LOW-Level”. If the “switch on” conditions are fulfilled the change to “HIGH-Level” is taken place after 180ms. Power Fail: In the “unit on“ mode the voltage levels of the 1.9V and 2.9V voltage supply shall be monitored. If one of this voltages drops below a certain level for longer than 10s, the RESET signal switches to “LOW-Level”. |
| Switch off of the phone in the case of overvoltage at the battery | VDD | In the case of a too high voltage at the VDD,(voltage level 5.8V  0.2V within 1s), the mobile phone will switch off. |
| Voltage Supply for the Logic | VREG2 U2V0 | The linear controller is designed for 1.92V(3%) and a maximum current of 100 mA. It consists basically of an internal operation amplifier, an integrated p-channel output transistor as well as an external capacitor (C = 2.2F) for stabilising the voltage. This regulated voltage is measured internally. In case of an internally measured voltage >3.1V , the output transistor will switch off. |

| Voltage Supply for the Logic | VREG3 U2V65 | Not used |
| --- | --- | --- |
| Voltage Supply for the Logic | VREG1 U2V9 | The linear controller is designed for 2.92V(3%) and a maximum current of 140 mA. It consists basically of an internal operation amplifier, an integrated p-channel output transistor as well as an external capacitor (C = 2.2F) for stabilising the voltage. This regulated voltage is measured internally. In case of an internally measured voltage >3.3V , the output transistor will switch off. |
| Voltage Supply for #### SIM-CARD | CCVZQ CCVCC | The voltage supply for the SIM-CARD (CCVCC) is switched by the signal CCVZQ from the EGOLD+ via a “low” at pin 12 The output signal CCVCC can be measured at pin 13 (2,8V) The picture 5212 shows the CCVCC depending on the signal 5211 CCVCZQ from EGOLD+ |
| #### Charge Support | CHARGE, CHARGE_uP, TBAT | For controlling the battery charge function, a charge support is integrated in the ASIC. It consists basically of an internal current source, a temperature sensor, an external charge FET with a Pull-Up resistor between the source and the gate of the charge FET. The current source is switched on trough a rising edge of the CHARGE_UP signal and generates an “LOW” at pin 3 (Charge). With this “LOW” the charge FET becomes conducting. Exceptions: - The temperature comparator does give a signal for high temperature - An overvoltage is present at the VDD. C) A falling edge at the CHARGE_UP. |
| #### Low Voltage Regulator for RTC | VDD_RTC | Voltage regulator to provide the Real Time Clock with the required 2V. Current consumption is ~1mA |

2,65V  220mA

2, V  100mA

Example of a timing diagram  (switch on by the keypad)

Example of a timing diagram  (Watchdog Observation)

2. WD-Observation

1. WD-Observation

3. WD-Observation

6. Battery and Charging

#### 6.1 Battery

A NiMH battery with a nominal capacity of 600mAh is used for the Axx series 		Notes

A temperature sensor (22k at 25°C)  is integrated to monitor the battery

temperature.

Battery connector:

## 6.2 Charging Concept

The battery is charged in the unit itself. The hardware and software is designed for

for NiMH batteries. The battery will be charged as long as the GAIM part of the

EGOLD+ will measure changes in the values of the battery voltages during the

charging process.

There are two ways to charge the battery:

- Normal charging also called “fast charging”

- Trickle charging

Normal Charging:

As soon as the phone is connected to an external charger, charging starts.

(The “Charge” symbol in the display shows the charging process).

Charging is enabled via a MOS-FET-Switch(V200). This MOS-FET-Switch activates

the circuit form the external charger to the battery.

A “low” signal at the gate of this transistor is required. To get this “low” the following

conditions must be fulfiled :

- The CHARGE_UC signal at pin 31 of the ASIC must be “H”

- The temperature information at pin 2 of the ASIC must be “L” < 55°C

- The overvoltage detection at pin 22 VDDLP has not to be active < 5,8V

A disable function in the ASIC´s hardware can interrupt charging in case of:

- A too high temperature >55°C

- A too high battery voltage >5,8V

For temperature detection, a NTC resistor (22k at 25°) is assembled in the

battery pack. Via the pin 2 of the battery connector and a resistor combination R115,

R116,R117, the EGOLD+ (GAIM L3) is carrying out the measurement.

To get the CHARGE_UC (Miscellaneous E9) signal, the EGOLD+ needs the information

that an external power supply is connected. This is realized by feeding the POWER

signal through the voltage divider R204 and the resistor R202 as a

EXT_POWER_UC to the EGOLD+ (Miscellaneous A2)

Trickle  Charging

If the phone has not been used for a longish time (longer than approx. 1 month), 	Notes

the battery could be totally self-discharged (battery voltage is below  3.2 V), so that

it is not possible to charge the battery via the normal charging circuit. In this case

only trickle charging is possible The charging current for trickle charging is  <10mA.

After approx. 10 hours of trickle charging the mobile switches to “normal charging”

The trickle charging circuit consists of the resistor R200 and the 2 diodes V202

and V204.

Charging Circuit

Trickle charging by pass

Charging activation

Power supply recognition

!! Attention!!

   a charger voltage >15V can destroy resistors or capacitors in the charging path

- a charger voltage >20V can destroy the MOS-FET switch transistor in the

charging path.

7. Logic Part

7.1 Overview  Logic

The logik part of the Axx consists of:

- The EGOLD+ 									NOTES

- Hardware µC-part

- Mikrocontroller

- Systeminterfaces for internal and external peripherals

- Onchip peripherals and memory chips

- Software µC-part

- Controlling of the MMI (Man Machine Interface) functions

(Keypad, LCD, Illumination, ... )

- GSM Layer 1-3

- Controlling Radio part (Synthesizer, AGC, AFC, ...),

- Controlling of the Baseband part (EGAIM)

- Hardware SP (Signal Processor) part

- DSP Signalprocessor

- Software SP (Signal Processor) part Equaliser

- Channel Coder

- Channel Decoder,

- Voice Encoder

- Voice Decoder

- EGAIM inside the EGOLD+

- 2 Sigma Delta A/D-converter for RX/Battery

- 2 D/A- converter for GMSK TX Inphase- und Quadratursignals,

- 1 D/A- cverter for the Power Ramping signal,

- 1 Sigma Delta A/D- and D/A-converter for the voiceband part

- RTC (Real Time Clock)

2)    The Flash IC

7.2 Overview  EGOLD+

7.3 Overview  EGAIM inside the EGOLD+

Tasks of the EGAIM inside the EGOLD+

- Measurement of Battery and Ambient temperature				Notes

- Measurement of Battery Voltage

- A/D conversion of MIC-Path signals incl. coding

- D/A conversion of EP-Path signals incl. decoding

- Generating of the PA-Control Signal “PA_Ramp”

- Generating of the Ringer Signal on EPP

- Measurement of Battery and Ambient Temperature

The temperature is measured as a voltage equivalent of the temperature on the	Notes

voltage dividers R115,R116,R117 for the battery temperature and R118,R119,

R120 for the ambient temperature from the EGAIM.

For this, the integrated  converter of the EGAIM of the RX-I base band

branch is used. This  converter compares the voltage of TBAT and TENV

internally with a reference voltage BREF.

Via an analog multiplexer, either the RX-I base band signal, or the TBAT signal

and the TENV signal can be switched to the input of the converter.

The signal MEAS_ON from the EGOLD+(GSM TDMA-TIMER G11)  activates the

measurement and is used to generate to BREF by the help of R121,R122

## Measurement of the Battery Voltage

The measurement of the battery voltage is done in the Q-branch of the EGAIM.

For this BATT+ is connected via a voltage divider R113, R114 to the EGOLD+

(GAIM N2) (Input limitation 1.33V to 5.91V) .An analog multiplexer does the

switching between the baseband signal processing and the voltage

measurement.

- A/D conversion of MIC-Path signals incl. coding

The Microphone signals (HF_MICRO, MICN2, MICP, MICN) arrive at	Notes

the voiceband part of the EGAIM. For further operations the signals will

be converted into digital information, filtered, coded and finally formed

into the GMSK-Signal by the internal GMSK-Modulator.

This so generated signals (IT, ITX, QT, QTX) are given to the SMARI IC

(pins 9-12) in the transmitter path.

- D/A conversion of EP-Path signals incl. decoding

Arriving at the Baseband-Part the demodulated signals (RX_I, RX_IX, RX_Q,

RX_QX) will be filtered and A/D converted. In the voiceband part after decoding

(with help of the uC part) and filtering the signals will be D/A converted amplified

and given as (EPP1, EPN1, EPP2, EPN2) to the internal earpiece or the external

loudspeaker.

- Generation of the PA Control Signal (PA_RAMP)

The RF output power amplifier needs an analog ramp up/down control voltage.

For this the system interface on EGOLD+ generates 10 bit digital values which

have to be transferred serially to the power ramping path. After loading into an

10 bit latch the control value will be converted into the corresponding analog

voltage with a maximum of ~2V

Key-Value

8. MMI-Functions

8.1 Acoustics									Notes

Electro-Acoustic components are:	a) VIBRA

b) Microphone

c) Loudspeaker

8.1.1 VIBRA

The VIBRA is activated and controlled from the EGOLD+(Miscellaneous E10)

via the signal, VIBRA_ON. For this the VIBRA is connected to XG230 with BATT+.

GND is given by the shielding of the RF-Part. With a “H” of the VIBRA_ON signal

the transistor V230 is switched on. The signal VIBRA_ON appears also at the

EGOLD+(Miscellaneous M6). This “second” VIBRA_ON is only used for ~40 to 65ms

due to a higher voltage required from the VIBRA in the beginning. The diode V231

is used to protect  the circuit against over voltage and switching spikes.

EGOLD+

timing of VIBRA_ON signal

8.1.2 Loudspeaker (Ringer) and Microphone

Loudspeaker (EPP1, EPN1, EPP2, EPN2) and Microphone (HF_MICRO, MICN2, 	Notes

MICP, MICN) are connected directly to the Voiceband-Part of  EGOLD+

Loudspeaker(Ringer)

Beside the normal Loudspeaker function the Loudspeaker is acting as a Ringer

controlled by the EGOLD+

The EGOLD+ ringer function is implemented as a special mode of operation of the

earpiece buffer EP1. In ringer mode the output of  is connected to a specific

driver circuit which guarantees a definite slew rate of the ringer output signal. This

feature ensures that excessive load of power supply circuits and overshoots at the

ringer output pins due to inductive load are avoided.

Microphone

Two Microphones can be connected to the EGOLD+ (external/internal) with the		Notes

lines HF_MICRO and MICP. Both Microphones are supplied with VMIC from the

EGOLD+(Voiceband E1). To avoid LF distortions, VMIC is low pass filtered R173, C175

and grounded (R170-1, R176).

The internal Microphone is connected to XG170. The capacitors C177,C179 are used

to suppress HF distortions.

The input ports a the EGOLD+ are protected versus DC voltages with the capacitors

C115-C118. An EMC protection is realised by C132,C129.

Voiceband part inside the  EGOLD+

8.2  Light (keypad and backlight)

All LED´s (Keypad and Backlight) are connected to BATT+. The LED´s are switched	Notes

on by the EGOLD+ (miscellaneous N7) controlled signal LIGHT_ C via the

transistor V193. The signal LIGHT_OFF can be used to switch off the light during

the TX-Burst is active.

The Illumination:	a) Display

b) Keypad

8.3  SIM-Card- and Display Connector

The SIM-CARD is supplied at pin2 with CCVCC. To get CCVCC (2,9V) the 		Notes

ASIC D200 pin12 is used to switch 2,9V pin 14 to pin 13with the EGOLD+(SIM L18)

signal CCVZQ. If no SIM-CARD is connected the EGOLD+ tries 3 times to connect

the SIM-CARD. After this time the EGOLD+ stops trying.

That means, if the EGOLD+ is losing the connection while normal operation of the

mobile phone, the mobile must be switched off and on again.

The communication between the EGOLD+ and the SIM-CARD is done via the CCIO

pin5 by using CCCLK pin6 as a clock signal.

The diode V351 is used to protect CCCLK and CCIO versus switching peaks.

from/to EGOLD+

The display is provided with 2,65V from the ASIC D200. The communication with the

EGOLD+ is realised via the serial signals SCL and SI

  - Connectors

8.4.1  I/O connector

Notes

| Pin | Name | IN/OUT | Notes |
| --- | --- | --- | --- |
| 1 | GND |  |  |
| 2 | SB | I/O | Charger coding and charger control. |
| 3 | POWER | I | Charging Current |
| 4 | FBatt+ | O | Power supply for the accessories. |
| 5 | TX | O | Serial interface |
| 6 | RX | I | Serial interface |
| 7 | ZUB_CLK | I/O | Clock line for accessory bus Use as DTC In data operation |
| 8 | ZUB_DATA | I/O | Data line for accessory bus. Use as CTS in data operation |
| 9 | GND_MIC |  | For external microphone |
| 10 | HF_MIC | I | External microphone |
| 11 | AUDO | O | Trigger for external loudspeaker |
| 12 | GNDA |  | For external loudspeaker |

8.4.2 Battery connector

| Pin | Name | IN/OUT | Notes |
| --- | --- | --- | --- |
| 1 | GND |  |  |
| 2 | Akku_Temp | O | Temperature control of the battery pack. |
| 3 | POWER + | I/O | Battery voltage |