| WIN FLASH MEM | USER GUIDE |
| --- | --- |
| Project Name : WIN FLASH MEM |  |
| Author : VENKI |  |

- History of Change

| Version | Date | Editor | Status | Description |
| --- | --- | --- | --- | --- |
| 01 | 29-04-2004 | Venki | Released | First Win Flash Mem release |
| 02 | 07-10-2004 | Venki |  | Added Support for Cmd line version of win flash mem. Added the supported command line options in the user guide |
| 03 | 19-10-2004 | Venki |  | Support for FLASH ini files and Project INI files are added. |
| 04 | 19-12-2004 | TKozlowski |  | Verify checksum button added, default project list deleted, flash gap supporting. |

Contents

History of Change	1

1	Introduction	3

2	Tool Startup	3

3	Features Of GUI Version	3

3.1	Settings	3

3.2	Start	5

3.3	FLASH READ	5

3.4	FLASH WRITE	5

3.5	FLASH ERASE	6

3.6	FLASH INI	6

3.7	INFO	6

3.8	MEMORY VIEW	7

3.9	Logging	7

3.10	SWITCH OFF MOBILE	7

3.11	EXIT	7

4	Features of Command Line Version	7

5	Example BOOT PIN INI file	9

6	Example PROJECT INI file	10

7	Example FLASH INI file	10

# Introduction

WinFlashMem is a tool developed to run on host pc to communicate with the mobile phone using the protocol “SW update on Siemens Mobile Phone”. This tool can be used to read the read the flash contents to a SREC/BIN file format or view the flash contents or erasing a flash block. All the supported features are explained in detail below.

# Tool Startup

Running the program “WinFlashMem.exe” brings up the GUI.  The main window has various frames/blocks. They are explained below. The command line version can be invoked with the same program, “WinFlashMem.exe”, with the command line options. The list of supported command line options and their description is explained in this document.

Be aware if flashmem.ini is present in program directory.

# Features Of GUI Version

The features of the tool are explained in detail.

## Settings

In this frame, one can configure all the setting required by the WinFlashMem to start the mobile. All these values are used when the mobile is started. After the mobile is started, changing these values doesn’t take any effect.

One can configure the following parameters.

- Configuration

  - Clicking on this button, brings a new dialog. Using this, user can configure the project settings and flash settings.

  - Project INI file

    - The project INI file contains the list of project names, id, start address, end address & dll to be used for each project respectively.

    - The tool has a default INI file which is parsed at the init time of the tool.

    - User can have his own INI file for new projects and add them to the tools project list by clicking on the button “Project INI File” to select the INI file and click on the respective “Apply Settings button”. User can repeat this operation for multiple INI file. New projects are appended to the project list. Existing projects are overwritten with the latest INI file values.

    - For an example INI file, please take a look at “FlashMem.INI” from the installation folder.

  - Flash INI file

    - The Flash INI file contains the information about flash layout.

    - It contains the flash sections with name, start address & end address.

    - There is no default INI file, used by WinFlashMem.

    - User can add a INI file and use the button “Apply Settings”.

    - Using a new INI file will clear all the previous INI file’s settings.

  - Click on the button “BACK” to come to the main dialog.

- Project  Selection

  - The list of projects parsed from the default INI file and user supplied INI file are presented to the user as a list.

  - User can select the project from the list of projects available for the mobile platform.

- Load dll

  - Clicking on this button will load the selected project’s settings and load the respective project’s dll.

- Update S/W

  - Each platform dll has a built in Update S/W. By default, this is used by the tool. If the user wants to load his own Update S/W then he can do that by the following steps.

    - Click on “Update S/W from file”.

    - Select the BIN file which has the Update S/W.

- Com Port

  - This tool detects the number of com ports available in the host pc and lists them. It also stores the last used comport in the registry and uses the same in the next invocation.

- BAUD RATE

  - User can select the baud rate from the list. The tool does not know if the comport H/W supports the selected baud rate. It is the responsibility of the user to select the correct baud rate. The last used baud rate is stored in the registry and uses the same in the next invocation.

- Boot With Pin

  - User can boot the mobile even if the mobile is in customer mode.  For this a boot pin of 16 bytes is to be supplied to the mobile. By selecting “Boot Win Pin”, user can either enter the boot pin manually or supply the INI file with the list of boot pins for selection. Follow the steps given below.

    - Select “Boot With Pin”. This opens a new window.

    - In the new window, one can enter the 16 bytes boot pin value  in the right side edit window with a space between each bytes and click “OK”.

    - In the new window, one can click on “Get Pin From File” and select a INI file.

      - The tool will scan the files for the boot pins and the associated names.

      - It lists all the pins with their names in the list box for selection.

      - Now the User can select the Boot Pin he want to send and click “OK”.

      - For an example INI file, look at the bottom of this user guide.

## Start

In this frame, user can start the mobile by downloading the boot strap loader and update s/w to the mobile. After configuring all the setting required, one needs to click on the button “Start” to start the mobile.

## FLASH READ

In this frame, user can read the data from start address to end address and store the data in SREC/BIN file format. Each S3 record contains 16bytes of data. The tool throws an error if the address range entered is wrong.

Follow the steps below.

- Enter the start address in the edit box right to “Start Add”

- Enter the end address in the edit box right to “End Add”

- Click on “File Name” to specify the file name. If the file already exists then it will be overwritten. The file extension should be either “SREC” or “BIN”.  According to the extension, the file format is selected.

- Click on “READ FLASH” to start the operation. The progress bar indicates the completion percentage of the operation.

- When the operation is completed, user gets a notification window with the message as “Reading From Flash is Over”.

## FLASH WRITE

In this frame, user can write a SREC/BIN file to the flash.

- For SREC Files

  - The tool reads the specified SREC file. It scans the file to find out the start address and end address. It erases all the blocks between these 2 addresses and writes the data to the flash. It is the responsibility of the user to make sure that the SREC file is block aligned. Otherwise, data will be lost.

  - If the file contains unknown records then the operation is aborted.

- Click on “File Name” to specify the file name. The extension of the file should be either “SREC” or “BIN”. According to the extension, the file format is selected.

- Click on “WRITE FLASH” to start the operation.

  - If  the selected file is BIN file then

    - A new dialog appears for user to enter the start address and end address.

    - Clicking on OK will continue the operation.

    - Clicking on Cancel will cancel the write operation.

  - User gets a warning window with the message about the block range that will be erased. User can decide to continue or abort at this point.

  - The progress bar indicates the completion percentage of the operation.

  - When the operation is completed, user gets a notification window with the message as “Writing to Flash is over”.

## FLASH ERASE

In this frame, user can erase the blocks between two addresses or erase a range of blocks directly.

Follow the steps below

- Enter the start address and end address. It is the responsibility of the user to give a block aligned address. Otherwise, data may be lost.

- Click on “Erase Add Range”. User gets a warning window with the message about the block range that will be erased. User can decide to continue or abort at this point. Continuing will erase the blocks.

- User can enter the start block and end block in “St Blk” and “End Blk” and click on “Erase” to erase the block range.

## FLASH INI

Clicking on the button, “FLASH LAYOUT”, will open a new window. In this window, user can perform flash operations on the sections specified in the FLASH INI file. The elements of the window are explained below.

- The sections mentioned in the FLASH INI file are listed in the list box.

- Each item contains the section name & address ranges. User can select any of the section to perform the operations.

- User can perform a READ/WRITE using the buttons “READ” & WRITE in bin or srec format.

- User can also erase the entire section using the “ERASE” button.

## INFO

This frame displays the s/w and h/w information about the mobile. Clicking on the button “MOBILE INFO” brings a new window and this displays the mobile s/w, h/w and flash information. User can go back to the main window by clicking on “BACK”.

## CHECKSUM

Clicking on the button, “VERIFY CHECKSUM”, You’ll get information about checksum and if the calculated chksum is equal to written in flash memory.

## MEMORY VIEW

In this frame, user can view the flash contents. User can enter the address and the data is displayed in HEX and ASCII format.

Follow the steps below

- Enter the address in the box next to “Address”

- Click on “Show”. This will display 256 bytes of data in the window.

- Clicking on “Prev” and “Next” will show the previous or next 256 bytes of data respectively.

- User can use the scroll bar to view next/previous 16 addresses by clicking on the bottom/top arrow marks respectively.

- Clicking on the scroll bar will show previous/next 256 addresses.

## Logging

This is a information frame. Important messages are displayed by the tool.

## SWITCH OFF MOBILE

This button switches the mobile OFF. Now all setting in the tool can be reconfigured.

## EXIT

Switches the mobile off and closes the application

# Features of Command Line Version

The supported commands are listed below with the syntax and the description. The same list can be attained by running the command “WinFlashMem.exe –h”

Option                              Explanation

-p/--Project        syntax: -p <id>  / --Project <id>

The project id of the mobile s/w.

The id is an integer value, given in decimal format

Eg: -p 320   or --Project 320

NOTE: If this option is not given, previously used

project id is used, by default.,

-c/--ComPort        syntax: -c <Number> / --ComPort <Number>

Selection of the com port to use.

Eg: -c 1 or --ComPort 1

NOTE: If this option is not given, previously used

comport is used, by default.,

-b/--BaudRate       syntax: -b <bps> / --BaudRate <bps>

Selection of the baud rate to use for communication

with the mobile over the com port.

Eg: -b 115200 or --BaudRate 921600

NOTE: If this option is not given, previously used baud

rate is used, by default.,

-h/--Help           Displays this message.,

-P/-BootPin         syntax: -P <code>  / -P <file,code_label>

Boot Pin code for secured boot mode.

The boot pin can be supplied in the command line.

The boot pin can be selected from a INI file.

Eg.1: -P 0xab,0xcd,0xef [ without space between bytes

and comma]

Eg.2: -P bootPin.INI,IMEI_004999511629841 [ with out

space between file name, code label and comma ],

-r/-ReadArea        syntax: -r <start_address>-<end_address> or <address,len>

Reads the specified flash memory range. Multiple Read

commands can be given in single execution of this program.

Eg.1: -r 0xa0000000-0xa001ffff [ without space between

address and hifen ].

Eg.2: -r 0xa0010000,0xffff  -r 0xa0000000,0xff

[without space between address and comma ].,

-w/-WriteArea       syntax: -w <start_address>-<end_address> or <address,len>

Writes to the specified address range. It erases the entire

blocks in the given address range and programs them with

the input data. The input data should come from a file.

Multiple Write commands can be given in single execution

of this program. For BIN formats, a special value for

end_address, END, makes the program write all the data

from the input file.

Eg.1: -w 0xa0000000-0xa001ffff [ without spaces between

address and hifen ].

Eg.2: -w 0xa0000000,0x1ffff -w 0xa0040000,0x1ffff

[ without spaces between address and comma ],

-e/-EraseArea       syntax: -e <start_address>-<end_address> or <address,len>

Erases the blocks in flash address range given.

It is the responsiblity of the user to make sure that

address range is block aligned.

Eg.1: -e 0xa0000000-0xa001ffff [ without spaces between

address and hifen ]. Erases block 1.,

-o/--OutFile        syntax: -o <file_name>

The output of the read options can be redirected to a

single output file. This option can only be used with

read options. This option is to be used only once per

execution.

Eg.1: -r 0xa0020000-0xa003ffff -o block_2.srec,

-i/--InFile         syntax: -i <file_name>

The input data to the write options are supplied by this

option. This option can only be used with write options.

This option is to be used only once per execution.

Eg.1: -w 0xa0020000-0xa003ffff -i block_2.srec,

-f/--FileFormat     syntax: -f <format>

This option specifies the file format to be used for

read options and write options.The format could be

SREC or BIN. By default, SREC is the selected option.

Eg.1: -f BIN

Eg.2: -f SREC,

-d/--display        syntax -d <start_address-endaddress> or <address,len>

Displays the address range in the command window.

This could be used to view a short range of memory.

Eg.1: -d 0xa0000000-0xa00000ff

Eg.2: -d 0xa0000000,0xff,

-u/--UploadRamSw    syntax -u <file_name>

The RAM sw can be loaded from a binary file

Eg.1: -u new_ram_sw.bin,

-l/--Dll            syntax -l <dll_file_name>

The swup dll can be selected by this option

Eg.1 -l wsw_swup75.dll,

-s/--Supress        syntax -s

All warnings can be suppressed by this option

A bit dangerous to us with erase & write operations

Eg.1: -s,

--AddressRange      syntax --AddressRange <start_address-end_address>

The supported address range of a project can be

modified by this option.

Eg.1: --AddressRange 0xa0000000-0xb1ffffff,

--ByPass            syntax --ByPass

Using this option, the process of loading the boot

block & RAM sw can be bypassed and flash operations

can be directly performed

-v                 Veryfying checksum

# Example BOOT PIN INI file

[Default]

PIN=0x1b 0xf9 0x7d 0x21 0xdc 0x53 0x28 0x1a 0xab 0x57 0x5c 0x90 0x61 0x93 0x73 0xfd

[004999511629841]

PIN=0x63 0x41 0x4B 0x07 0xC4 0x0D 0x5E 0x7F 0xF5 0xF1 0x95 0x4B 0x1E 0x43 0x6F 0x5C

[351011372027047]

PIN=0x1b 0xf9 0x7c 0x21 0xde 0x53 0x68 0xaa 0xab 0x57 0x5c 0x90 0x61 0x93 0x73 0xfd

[PIN_01199_00178]

PIN=0xd1 0x89 0x70 0x8f 0xd7 0x15 0xe6 0xbc 0x23 0xd1 0xc1 0xd8 0xb1 0x83 0xe9 0x65

# Example PROJECT INI file

[X75_NEXT_GEN]

Id = 4098

StartAddress = 0xa0000000

EndAddress = 0xa1ffffff

Dll = wsw_r65.dll

[X75_NEXT_GEN_2]

Id = 4097

StartAddress = 0xa0000000

EndAddress = 0xa1ffffff

Dll = wsw_r65.dll

[X75_NEXT_GEN_3]

Id = 411

StartAddress = 0xa0000000

EndAddress = 0xa7ffffff

GapStartAddress = 0xa2000000

GapEndAddress = 0xa3ffffff

Dll = wsw_x75.dll

# Example FLASH INI file

[GENERAL]

NumOfInstances = 8

NumOfOtpBlocks = 4

[OTP]

QueueLength = 2

OtpLength0 = 8

OtpLength1 = 24

OtpLength2 = 32

OtpLength3 = 64

[INSTANCE_0]

Name = EEFULL

Mode = FM_PRV_FSS

PartitionMode = FM_PRV_DATA

Reserve = 8192

QueueLength = 6

NumberOfEraseBlocks = 2

EraseBlockSize = 0x20000

NumberOfBlocks = 500

NumberOfDirentriesInOneEraseBlock = 8192

EraseBlockAddress0 = 0xA0220000     ; address on mobile = 0xA0220000

[INSTANCE_1]

Name = FFS

Mode = FM_PRV_FSS

PartitionMode = FM_PRV_DATA

Reserve = 0

QueueLength = 6

NumberOfEraseBlocks = 89

EraseBlockSize = 0x20000

EraseBlockAddress0 = 0xA04E0000     ; address on mobile = 0xA04E0000

EraseBlockAddress25 = 0xA1800000     ; address on mobile = 0xA1800000

FFS_SysArraySize = 65530

FFS_ClusterSize = 1024

[INSTANCE_2]

Name = FFS_B

Mode = FM_PRV_FSS

PartitionMode = FM_PRV_DATA

Reserve = 0

QueueLength = 6

NumberOfEraseBlocks = 4

EraseBlockSize = 0x20000

EraseBlockAddress0 = 0xA0260000     ; address on mobile = 0xA0260000

FFS_SysArraySize = 4096

FFS_ClusterSize = 1024

[INSTANCE_3]

Name = FFS_C

Mode = FM_PRV_FSS

PartitionMode = FM_PRV_DATA

Reserve = 0

QueueLength = 6

NumberOfEraseBlocks = 16

EraseBlockSize = 0x20000

EraseBlockAddress0 = 0xA02E0000     ; address on mobile = 0xA02E0000

FFS_SysArraySize = 16384

FFS_ClusterSize = 1024

[INSTANCE_4]

Name = EELITE

Mode = FM_PRV_EEL

PartitionMode = FM_PRV_MIXED

Reserve = 0

QueueLength = 2

NumberOfEraseBlocks = 1

EraseBlockSize0 = 0x20000

NumberOfBlocks = 350

NumberOfDirentriesInOneEraseBlock = 8192

EraseBlockAddress0 = 0xA0FE0000     ; address on mobile = 0xA0FE0000

[INSTANCE_5]

Name = EXIT

Mode = FM_PRV_RAW

PartitionMode = FM_PRV_DATA

Reserve = 0

QueueLength = 2

NumberOfEraseBlocks = 1

EraseBlockSize0 = 0x20000

EraseBlockAddress0 = 0xA0200000     ; address on mobile = 0xA0200000

[INSTANCE_6]

Name = BCORE

Mode = FM_PRV_RAW

PartitionMode = FM_PRV_MIXED

Reserve = 0

QueueLength = 2

NumberOfEraseBlocks = 1

EraseBlockSize0 = 0x20000

EraseBlockAddress0 = 0xA0000000     ; address on mobile = 0xA0000000

[INSTANCE_7]

Name = __FM__

Mode = FM_PRV_EEL

PartitionMode = FM_PRV_MIXED

Reserve = 0

QueueLength = 2

NumberOfEraseBlocks = 1

EraseBlockSize0 = 0x1400

NumberOfBlocks = 20

NumberOfDirentriesInOneEraseBlock = 256

EraseBlockAddress0 = 0xA0000C00     ; address on mobile = 0xA0000C00