ATT – AT Command Test Tool

USERMANUAL

Version 1.1

Table of Contents

1.	Introduction	4

2.	First Steps – Quick Start	4

2.1.	Getting ATT, basic setup	4

2.2.	Generating reference files	6

3.	The script and the project files	7

4.	Start and Initial Setup of ATT	8

4.1.	Setting the comports	9

5.	Project settings	11

6.	Running the project	12

History of change

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0_001 | 17.07.2003 | Jillissen | Created new document for ATT 1.7 |
| 1.1 | 27.05.2004 | Weiden | Added some notes, “First steps” guide introduced |

- Introduction

With ATT, a test tool for AT cellular commands you can send AT commands to the mobile and receive the replies from the mobile. ATT can be used for sending single commands typed in manually or used for a set of commands by running scripts. When running scripts the replies of the mobile are checked if they are okay or not. This is done with the help of reference files. These reference files have a special format and contain all AT commands, comments and all other instructions of the original source script and expected replies to the AT commands. All the results of the run are recorded into an HTML file. General information like project name and location, date and time, name of the tester, department, product, test run, software version, software net, number of tests passed and number of tests failed are recorded in this file. If the reply is as expected then only the reply is written in the result file. If the reply is not as expected the actual reply and the expected reply are written into the result file. It is possible to run several scripts in one go. Also it is possible to communicate to up to 6 different devices over virtual ports (NOTE: Coming soon – not yet completely implemented for automated script runs).

- First Steps – Quick Start

## Getting ATT, basic setup

fig. 1: ATT directory structure

All ATT related stuff is currently located on KLF drive K:\Entw\PROJ\R65_Ulysses\PE-Team\I&T\Application\Mobile\ATC (or in future on testspec server).

- In ATC Subdirectory, copy the folder ATT (including the subfolders bin and TestScripts to your local harddrive.

The following ALWAYS refers to your local copies:

- In your local subfolder bin you’ll find the executable att.exe and the inifile att.ini. For convenience, create a shortcut to att.exe on your desktop.

- Open the file att.ini (eg. with notepad) and set up some basic att settings. Follow the hints in att.ini.

[VARIABLES]

# Mobile phones MODEL, returned by AT+GMM

MODEL="CX65"

# Manufacturer, returned by AT+GMI

MANUFACTURER="Siemens"

- set variable MODEL to the model you’re about to test, eg. “CX65”. (That’s the value which should be returned by AT+CMM). MANUFACTURER normally is “Siemens”

# --------------------------------------------------------------------------------

# Phone Numbers

# Service Center Address: Phone number of SMS service center (can be found in SMS setup)

SCA=+49<add your SMS service center address>

# Destination Addresses: Phone numbers (always in international format, eg. +492842950)

# primary mobile phone number (the one attached to ATT via serial port)

DA1=+49<add your own mobile number here>

# secondary mobile phone number (for establishment of data calls, as SMS receiver etc.), not attached to ATT

DA2=+49<add a second mobile number>

# Primary stationary phone number (must have a display with ability of calling number presentation (CLIP))

DA4=+49<add your own stationary phone here)>

# Secondary stationary phone number (for call forwarding, multiparty etc).

DA3=+49<add a 2nd phone number here, which is not more than an armlength away :-)>

- set SMS service center address (SCA) and the phone numbers DA1…DA4 as described in the comments.

# --------------------------------------------------------------------------------

# Variables for GPRS

# Provider-dependent access point name (can be found in your mobile's WAP/internet settings)

APN=web.vodafone.de

- Set the access point name of your provider, which is used for GPRS connections. Save the ini file. Now you have to recompile all script files, as follows.

- All ATT test scripts are located in model-specific subfolders of ATT\TestScripts. 
For Ulysses, the subfolder R65_platform\ReadyForUse\_MMIVerify contains the latest versions of all relevant test scripts. Each other subfolder of R65_platform can be ignored for now. In your local copy, you can delete these. 
In _MMIVerify, the scripts for all AT commands are grouped by category, in category-specific subfolders. Each of these category-specific subfolders contains at least one ATT project file (*.prj), grouping together the script files (*.txt) for specific AT commands. These project files can be run from within ATT, as described below.

NOTE: Currently, some AT command categories aren’t completely automated. These mainly are Bluetooth and FAX related commands.

## Generating reference files

ATT’s reference files (*.ref) contain all the answers to specific AT commands which are expected after a command was send to a mobile. These reference files are generated from your script files (*.txt) automatically if you start a script for the first time or if you press the COMPILE button on ATT’s gui.

IMPORTANT:

- After any change in att.ini, you have to delete and recreate ALL *.ref files of your ATT projects. That’s because variables defined in att.ini are resolved during compile time of the reference file, NOT during script runtime (this will be changed in ATT2.0, coming soon…).

- After changes in a single script file, you have to recompile the corresponding reference file.

So, before starting an ATT test project for the first time, you have to delete ALL *.ref files in ALL subdirectories containing ATT command scripts.

For re-generating them,

- Connect a mobile which is known to response correctly to most AT commands. Eg., connect an S55(!) for generating references for platform 65. That’s because some responses are taken from the connected mobile and stored in the reference file to be used as reference in future test runs.

- Start ATT

- Do COM port setup, as described in 1.3, “Setting the comports”.

- Load a project whose reference files shall be generated. See 5, “Project settings”

- Press the COMPILE button on ATT’s gui. ATT will execute the project without any user feedback, recording all required responses from mobile for generating the reference files. This may take some time, so please be patient. After compiling finishes, the project is ready to run for testing.

- The script and the project files

A new project always starts with writing one or more scripts. Any simple text editor will do. Each line in the file should contain a command. The AT commands starts with AT. All AT commands will be send by default to the first serial port. But it is also possible to send  AT commands to one of the other 5 possible serial ports. A message send to the second serial port will look like: 2: AT+CGMM. Comments start with COM and will be copied to the result file for better understanding. A message to the user starts with MESSAGE. With messages the user can get notices or instructions, e.g. “MESSAGE Call the connected mobile”. These script files should be saved with the txt extension.

Example of getManufModel.txt:

COM Get manufacturer ID code

AT+CGMI

COM Get model ID code

AT+CGMM

A project file with the .prj extension contains the names of one or more text files without the .txt extention.

Example of GereralTest.prj:

GetManufModel

CallNumber

EndCall

Although it is not necessary it is easier to put the  script and the project files in the same directory. It is not possible to use script names that include the absolute path but it is possible to use relative paths where the subdirectory which contains the project file is the starting point.

So the tester needs to write two types of files: the scripts and the project files which tell which script files should be run. For both types of filenames it is useful to use descriptive names. Don’t use names like test123.

- Start and Initial Setup of ATT

Start the ATT tool by executing ATT.exe.

fig. 1: The main screen of ATT

For sending AT command manually enter an AT command in the “send” window. To send this command press the enter key or click the send button. All manually entered commands and replies are saved in c:\ATT.log. By default the command isis send to port 1. It is possible to send commands to other ports by changing the port selection under the send button. Before the relevant port data should be set. To save typing time it is possible to select add AT+. This way instead of typing AT+CGMM only CGMM has to be typed. Ticking the “add AT^” box makes the typing of AT^ redundant. Most of the commands should be terminated with a carriage return (CR). For this reason “add CR” is selected by default. However when sending PDU-data the string should be terminated with control-Z and the “add CTR-Z” boxshould be ticked.

With the button “project set up “ all setting for the project like project file, timing, and project specific details can be changed. With the button “run project it is possible to run the project. If the project is run for the first time and no reference file exists the user will be asked if they should be generated. In these  reference files all data of the script files is copied in a special format. Also the responses of the mobile are recorded in this file. In future the expected results will be extracted from this file. Because of this fact there is a warning which tells the user to check the newly generated reference file for correctness.

## Setting the comports

Before it is possible to start any communication it is necessary to set up the comports. Press the button “RS 232 setup” in the main screen. By default Port_1 is selected. Select the port that you want to configure. Then click on the pull down list below the port selection list to select the serial port COM1…COMn. Only existing ports that are still available will be shown. Also it is possible to remove previously configured port data with the button “remove port”. NOTE: Currently only settings for default Port_1 will be recognized !

fig. 2: Comport selection

After selecting the comport press the “Setup” key the window for changing the port properties will be opened. In this window the baud rate, number of data bits, parity, number of stop bits and the control can be set up. With OK the new properties can be saved. Normally, set them to the values shown in fig.3, probably with different baud rates:

fig 3: Port properties

To test all the initalised ports click on the button  “RS232 test”.

NOTE: This currently fails due to a bug in ATT! Sorry. 

fig. 4: Results of checking the initialised ports.

- Project settings

In the ATT mainscreen select “Project setup””. The project setting window will be opened.

fig. 5: the Project settings window

The project file can be selected by pressing the “Select project” button. All data in the details field, user name, department, product, testrun, SVN, SW-net and comment can be edited freely. The number of attempts is the maximum number of retries when the response from the mobile is not OK. The value “delay receive-send” is the pause between receiving an answer from the mobile until the next command is send. The “delay send-receive” is the time the program waits after sending an AT command to the mobile and starting with receiving. The “timeout receive” is the maximum time that is waited after receiving an AT command when the mobile doesn’t reply. If the “ERROR messagebox” is ticked and a reply is not okay then the user gets a message on the screen. After confirming this message the  rest of the commands will be run. As default this optionbox is not ticked because it is better for long scripts, so the script can run without user interfearance, even when errors occur. Also it is possible to suppress user messages with the MESSAGE command. To tick the option box “Suppress user messages” is not a good idea if the user has to perform certain actions.

NOTE: Setting attempts=1, delay recv-send=300ms, delay send-recv=300ms, timeout receive=30000ms occurred to be useful. If timeouts occur during script runs, try to increase the latter.

- Running the project

After setting up the comports, selecting a project and adjusting the project settings it is possible to start the project by pressing the “Run project button”. The results will be saved in a html file.