Content

History of change	3

1	Introduction – what is ATT?	4

2	Overview and basic setup	6

2.1	ATT2.0 directory structure	6

2.2	Initial setup of configuration files	7

3	ATT file types	8

4	Start and Initial Setup of ATT2.0	9

4.1	Device setup	10

5	Loading and setting up projects	12

6	Running the project	13

7	Creating new projects	14

8	Scripting	15

8.1	Introduction	15

8.2	ATT Command Levels	16

8.3	ATT command reference	17

8.3.1	High level commands	17

8.3.1.1	AT commands	17

8.3.1.2	Strings, Variables and Python Expressions in AT commands	18

8.3.1.3	WAITFOR / WAIT FOR	20

8.3.1.4	COM	22

8.3.1.5	MESSAGE	23

8.3.1.6	WAIT	24

8.3.2	Mid level commands	25

8.3.2.1	About receive buffers	25

8.3.2.2	Variable argument lists	25

8.3.2.3	ATCMD()	26

8.3.2.4	WAITFOR()	27

8.3.2.5	COM()	29

8.3.2.6	MESSAGE()	30

8.3.2.7	USERTEST()	31

8.3.3	Low level commands	32

8.3.3.1	send(), sendln()	32

8.3.3.2	receive()	33

8.3.3.3	ClearReceiveBuffer() / ClearAllReceiveBuffers()	34

8.3.3.4	SetReceiveTimeout()	35

8.3.3.5	ExtractParameter ()	36

8.4	Compatibility and scripting notes	38

History of change

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0_001 | 17.07.2003 | Jillissen | Created new document for ATT1.7 |
| 1.1 | 27.05.2004 | Weiden | Added some notes, “First steps” guide introduced |
| 2.0 | 12.08.2004 | Weiden | Started adaptation to ATT2.0 |
| 2.1 | 31.08.2004 | Weiden | Page layout improved, added ATT command reference guide |
| 2.2 | 16.09.2004 | Weiden | Last changes, no longer ‘preliminary’. |

# Introduction – what is ATT?

Figure 1.1: ATT2.0 Main dialog

ATT is the “automatic test tool for AT cellular commands”.

ATT allows the user to communicate with up to 6 mobiles connected via serial COM ports using AT cellular (ATC) commands. Replies from the mobiles are displayed as soon as ATT receives them. All activated COM ports are internally mapped to 6 logical ATT ports.

ATT2.0 can be used for sending single commands typed in manually at ATT’s command line or automatically by running test scripts.

When running scripts the replies of the mobiles are evaluated and the test results are reported in HTML result files.

It is possible to group several test scripts in one ATT project, enabling the contained scripts to execute in one go.

ATT2.0 key feature overview:

- Uses PYTHON (www.python.org) as script engine, providing features nowadays expected from high-level scripting languages. The very basic ones: Variables, conditional execution via if statement, for and while loops…

- Easy-to-program test scripts: For most AT command variants, no special syntax is required: They can be scripted as ‘pure AT commands’. The ATT2.0 precompiler converts them internally to PYTHON-compatible command syntax.

- Multithreaded I/O: ATT listens for incoming data from all connected mobiles asynchronously in background, thus avoiding data loss or blocked graphical user interface while receiving.

- Detailed result reporting in HTML files.

- Modular device concept using MAX, the Modular Application Toobox: For future use, other than serial communication via COM ports can be implemented quite easily, e.g.  sockets for network communication. Ideas are always welcome. 

# Overview and basic setup

## ATT2.0 directory structure

Figure 2.2: ATT2.0 directory structure

After you copied or unzipped the ATT folder to your local drive, you will find all ATT2.0 related stuff in subfolders of ATT20Release (see fig. 1 above).

- In subfolder Bin you’ll find the executable ATT20.exe and the inifile ATT20.ini. For convenience, create a shortcut to ATT20.exe on your desktop.

- Subfolder ATTDocumentation contains all available documentation, e.g.  the document you’re currently reading. 

- ATTTestScripts contains two subfolders: Demo contains several example scripts for demonstrating ATT scripting methods. The script file ATTSpecificPythonCommands.txt demonstrates all currently available ATT-specific PYTHON extension commands. TwoMobiles.txt is a typical example showing how two connected mobiles can be tested interactively. All demo files are commented in detail, so looking at these should be a good start for own scripting experiments.
The subfolder R65_Platform contains scripts originally written for ATT1.0 and the R65 platform/Ulysses. They were ported to run with ATT2.0. See the README in this folder!

- Folder AttPythonGlobals contains all files with global data and functions, imported from most ATT2.0 scripts.This mainly is the file attglobals.py, which defines globally used variables (Phone numbers!) and constants. Files in this folder are imported from test scripts with the PYTHON import <FileName> resp. from <FileName> import * statements.

- ATTLog is the subfolder for ATT ‘s application logging. Per default, ATT writes its application specific logfiles to this directory. In strange error cases, these files might be useful for debugging or tracing of data transferred to/from connected mobiles.

- ATTTestResults can be used as output folder for the test result HTML files. This depends on the project settings of the currently loaded ATT2.0 project and can be specified in ATT’s Project setup dialog (see below).

## Initial setup of configuration files

Open the file AttPythonGlobals\attglobals.py (e.g.  with notepad) and set up some variables. This file contains variables for phone numbers (PrimaryMobilePhoneNumber, SecondaryMobilePhoneNumber…), the GPRS access point name and maybe some more. Follow the comments in attglobals.py.

That’s all for basic setup. You can now start Att20.exe.

For more experienced users, the following might be useful (completely optional):

Open bin/att20.ini.

You might want ATT2.0 to write its application loggings to somewhere else than the default ATTLog, e.g.  C:\temp. In this case, set variable LOGDIRECTORY accordingly. Additionally, you can change variable PYTHONIMPORTPATH to your needs, if you have more or other than the standard AttPythonGlobals directory for PYTHON files to be imported. Use ‘;’ to separate paths from each other.

# ATT file types

ATT2.0 uses several types of files:

- Script files written by user, containing all required commands for testing. These scripts have the extension *.txt and can be created with any ASCII text editor. Watch out the examples in the demo subdirectory.

- The PYTHON executable files with extension *.py. These files are created from the script files (*.txt) if they don’t already exist or if the *.txt file modification time is newer than the *.py file creation time. *.py files contain the script code transformed into a command syntax that is understandable for ATT’s built-in python interpreter. 
If necessary, creation is done automatically by ATT2.0’s precompiler after pressing the Compile or Run button on ATT’s main dialog. 
Attention: Never change the output of the *.py files directly, as they will be overwritten by precompiler again! Nevertheless, having a look at these files might be informative if the precompiler seems to have become confused about an input script. 

- An ATT Project file (*.prj) contains simply the name(s) of one ore more script file(s) (without the *.txt) extension. ATT always runs projects, thus executing all script files specified in the currently loaded project file. Projects can be created using ATT’s new project dialog.

- Probably the python interpreter itself will create *.pyc files. These are precompiled *.py files enabling the python interpreter to run scripts faster.

You are missing the reference files *.ref from ATT1.0? They are no longer required within ATT2.0!

# Start and Initial Setup of ATT2.0

Start ATT2.0 by executing ATT20.exe. First of all you should setup your devices/ports. Please refer to 4.1, “Device setup”.

Port selector for command line

Command line

Trace window for runtime messages

Device and project configuration

Project execution control

Figure 4.3: The main screen of ATT2.0

For sending an AT command manually enter an AT command in the command line and press the SEND button. By default the command is send via logical port 1. Use the “Port selector” radio buttons under the send button to change this.

To save typing time it is possible to activate the “add AT+” check box. This way instead of typing AT+CGMM only CGMM has to be typed. Ticking “add AT^” does the same with AT^. Most of the commands should be terminated with a carriage return (CR). For this reason “add CR” is selected by default. However, when sending PDU-data etc., the data needs to be terminated with control-Z, so “add CTR-Z” should be ticked instead.

With the button “PROJECT SETUP“ all settings for a project like project file, timing, and project specific details can be changed. With the button “run” it is possible to run a previously loaded project. The “COMPILE” buttons creates *.py files from the script files used in the loaded project. This will also be done automatically by pressing “run”, if the *.txt file was changed.

## Device setup

Before it is possible to start any communication it is necessary to set up ATT’s logical ports and map them to physical COM ports. Press the button “Device setup” in the main dialog. The dialog shown in Figure 4.4 opens.

The port mapping is done by selecting a logical port in the ‘Logical port’ dropdown box first and afterwards selecting an available physical device in the ‘physical device’ list.

Attention: A logical port can only be mapped to exactly one physical device and vice versa. If this is ignored, a message box will open with a warning message.

To delete an existing port mapping, press the “Clear mapping” button. This will clear the mapping between the currently selected logical port and the physical device.

Figure 4.4: Device setup dialog

After selecting the COM port press the “Setup device” button.

The dialog for changing the port properties (Figure 4.5: Port properties) will open. In this window the baud rate, number of data bits, parity, number of stop bits etc. can be configured.

Normally, this would be >=57600 baud, 8 databits, 1 stopbit and no parity (flow control is currently ignored).

Do this for each COM port you mapped to a logical port.

Figure 4.5: Port properties

To test all your settings, return to ATT’s main dialog and press the “device test” button. The dialog shown in Figure 4.6: Device test results will inform you about your port mappings, device states and currently connected mobiles, if any.

Figure 4.6: Device test results

# Loading and setting up projects

In the ATT2.0 main dialog select “Project setup””. The project settings window will open:

Figure 5.7: the Project settings window

The project to be executed can be selected by pressing the “Select project” button. This opens a dialog enabling you to select existing project files *.prj or to specify a folder for new projects to be created.

All HTML result files will be written to the directory selected with the “Select directory” dialog.

All data in the details field, user name, department, product, testrun, SVN, SW-net and comment can be edited can be edited for documentation of tests. The value “delay receive-send” is the pause between receiving an answer from the mobile until the next command is send. The “delay send-receive” is the time the program waits after sending an AT command. The “timeout receive” is the maximum time ATT waits for the expected answer from mobile after sending an AT command. The default values delay recv-send=25ms, delay send-recv=25s, timeout receive=15000ms occurred to be useful. If timeouts occur during script runs, probably try to increase the latter.

# Running the project

After setting up the devices, selecting a project and entering the project settings, pressing the “run” button will start project’s execution. Running projects can be aborted with the “STOP” button.

The testrun results are saved in a HTML result file in the directory selected during project setup (see above). Each result file contains a detailed logging of every test case.  A summary is appended at the end of the file, summarizing the project run with the number of executed, passed and failed tests. This summary also includes the general information from the project settings, as shown below:

Naming of result files follows the following conventions:

<Project name>_SVN<software version number>_net<software net>_<date>_<time>.htm

With:

<software version number> and <software net>: Values previously set in the ‘project settings’ dialog.

<date> and <time>: Date/time when test run finished. Format: YYYYMMDD / HHMMSS

# Creating new projects

An ATT project is a collection of test script (*.txt) files, which are grouped together for being executed one by one. ATT always executes projects, so you need to create one even if it contains only a single script file.

New projects are created using the ‘new project’ dialog. But first, open the project setup dialog (see 5, “Loading and setting up projects”), and select the project path for the new project to be located.

In the new project dialog, first enter the name of the new project in the corresponding edit field.

Figure 7.8: The New Project dialog

The list box ‘source files’ shows all script (*.txt) files found in the current project path. You can add or remove them to / from your new project using the arrow buttons. The order of the scripts in your project can be changed with the ‘file up’ button.

Finally, press SAVE. Open project setup dialog (chapter 5) again and enter additional project information. Afterwards, the project is ready for execution.

The created project file (*.prj) can also be edited using any text editor. That might be required if script files not contained in the current project path should be added to the project. Add them with the complete path, but without the *.txt resp. *.py extension.

# Scripting

## Introduction

Creating new projects from scratch always starts with writing one or more scripts. Any simple text editor will do. Each line in the file should contain exactly one command.

Commands can be:

- AT cellular commands, like ATD, AT+CCLK?, AT+CMGI=?, etc.

- ATT-specific commands: ATCMD(), WAITFOR(), MESSAGE, USERTEST(), COM…

- PYTHON build-in statements of the python scripting language: if, for, i=i+1, …
Shortly, all typical programming language commands. Refer to the documentation section at http://www.python.org to learn all about PYTHON.

By default, all AT cellular commands in a script will be sent to logical port #1, which is mapped to a physical COM-port via ATT’s device setup dialog. Sending AT commands to any of the 6 possible logical ports is possible by adding the port number in front of the AT command (or the corresponding WAITFOR) like:

2:AT+CGMM.

Note that there must not be a blank between the port number (2:) and the command!

Responses from mobile can be evaluated using the WAITFOR command. Each WAITFOR defines a testcase for comparing the received response with the expected one, as specified by the WAITFOR parameters. Testcase results are written to the HTML result file.

Comments start with COM and will be added to the HTML result file for better understanding. A message to the user starts with MESSAGE. With messages the user can get notices or instructions, e.g. “MESSAGE This script will overwrite SMS indices 1-10!”.

Example of a very simple ATT script:

COM Get manufacturer ID code

AT+CGMI

WAITFOR OK

COM Get model ID code from mobile at port 2

2:AT+CGMM

2:WAITFOR OK

MESSAGE Script ended!

See also examples in the demo folder. All ATT-specific commands are documented in detail in the following command reference.

## ATT Command Levels

ATT2.0’s scripting engine knows three hierarchical levels of commands: High-, mid- and low level commands. Of course, all of these commands can be used in script files.

HTML

result

file

PRECOMPILER

PYTHON executable file (*.py)

ATT script file (*.txt)

HIGH LEVEL COMMANDS

All AT commands

COM

MESSAGE

WAITFOR / WAIT FOR

WAIT

ATT1.0 variables

…

MID LEVEL COMMANDS

ATCMD(…)

COM (…)

MESSAGE (…)

WAITFOR (…)

USERTEST (…)

…

LOW LEVEL COMMANDS

All PYTHON commands

send(…) / sendln(…)

receive (…)

ClearReceiveBuffer(…)

ClearAllReceiveBuffers()

ExtractParameter(…)

SetReceiveTimeOut(…)

…

Figure 8.9: ATT command levels

- High Level Commands are those without the typical function call syntax of common programming languages, that means they don’t have the format functionname (<parameter1>, <parameter2>, …). This obviously are all AT commands, but also those ‘simplified’ ATT-specific commands like MESSAGE, COM, WAITFOR etc., as already known from ATT1.0. All high level commands need to be processed by ATT’s precompiler. The precompiler automatically  transforms them into mid level commands before script runs, if the script file was changed. Thus, the resulting PYTHON executable files (*.py) don’t contain high level commands any more.

- Mid Level Commands extend the PYTHON language with ATT-specific functionality. They all have the function call syntax directly understandable by ATT’s embedded PYTHON interpreter. Most mid level commands are closely related to AT command execution and testing. Usually their results will be written to the HTML result file.

- Low Level Commands are quite similar to mid level commands. Main difference is that they don’t write output to the HTML result file. They mainly provide basic input/output and control functionality. In this function hierarchy, also python’s build-in, “native”  functions could be subsumed here.

## ATT command reference

### High level commands

#### AT commands

ATT2.0 is intended to handle all AT commands according to 3GPP TS 27.007, 3GPP TS 27.005 and ITU-T V250 directly, without any modifications, in ATT scripts. This includes the siemens-specific extensions to the standards. For a basic introduction e.g.  refer to the reference manual “AT command set for Siemens mobile phones and modems”.

All AT commands sent to any connected mobile are written to the HTML result file.

3GPP AT cellular commands always start with ‘AT’, followed by +, ^, or &, followed by the remainder of the command.

Examples:

AT+CCLK?					# execute command

AT^SACM=?					# check command

AT+CMEC=2					# write command

ITU-T V250 (“Hayes standard commands”) are the “classical” modem commands. They all start with “AT”, followed by further letters and numbers. They MUST NOT contain parenthesis () or the equation sign “=”, because otherwise ATT’s precompiler assumes them to be PYTHON function calls or variable assignments.

Examples:

ATE0

ATD02842950;

ATD>2;

These all are valid AT-Commands which can be used directly in ATT scripts. But the following commands will be left unprocessed by the precompiler, either causing PYTHON errors/script termination or strange behaviour during execution:

ATS3=9	will be treated as a variable assignment  (“Let variable ATS3 be 9”)

ATD(MyPhoneNumbersArray[5]);	is recognized as function call to function ATD()

Of course, these examples are quite exotic and somewhat constructed. Anyway, if in trouble, use the mid level command ATCMD() (see section 8.3.2.3) in conjunction with the questionable AT command.

#### Strings, Variables and Python Expressions in AT commands

Variables:

ATT2.0 recognizes two variants of variables:

- Old style ATT1.0 variables: ATT1.0 syntax used angle brackets ‘<’ and ‘>’ to mark variables: <MyVarName> = … or ATD <MyVarName>;. For compatibility reasons, this syntax is still recognized by ATT2.0. The precompiler transforms them to ‘new style variable syntax’ as described in section 8.4, “Compatibility and scripting notes”. Unlike ATT1.0, ATT2.0 does variable value assignment/resolving during script runtime, not while compiling, thus providing much more flexibility. Old style variables shouldn’t be used any more in new scripts.

- New style variables are not marked with angle brackets. Their first chararacter must be either a letter or an underscore ‘_’. Actually, ATT2.0’s variables are PYTHON variables and completely handled by the PYTHON interpreter. Please refer to the python documentation for all about value assignment, PYTHON data types and type conversion.  Of course, this means that also object oriented programming is possible, as provided by PYTHON.

Variables can be used in AT write commands (AT+…=n) and the dial command ATD:

# call phone with  PrimaryMobilePhoneNumber

ATD PrimaryMobilePhoneNumber;

# reads entry iIndex from short message storage

iIndex=17

AT+CMGR=iIndex

# send message from Index iIndex to Mobile with SecondaryMobilePhoneNumber

AT+CMSS=iIndex, SecondaryMobilePhoneNumber, 145

Strings:

Strings in AT write commands (AT+…=n) always have to be scripted using double quotes, like "This is my simple demo string".

Correct example:

AT+CPMS="MT","MT","MT"

Incorrect example:

AT+CPMS=MT, MT, MT

In the latter case, the precompiler would handle MT as a python variable. This will cause the PYTHON interpreter to terminate with error, because MT is an unknown identifier (Of course not if MT really is a variable and defined somewhere before… ).

The double quotes “…” are part of the AT command and will be sent to the mobile. If that’s not desired, please use the mid level command ATCMD() or use single quotes, as used by python itself (see below).

Please note that phone numbers in AT write commands are also strings, so the above is also valid. The one and only exception is the command ATD: Here, phone numbers can be used directly:

ATD +492842950;

ATD 02842950;

These both are possible commands.

Note that PYTHON itself uses single quotes ‘…’ (the apostrophe) to mark strings:  ‘This is my simple demo string. ‘

So, in all expressions other than AT write commands, use this python syntax, e.g.  for variable assignment:

PrimaryMobilePhoneNumber=’+492842950’

ATD PrimaryMobilePhoneNumber;

Globally used phone numbers are assigned to global variables in the file attglobals.py. Import this file at the beginning of your script with from attglobals import *, if required.

Python Expressions:

In fact, everything that is not recognized as a string (not embedded in double quotes “…”) or a simple number will be treated as a python expression. This includes variables. Python expressions are evaluated by the PYTHON interpreter during execution and may contain mathematical expressions, function calls, etc.:

# reads entry iIndex with offset 10 from short message storage:

AT+CMGR=10+iIndex

# reads entry 21 from short message storage:

AT+CMGR=3*7

# Function call. Of course, GetCurrentIndex() must be defined somewhere!

AT+CMGR=GetCurrentIndex()

# set service center address SCA to a python string expression

AT+CSCA='+49234599999'

Again, if you really want to send a string like “3*7” to the mobile, don’t forget the double quotes “…” or use the mid level command ATCMD()!

#### WAITFOR / WAIT FOR

Most AT commands are followed by WAITFOR resp. WAIT FOR (no difference in behaviour) to check mobile’s answer.

Syntax:

n:WAITFOR 	strExpectedData 						resp.

n:WAIT FOR 	strExpectedData

Parameters:

n:

Logical port [1..6] to wait for result. Optional, can be left out. Default: 1

strExpectedData

(Sub-) String which has to be part of mobile’s response. The command always interprets strExpectedData as an ASCII string. Variables and python expressions are not recognized. If required, use the mid level command WAITFOR() (see 8.3.2.4, ”WAITFOR()”).

Remarks:

WAITFOR waits until either the expected answer from mobile is received or receive timeout exceeds. Timeout time can initially be set in project settings dialog (see 5, “Loading and setting up projects ”) or be changed during script runtime using the low level command SetReceiveTimeout() (see 8.3.3.4).

Each WAITFOR defines a testcase. Its result will be written to the HTML result file. The testcase is passed if strExpectedData is contained somewhere in the received data. If strExpectedData is not part of the received data, the testcase fails after timeout. The corresponding entry in the HTML file will be marked with a red background (or green if passed).

Example:

# get time and date from mobile

AT+CCLK?

WAITFOR +CCLK:

The mobile will respond to AT+CCLK? with something like:

AT+CCLK?

+CCLK: "03/01/02,15:59:24"

OK

As ‘+CCLK:’ is part of the received answer, this testcase is passed. For exact matching (received exactly equal to expected data) you would need CR and LF (and probably variables etc.) to be part of strExpectedData. Use the mid-level command WAITFOR() in such cases.

Please note: Some responses from mobile may be received in multiple segments. It may take some time until the complete response is received. Imagine the following long response to AT+COPN:

AT+COPN

+COPN: "21303","STA-MOBILAND"

+COPN: "42402","UAE ETISALAT"

…

… many more lines like above

…

+COPN: "52023","GSM 1800"

OK

A statement like WAITFOR +COPN: "21303" will pass at the very beginning of mobile’s answer, causing ATT to continue script execution immediately, regardless of mobile is still sending its remaining response. In consequence, the data in receive buffer might get corrupted, if in the meanwhile ATT sent subsequent AT commands.

To avoid such problems, simply add a WAITFOR OK, insuring that mobile finished its response completely. The complete example script:

AT+COPN

WAITFOR +COPN: "21303"

WAITFOR OK

#### COM

COM adds a comment to the HTML result file. There, comments always occur with a yellow background and can contain any descriptive information.

Syntax:

COM strMyComment

Parameters:

strMyComment

Comment string to be written to the result file. The precompiler always interprets strMyComment as an ASCII string. Variables and Python expressions are not recognized. If required, use the mid level command COM() (see section 8.3.2.5).

Remarks:

You can use all HTML tags in strMyComment, allowing a lot of sophisticated output formatting, like <H1>…</H1> for headline style or <br> to add a linefeed.

Note that your scripts should also make massive use of the standard python comment (beginning with the ‘#’), e.g.  for structuring and commenting the script file itself!

Example:

COM <H1>This is a comment in headline style!</H1>

COM This is the first line <br> This is the second line

# This is a PYTHON comment.

# Python comments of course will not be written to HTML result file.

These lines will produce the following output in HTML file:

#### MESSAGE

MESSAGE opens a message box, displaying any information to the user. Script execution waits until user clicks the ‘OK’ button in the message box.

Syntax:

MESSAGE strMyMessage

Parameters:

strMyMessage

Information string to be displayed in the message box. The precompiler always interprets strMyMessage as an ASCII string. Variables and Python expressions are not recognized. If required, use the mid level command MESSAGE() (see 8.3.2.6, “MESSAGE()”).

Remarks:

MESSAGE is intended for displaying simple information. MESSAGE does not write any output to the HTML result file.

If you want the user to execute a test manually, also see mid level command USERTEST(), section 8.3.2.7.

Example:

MESSAGE Attention! This script will overwrite messages in SMS storage!

#### WAIT

WAIT pauses the script execution for the time specified. (Equivalent to Windows/MFC’s Sleep()).

Syntax:

WAIT iWaitTime

Parameters:

iWaitTime

Non-negative integer: Time to wait in milliseconds.

Remarks:

The precompiler transforms WAIT to the Python build-in command time.sleep(). This requires the python module time to be imported. This is done implicitly if you import attglobals.py, which is standard in most cases. So add one of the following statements at the beginning of your script:

from attglobals import *

OR

import time

Example:

from attglobals import *

# …

# some code

# …

# Wait 500ms

WAIT 500

### Mid level commands

#### About receive buffers

Each logical port of ATT has its own, independent receive buffer. Data received from connected mobiles will be appended to the corresponding receive buffer. This continuously happens in background, thus buffering all incoming data for later evaluation without data loss.

Every time an AT command is send, all receive buffers (of all logical ports!) are cleared automatically. This happens implicitly during execution of the mid-level command ATCMD() (see 8.3.2.3, “ATCMD()”). In this way, ATT is ready to receive the responses caused exactly by the last AT command from all affected mobiles, enabling interactivity tests. As one example, simply imagine one mobile connected to logical port 1, calling another one at port 2:

| TEST SCRIPT | Buffer 1 (Responses from Mobile 1) | Buffer 2 (Responses from Mobile 2) |
| --- | --- | --- |
|  | …old content… | …old content… |
| 1:ATD SecondaryPhoneNumber; | cleared | cleared |
|  | ATD01722595500; dialling OK |  |
| 1:WAITFOR OK |  |  |
| 2:WAITFOR RING |  | RING |

Receive buffers can be cleared explicitly using low level commands ClearReceiveBuffer() and ClearAllReceiveBuffers() (see section 8.3.3.3) if required.

#### Variable argument lists

Several mid level commands, like ATCMD(), COM() and USERTEST(), accept a variable number of parameters (at least one), known as variable argument list. This argument list can be a mixture of python strings, variables and python expressions. The related commands will concatenate all given argument values to one single string. Variable argument lists are identified by the closing Param1, …) in the command’s syntax documentation. Examples:

ATCMD (1, 'ATD', SecondaryMobilePhoneNumber, ';')

will result in something equivalent to:	ATD01729594857;

iIndex=17

COM ('Index+4 = ', iIndex+4)

will result in:	Index+4 = 21

#### ATCMD()

ATCMD()is used for sending an AT command via a specified logical port.

Syntax:

ATCMD (iLogicalPort, Param1, …)

Parameters:

iLogicalPort

Logical port (1..6) for sending the AT command

Param1, …

Variable argument list containing the AT command itself (as a string) and probably several further parameters. See 8.3.2.2, ”Variable argument lists”.

Remarks:

All AT commands directly used in ATT scripts will be transformed to parameter(s) within ATCMD() by ATT’s precompiler. Logical port 1 is default, if the AT command does not explicitly specify another.

Each ATCMD() command internally clears all receive buffers of all logical ports, as described in section 8.3.2.1, ”About receive buffers”.

ATCMD() adds all sent AT commands to the HTML result file in the following form:

Logical port

AT command, as sent to mobile

(after processing variable argument list)

Command’s file location

Project-global AT command counter

Script execution will terminate with error if iLogicalPort is invalid or not open.

Examples:

ATCMD (1, 'AT+CCLK?')		# simple AT command with no variables etc.

MyPhoneNumber='+492842950'

ATCMD (1, 'ATD', MyPhoneNumber, ';')	# will result in ATD+492842950;

ATCMD (1, 'AT+CMGR=', GetCurrentIndex())	# function call also possible

#### WAITFOR()

Mid level equivalent to WAITFOR. Defines a testcase for checking mobile’s response to a previously sent AT command and compares it with the expected answer. WAITFOR() allows waiting for variable values, results of function calls, etc., and further processing of received responses.

Syntax:

strReceivedData WAITFOR (iLogicalPort, Param1, …)

Parameters:

iLogicalPort

Logical port (1..6) for listening to incoming data.

Param1, …

Variable argument list defining the expected data to be received. This may be composed using strings, variables and python expressions. See 8.3.2.2, ”Variable argument lists”.

Return Value:

WAITFOR() returns the complete content of the receive buffer for further evaluation. If no data was received (WAITFOR() timed out), python object None is returned.

Remarks:

See also the Remarks and Please Note  section of high level WAITFOR, chapter 8.3.1.3.

WAITFOR() writes its result to the HTML file, like follows:

Command’s file location

Project-global testcase counter

Expected data, as specified in WAITFOR()

Logical port

Complete data received from mobile, if any

Result

(PASSED / FAILED)

Example 1:

# get current date/time from mobile connected to port 2

2:AT+CCLK?

strResult=WAITFOR (2, '+CCLK:')

if strResult is not None:

# Mobile answered -> write result to HTML file

COM ('Response to AT+CCLK:\r\n', strResult)

else:

# receive timeout -> display message

MESSAGE No answer from mobile at port 2!

Example 2:

# check complete response, including variable values and CR+LF (hex 0D/0A)

iValue=2

AT+CMEC=iValue

The Mobile will response with:

AT+CMEC=2

OK

For verifying the complete response, including variable value and carriage return/linefeed, the appropriate WAITFOR looks like:

WAITFOR (1, 'AT+CMEC=', iValue, chr(0x0D), chr (0x0D), chr (0x0A), 'OK')

#### COM()

COM() adds a comment to the HTML result file. Mid level equivalent to COM (see 8.3.1.4, “COM”).

Syntax:

COM (Param1, …)

Parameters:

Param1, …

Comment to be written to HTML file, as variable argument list. This may be composed using strings, variables and python expressions. See 8.3.2.2, ”Variable argument lists”.

Remarks:

See Remarks section of high level command COM, section 8.3.1.4.

Example:

iIndex=17

COM ('Index+4 = ', iIndex+4)

# also possible to add carriage return/linefeed:

COM ('Index+4:\r\n', iIndex+4)

#### MESSAGE()

MESSAGE() opens a message box, displaying any information to the user. Script execution waits until user clicks the ‘OK’ button in the message box.

Mid-Level equivalent to MESSAGE, (see 8.3.1.5, “MESSAGE”), allowing usage of variables etc.

Syntax:

MESSAGE (Param1, …)

Parameters:

Param1

Variable argument list specifying information to be displayed in the message box. This may be composed using strings, variables and python expressions. See 8.3.2.2, ”Variable argument lists”.

Remarks:

MESSAGE() is intended for displaying simple information. MESSAGE() does not write any output to the HTML result file.

If you want the user to execute a test manually, see also mid level command USERTEST(), section 8.3.2.7.

Example:

PhoneNumber='+492842950'

MESSAGE ('Dialling number ', PhoneNumber, '! Please wait…')

#### USERTEST()

Quite similar to MESSAGE(), USERTEST() opens a dialog box, asking the user to perform a test manually, e.g. , check a display content or check if a called phone really rings. The opened dialog always has three buttons: ‘PASSED’, ‘FAILED’ and ‘SKIP’, allowing the user to react in an appropriate way. Script execution waits until user clicks any of the three buttons. The usertest result will be written to the HTML result file.

Syntax:

iResult USERTEST (Param1, …)

Parameters:

Param1

Variable argument list specifying the test the user should perform. This may be composed using strings, variables and python expressions. See 8.3.2.2, ”Variable argument lists”.

Return value:

-1, 0 or 1, depending on the button the user clicked:

-1:	‘FAILED’

0:	‘SKIP’

1:	‘PASSED’

Example:

PhoneNumber='02842958644'

ATD PhoneNumber;

iResult=USERTEST ('Dialling ', PhoneNumber, '...\r\nPlease check if called phone rings!')

if iResult==0:

MESSAGE Ooohps! Lazy today? ;-)

Depending on the button the user clicked, USERTEST() writes an output to the HTML result file like:

### Low level commands

#### send(), sendln()

Low level functions for sending any data via a specified logical port.

Syntax:

send (iLogicalPort, strSendBuffer)

sendln (iLogicalPort, strSendBuffer)

Parameters:

iLogicalPort

Logical port (1..6) to be used for sending

strSendBuffer

Python string with data to be sent. Variable argument lists are not supported.

Remarks:

send() sends the content of  strSendBuffer, without any modification. sendln() appends carriage return / linefeed (0x0D / 0x0A).

As low level commands, both don’t write any information to the HTML result file.

Script execution will terminate if iLogicalPort  is invalid or port not opened.

Examples:

send (1, 'AT+CCLK?')

# demonstrate sendln() in conjunction with python string operations

MyPhoneNumber='+492842950'

sendln (1, 'ATD'+MyPhoneNumber+';')		# will result in ATD+492842950;

#### receive()

Low level function for receiving incoming data via a specified logical port.

Syntax:

strReceivedData receive (iLogicalPort)

Parameters:

iLogicalPort

Logical port (1..6) for listening to incoming data.

Return Value:

Complete receive buffer content of the corresponding logical port, if data available.

If no data was received (receive() timed out), python object  None is returned.

Remarks:

If the logical port's receive buffer contains previously received data, receive() returns immediately, without waiting for further incoming data.

If no data is available, receive() waits until either data is received or receiving times out. Timeout time can initially be set in project settings dialog (see 5, “Loading and setting up projects ”) or be changed during script runtime using the low level command SetReceiveTimeout() (see 8.3.3.4).

Received data will be appended to the internal, port-specific receive buffer without deleting already existing content. If an empty buffer is desired before receiving, call ClearReceiveBuffer() (see section 8.3.3.3 and 8.3.2.1, “About receive buffers”).

Example:

# check if a mobile is connected to port 2

# first set timeout time to 1s

iOldTimeout=SetReceiveTimeout(1000)

# now simply send 'AT'

sendln (2, 'AT')

strReceived=receive(2)

if strReceived is not None:

# Mobile answered -> show receive result in message box

MESSAGE ('Response to AT:\r\n', strReceived)

else:

# receive timeout -> display message

MESSAGE No answer from mobile at port 2!

# reset timeout time to old value

SetReceiveTimeout(iOldTimeout)

#### ClearReceiveBuffer() / ClearAllReceiveBuffers()

Functions for clearing internal receive buffers.

Syntax:

ClearReceiveBuffer(iLogicalPort)

ClearAllReceiveBuffers()

Parameters:

iLogicalPort

Logical port (1..6) whose receive buffer is to be cleared.

Remarks:

The first function clears the internal receive buffer of the specified logical port, ClearAllReceiveBuffers()clears all receive buffers. See 8.3.2.1, “About receive buffers” for more information about receive buffer handling.

#### SetReceiveTimeout()

Sets the timeout time for all commands which actively wait for incoming data (WAITFOR(), receive()).

Syntax:

iOldTimeoutTime SetReceiveTimeout (iNewTimeoutTime)

Parameters:

iNewTimeoutTime

Non-negative integer with new timeout time (in milliseconds) to be set.

Return Value:

Old timeout time which was active before the call to SetReceiveTimeout().

Remarks:

Initial timeout time is defined in project settings dialog (see 5, “Loading and setting up projects ”).

As a general idea, it’s worth considering setting initial project’s timeout time to a relatively short value, e.g.  5 seconds. This will shorten the time ATT waits for the expected response from mobile if a testcase fails, thus accelerating the whole testrun. As most AT commands will respond in a relatively short amount of time (also in error cases), this will mostly be appropriate.

Only a handful of commands take longer. These especially are all commands with network interaction (ATD, sending SMS, checking/setting call forwarding, etc.). Before sending such a command, simply increase timeout time temporarily, using SetReceiveTimeout(). Afterwards, reset timeout time to the old value again, as returned by the first call to SetReceiveTimeout().

Example:

See example receive(), section 8.3.3.2.

#### ExtractParameter ()

ExtractParameter() is useful for extracting a specified parameter value (a substring) out of a complete string returned from mobile.

Syntax:

strParameterValue ExtractParameter (strInput, iNoOfParameterToExtract,

strStartAfter=’’, strSeparatorTokens=’,’)

Parameters:

strInput

Input string from which a substring is to be extracted as parameter.

iNoOfParameterToExtract

Parameter number (non-negative integer), specifying the parameter to extract. Counting of parameters starts after strStartAfter. Parameters are separated by one of the chars contained in strSeparatorTokens.

strStartAfter

Starting string sequence (optional). Parameter extraction starts after the first occurrence of strStartAfter in the given string strInput. If strStartAfter is not contained in strInput, ExtractParameter() fails. This parameter is optional. By default, the parameter extraction starts at the beginning of strInput.

strSeparatorTokens

(Optional) string containing the separator(s) which separate(s) the parameters in strInput. This parameter is optional. Default: ','.

Return Value:

Extracted parameter value (maybe an empty string).

Example:

This example demonstrates how to extract the SMS service center address (SCA) and its number format from mobile, as returned from command AT+CSCA?.

The mobile will response to AT+CSCA? like:

AT+CSCA?

+CSCA: "+491722270333",145

OK

Parameter extraction should start after +CSCA: ", so that’s the value for strStartAfter.

The SCA and its number format (“145”) a separated by “ (double quote) and , (comma), so these both will be used as separator tokens. Additionally, the number format is followed by a line feed (\n), so that’s also part of strSeparatorTokens.

The corresponding script:

# get service center address (SCA) from mobile

AT+CSCA?

strCSCAResult=WAITFOR (1, '+CSCA:')

# extract SCA (first parameter after +CSCA: ")

strSCA=ExtractParameter (strCSCAResult, 1, '+CSCA: "', '",\n')

# extract SCA number format (second parameter after +CSCA: ")

iSCAFormat=int (ExtractParameter (strCSCAResult, 2, '+CSCA: "', '",\n'))

See also demo script AttSpecificPythonCommands.txt for another example.

## Compatibility and scripting notes

As ATT2.0 uses a completely new architecture, especially the PYTHON interpreter, there might be compatibility problems if you try to run old scripts written for ATT1.0.

ATT2.0’s precompiler tries to get the old scripts running without any changes. That should be successful if no variables or some strange command variants are used in ATT1.0 scripts.

For ATT1.0 variable resolving, the [COMPATIBILITY] section was introduced to the ini file att20.ini. Old variables contained in this section are simply mapped to new names used by ATT2.0. Names of old variables not found in this section will be left unchanged.

Variable resolving in conjunction with AT commands may lead to ambiguity (with both ATT2.0 and 1.0 scripts), causing the PYTHON interpreter to terminate with errors.

Imagine the following AT command in a script file:

AT+CPMS=MT,MT,MT

Now, what’s MT? Is this a PYTHON variable or a string?

Per default, ATT2.0 presumes all not recognized expressions to be a python expression, in this case a variable. But of course, at runtime, the PYTHON interpreter terminates because MT is an unknown identifier.

As described in 8.3.1.1, “AT commands”, the solution is: Always use double-quotes “…” for marking string expressions, in this case:

AT+CPMS=”MT”, “MT”, “MT”

If that’s not possible, use the ATT2.0 specific PYTHON command ATCMD():

ATCMD (1, ‘AT+CPMS=MT, MT, MT’)

This will be the solution for all (I hope! …) cases the precompiler fails to interpret a command correctly. This also is valid for WAITFOR, COM etc.: If in trouble, use the ‘mid level’ python commands WAITFOR (), COM () etc.

See also the files in demo folder for examples!