- ICM MP AD RD

- Bluetooth Stereo Headset (BTSTHS)

- AT Command List

Status:	in work, in review, released

Version:	0.2

Date:	24.08.2004

Responsible:	?

Author(s):	Jens Knapmeyer, ICM MP AD RD 1 PL

Release:  <name>, <date>

Distribution List

Name (alphab.)	Department	Location

Ebert, Volker	ICM MP AD RD 1	Mch

Galtier, Antonin	Comneon	Mch

Guhl, Stefan	ICM MP PD SW 22 KLF 1	Klf

Höfelschweiger, Martina	ICM MP AD RD 1	Mch

Reutlinger, Winfried	ICM MP PD SW 3 Ulm 3	Ulm

History

| Ver. | Date (yy-mm-dd) | Name | Status | Changes |
| --- | --- | --- | --- | --- |
| 0.1 | 04-08-20 | Knapmeyer | In work | First draft based in CK Bluetooth 75 (also CK BT SIM-access and CK-BT-Portable) |
| 0.2 | 04-08-24 | Knapmeyer | In work | Slight corrections after discussion with Antonin Galtier, Not yet combined to a single document with the CK requirements |

Introduction

Bluetooth Stereo Headset (BTSTHS) is a hands free unit for portable use. It does only support connectivity over Bluetooth. It uses the Bluetooth Hands Free Profile (HFP), the Advanced Audio Distribution Profile (A2DP) and the Audio Video Remote Control Profile (AVRCP), as well as the related profiles and protocols. It also uses ATC commands not specified in the mentioned profiles to provide advanced functionality. Furthermore, the BTSTHS uses BT Sync-Profile and/or SyncML (in parallel to the HFP) to synchronize address bool contents. In the context of this document we will call this mode of operation “BT HFP MODE” (even if other BT profiles are involved). BTSTHS is an ODM product.

AT Commands

Note: Almost the same commands as for the Bluetooth CarKits are used, therefore the markings are similar as in the document “ATC_Command_Set_x75_Bluetooth_Car_Kits_v0_4_.doc”

Blue entries denote changes to previous version of this document.

Red entries denote extensions to existing commands or completely new commands.

Green entries are additional new extensions for the BTSTHS

Grey entries denotes that the requirement is TBC.

Green entries with yellow background are requested again for the BTSTHS, since the implementation of SyncML and Synchronisation Profile is not confirmed for the accessories

| Commands specified in Hands Free Profile |  |
| --- | --- |
| Command | Usage |
| ATA | Accept incoming call |
| ATDdd..dd; | Dial number |
| ATDL | Dial last number. Only required in non-bluetooth mode if phone does not support LD phone book. |
| AT+BLDN | Dial last number (only used if phone does not support LD or DC book) (BT specific command) |
| AT+BRSF=31 | Bluetooth report and retrieve supported features. Report the following: Local ECNR: Not Supported, Three-Way-Calling Features: Supported, CLI Representation: Supported, Voice Recognition Activation: Supported, Remote Volume Control: Supported |
| AT+BVRA… | Enable / disable voice recognition (BT specific command). Command is not required if phone does not support voice recognition. |
| AT+CCWA… | Enable ‘call waiting notification’ URC |
| AT+CHLD=2 | Handle waiting calls  Swap active / waiting / held call |
| AT+CHLD=? | Test command. Check availability of multi party call service. |
| AT+CHUP | End call, end call set up |
| AT+CIND=? | Retrieve mapping of indicators |
| AT+CIND? | Get current status of indicators ( call status) |
| AT+CLIP=1 | Enable CLIP |
| AT+CMER… | Enable indicator events (call status, …) |
| AT+NREC | Disable Noise Reduction and Echo Cancellation of mobile phone. A neutral parameter set has to be selected. |
| AT+VGS | Send current speaker gain to mobile phone. (BT specific command) |

| URCs specified in Hands Free Profile |  |
| --- | --- |
| Command | Usage |
| OK |  |
| ERROR |  |
| RING | Incoming call URC. Handle incoming calls |
| +CCWA | Waiting call URC. |
| +CIEV… | Indicator event URC. Call handling. (Battery / network status) |
| +CLIP: “… | Display number of calling line. |
| +BVRA | Voice recognition termination URC. Voice recognition handling. URC is not required if phone does not support voice recognition. |
| +VGS | URC. Set speaker gain in car kit. |
| +BSIR | URC. In-band ringing tone settings of mobile phone have changed. |

| Commands NOT specified in Hands Free Profile |  |
| --- | --- |
| Command | Usage |
| AT+CLCC=? | CLCC test command |
| AT+CLCC | Read number and name of current call. |
| AT+CPBS=? | Test command. Check which books are available |
| AT+CPBS? | Read the currently selected phone book storage and the usage status. The response shall comprise the parameters <used> and <total>. |
| AT+CPBS=”DC” | Select the DC (dialed call) phone book. |
| AT+CPBS=”LD” | Select the LD (last dialed) phone book. |
| AT+CPBS=”MC” | Select the MC (missed calls) phone book. |
| AT+CPBS=”RC” | Select the RC (received calls) phone book |
| AT+CPBS=”SM” | Select the SIM phone book. |
| AT+CPBR=? | Check the valid range of book indexes (including empty ones) |
| AT+CPBR=x,y | Read entries of the selected phone book (DC, LD, MC, RC, SM) |
| AT+CSCS=? | Get list of available character sets. |
| AT+CSCS=”GSM” | Activate character set GSM |
| AT+CSCS=”UCS2” | Activate character set UCS2 |
| AT+COPS? | Read current network operator |
| AT^SABD=XYZ | Write accessory ID, type (= car kit) and software version |
| AT^SDBR=? | Get number of address book entries |
| AT^SDBR=x | Get address book entry with index x. |
| AT^SPBA=? | ‘Get current default book’ test command |
| AT^SPBA? | Get current default book (Is address book or phone book active?) |
| AT^SPBG=? | Get number of sorted phone book entries |
| AT^SPBG=x | Get sorted phone book entry with index x. |
| AT^SPBS=? | SPBS test command |
| AT^SPBS=”CS” | (Re-)select “common sorted” phone book as this is not possible with AT+CPBS… . |
| New PoC command (mandatory) | Accept Incoming Session |
| New PoC command (mandatory) | Reject incoming session |
| New PoC command (mandatory) | Leave / End Session |
| New PoC command (mandatory) | Request right to speak |
| New PoC command (mandatory) | Release right to speak |
| New PoC command (mandatory) | Start ad-hoc session with 4efault user |
| New PoC command (mandatory) | Get session info (a command similar toC LCC). For very participant: - status (inviting, connecting, connected, speaking, …) - name - URI - URI type - ID - others? |
| New PoC command (TBC) | Read PoC phone book |
| New PoC command (TBC) | Start PoC session with any group / person |
| New PoC command (TBC) | Get PoC service status (online / offline) |
| New Media-Player-Status command (mandatory) (TBC) | Test command (SABD or SACD) (mobile may reply with the possible range of answers) |
| New Media-Player-Status command (mandatory) (TBC) | Request Media Player Status Information (SABD or SACD) (requested by BTSTHS from mobile phone, mobile answers with its actual status Play / Pause / Stop / Title Select) |
| New Enable URC for Media-Player file information (mandatory) (TBC) | (SABD or SACD) Enables the URC to send file information towards the Bluetooth Headset, see URCs |

| URCs NOT specified in Bluetooth Profiles |  |
| --- | --- |
| Command | Usage |
| BUSY |  |
| NO_DIALTONE |  |
| NO_CARRIER |  |
| New PoC URC (mandatory) | Session invitation (sender name, sender URI) |
| New PoC URC (TBC) | Right to speak status |
| New PoC URC (TBC) | Session info (status (something like ‘inviting’, ‘connected’, ‘speaking’, …), participant name, participant URI, ID?, URI type?)  whenever session state changes |
| New PoC URC (TBC) | PoC service status (online / offline) |
| New Media-Player file information URC (mandatory) (TBC) | (SABD or SACD) The mobile phone reports the currently selected media “file name”, an “index" for this file (for reference), and “ID3-Header information” (at least duration of song) whenever a new file is selected or highlighted, Character set as requested by command AT+CSCS. *In case a tuner is integrated in the mobile phone similar information about frequency, band and channel name are requested. |

* only necessary if a tuner is integrated in the mobile phone

| Unused commands of Handsfree Profile |  |
| --- | --- |
| Command | # Usage |
| AT+BINP | Connect a telephone number to a voice tag. (BT specific command) |
| ATD>nnn; | Standard ATC command. Memory dialling. nnn (Dial the telephone number from the current telephone book location number nnn.) |
| AT+VGM | Send current microphone gain to mobile phone. (BT specific command) |
| AT+VTS | Standard ATC command. DTMF generation |
| +VGM | URC. Set microphone gain in car kit. (BT specific command) |

Other Commands

In the table below commands of the AVRCP used by the BTSTHS are listed

| Used Commands of Audio Video Remote Control Profile |  |  |
| --- | --- | --- |
| Operation ID | In Category 1 (Player) | In Category 3* (Tuner) |
| Channel up | No | Yes |
| Channel down | No | Yes |
| Power | Yes | Yes |
| Play | Yes | No |
| Stop | Yes | No |
| Pause | Yes | No |
| Rewind | Yes | No |
| Fast Forward | Yes | No |
| Forward (skip forward) | Yes | No |
| Backward (skip backward) | Yes | No |

* only necessary if a tuner is integrated in the mobile phone