ICM MP P SW 1

ODM Support of Mobile Phone Manager

Mobile Phone Interfaces for MPM Support

DRAFT VERSION

Copyright  Siemens AG 2003

Copying of this document and giving it to others and the use or communication of the contents thereof, are forbidden without express authority. Offenders are liable to the payment of damages. All rights reserved in the event of the grant of a patent or the registration of a utility model or design.

Department:	ICM MP P SW 1

Name:		Bill Holohan

Tel:	+49 89 636-80110

E-Mail:	william.holohan.extern@mch.siemens.de

File:		ODM Support Requirements for Siemens Mobile Phone Manager.doc

Table of Contents

1	General	5

1.1	Document History	5

1.2	Abbreviations and Definitions	5

1.3	Purpose of this document	9

1.4	References	9

1.4.1	OBEX®	9

1.4.2	IrMC®	9

1.4.3	Siemens OBEX	9

1.4.4	Siemens AT	9

1.4.5	Siemens SMS Archive Files	9

1.5	Editor notes	10

1.5.1	Version 0.3	10

1.5.2	Version 0.4	10

1.5.3	Version 0.5	10

1.5.4	Version 0.6	10

1.5.5	Version 0.7	10

1.5.6	Version 0.8	11

1.5.7	Version 1.0	11

1.5.8	Version 1.1	11

1.5.9	Version 1.2	11

1.5.10	Version 1.3	11

1.5.11	Version 1.4	11

2	Overview	12

2.1	Structure of this document	12

2.2	How this document should be used	12

2.3	Connectivity	13

2.4	Standards	13

2.4.1	General remarks about AT commands	13

2.4.2	OBEX	13

2.4.3	General remarks about IRMC	14

2.5	Switching between AT and OBEX	14

3	Components	14

3.1	General Applications support (MPM Framework functions)	14

3.1.1	Use Cases	14

3.1.2	Interface	15

3.2	Contacts	17

3.2.1	Use Cases	17

3.2.2	Interface	18

3.2.2.1	Phone Book Access	18

3.2.2.2	Address book access	20

3.2.3	Contact Fields Supported	21

3.3	Messenger	23

3.3.1	Assumptions	23

3.3.2	Use Cases	23

3.3.3	Interface	24

3.4	Sounds	28

3.4.1	Use Cases	28

3.4.2	Interface	28

3.5	Photo Suite and Photos-To-PC	32

3.5.1	Use Cases	32

3.5.2	Interface	32

3.6	WAP-Browser	34

3.6.1	Use Cases	34

3.6.2	Interface	34

3.7	Backup Restore	36

3.7.1	Use Cases	36

3.7.2	Interface	36

3.8	SmartSync	39

3.8.1	Use Cases	39

3.8.2	Interface	39

3.9	GPRS Modem Assistant	40

3.9.1	Use Cases	40

3.9.2	Interface	40

3.10	Phone Explorer	42

3.10.1	Use Cases	42

3.10.2	Interface	43

4	AT Command List	48

4.1	DIR values for AT^SBNR command on a Siemens S65.	51

4.2	AT^SLNG Language values returned by a Siemens S65	52

4.3	AT^SPBS phone book values	52

4.4	AT^SSET parameter values	54

4.5	AT+CRMP call type values	55

5	Appendix A: Multimedia Message Service Handling	56

5.1.1	Test Option	57

5.1.2	Read Option	57

5.1.3	Write Option	58

5.1.4	Unsolicited Result Codes	58

5.2	Use cases	58

5.2.1	Query the import/export path	59

5.2.2	Read all MMS messages from phone	61

5.2.3	Query the pending state	63

5.2.4	Delete a message	64

5.2.5	Create new message and import to phone	65

5.2.6	Create new message, import and send it	67

5.2.7	Read a single message (export message)	69

5.2.8	Send a message which is already on the phone.	71

6	Appendix B: AT Commands which are not yet in the official document	72

6.1	Get Device Info AT Command	72

7	Appendix C: Sequence Diagrams / Call Flows	73

7.1	IrDA connection	73

7.2	Serial Cable connection	75

7.3	USB connection	76

7.4	BlueTooth connection	76

8	Appendix D: Check lists	77

8.1	Checklist 1: Use Cases	77

8.2	Checklist 2: Support of OBEX commands	81

8.3	Checklist 3: Support of AT Commands	82

# General

## Document History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 0.1 | 09.10.03 | Mario Tomaselli | Document created |
| 0.2 | 08.12.03 | Andreas Wetterich | First draft |
| 0.3 | 18.06.03 | Andreas Wetterich | First reviewed version |
| 0.4 | 07.07.04 | Bill Holohan | Following use case review |
| 0.5 | 14/07/04 | Bill Holohan | Tidy up and add results of AT commands review in Brazil |
| 0.6 | 21/07/04 | Bill Holohan | SmartSync & AT list |
| 0.7 | 26/07/04 | Bill Holohan | Minimal changes |
| 0.8 | 29/07/04 | Bill Holohan | SGDI |
| 0.9 | 24/08/04 | Andreas Wetterich | Remove AT+SSOC and AT+SRTC (obsolete); AT^SPBS –phonebook types |
| 1.0 | 01/09/04 | Frank Eppink | Added ref. SMS_ON_MMC.doc, reviewed SmartSync, added comment to Sounds M4.10 |
| 1.1 | 09/09/04 | Bill Holohan | Corrected major error in address book description of OBEX usage. We use level 4 not 3. |
| 1.2 | 14/09/04 | Frank Eppink Bill Holohan | Added more details about MMS-Sync. Added information about AT^SSET parameters. |
| 1.3 | 07/10/04 | Bill Holohan | Moved AT commands for older phones only to “not needed from ODMs” table. Explained “SMS Archive”. |
| 1.4 | 14/10/04 | Bill Holohan | Added <call type> table for AT+CRMP. Added Sequence Diagrams for MMS. Added call flows for IrDA and Serial Cable. |

## Abbreviations and Definitions

| Term | Definition | See also |
| --- | --- | --- |
| address book | An application or database on the phone that organises contact information such as, for example, name, phone numbers, e-mail addresses, or postal addresses. |  |
| AT | A simple data exchange protocol between mobile phone and PC based on the standard used in Hayes Modems. All (or nearly all) commands are user readable texts starting with ‘AT’ (or ‘at’, but not ‘At’ or ‘aT’). |  |
| Backup-Restore | An MPM component, which creates backups of phone data to allow the user to restore the phone data to a previous state. The user can select which folders or files should be included in the backup (e.g. only the address book, only phone memory folders etc.) |  |
| Contacts | An MPM component application for managing address book and phone book on the phone and exchanging contact data between the phone and PC (address book and MS Outlook) |  |
| Connectivity | The ability to exchange data between a mobile device and a PC. |  |
| Data Exchange Layer | A protocol layer on the PC that encapsulates the communication with the mobile phone and provide services for the application layer. The Data Exchange Layer is implemented in a process referred as phone server. |  |
| GPRS Modem Assistant (GPRS MA) | An MPM component, which emulates a modem to allow the user to establish an IP connection over GPRS with the phone. |  |
| IMEI | International Mobile Station Equipment Identity |  |
| IrDA | Infrared Data Association. Besides standards for physical layers and transport layers over infrared, the organisation is also responsible for the high level protocol standards OBEX and IrMC. OBEX and IrMC are not bound to infrared communication but applicable for other bearers (e.g. USB, Bluetooth). Sometimes the term IrDA is simply used in the meaning “wireless infra-red communication”. |  |
| IrMC | Infrared Mobile Communications IrMC® is a standard protocol of IrDA. This document refers to Version 1.1 of the IrDA IrMC specification. The main scope of IrMC is the definition of data formats and access protocols to standard applications on mobile devices. Standard applications are for example address books, calendars, SMS clients and notepads |  |
| Messages | Short Messages (SMS), Extended Short Messages (EMS) or Multimedia Messages (MMS). This Messaging protocols are subject of ETSI standardisation. Messages are stored in a repository on the phone. Message data includes status information, so that applications are able to decide whether a message has been received, read, send, stored to be sent in the future or saved for further editing. |  |
| Messenger | An MPM component application used to create, edit, read and send Messages (SMS, EMS, and MMS) on the phone. |  |
| MPM | Mobile Phone Manager. An application for Microsoft Windows®. Siemens AG provides this application along with some Siemens Mobile Phones. MPM is a collection of PC connectivity tools e.g. Contacts, Messenger, and a Shell-Extension for MS Windows Explorer®. |  |
| MPM Framework | An MPM component responsible for detecting connected phones and providing a user interface (Launch bar) to start other MPM applications. |  |
| OBEX | object exchange protocol IrOBEX® is a standard protocol of IrDA. OBEX in this document refers to the Siemens customisation of IrOBEX 1.2. The main task of OBEX is to exchange so called objects between devices It is a session protocol as well as an application protocol. |  |
| ODM | Original Device Manufacturer. A manufacturer who produces a phone according to Siemens specifications where the phone is sold under the brand name Siemens. |  |
| phone book | A repository on the SIM card of the mobile phone that contains a list with names and numbers. |  |
| Phone Explorer | An MPM component implemented as a Windows Shell namespace extension. It shows folder views in the Windows Explorer® window and Open/Save file dialogs in different applications that need access to the phone file system. |  |
| phone server | See Data Exchange Layer |  |
| Photo Suite | An MPM component application that deals with bitmap graphics on the phone (camera JPEGs, wallpapers etc.) |  |
| Photo-To-PC | An MPM component for handling manual or automatic (scheduled) download of camera images from the mobile to the PC |  |
| SDS | Siemens Data Suite: The name of a fore-runner to Mobile Phone Manager. No longer used. |  |
| Siemens Phone | In this document a Siemens phone is defined as a phone which responds to the AT+CGMI command with a string starting: “<CR><NL>SIEMEMS<CR><NL>” |  |
| SIM | Subscriber Identity Module The main purpose of SIM is identifying customers for network operators. However SIM cards are often used to store user specific data. Information stored on the SIM card is immediately available for other phones if the user decides to use the card with other phones. This is why data repositories on SIM cards are still supported by modern phones even when they have extended non-volatile phone memory (flash file system, multi-media card). |  |
| Sounds | An MPM component formerly known as “ringing tone editor”. This application deals with sounds on the phone (Ringing tones, MP3 files etc.) |  |
| TE | Terminal Equipment |  |
| vCalendar | A versit Consortium (http://www.imc.org/pdi/) standard for calendar entries. |  |
| vCard | A versit Consortium (http://www.imc.org/pdi/) standard for electronic business cards. There are also IrMC extensions to the vCard 2.1 standard. These are described in vchgs_1p0.pdf available at http://www.irda.org/. |  |
| WAP-Browser | An MPM Component application. While browsing WAP pages with the phone, the user saves bookmarks (links) to sites he is visiting. The bookmarks will be stored in a repository on the phone. MPM provides a WAP browser application, too. The user will visit WAP sites and store the bookmarks on the PC. It is possible to exchange bookmarks between the MPM WAP-browser and the WAP browser application on the phone. |  |

## Purpose of this document

Mobile Phone Manager (MPM) is a common platform for PC connectivity tasks on a large range of Siemens mobile phones. In order to simplify the integration of new phone models in this environment and shorten the time to add support for new functionality, this document will define a set of PC connectivity interfaces. A phone that is able to support a function should implement the interface considering the recommendations described in this document.

The requirements listed in this document are based on already existing applications for platform 65 phones. Enhancements in new versions of MPM have to be added here.

Note that only those Use Cases are mentioned that involve communication with the phone. MPM itself supports more Use Cases.

## References

### OBEX®

OBEX is a standard defined by the Infrared Data Association. You can find version 1.3 of the OBEX specification at http://www.irda.org/

### IrMC®

IrMC is a standard defined by the Infrared Data Association. It is based on OBEX®. You can find version 1.1 of the IrMC specification at http://www.irda.org/. We require support of IrMC Levels 2 and 4.

### Siemens OBEX

A description of Siemens customer defined extensions to the OBEX protocol is given in the document

DES-OBEX-9.doc

### Siemens AT

The complete reference to AT Commands for Siemens Mobile Phones (Document-Id: A30880-A10-A001-13-D376 from 20 July, 2004) for platform 65 phones

ATC_Command_Set_s65.pdf

### Siemens SMS Archive Files

The following document contains the specification of the SMS archive files.

SMS_ON_MMC.doc

## Editor notes

### Version 0.3

We decided to split the former document into two separate documents. This document now only describes MPM interoperability with phones that means the core interface specification for communication between the phone and the PC. The second document “Phone Server Requirement Specification” contains information for developers that have to implement a MPM compatible data exchange layer between phone and PC or want to adapt an existing phone-to-PC interface for its use with MPM.

The documentation now incorporates IRMC Level 1.1.

Some components have been renamed. Ringing tone editor is now called “Sound Manager”. The name “Bitmap editor” has been changed to “Photo suite”.

The following components have been added: WAP-Browser, Backup-Restore and GPRS-Modem-Assistant.

### Version 0.4

- The use cases have been updated following feed-back from the developers of the various components.

### Version 0.5

- The correct name for the “Sound manager” component is “Sounds”. Similarly the “Contact Manager” component is called “Contacts”.

- Formatting has been changed in some places.

- The results of the investigation of AT command usage, done in Brazil have been included in the document.

- This version is still a “work in progress” and is being released to satisfy urgent requirements of some recipients. Chapter 4 needs to be re-worked (and I need to be persuaded that it is useful – if you think so let me know).

### Version 0.6

- Added SmartSync to list of components.

- Added a list of AT commands with comments on our usage.

- Included a draft of the AT^SMMS documentation. This should be removed when the Siemens AT documentation has been updated to include it.

### Version 0.7

- Some tidying up – minimal.

### Version 0.8

- Added description of AT^SGDI. This will be removed when the Siemens AT documentation has been updated to include it.

- Added list of contact fields supported. This will expand to allow for future phones.

### Version 1.0

- Added reference to SMS_ON_MMC.doc-document that specifies format of SMS archive files.

- Reviewed SmartSync Chapter.

- Added comment to Sounds M4.10

### Version 1.1

- The description of Contacts usage of OBEX for accessing the Address Book seems to have been based on a planned usage which was never implemented. We use IrMC Level 4 and not 3, as previously described here.

### Version 1.2

- Added more details to the MMS-Sync Use Cases "Read MMS from phone" and “Create new message, import and send it”

- Added some values for the AT^SSET parameters. These were extracted from our code. The current ATC documentation doesn’t seem to have them.

### Version 1.3

- Some of the AT commands in our list will never be used with “newer” phones. Moved these to a table of commands which an ODM need not implement.

- Note 1 in the Messenger description has been expanded to include a description of how archived messages (SMS) are recognised.

### Version 1.4

- Added list of <call type> values for AT+CRMP command in 65 generation phones.

- Added sequence diagrams and some explanatory text to the MMS Use Cases.

- Added call flows for Serial Cable and IrDA connections.

# Overview

## Structure of this document

Chapter 1.2 (see above) provides a detailed glossary of terms and definitions. This document will use these definitions without further explanation. Please refer to the glossary if necessary.

The specification is separated into one chapter per MPM component (application or plug-in).

Each chapter is divided into two or three parts.

- Part 1 lists and describes the use cases for each application. Use cases with ‘M’ at the start of their ordinal form part of the minimum requirement for a component. The minimum requirement must be supported in order to support the component (assuming the component should be supported at all. This will be specified elsewhere). If the use case is marked as ordinal starts with ‘E’ (for extended) and the phone does not support it then the component is able to operate with the phone but this component feature will not be available.

- Part 2 refers to the use cases in part 1 to describe which interfaces have to be implemented in order to support the use case.

- Part 3 (optional) contains examples how the interface is used in existing applications.

## How this document should be used

The document provides a starting point for analysing or planning data exchange between the phone and the PC. We want to reduce the time for integrating new phones in an existing environment designed to work with a wide range of different phones. So the process has to start with the following questions:

- Can the phone support all use cases, or at least the mandatory ones?

Should a phone not be able to support all use cases for a component then it is possible that it can still work with that component by supporting a sub-set of the use cases. MPM has the ability to scale the functionality of its applications depending on the connected phone. The MPM developing team needs a detailed list of use cases supported. You can use checklist 1 as a template for supplying this information. It will not necessarily be possible to pick and choose at the use case level due to interdependencies between the protocol usage of the use cases.

- What must the interface between phone and PC looks like in order to add support for the phone without further modifications of the data exchange layer? (Presuming that the phone supports the use cases)

This document aims to answer that question.

- Is there any functionality that is provided or can be provided by the phone that is not described in the list of use cases?

Our aim is to enable the PC user to access as many features as possible on the phone. So we would be grateful for your suggestions for improvements. If you have any ideas for functionality that would improve the user’s experience with MPM, please inform the MPM development team.

## Connectivity

We support connection to current phones via Bluetooth, Infrared, USB, and Serial (including the Siemens USB-Serial adaptor cable).

MPM scans all serial connections on the PC at a fixed baud rate of 19200 (and no other value) to detect supported phones. This implies that a supported phone is able to support this baud rate. When a phone has been detected we attempt to communicate with it at the fastest possible baud rate. We try 921600, 460800, 230400, 115200, 57600, 38400, and 19200 until we get a response "OK" to an "AT" command. Future versions of MPM will allow for baud rate customisation (i.e. setting the required for a specific phone type in MPM’s configuration file).

## Standards

### General remarks about AT commands

AT commands are used for all kinds of remote calls to phone functions. Some data is not organised as files, but stored in special repositories. For this reason there are AT commands to binary read or write the entries from such repositories. Most of these access routines apply to data in SIM card storages, an older standard that is still used for modern phones, even when most phones support the automatic import of SIM data to other repositories (e.g. SIM phone book to address book).

AT Commands are ASCII-character strings starting with “AT” and ending with a carriage-return-character (ASCII: 0x0D). The sender has to wait for an acknowledgement from the receiver (either “OK” or “Error”) before sending the next command. If the next command or simply a string “AT” is send before the acknowledgement is received, the current command will be cancelled. Please read document 3 for more information about the basics of AT. It is also a reference of all AT codes available for Siemens phones.

In most cases MPM needs a sequence of AT commands to perform a certain action. This document will only give examples of those sequences. It may be possible that the data access layer of MPM will vary from that order (if it is necessary and logically correct), so no presumptions should be made that there is a distinct order.

### OBEX

OBEX was originally designed for Infrared connections. The communication layer of the MPM supports this protocol for serial cables and Bluetooth as well. OBEX is used for operations that involve access to the file system of the mobile phone. For further particulars we refer you to the official specification by the Infrared Data Association (www.irda.org).

### General remarks about IRMC

IrMC is based on the OBEX layer. It defines a couple of data formats for database record sets along with methods to access the application databases. The main resource for this subject is the official IrDA IrMC specification version 1.1. The specification is available directly from IrDA (www.irda.org).

## Switching between AT and OBEX

The PC as client application determines the connection protocol (AT or OBEX) used. It switches from AT mode to OBEX mode by use of the AT command:

AT^SQWE=3<CR>

The phone answers with "OK<CRLF>" The connection is then in OBEX-mode. This means the next byte sent by the PC or by the phone is the start byte of an OBEX package.

It has been found to be necessary with some older phones to use the command AT^SQWE=0<CR> before sending the above command. This is currently done for all phones.

The PC switches back from OBEX mode to AT mode using the escape sequence:

<At least 1 second silence>+++<at least one second silence>.

Following this sequence the connection is in AT mode.

We do not use the sequence AT+CPROT=0 for switching to OBEX mode. At the time of writing this document current Siemens phones do not support this command.

# Components

## General Applications support (MPM Framework functions)

### Use Cases

M1.1	When a phone is connected to the PC the MPM wants to show status information in the main application window:

M1.1.1	Read the manufacturer name from the phone.

M1.1.2	Read the phone model from the phone.

M1.1.3	Read the status of the battery charge from the phone.

M1.1.4	Read the antenna status (signal strength) from the phone.

E1.1.5		Read the software revision from the phone.

MPM can query the phone for this information any time after the phone has been connected.

M1.2	MPM wants to know the manufacturer and model in order to decide whether the phone is supported or not. MPM can query this value any time after the phone has been connected.

M1.3	MPM configures itself depending on what features are supported by the phone. There are different configurations of MPM for different types of phones. MPM will load the correct configuration and disable and hide unavailable functions from user access. Therefore MPM can query manufacturer and model any time after the phone has been connected.

M1.4	Configurations, user data and settings might be different for different phones even on the same model. A phone is identified by a global unique serial number called IMEI. MPM can query this value from the phone any time after the phone has connected with PC.

E1.5	The battery charge could change while the phone is connected with the PC and MPM is running. MPM could poll the phone server periodically for the new status. Polling however is a waste of time in multiprocessing environments. It is more effective, if the phone informs the MPM about changes and MPM updates the status information driven by those events. Future versions of MPM will be able to receive change notifications for battery charge and signal quality.

### Interface

AT Commands usage:

| Required for use case | AT Commands | Comment |
| --- | --- | --- |
| 1.1.1, 1.2, 1.3 | AT+CGMI | Get manufacturer. AT Command according to GSM 07.07. The response to this command contains the required information. The phone must reply with a string starting “<CR><NL>SIEMEMS<CR><NL>” if it is to be used with the MPM. |
| 1.1.2, 1.2, 1.3 | AT+CGMM | Get phone model. AT Command according to GSM 07.07. The response to this command contains the required information. The reply to this command must be registered with the MPM team if the phone is to be used with the MPM. |
| 1.1.3 | AT+CBC | Get battery charge AT Command according to GSM 07.07 The response to this command contains the required information in the second parameter (bcl). The first parameter is ignored. |
| 1.1.4 | AT+CSQ | Get signal quality AT Command according to GSM 07.07 The response to this command contains the required information in the first parameter (rssi). The second parameter is ignored. |
| 1.1.5 | AT+CGMR | Get the software revision AT Command according to GSM 07.07 The response to this command contains the required information. |
| 1.4 | AT+CGSN | Get IMEI AT Command according to GSM 07.07. The response to this command contains the required information. |
| 1.5 |  | Not currently used by MPM. |

See Siemens AT Commands  for a detailed description of each AT Command.

In order to check for the current values of the battery charge and the signal quality, MPM polls the phone using the commands "AT+CBC" and "AT+CSQ".

MPM may also use the commands "AT+CGMM", "AT+CGMR", "AT+CGMI" and "AT+CGSN" periodically in order to check that the phone is (still) connected.

The phone software however should not rely on these commands being called periodically because certain MPM configurations may be using only some of the commands or not polling at all.

During all activities it is possible that the commands “ATE0” and “ATE1” can be used to change the echo behaviour of the connected phone. The command “AT+CMEE=1” can also be used at any time to set the required extended error format. Although we currently only use the value “1” it is recommended that all defined values be implemented.

## Contacts

### Use Cases

M2.1	The Contacts plug-in accesses a phone book.

M2.1.1	A phone book entry can contain two fields, name and phone number.  The phone number entry is mandatory.

M2.1.2	MPM reads and displays the entries. The fields name and number are listed in separate columns.

M2.1.3	The user changes the entry (both name and number are changeable). The changes are written to the phone book when the user confirms the changes.

M2.1.4	The user creates a new entry and fills in name (optional) and number. After confirmation the new entry is added to the phone book.

M2.1.5	The user deletes an entry. The change is written to the phone book after confirmation.

M2.1.6	The user cuts an entry from the phone book. The original entry in the phone book will be removed after pasting it to another location.

M2.1.7	The user copies an entry from the phone book. The original entry is not affected by a subsequent paste action.

M2.1.8	The user pastes an entry to phone book. The entry is written to the phone book immediately.

M2.2	The Contacts plug-in accesses the address book.

M2.2.1	An address book entry contains at least two fields, a name (can be last or first name) and phone number (can be home, office or mobile number). Other entries, which are described in the vCard specification, are optional.

M2.2.2	MPM lists address book entries. Each field of the entry is listed in a separate column. The number of columns may differ on different phones.

M2.2.3	The user changes each field in a dialog box (only fields that are supported by the phone are editable). The change is written to the address book on phone when the user confirms the change.

M2.2.4	The user creates a new contact. It is possible to fill in fields that are supported by the phone address book. After confirmation the new contact is written to the address book on phone.

M2.2.5	The user selects one or more entries from the list and deletes them.

M2.2.6	The user selects one or more entries from the list and cuts them. Later the user pastes the entries to another location. The original entries are then deleted.

M2.2.7	The user selects one or more entries from the list and copies them. Later the user pastes the item to another location. The original entries remain in place.

M2.2.8	The user pastes an entry. The entry is transferred to the phone book immediately. If the entry was “cut” from the previous location it is then deleted from there.

General Note: The Contacts plug-in shows address book, phone books and PIMs on the PC (e.g. Microsoft Outlook) as different folders. In principle it is possible to copy or move entries between the folders. However when transferring data to a destination that is capable of holding less information than the source then any unsupported information will be lost. Copying an address book entry to the phone book for example means losing all information except name and main phone number.

### Interface

#### Phone Book Access

AT Commands are used to access the phone book as follows:

| Function | AT Commands | Comment |
| --- | --- | --- |
| 2.1.2 Read phone book entries | AT^SPBS=? AT^WSSL=? AT+CPBS=? [one of these depending on the phone type] | Command to get a list of supported phone books. MPM expects a comma separated list of two character phone book identifiers (e.g. ^SPBS: ("SM", “ME”, "FD", "ON","MC","RC","LD","OW","MS","CD","BL","RD","CS") is returned from an M55). |
|  | AT^SPBS="xx” AT^WSSL=”xx” AT+CPBS=”xx” [one of these depending on the phone type] | Select the required phone book. The value of xx is one of the phone book identifiers from the response to the command above. |
|  | AT+CPBR=? AT^WSRE=? [one of these depending on the phone type] | The response to this command contains information about the range of indices and the length of an entry. |
|  | AT+CPBR=<P1>,<P2> AT^WSRE [one of these depending on the phone type] | The response to this command contains the phone book entries of the records between positions <P1> to <P2> in the phone book, if present. <P2> will not be supplied if reading a specific entry. |
|  | AT+CSCS="UCS2" | Select TE character set |
|  | AT+CSCS="GSM" | Select TE character set |
| 2.1.3 Overwrite existing phone book entry (edit entry) | AT^SPBS=? AT^WSSL=? AT+CPBS=? [one of these depending on the phone type] | Command to get a list of supported phone books. MPM expects a comma separated list of two character phone book identifiers (e.g. ^SPBS: ("FD","SM","ON","LD","MC","RC","OW","MS","CD","BL","RD","CS") is returned from an M55). |
|  | AT^SPBS="xx” AT^WSSL=”xx” AT+CPBS=”xx” [one of these depending on the phone type] | Select the required phone book. The value of xx is one of the phone book identifiers from the response to the command above. |
|  | AT+CPIN=<pin> | Writing requires PIN2 authorisation. |
|  | AT+CPBW=[<index>], <number>, <type>, <text> AT^WSWE=<namelen>,<name>,<numlen>,<num>[,<datelen>,[<date>]] [one of these depending on the phone type] | Where <index> is the index of the contact in the phone book (the existing data for this index) is overwritten, <number> is the phone number, <type> is the type of number, and <text> is the encoded name for the contact. In the second format <name> is the contact name, <num> is the contact number and <date> an optional date value for some phone book types. |
|  | AT+CSCS="UCS2" | Select TE character set |
|  | AT+CSCS="GSM" | Select TE character set |
| 2.1.4 Create a new Phone book entry | see 2.1.3 | The AT Commands are identical to that used for overwriting existing entries. MPM must use a free index with the AT+CPBW command in order to create a new entry. |
| 2.1.5 Delete an entry | AT^SPBS=? AT^WSSL=? AT+CPBS=? [one of these depending on the phone type] | Command to get a list of supported phone books. MPM expects a comma separated list of two character phone book identifiers (e.g. ^SPBS: ("FD","SM","ON","LD","MC","RC","OW","MS","CD","BL","RD","CS") is returned from an M55). |
|  | AT^SPBS="xx” AT^WSSL=”xx” AT+CPBS=”xx” [one of these depending on the phone type] | Select the required phone book. The value of xx is one of the phone book identifiers from the response to the command above. |
|  | AT+CPBW=<index> AT^WSEL=”FD” [one of these depending on the phone type] | Where <index> is the index of the contact in the phone book. Because there are no further arguments, existing data is erased and the cell is available again. The second format (for DECT) seems a little drastic but is probably followed by writing all current entries but the entry to be deleted to the phone book. |
|  | AT+CSCS="UCS2" | Select TE character set |
|  | AT+CSCS="GSM" | Select TE character set |
| 2.1.6 Cut and paste | A sequence of calls of the commands above (2.1.2, 2.1.3, 2.1.4, 2.1.5) | To cut and paste (or move) an entry it is first read. Then a “create new” or “overwrite” action is used place the destination entry and finally the original entry will be deleted. |
| 2.1.7 Copy and paste | A sequence of calls of the commands above (2.1.2, 2.1.3, 2.1.3) | To copy and paste an entry it is first read. Then a “create new” or “overwrite” action is used place the destination entry. |

Notes:

- This table is not complete. There are although Siemens specific AT Commands used for phones which stores its contacts alphabetically sorted. The commands should substitute the equivalent command if required. Please refer to the AT Command definition for further information.

- The commands AT+CPBS=x would be used with a non-Siemens phone. But since MPM will not work with a non-Siemens phone they will not occur in practice.

- The AT^WXXX commands are used with non-GSM phones (e.g. DECT phones). They are mentioned here in the use cases but are not described anywhere else in this document.

See Siemens AT Commands  for a detailed description of each AT Command.

#### Address book access

For address book access MPM will use IrMC. This section does not provide a complete description of the protocol but only a brief overview to each use case. The exact specification is provided by the official IrMC and OBEX specifications which are available from IrDA.

IrMC defines four level of access to IrMC applications.

Level 1 provides the ability to push objects to an “Inbox”. We do not use this.

Level 2 provides the ability to read and write complete object stores. We use this in the Contacts plug-in for reading the contents of the address book (telecom/pb.vcf).

Level 3 provides access to object stores for reading and writing using static indices. We do not use this.

Level 4 provides access to object stores for reading and writing using unique non-reusable indices. The Contacts plug-in requires level 4 for updating (including deleting) individual address book entries. Level 4 is also used by the SmartSync plug-in.

2.2.2	MPM reads the number of entries in the database from the file ‘telecom/pb/info.log’. It uses an OBEX Get command to read all vCards in the address book using the Level 2 object ‘telecom/pb.vcf’.

2.2.3	MPM uses an OBEX PUT command to ‘telecom/pb/luid/xxx.vcf’ (where xxx is the index/LUID of the entry that has been changed) to write the changed entry back to the address book.

2.2.4	MPM uses an OBEX PUT command to ‘telecom/pb/luid/.vcf’ to write the new entry to the address book and expects the response to contain an App Parameters header with Tag 0x01 giving the LUID of the new entry.

2.2.5	MPM uses an OBEX PUT command to ‘telecom/pb/luid/xxx.vcf’, where xxx is the index/LUID of the entry that has to be deleted, without a body to remove the entry from the address book.

2.2.6	A cut and paste operation consists of a read (source), overwrite or create new (destination) and delete (source) actions.

2.2.7	A copy and paste operation consists of a read action (source), and an overwrite or a create new (destination) action.

### Contact Fields Supported

| Description | VCard |
| --- | --- |
| Family name | N:<family name>;<forename> |
| Forename | N:<family name>;<forename> |
| Company | ORG |
| Email | EMAIL;INTERNET |
| Home phone | TEL;HOME |
| Work phone | TEL;WORK |
| Mobile phone | TEL;CELL |
| Fax | TEL;FAX |
| Street | ADR:;;<street>;<city>;;<post code>;<country> |
| Post code | ADR:;;<street>;<city>;;<post code>;<country> |
| City | ADR:;;<street>;<city>;;<post code>;<country> |
| Country | ADR:;;<street>;<city>;;<post code>;<country> |
| URL | URL |
| Group | X-ESI-CATEGORIES |
| LUID | X-IRMC-LUID |
| Home E-mail | EMAIL;INTERNET;HOME |
| Date of birth | BDAY |
| Home Fax | TEL;FAX;HOME |

Fields which are not in this list will be ignored.

It is intended, in future releases, to use the contents of the info.log file to dynamically determine which fields are present.

## Messenger

### Assumptions

A message consists at least of the following parts:

- A key or index that identifies the storage location of the message on the phone.

- The length of the message.

- The data (PDU) of the message. Besides the user data, the PDU contains information needed (and defined) by the protocol (SMS, EMS, MMS), for example  the destination of the message, the sender of the message, a subject line However messaging protocols are not discussed here. Messaging is subject of ETSI- standardisation.

The storage model of messages is not subject of these requirements. However it is assumed (required) that the list of messages can be divided into logical groups as follows:

- Received messages - not read yet.

- Received messages - already read.

- Draft messages - created by the user and not yet marked as finished.

- Outgoing Messages - not to be sent.

- Outgoing Messages - scheduled to be sent.

- Sent messages.

### Use Cases

M3.1	MPM reads messages stored on the phone.

M3.2	The user edits a message that has not been send yet. MPM stores the message on the phone with the option to send it later. The existing message content is overwritten.

M3.2.1	The message was already on the phone.

M3.2.2	The message was on the PC.

M3.3	The user creates a new message.

M3.3.1	MPM stores the new entry on the PC.

M3.3.2	MPM stores the new entry on the phone. The message is stored so that the user can manually send it later.

M3.3.3	MPM stores the new entry on the phone. The message is sent immediately (or as soon as possible).

E3.3.4		The message is scheduled for sending at a specific date and time.

M3.4	The user sends a message which is stored on the phone.

M3.4.1	The message is to be sent immediately (or as soon as possible).

E3.4.2		The message is to be sent at a specific date and time.

M3.5	The user uses MPM to delete an existing message on the phone.

M3.6	MPM changes the status of a received message.

M3.6.1	The message is marked as “read” either on user request or automatically after the message has been shown.

M3.6.2	The message is marked as “not read” on user request.

M3.7	Before Text Messages can be sent or received the user has to configure a SMS Service Centre Address. MPM provides the ability to manage this setting from the PC.

M3.7.1	MPM read the current Service Centre Address from the phone

M3.7.2	MPM change the Service Centre Address on user request.

E3.8	MPM is informed when new messages are received while the messenger application is running, so that the messages can be shown immediately.

### Interface

We do not support the IrMC standard for messaging (IrMC specification chapter 9) yet.

GSM 07.05 commands are used for operating the SMS functions of the GSM mobile phone. The GSM module mobile supports the SMS PDU mode.

| Function | AT Commands | Comment |
| --- | --- | --- |
| 3.1 Read messages | AT+CMGF=0 | Set SMS PDU mode |
|  | ATE0 | Echo off |
|  | AT+CPMS=? | The response to this message contains information which message storage is currently preferred for a) already read, b) stored and c) not-read (received) messages. |
|  | AT+CPMS=<mem1>, <mem2>[, <mem3>] | Where mem1 is the preferred read/delete store, mem2 is the preferred write store, and mem3 is the preferred unread store. Possible values for mem1, mem2, and mem3 are SM (SIM card), ME (Mobile Equipment storage) or MT (both) |
|  | AT+CMGL=<type> AT^SMGL=<type> [one of these commands will be used based on phone type and context] | Where type is defined as: 0=unread msgs, 1=read msgs, 2=unsent msgs, 3=sent msgs, 4= all. The response to this command contains all SMS of the required type. An entry in that list is structured as follows: <index>,<type>, <length>,<CR,LF><PDU><CR,LF> |
|  | It is possible to read the content of a single SMS, too, if the index of the entry is known (command AT+CMGR). However MPM prefers to read the entire list. |  |
| 3.3 Create a new message | AT+CSCA? | Get the service centre number |
|  | AT+CMGF=0 | Set SMS PDU mode |
|  | ATE0 | Echo off |
|  | AT+CPMS=? | The response to this message contains information which message storage is currently preferred for a) already read, b) stored and c) not-read (received) messages. |
|  | AT+CPMS=<mem1>, <mem2>[, <mem3>] | Where mem1 is the preferred read/delete store, mem2 is the preferred write store, and mem3 is the preferred unread store. Possible values for mem1, mem2, and mem3 are SM (SIM card), ME (Mobile Equipment storage) or MT (both) |
|  | AT+CMGW=<LEN><CR><PDU><CTRL+Z> | Write <LEN> bytes of <PDU>. See comments for AT+CGMS, below. |
| 3.3.3 Create a message and send it immediately | AT+CSCA? | Get the service centre number |
|  | AT+CMGF=0 | Set SMS PDU mode |
|  | ATE0 | Echo off |
|  | AT+CMGS=<LEN><CR><PDU><CTRL+Z> | Write <LEN> bytes of <PDU> and send it. The PDU followed by CTRL+Z (0x1A) is output when a response (“>”) from the first part of the command has been received. |
| 3.4 Send a message existing on the phone | This is done using the same sequence of commands used in 3.3.3. The information about the message has previously been read from the phone. |  |
| 3.5 Delete a message | AT+CPMS=? | The response to this message contains information which message storage is currently preferred for a) already read, b) stored and c) not-read (received) messages. |
|  | AT+CPMS=<MEM1>, <MEM2>, <MEM3> | MEM1: preferred read/delete store MEM2: preferred write store MEM3 preferred unread store values for MEM<n> are SM (SIM card) or ME (Mobile Equipment storage) or MT (both) |
|  | AT+CMGD=<index> | Delete a message given by index. The message is moved to the delete store |
| 3.6 Set the “read” flag for a message | AT+CPMS=? | The response to this message contains information which message storage is currently preferred for a) already read, b) stored and c) not-read (received) messages. |
|  | AT+CPMS=<MEM1>, <MEM2>[, < MEM3>] | MEM1: preferred read/delete store MEM2: preferred write store MEM3 preferred unread store values for MEM<n> are SM (SIM card) or ME (Mobile Equipment storage) or MT (both) |
|  | AT+CMGR=<index> | Read the message identified by <index> setting the status to “read”. |
| 3.7.1 Get the current service centre address | AT+CSCA? | The response to this command contains a string with the service centre address |
| 3.7.2 Set the service centre address. This use case is not currently supported by MPM. | AT+CSCA=<ADDR> | This command sets the service centre address on the phone. |

- Notes

- In 3.1 OBEX is used to read the contents of “SMS archive” sub-directories “Draft”, “Inbox” and “Sent”. Please consult the document “SMS_ON_MMC.doc” for the specification of the SMS archive files. The location of the Archive directories and the file suffix for the archived messages in that sub-directory can be specified for each phone type in MPMs configuration data (e.g. “\Data\Sms archive\Draft\*.smo” specifies that all files with suffix ‘smo’ in the \Data\Sms archive\Draft sub-directory are archived messages). Four types of messages are catered for, namely: Draft, Inbox, Sent, and Unsent. These will not necessarily be present for all phone types.

- In 3.1 the AT+CPMS command is sent with parameters “SM”,”SM” and “ME”,”ME” and in each case followed by an AT^SMGL=4 command.

- In 3.5 OBEX might be used to delete the message (e.g. if it is in the “Archive”).

- In 3.6 if a Siemens main-stream (i.e. non-Symbian) phone is connected MPM currently only supports setting the “read” status flag (i.e. it does not support resetting it; presumably because no Siemens mainstream phone can do this).

See Siemens AT Commands  for a detailed description of each AT Command.

## Sounds

### Use Cases

M4.1	The user changes the alert mode setting of the phone.

M4.2	The user changes the vibrator alarm mode of the phone.

M4.3	The user changes the ringing tone selection for the phone.

M4.4	The user changes the volume of the ringing tone.

M4.5	The user starts playing a sound on the phone.

M4.6	The user stops a sound that is played on the phone.

M4.7	The user reads a ringing tone from the phone.

M4.8	The user writes a ringing tone to the phone.

M4.9	The user uploads MP3 files for built-in MP3 players on the phone.

M4.9.1	MPM must know how much memory is available for MP3 files on the target drive before starting the upload.

M4.9.2	The music files are transferred to a standard directory that is used by the MP3 player application.

M4.10	The user converts one sound format to another.

### Interface

Like contacts and messages ringing tones are not necessarily accessible by the file system. The phone may prefer to keep the ringing tones in an internal repository on the SIM-card. For this reason reading and writing ringing tones is has a file independent interface based on AT commands.

Other sound files like MP3 music files for MP3 players on the phone are accessed by normal file transfer. File transfer is described in detail in the “Phone Explorer” chapter below.

AT Commands:

There are several commands here that are not in the original GSM command set. Siemens defined additional commands for

- Select a ringing tone and change its volume.

- Start and stop playing a sound file on the phone.

- Read and write ringing tones from and to the phone.

| Function | AT Commands | Comment |
| --- | --- | --- |
| 4.1 Change the alert mode (ring tone) | AT+CALM? This command is only used when the component starts up and not each time the use case is invoked. | The response of this command is one of the following values: 0 normal mode (ring tone on), 1 silent mode (ring tone off), and 2 beep |
|  | AT+CALM=<mode> | Set a new value for the alert mode <mode>. |
| 4.2 Change the vibrator alarm mode. | AT+CVIB? This command is only used when the component starts up and not each time the use case is invoked. | The response of this command is one of the following: 0 disabled, 1 enabled, and 16 first vibrate than ring |
|  | AT+CVIB=<mode> | <mode> values currently used by MPM are: 0: disabled, and 1: enabled. |
| 4.3 & 4.4 Change the ringing tone and its volume (CRMP commands) | AT+CRMP=<call type>, <volume> | Where <call type> is the index of the ringing tone in the phone repository and <volume> is its volume. Plays the specified ringing tone at the specified volume. |
|  | AT+CRMP | If an MTC arrives while the test ring is active, the latter is switched off and the "normal” ring is switched on. |
|  | AT+CSCS="GSM" | Select TE character set. |
|  | AT^SSET=<action>,<application>,<keys>, <full-name> | Where <action> is an integer value indicating the action to be performed, <application> an integer value indicating the application type (always set to 1), <keys> an integer value indicating the feature (e.g. incoming SMS ringer melody, background picture), and the <full-name> parameter should be given according to the character setting defined in command AT+CSCS. Example: “A:\Sounds\jump.mid” |
| 4.5 Start playing the ringing tone. | AT+CRMP=<call type>, <volume> | Where <call type> is the index of the ringing tone in the phone repository and <volume> is its volume. Plays the specified ringing tone at the specified volume. |
| 4.6 Stop playing the ringing tone. | AT+CRMP | Stops the ringing tone that is currently played |
| 4.7 Read a ringing tone | Ringing tones are read using OBEX Commands. |  |
| 4.8 Write a ringing tone | Ringing tones are written using OBEX Commands. |  |

Notes:

- The command sequences 4.1 and 4.2 always occur together.

See Siemens AT Commands  for a detailed description of each AT Command.

File transfer:

  - Sounds use methods of the standard file transfer (Phone Explorer) to read the free space and copy MP3 files to the standard sound folder on the phone. The phone should provide a standard folder from which MP3 files are loaded in MP3 player applications. The standard folder must not change.

M4.10	The user converts one sound format to another.

This functionality is implemented completely on the PC-Side therefore no communication with the mobile is necessary here.

## Photo Suite and Photos-To-PC

### Use Cases

M5.1		The user reads the phone settings (logo, screensaver, wallpaper).

M5.2		The user writes phone settings (logo, screensaver, wallpaper).

M5.3		The user uploads a JPEG image to the phone (e.g. in order to send it with a MMS client application).

M5.4		The user downloads an image from a standard picture folder on the phone (e.g. a photo made with the integrated camera, or a picture received via MMS).

### Interface

File handling is done with standard file transfer methods (e.g. OBEX as used in the Phone Explorer described below).

AT Commands:

| Function | AT Commands | Comment |
| --- | --- | --- |
| 5.1 Read wallpaper from phone | AT+CSCS="GSM" | Select TE character set. |
|  | AT^SSET=2,1,102 | Read the name of the logo picture file from the phone. |
|  | AT+CSCS="GSM" | Select TE character set. |
|  | AT^SSET=2,1,101 | Read the name of the screensaver picture file from the phone. |
|  | AT+CSCS="GSM" | Select TE character set. |
|  | AT^SSET=2,1,103 | Read the name of the wallpaper picture file from the phone. |
| 5.2 Write wallpaper to phone | AT+CSCS="GSM" | Select TE character set. |
|  | AT^SSET=0,1,102[,”<phone file path>”] | Set the name of the logo picture file to the supplied value. The value, if present, must be a valid path for a file on the phone. |
|  | AT+CSCS="GSM" | Select TE character set. |
|  | AT^SSET=0,1,101[,”<phone file path>”] | Set the name of the screensaver picture file to the supplied value. The value, if present, must be a valid path for a file on the phone. |
|  | AT+CSCS="GSM" | Select TE character set. |
|  | AT^SSET=0,1,103[,”<phone file path>”] | Set the name of the wallpaper picture file to the supplied value. The value, if present, must be a valid path for a file on the phone. |

See Siemens AT Commands  for a detailed description of each AT Command.

File transfer:

5.3 & 5.4

Photo Suite uses standard file transfer methods (cf. Phone Explorer) to read image files from the phone and write image files to the phone. The phone should provide a standard folder for application images. The standard folder must not change.

## WAP-Browser

### Use Cases

M6.1	The user exports WAP-Browser bookmarks to a bookmark list file and transfers this file to the phone. The WAP browser on the phone automatically imports the bookmark list.

M6.2	The user imports a bookmark list from the phone.

M6.3	The user opens a WAP bookmark on the PC to view the WAP page using an existing internet connection.

M6.3.1	The WAP browser on the PC must be able to display pages conforming to the current WAP standard.

M6.3.1	The user must be able to view WAP pages on the PC when behind a firewall. Proxy settings must be user-editable.

E6.3.2		In future versions the WAP browser must be able to use the proxy settings from Internet Explorer on user request.

E6.4	In future versions the user must be able to automatically synchronise the WAP bookmarks on the PC and the mobile phone.

### Interface

The Netscape-Bookmark-file format is an XML file containing a list of hyperlinks. Microsoft Internet Explorer provides export/import functions for that format.

Important: The WAP-Browser does not support nested lists. The structure of the list has to be level.

Example:

<!DOCTYPE NETSCAPE-Bookmark-file-1>

<!-- This is an automatically generated file.

It will be read and overwritten.

Do Not Edit! -->

<TITLE>Bookmarks</TITLE>

<H1>Bookmarks</H1>

<DL><p>

<DT><A HREF="http://www.my-siemens.com/MySiemens/CDA/Channel/WML/Index/1,1291,HQ_en_2_news%253A_0_15994_0_0,00.wml" ADD_DATE="1074006571" LAST_VISIT="1074176649" LAST_MODIFIED="1074006571">Siemens mobile (1)</A>

<DT><A HREF="http://wap.my-siemens.com/wap/uk/index.wml" ADD_DATE="1074001171" LAST_VISIT="1074176649" LAST_MODIFIED="1074001171">Siemens mobile</A>

</DL><p>

The phone has to save its bookmark list to a certain location on the phone. The location and the name of the bookmark file must not change. The WAP browser application on the phone loads the exchange file either automatically or by user request.

The standard file exchange interface (cf. Phone Explorer) is used to exchange the bookmark files between PC and phone.

AT Commands (for WAP Browser):

| Function | AT Commands | Comment |
| --- | --- | --- |
| 6.1 Export the bookmarks to the mobile | MPM uses OBEX to write the contents of “\Data\Misc\bookmark.htm” |  |
|  | AT^SBMH=1 | Import the bookmark list (presumably from \Data\Misc\bookmark.htm). |
| 6.2 Import the bookmarks from the mobile | AT^SBMH? | Export the bookmark list (presumably to \Data\Misc\bookmark.htm). |
|  | MPM uses OBEX to read the contents of “\Data\Misc\bookmark.htm” |  |

## Backup Restore

### Use Cases

M7.1	Select data for backup and restore

M7.1.1	The user can select from a list of available data for a connected phone to be included in a backup

M7.1.2	The user can select from a list of available data in a backup file to be restored on a connected phone.

M7.2	Address book

M7.2.1	The user creates a backup of the contents of the address book

M7.2.2	The user restores the address book

M7.3	Phone book

M7.3.1	The user creates a backup of the contents of the phone book

M7.3.2	The user restores the phone book

M7.4	Calendar (Appointments)

M7.4.1	The user creates a backup of the calendar data

M7.4.2	The user restores the calendar from backup.

M7.5	Phone directories

M 7.5.1	The user creates a backup of selected folders of the connected phone.

M 7.5.2	The user restores selected folders from a given backup to the connected phone.

M7.6	Phone Settings

M7.6.1	The user creates a backup of phone settings

M7.6.1	The user restores phone settings from backup

### Interface

7.2 & 7.3	MPM access address book with IrMC and file transfer.  Use AT commands for phone book access. The interface is identical to that described in the chapter “Contacts” above.

7.4	MPM use IrMC and file transfer to access calendar data. The approach is nearly identical to address book access with IrMC. For more information about VCAL and IrMC see the official IrMC specification from IrDA.

7.5 & 7.6	MPM uses simple file transfer for the phone directories. File access is described in detail in chapter “Phone Explorer” below.

AT Commands (for backup/restore settings):

| Function | AT Commands | Comment |
| --- | --- | --- |
| 7.1.1 | AT^SPBS=? | Test command. Used to get a list of all available phone book types. |
|  | AT^SPBS=<phonebook> | Select specific phone book type |
|  | AT+CPBR=? | Test Command. We expect the list of indices to be in the form: (<start>-<end>). [e.g. a typical response would be +CPBR: (1-100),20,17] |
|  | AT+CSCS="UCS2" | Set the character set to UCS2. |
|  | AT+CPBR=<index1>,<index2> | Read a telephone-book entry. The <index1> value identifies the first entry to be retrieved and the <index2> value identifies the last entry. |
|  | AT+CSCS="GSM" | Set the character set to GSM. |
| 7.1.2 | AT^SPBS=? | Test command, it inform all available phone book types |
|  | AT^SPBS=<phonebook> | Select specific phone book type |
|  | AT+CPBR=? | Test Command. We expect the list of indices to be in the form: (<start>-<end>). [e.g. a typical response would be +CPBR: (1-100),20,17] |
|  | AT+CSCS="UCS2" | Set the character set to UCS2. |
|  | AT+CPBR=<index1>,<index2> | Read a telephone-book entry. The <index1> value identifies the first entry to be retrieved and the <index2> value identifies the last entry. |
|  | AT+CSCS="GSM" | select character set “GSM” |
|  | AT^SPBS=? | Test command. Used to get a list of all available phone book types. |
|  | AT^SPBS=<phonebook> | Select specific phone book type |
|  | AT+CPBW=<index> | This command erases an entry from the previously selected phone book. |
|  | AT^SPBS=? | Test command. Used to get a list of all available phone book types. |
|  | AT^SPBS=<phonebook> | Select specific phone book type |
|  | AT+CSCS="UCS2" | Set the character set to UCS2. |
|  | AT+CPBW=<index>, <number>, <type>, <text> | This command writes an entry to the previously selected phone book. Where <index> is the entry index, <number> is the phone number, <type> is the type of entry and <text> is the text of entry. |
|  | AT+CSCS="GSM" | select character set “GSM” |
| 7.6.1 | AT+CSCS=”GSM” | select character set “GSM” |
|  | AT^SSET=2,1,<index><CR><NL> | Siemens specific command to get the profile settings in mobile. Where <index> refers to the setting that should be read. The response to this command contains the required data. |
| 7.6.2 | AT+CSCS=GSM | select character set “GSM” |
|  | AT^SSET=1,1,<index>,<setting>,<EOL> | Siemens specific command to set the profile settings in mobile. Where <index> refers to the setting that should be changed, and <setting> contains the data itself. |

See Siemens AT Commands  for a detailed description of each AT Command.

## SmartSync

### Use Cases

M8.1	Synchronise changed contacts between the phone and PC.

M8.1.1	Overwrite contacts on phone with contacts from PC.

M8.1.2	Overwrite contacts on PC with contacts on phone.

M8.1.3	Synchronise in both directions according to predefined rules for regulation conflicts.

M8.2	Synchronise changed appointments between the phone and PC.

M8.2.1	Overwrite appointments on phone with appointments from PC.

M8.2.2	Overwrite appointments on PC with appointments on phone.

M8.2.3	Synchronise in both directions according to predefined rules for regulation conflicts.

M8.3	Synchronise changed tasks between the phone and PC.

M8.3.1	Overwrite tasks on phone with tasks from PC.

M8.3.2	Overwrite tasks on PC with tasks on phone.

M8.3.3	Synchronise in both directions according to predefined rules for regulation conflicts.

M8.4	Synchronise changed notes between the phone and PC.

M8.4.1	Overwrite notes on phone with notes from PC.

M8.4.2	Overwrite notes on PC with notes on phone.

M8.4.3	Synchronise in both directions according to predefined rules for regulation conflicts.

M8.5	Synchronise changed emails between the phone and PC.

M8.5.1	Overwrite emails on phone with notes from PC.

M8.5.2	Overwrite emails on PC with notes on phone.

M8.5.3	Synchronise in both directions according to predefined rules for regulation conflicts.

### Interface

All use cases are handled using IrMC Levels 2 and 4.

## GPRS Modem Assistant

### Use Cases

M9.1	Modem assistant detects available phones

M9.2	Modem assistant sets up a connection and dial-in on user request

M9.2.1	The user must be able to select an operator / service provider from a list sorted by countries. Default selection should be the country set in Windows.

M9.2.2	The user must be able to use own dial-in parameters.

E9.2.3		In future versions the user must be able to modify existing dial-in parameters and save them as custom settings

E9.2.4		In future versions the Modem Assistant should be able to read out the currently selected operator / service provider from the phone and set the parameters accordingly

M9.3	Modem assistant hangs up the connection on user request.

M9.4	The user must be able to update the operator / service provider list by executing a separate "Settings Update" program

### Interface

The modem assistant is using AT Commands to setup the modem and establish a connection:

| Function | AT Commands | Comment |
| --- | --- | --- |
| 9.1 Phone detection | AT | Break previous commands |
|  | AT+CGMI | Retrieve the manufacturer of the Phone (must be SIEMENS) |
|  | AT+CGMM | Retrieve the name of the mobile |
| 9.2 Dial-in | AT&F | Reset all current parameters of AT commands to their factory settings. A list of commands affected by AT&F is given in the AT command specification |
|  | ATE0 | Switch off echoing characters |
|  | ATV1 | Display acknowledgements as text |
|  | ATD2 | Set the data terminal ready (DTR) behaviour |
|  | AT&C1 | Set the received line signal detector (DCD) behaviour |
|  | ATQ0 | Display acknowledgements |
|  | ATS0=0 | Write value 0 to S register 0 |
|  | AT+CGQREQ | Quality of Service Profile (Requested) |
|  | AT+CGDCONT | Define PDP context |
|  | AT^SGAUTH | Select type of Authentication for PPP connection |
|  | ATS7=60 | Write value 60 to S register 7 |
|  | AT\Q<n> | Local flow control selection (DTE, DCE) can be customised. <n> can take one of the following values: 0 Disable flow control 1: XON-XOFF software flow control 2: CTS only flow control 3: RTS/CTS flow control |
|  | ATD*99***1 | Establish a connection via service code 99 |
| 9.3 Hang-up | ATH | Release existing connection |
|  | AT+CHUP | Terminate all active calls and all calls on hold |

## Phone Explorer

### Use Cases

M10.1	The phone provides a file system with hierarchical tree structure. The user browse the file system with the windows explorer as common for storage mediums connected with the PC. The phone may use volumes to structure logical parts of memory on the same storage, e.g. a volume for user data, a volume for cache data etc.

M10.2	The explorer displays the attributes of an element in the phone file system and allows the user to change some of them.

Read only attributes of the phone are:

- File size (in bytes)

- File Modification date and time

- Folder creation time

- Volume name

- Volume total space

- Volume free space

The following attributes are read and writeable by the user

- File name

- Folder name

- File attributes (read-only, archive, hidden, system)

- Folder attributes (read-only, archive, hidden)

M10.3	The user deletes a file or folder.

M10.4	The user creates new subfolder.

M10.5	There are some copy & paste or drag & drop scenarios that the explorer provides to copy or move data from one storage to another or between locations in the same storage. For the phone these scenarios will result in eight main use cases:

M10.5.1	Phone files are copied to another location on the phone.

M10.5.2	Phone files are copied to a destination on the PC (on other storage mediums).

M10.5.3	Phone files are moved to another location on the phone.

M10.5.4	Phone files are moved to a destination on the PC (on other storage mediums).

M10.5.5	Phone folders are copied to another location on the phone.

M10.5.6	Phone folders are copied to a destination on the PC (on other storage mediums).

M10.5.7	Phone folders are moved to another location on the phone.

M10.5.8	Phone folders are moved to a destination on the PC (on other storage mediums).

M10.6	The user formats a volume for use with the file system.

M10.7	The user creates standard directories on a volume.

E10.8	When the user removes a removable medium from the phone (ejecting the multi media card) the change is immediately visible in the explorer.

### Interface

For file operations MPM’s data exchange layer uses the OBEX protocol. The IrOBEX protocol is slightly extended in order to support file operations that are not available in the original protocol specification.

The current implementation use OBEX over the following communication bearers:

- Infrared (MS TinyTP layer)

- Bluetooth (RFCOMM profile)

- USB (raw data transfer with hardware flow control over a serial line, baud rate 119 )

This chapter will give a very short introduction to OBEX and discuss how the use cases are realised with OBEX.

OBEX is a session layer protocol that although contains a model for representing objects. The purpose of the protocol is the exchange of objects and is based on a connection oriented, reliable transport layer.

Objects are a more general concept than files. While a file simply contains data, an object contains data plus a parse-able header with information about the representation of the data. Therefore OBEX can although be used to exchange sequences of controls and commands or to perform data base transactions. The use of headers in OBEX is similar to its use in HTTP.

The main OBEX-operations are CONNECT, PUT, GET, SETPATH, DISCONNECT and ABORT. CONNECT is used to start a session between both parties, disconnect to end the session. The party that send the connect request is the client, the party that accepts this request is the server.

While a session is running client and server are exchanging headers and data as follows:

The client sends a PUT-command to upload an object to the server. A PUT command starts with byte 0x02. This code indicates that all from now until the command 0x90 is sent belongs to the PUT-command.

The client continues with two bytes containing the packet length and a sequence of headers. The first headers may contain information describing the body of the object, followed by body headers containing the user data and an “End of Body” chunk. After sending the last byte of the request a frame starting with 0x90 indicates that the PUT request is finished now.

The server must acknowledge received intermediate packages with the sequence “0x90 0x0003” and must send the final response 0xA0 0x0003. (0x003 is the length of the header, by the way).

The client sends GET commands to download data from the server. The GET-request starts with 0x03 followed by the packet length and a sequence of headers. The server is sending a response code either 0x90 or 0xA0 (or a code indicating an error) followed by headers and user data (body headers). 0x90 indicates that the last byte was not sent yet. If it is not possible to transfer the object in a single GET, the client must repeat with GET requests until a 0xA0 Packet is received.

PUT and GET along with another command called SetPath can be used to implement a simple generic file transfer mechanism. To copy a file from the PC (client) to the mobile (server) the PC will call SetPath to define the destination path of the file as a “current directory” for the running session. The second step is to call the PUT command which contains some headers containing the file properties (name, size, access permissions, last modified time and date) and the file content (the body). The reverse process, copying data from the phone to the PC, is implemented in a similar way, of course using the SetPath command to specify the source path along with the GET command to copy the data.

Siemens OBEX extend the standard OBEX commands by specifying a so called application header with the CONNECT command.

A detailed description of standard OBEX can be found in the IrDA OBEX specification. Details on Siemens additions to the OBEX protocol are described here: DES OBEX specification.

The following table should help to find related topics in either the IrDA (Standard) or Siemens specifications.

| Function | key words in specification | specification |
| --- | --- | --- |
| List files contained in a folder | Folder-listing object | Standard |
| List subfolders | Folder-listing object | Standard |
| Read the properties of a file or folder (attribute, size, date) | Folder-listing object | Standard |
| Get the properties of a volume (free space, capacity) | Get - MMCAttrib | Siemens |
| Change the file attribute of a file or folder | Set Attribute, Attributes | Siemens |
| Write a file to the phone. Copy a file from PC to phone. | Generic File Object | Standard |
| Read a file. Copy a file from phone to PC. | Generic File Object | Standard |
| Create a new folder | Create-Empty | Standard |
| Remove a file. | Put-Delete | Standard |
| Remove an empty folder. | Put-Delete | Standard |
| Format a volume | Format | Siemens |
| copy phone files to other locations on the same phone | Move, Copy | Siemens |
| move phone files to other locations on the same phone | Move, Copy | Siemens |
| move folders to other locations on the same phone | Changing Folders | Siemens |
| Rename files | Move, Copy | Siemens |
| Rename folders | Move, Copy | Siemens |
| Create standard folders for a volume | Create Std Dir | Siemens |

The use cases enumerated in the first section of this chapter are realised as follows:

10.1	Retrieving the contents of folders is a standard OBEX feature. See the IrDA OBEX specification for details. The file system of the phone should be similar to the common PC file systems. The following rules have to be observed:

  - a  file name must not contain one of the following special characters

? * \ " < > | /

  - The following names are reserved and must not be valid:

CON, PRN, AUX, CLOCK$, NUL, COM1, COM2, COM3, COM4, COM5,

COM6, COM7, COM8, COM9, LPT1, LPT2, LPT3, LPT4, LPT5, LPT6, LPT7, LPT8, LPT9

  - Paths should use the “\” character as delimiter.

  - Filenames are stored and shown case-sensitive but used case-insensitive. Therefore it is not allowed to have two files or folders in a folder whose name only differs in cases.

  - The maximum length for a file or folder name is 256 wide characters

  - Only the root of the file system should contain volumes. It is recommended to structure the first level of the file system hierarchy with volumes only. This approach is used for all phones of the 65 platform. Volumes are not erasable but may be physically removable (e.g. multi-media cards).

10.2	Handling properties of file system items is mainly performed with standard OBEX except the following:

10.2.4 & 10.2.5	Getting volume properties capacity and total space requires application header “MMCAttrib” with GET.

10.2.6 & 10.2.7	Reading file names and folder names are standard OBEX. See the IrDA OBEX specification for details. Changing the name (rename) requires Siemens application header “Move” along with a PUT command.

10.2.8	Reading file and folder attributes is standard OBEX. Changing the attribute requires the Siemens application header “Set Attribute” along with a PUT command.

10.3	Deleting a file or an empty folder is standard OBEX (Put command on a file / folder without body). See the IrDA OBEX specification for details. The attempt to delete a file with read-only attribute must fail. The attempt to delete a folder that is not empty must fail. Deleting files and / or folders recursively or changing the attributes before deleting is not the responsibility of the phone.

10.4	Creating an empty subfolder is standard OBEX. See the IrDA OBEX specification for details.

10.5.1	Copying a file within the phone requires the Siemens application header “Copy”.

10.5.2	Copying a file to the PC is a Standard OBEX operation (GET command, see introduction above). See the IrDA OBEX specification for details.

10.5.3	Moving a file within the phone requires the Siemens application header “Move”.

10.5.4	Moving a file to PC can be done with standard OBEX (Get command to transfer the file in sequence with an “Empty-Body-PUT” operation to delete the original file on the phone).

10.5.5	Now additional interface required. Copying a folder within the phone is a sequence of operations described above: Get the contents of the folder (see 10.1). Create destination directories (see 10.4), copy any file to the new location (see 10.6.1).

10.5.6	No additional interface required. Copying a folder to the PC is a sequence of operations described above. Get the contents of the folder (see 10.1) and copy any single file (see 10.5.2).

10.5.7	No additional interface required. Moving a folder within the phone is a sequence of operations described above. Get the contents of the folder (see 10.1), move any single file (see 10.5.3). Remove the empty target directory after moving its contents (see 10.3).

10.5.8	No additional interface required. Moving a folder to the pc is a sequence of operations described above. Get the contents of the folder (see 10.1), move any single file (see 10.5.4), remove the empty target directories and files after moving its contents.

10.6	Formatting a volume requires the Siemens application header “Format”.

10.7	Creating standard directories requires the Siemens application header “Create Std Dirs”

# AT Command List

The texts “Test”, “Read”, “Write”, and “Execute” in the usage column refer to the formats “Test command”, “Read command”, “Write command”, and “Execute command” in the Siemens AT documentation. Unless otherwise stated all possible values described there are required.

If the text in “Name” column is in italics then this command is not yet described in the Siemens AT documentation and the “Name” is guessed from our usage.

Note: Since we do not have access to the code this table does not contain usage comments about commands used exclusively by the GPRS Modem Assistant.

| Command | Name | Usage |
| --- | --- | --- |
| +CALM | Alert sound mode | Test, Read, and Write. |
| +CBC | Battery charge | Execute |
| +CCLK | Clock | Read and Write. This is not currently used but is required for a future component. |
| +CGDCONT | PDP context Modify | Used by the GPRS Modem Assistant. |
| +CGMI | Issue manufacturer ID code | Execute. |
| +CGMM | Issue model ID code | Execute. |
| +CGMR | Output the GSM telephone version | Execute. |
| +CGQREQ | Quality of Service Profile (Requested) | Used by the GPRS Modem Assistant. |
| +CGSN | Output the serial number (IMEI) | Execute. |
| +CHUP | Hang up call | Used by the GPRS Modem Assistant. |
| +CMEE | Expanded error messages according to 3GPP TS 27.007 | Write. |
| +CMGD | Delete an SMS in the SMS memory | Write. |
| +CMGF | SMS format | Write. |
| +CMGL | List SMS [Sets SMS Status to read] | Write. See ^SMGL. |
| +CMGR | Read in an SMS | Write. |
| +CMGS | Send an SMS | Write. |
| +CMGW | Write an SMS to the SMS memory | Write. |
| +CMSS | Send an SMS from the SMS memory | Write. The current implementation of the Messenger does not use this command but it is required for a future enhancement to the Messenger. |
| +COPS | Commands concerning selection of network operator | Read and Write. This is currently not used but is required for future enhancements. |
| +CPBR | Read a telephone-book entry | Test and Write. |
| +CPBS | Select a telephone book | Test and Write. |
| +CPBW | Write a telephone-book entry | Write. |
| +CPIN | Enter PIN and query lock | Write. |
| +CPMS | Preferred SMS message storage | Test, Read, and Write. |
| +CPWD | Change password to a lock | Test and Write (P2) |
| +CRMP | Ring Melody Playback | Test, Write, and Execute. |
| +CSCA | Address of the SMS service centre | Read and Write. |
| +CSCS | Select TE character set | Write (UCS2 and GSM). |
| +CSQ | Output signal quality | Execute. |
| +CVIB | Vibrator mode | Read (missing in documentation) and Write. |
| +IPR | Fixed DTE rate | Write (115200). |
| &C1 |  | Used by the GPRS Modem Assistant. |
| &F |  | Used by the GPRS Modem Assistant. |
| \Q<n> |  | Used by the GPRS Modem Assistant. |
| D2 |  | Used by the GPRS Modem Assistant. |
| D*99***1 |  | Used by the GPRS Modem Assistant. |
| E0 | Echo off |  |
| E1 | Echo on |  |
| H |  | Used by the GPRS Modem Assistant. |
| Q0 |  | Used by the GPRS Modem Assistant. |
| S0=0 | Automatic answer (OFF). | Used by the GPRS Modem Assistant. |
| S7=60 | Wait for carrier after dialing (in seconds). | Used by the GPRS Modem Assistant. |
| V1 | Display acknowledgments as text | Used by the GPRS Modem Assistant. |
| ^SBMH | Bookmark Handling | Read and Write (action = 1, without folder parameter). |
| ^SGAUTH | Select Type of Authentication for PPP connection | Used by the GPRS Modem Assistant. |
| ^SLNG | Language Settings | Test and Read. Not currently used but it is planned to use this. |
| ^SMGL | List SMS (without status change from unread to read) | Write. |
| ^SMMS | Multimedia Message Service Handling See description below. | Read, Write, and Test. Intended for use in MMS handling but current Siemens mobiles do not yet support this. |
| ^SPBS | Select a telephone book (including Siemens-specific books) | Test and Write. |
| ^SQWE | Switch Mode for External Interface | Write (0, 3 and 4). We use 0 followed by 3 to change to OBEX mode. |
| ^SSET | Profile Settings Control (SET Melody and Picture settings in Mobile ) | Write. |

The following commands are implemented by Siemens phones but will not be used by MPM with newer phones. There is therefore no requirement from MPM for an ODM to implement these commands.

| Command | Name | Usage |
| --- | --- | --- |
| +CIND | Indicator Control | Test and Read. |
| ^SBNR | Binary Read | Test and Write (bmp, mid, DIR, and vcs). “vcs” does not appear in the latest AT commands document. It is used for reading appointments. |
| ^SBNW | Binary Write | Write (bmp, mid, and vcs). |
| ^SPIC | Output PIN counter | Execute. |

## DIR values for AT^SBNR command on a Siemens S65.

Command Format: AT^SBNR="DIR",X

The return value depends on the value of ‘X’ (see table below) and consists of two parts. The second part contains the directory name as an ucs2-string

Example: X=1 returns "\Data\Sounds" as follows:

Part 1: ^SBNR="dir",1,1,1

Part 2: "0C005C0044006100740061005C0053006F0075006E0064007300"

X value not recognised returns: "+CME ERROR: 21"

| X value | Name | Directory name on S65 |
| --- | --- | --- |
| 1 | OBJECT_FOLDER_RINGINGTONE | \Data\Sounds |
| 2 | OBJECT_FOLDER_RINGINGTONE_EMS | \Data\Sounds\EMS |
| 3 | OBJECT_FOLDER_RINGINGTONE_ONOFF | \Data\Sounds\On Off |
| 4 | OBJECT_FOLDER_RINGINGTONE_SLIDE | \Data\Sounds\ProSlide |
| 5 | OBJECT_FOLDER_PICTURES | \Data\Pictures |
| 6 | OBJECT_FOLDER_PICTURES_EMS | \Data\Pictures\EMS |
| 7 | OBJECT_FOLDER_PICTURES_LOGOS | \Data\Pictures\Logo |
| 8 | OBJECT_FOLDER_PICTURES_WALLPAPER | \Data\Pictures\Wallpaper |
| 9 | OBJECT_FOLDER_PICTURES_MMS | \Data\Picture\MMS |
| 10 | OBJECT_FOLDER_VIDEOS | \Data\Videos |
| 11 | OBJECT_FOLDER_ANIMATION | \Data\Animations |
| 12 | OBJECT_FOLDER_COLOURSKINS | \Data\Skins |
| 13 | OBJECT_FOLDER_JAM | \Data\Java\Jam |
| 14 | OBJECT_FOLDER_JAVA_JAM_APPLICATIONS | \Data\Java\Jam\Applications |
| 15 | OBJECT_FOLDER_JAVA_JAM_GAMES | \Data\Java\Jam\Games |
| 16 | OBJECT_FOLDER_THEMES | \Data\Themes |
| 17 | OBJECT_FOLDER_AVATAR | \Data\PhonePilots |
| 18 | OBJECT_FOLDER_SMS_ARCHIVE | \Data\Sms archive |
| 19 | OBJECT_FOLDER_SMS_ARCHIVE_INBOX | \Data\Sms archive\Inbox |
| 20 | OBJECT_FOLDER_SMS_ARCHIVE_SENT | \Data\Sms archive\Sent |
| 21 | OBJECT_FOLDER_SMS_ARCHIVE_UNSENT | \Data\Sms archive\Unsent |
| 22 | OBJECT_FOLDER_SMS_ARCHIVE_DRAFT | \Data\Sms archive\Draft |
| 23 | OBJECT_FOLDER_CAMERA | \Data\Pictures\Clip Gallery |
| 24 | OBJECT_FOLDER_MMS | \Data\System\mms |
| 25 | OBJECT_FOLDER_SYSTEM_EMAIL | \Data\System\email |
| 26 | OBJECT_FOLDER_MMC1_PICTURES | \MMCard\Pictures |
| 27 | OBJECT_FOLDER_MMC1_VIDEOS | \MMCard\Videos |
| 28 | OBJECT_FOLDER_INBOX | \Data\Misc\Data inbox |
| 29 | OBJECT_FOLDER_SYSTEM_SMS | \Data\System\SMS |
| 30 | OBJECT_FOLDER_SYSTEM_WAP | \Data\System\wap |

## AT^SLNG Language values returned by a Siemens S65

| Value | Language |
| --- | --- |
| 0 | German |
| 1 | English |
| 2 | Italian |
| 3 | French |
| 4 | Spanish |
| 8 | Portuguese |
| 11 | Greek |
| 12 | Turkish |
| 106 | Catalan |
| 107 | Portuguese (Brazil) |

## AT^SPBS phone book values

| Value | Description |
| --- | --- |
| BL | Blacklist dialling numbers (barred numbers from remote) |
| CD | Call-back dialling numbers (answered calls) |
| CS | Common sortable telephone book (sorted combination of “SM”, ”ME”, ”FD”; access only via ^SPBC, ^SPBG) |
| FD | SIM fix-dialling telephone book |
| LD | SIM last dialling number |
| MC | ME Missed Calls List |
| MS | Missed call numbers (unanswered calls) |
| ON | Own telephone numbers |
| OW | Own numbers |
| RC | ME Received Calls List |

## AT^SSET parameter values

The command is defined as:

AT^SSET=<action>,<application>,<keys>, <full-name>

action:

| Delete | 0 |
| --- | --- |
| Set | 1 |
| Get | 2 |

application:

always = 1

keys

| RingToneOtherCallsLine1 | 1, ///< Index 1: Unknown incoming call (for primary number) |
| --- | --- |
| RingToneOtherCallsLine2 | 2, ///< Index 2: Unknown incoming call for secondary number (E-Plus only) |
| RingTonePhoneBookCalls | 3, ///< Index 3: Incoming call from phonebook (without group) |
| RingToneGroupOne | 4, ///< Index 4: Incoming call from group 1 |
| RingToneGroupTwo | 5, ///< Index 5: Incoming call from group 2 |
| RingToneGroupThree | 6, ///< Index 6: Incoming call from group 3 |
| RingToneGroupFour | 7, ///< Index 7: Incoming call from group 4 |
| RingToneGroupFive | 8, ///< Index 8: Incoming call from group 5 |
| RingToneGroupSix | 9, ///< Index 9: Incoming call from group 6 |
| RingToneGroupSeven | 10, ///< Index 10: Incoming call from group 7 |
| RingToneGroupEight | 11, ///< Index 11: Incoming call from group 8 |
| RingToneGroupNine | 12, ///< Index 12: Incoming call from group 9 |
| RingToneOrganizer | 13, ///< Index 13: Event for organiser |
| RingToneAlarm_Clock | 14, ///< Index 14: Event for Alarm clock |
| RingToneMessage | 15, ///< Index 15: Arriving Message |
| RingToneSwitch_On_Melody | 16, ///< Index 16: switch on melody |
| RingToneSwitch_Off_Melody | 17, ///< Index 17: switch off melody |
| RingToneOpen_ProSlide | 18, ///< Index 18: slider open melody |
| RingToneClose_ProSlide | 19, ///< Index 19: slider close melody |
| RingTonePushToTalk | 20 ///< Index 20: Push to talk Over Cellular ring tone |
| ScreenSaver | 101, |
| ProviderLogo | 102, |
| WallPaper | 103, |
| StartUpAnimation | 104, |
| ShutDownAnimation | 105 |

## AT+CRMP call type values

For 65 Generation phones

| RingToneOtherCallsLine1 | 1, ///< Index 1: Unknown incoming call (for primary number) |
| --- | --- |
| RingToneOtherCallsLine2 | 2, ///< Index 2: Unknown incoming call for secondary number (E-Plus only) |
| RingTonePhoneBookCalls | 3, ///< Index 3: Incoming call from phonebook (without group) |
| RingToneGroupOne | 4, ///< Index 4: Incoming call from group 1 |
| RingToneGroupTwo | 5, ///< Index 5: Incoming call from group 2 |
| RingToneGroupThree | 6, ///< Index 6: Incoming call from group 3 |
| RingToneGroupFour | 7, ///< Index 7: Incoming call from group 4 |
| RingToneGroupFive | 8, ///< Index 8: Incoming call from group 5 |
| RingToneGroupSix | 9, ///< Index 9: Incoming call from group 6 |
| RingToneGroupSeven | 10, ///< Index 10: Incoming call from group 7 |
| RingToneGroupEight | 11, ///< Index 11: Incoming call from group 8 |
| RingToneGroupNine | 12, ///< Index 12: Incoming call from group 9 |
| RingToneOrganizer | 13, ///< Index 13: Event for organiser |
| RingToneAlarm_Clock | 14, ///< Index 14: Event for Alarm clock |
| RingToneMessage | 15, ///< Index 15: Arriving Message |
| RingToneSwitch_On_Melody | 16, ///< Index 16: switch on melody |
| RingToneSwitch_Off_Melody | 17, ///< Index 17: switch off melody |
| RingToneOpen_ProSlide | 18, ///< Index 18: slider open melody |
| RingToneClose_ProSlide | 19, ///< Index 19: slider close melody |
| RingTonePushToTalk | 20 ///< Index 20: Push to talk Over Cellular ring tone |

# Appendix A: Multimedia Message Service Handling

This will be included in future versions of the Siemens AT documentation, at which time it will be removed from here.

The syntax:

| AT^SMMS | Multimedia Message Service Handling |
| --- | --- |
| Write command AT^SMMS=<handle>[,<action>][,<message_Id>] |  |
| Read command AT^SMMS? | ^SMMS: <status> |
| Test command AT^SMMS=? | ^SMMS: (list of supported <handle>s) |

Description

This command allows an external application to access to multimedia messages.

For reading and sending a MMS it is necessary to send this command more than once using different parameter. Refer to example and use cases for more details.

Important note: If an interface is registered all other interface are not allowed to use any AT-Command concerning MM-handling. Furthermore the registration is the 1st step of all actions for MM-handling. That means that all other MMS-AT-Commands will issue an error if the interface used is not registered.

Defined values

<Handle>:

- End the MMS handling, the MMS storage is unlocked and the interface deregistered at MMS-client

- Initialise MMS handling, the MMS is locked to the current interface * and the interface register to the mobile MMS client.

- The pending state.
The following <action>s are allowed for this <handle>:  
0	request the mobile MMS-client to resume from pending state.
1	request the mobile MMS-client to go to pending state
2	query the mobile MMS-client state
		the query <action> returns one of the following values:
		1	the mobile MMS-client is started so it can be set pending state
		2	the mobile MMS-client is in pending state
		3	the mobile MMS-client is unknown/undefined

- query import/export path, which will return the path of the file system where to read/write imported/exported messages in character set specified by AT+CSCS.
The following <action>s are allowed for this <handle>:  
0	query the data path of mobile MMS client 
1	query the import path
2	query the export path

- Import message
The following <action>s are allowed for this <handle>:  
0	initialise the import of a message 
1	start the import of a message
2	stop import

- Export message
The following <action>s are allowed for this <handle>:  
0	initialise the export of a message 
1	start the export of a message, third parameter <message_Id> is required
2	stop export

- send MM, third parameter <message_Id> is required

- set a MMS to read or unread; third parameter <message_Id> is required
The following <action>s are allowed for this <handle>:  
0	set MMS defined by <message_Id> to unread 
1	set MMS defined by <message_Id> to read

- delete a MM from storage, third parameter <message_Id> is required

<Status>:

0	the MMS command is available

1	the MMS command is locked by this interface

2	the MMS storage is locked by another interface/application and not available

### Test Option

The test option returns the list of implemented/supported <handle>s and it’s <action>s of the write option.

### Read Option

The test option returns the state of availability of the MMS-command.

The command is available only for one interface at the same time. Furthermore the MMS-client may in use by another internal mobile application.

<Status>:

0	the MMS command is available

1	the MMS command is locked by this interface

2	the MMS storage is locked by another interface/application and is not available

### Write Option

The write option invokes a <handling> according parameter 1.

For several <handling>s (send, delete, import, export, etc.) the second parameter <action> and third <message_Id> is required.

* After initialising a MMS sequence the MMS storage is kept in “lock” state for 3 minutes.

If there is no MMS action within these 3 minutes the MMS storage is unlocked by a watchdog timer. The watchdog timer is restarted each time a new MMS action is invoked.

### Unsolicited Result Codes

None

## Use cases

General note: 
Each MM handling (excluding the query of the import/export path and the query of the MMS-client state has to be enclosed start and stop commands. Refer to the following command sequence:

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

AT^SMMS=0		// stop the MMS sequence, unlock the interface and deregister from MMS client

This commands sequence does not lock/unlock the UI or incoming message handler. To do this it is necessary to set the client into ‘pending state’ using the AT^SMMS=2,1 command. This will only be done by MPM when it is reading all messages. This is to stop anything from changing during the read.

A “smaller” lock occurs when exporting (i.e. reading from the phone) a message. In this case only the message (i.e. the directory containing the message’s files) will be locked but the client can do as it wishes with other messages whilst this lock is set. This lock is set when MPM sends the AT^SMMS=5,1,ID command which identifies the message and is reset by the following AT^SMMS=5,2 when MPM is finished.

### Query the import/export path

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

OK

AT^SMMS=3,0	// query MMS client data path

^SMMS:”path string in ACSII or UCS2 according to setting of AT+CSCS command”

OK

AT^SMMS=3,1	// query import path

^SMMS:”path string in ACSII or UCS2 according to setting of AT+CSCS command”

OK

AT^SMMS=3,2	// query export path

^SMMS:”path string in ACSII or UCS2 according to setting of AT+CSCS command”

OK

AT^SMMS=0		// end MMS sequence

OK

A potential example for how the phone directory structure in this area could appear is shown in the diagram below. In this directory structure the Data and Export paths contain one sub-directory for each message in that area. This sub-directory contains all files associated with the message.

There is no reason, from MPM’s point of view, why the Data and Export paths should not be the same. We know the message ID when accessing these areas (or can find all of them by listing the ‘path’ contents) and so we can access the appropriate directory. From the phone’s point of view it might be an advantage to have these paths the same since it will save moving and duplicating potentially large amounts of data.

In the case of the Import Path we do not know the message ID until after all files are copied onto the phone. However there is nothing to stop a phone implementing the Import path as a sub-directory of the Data path. We copy our files into it and when the Import is finished the sub-directory is renamed to the ID returned to us. The Import path sub-directory can then be created again (at this point or when the Import Init is received).

Figure 1 Message file-store structure

### Read all MMS messages from phone

including “Go and resume from pending state”

The purpose of the pending state is to be used in the Read all MMS messages from the phone Use Case.

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

OK

AT^SMMS=2,1	// request to go pending

OK

// AT^SQWE=0 might be sent but may be ignored (i.e. just respond OK)

AT^SQWE=3

OK

// the connection is now in OBEX mode

Read the MMS-Files from the data path of mobile MMS client (presumes that AT^SMMS=3,0 has been previously used to obtain path).

We expect to find a series of sub-directories under the Data Path (read using AT^SMMS=3.0). Each sub-directory should contain the files for one MMS. These sub-directory names should be the same as the message ID for the MMS.

This gives the following sequence:

- list contents of Data Path directory (this gives us list of message IDs), and

- for each sub-directory:

  - list the contents of the directory,

  - read all of the files,

  - if the sub-directory has further sub-directories then traverse them also, reading their contents, and

  - identify the SMIL file (the only one with extension .smil?).

<1 second inactivity on connection>+++<1 second inactivity on connection>

// the connection is now back in AT mode

AT^SMMS=2,0	// resume from pending

OK

AT^SMMS=0		// end MMS sequence

OK

Figure 2 Sequence Diagram for reading all MMS

### Query the pending state

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

OK

AT^SMMS=2,2	// query pending state of mobile MMS client

^SMMS: 1

OK

AT^SMMS=0		// end MMS sequence

OK

Figure 3 Sequence Diagram for querying pending state

### Delete a message

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

OK

AT^SMMS=8,,x	// where ‘x’ is the message ID

OK

AT^SMMS=0		// end MMS sequence

OK

Figure 4 Sequence Diagram for deleting a message

### Create new message and import to phone

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

OK

AT^SMMS=4,0	// init message import

OK

// AT^SQWE=0 might be sent but may be ignored (i.e. just respond OK)

AT^SQWE=3

OK

// the connection is now in OBEX mode

Copy the message (i.e. the SMIL and associated files) into mobile storage’s import path (via OBEX) (presumes that AT^SMMS=3,1 has been previously used to obtain path).

Does the AT^SMMS=4,0 make sure that the import path directory is empty?

<1 second inactivity on connection>+++<1 second inactivity on connection>

// the connection is now back in AT mode

AT^SMMS=4,1	// start message import, where ‘x’ is the message ID

x OK

AT^SMMS=4,2	// stop the message import

OK

AT^SMMS=0		// end MMS sequence

OK

Figure 5 Sequence Diagram for importing message to phone

### Create new message, import and send it

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

OK

AT^SMMS=4,0	// init message import

OK

// AT^SQWE=0 might be sent but may be ignored (i.e. just respond OK)

AT^SQWE=3

OK

// the connection is now in OBEX mode

Copy the message (i.e. the SMIL and associated files) into mobile storage’s import path (via OBEX) (presumes that AT^SMMS=3,1 has been previously used to obtain path).

Does the AT^SMMS=4,0 make sure that the import path directory is empty?

<1 second inactivity on connection>+++<1 second inactivity on connection>

// the connection is now back in AT mode

AT^SMMS=4,1	// start message import, where ‘x’ is the message ID

x OK

AT^SMMS=4,2	// stop the message import

OK

AT^SMMS=6,,x	// send message, where ‘x’ is the message ID

OK

AT^SMMS=0		// end MMS sequence

OK

Figure 6 Sequence Diagram for sending MMS from PC

### Read a single message (export message)

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

OK

AT^SMMS=5,0	// init message export

OK

AT^SMMS=5,1,x	// start message export, where ‘x’ is the message ID; All files for the MMS are exported to a sub-directory of the “export Path” with the name ‘x’.

OK

// AT^SQWE=0 might be sent but may be ignored (i.e. just respond OK)

AT^SQWE=3

OK

// the connection is now in OBEX mode

Copy message from mobile storage’s export path (via OBEX) (presumes that AT^SMMS=3,2 has been previously used to obtain path).

We expect that the files associated with the message will be stored in a sub-directory of the ‘export path’ with the same name as the message ID.

<1 second inactivity on connection>+++<1 second inactivity on connection>

// the connection is now back in AT mode

// it is possible to repeat the sequence from AT^SMMS=5,1,x to read other messages here

AT^SMMS=5,2	// stop the message export

OK

AT^SMMS=0		// end MMS sequence

OK

Figure 7 Sequence Diagram for reading a single MMS

### Send a message which is already on the phone.

AT^SMMS=1		// start the MMS sequence, lock the interface and register to mobile MMS client

OK

AT^SMMS=6,x	// send message, where ‘x’ is the message ID.

OK

AT^SMMS=0		// end MMS sequence

OK

Figure 8 Sequence Diagram for sending MMS from phone

# Appendix B: AT Commands which are not yet in the official document

Commands listed here will be included in future versions of the Siemens AT documentation, at which time they will be removed from here.

## Get Device Info AT Command

AT^SGDI

| AT^SGDI | Get Device Info |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test command | Response: |  |  |  |  |
| AT^SGDI=? | ^SGDI: (list of supported <device>s) OK/ERROR/+CME ERROR |  |  |  |  |
|  | <device> | Name of the device |  |  |  |
|  |  | BBERRY |  |  | Blackberry |
| Write command | Response: |  |  |  |  |
| AT^SGDI= <device> | OK/ERROR/+CME ERROR |  |  |  |  |
|  | Parameter: |  |  |  |  |
|  | <device> |  | See Test command |  |  |

# Appendix C: Sequence Diagrams / Call Flows

## IrDA connection

This diagram shows the polling sequence via IrDA:IrCOMM to see if the phone is (still) connected and the command sequence when the user performs an action which requires OBEX functionality.

Not shown here are:

- the (less frequent) polling using AT+CBC and AT+CSQ to get the battery and signal strength status, and

- other usage of the OBEX functionality (e.g. Backup/Restore, or Contacts)

Note:

It is proposed in the (near) future to no longer use a separate connection for OBEX but rather to switch on the IrDA:IrCOMM connection between AT and OBEX as done for a Serial Cable connection. In this implementation the socket will be opened when the phone connects and closed when it disconnects.

## Serial Cable connection

- PhoneObserver is part of MPM and watches for phones connecting via USB, Serial Port, Bluetooth or IrDA.

- ACTSer runs in a thread in PhoneObserver and watches for activity on all serial ports.

- PhoneServer is a part of MPM which interfaces between the MPM Components and a phone. The phone specific information is hidden by the PhoneServer from the other MPM Components.

- It is important to note that a phone which is to be connected via a serial port MUST support baud rates of 19200 and one of  921600, 460800, 230400, 115200, 57600, 38400, and 19200. 
i.e. the phone must support at least two baud rates (unless it only wishes to only support 19200).

Control Block for SetCommState

dcb.DCBlength = sizeof(DCB);

dcb.BaudRate = nBaudRate;

dcb.fBinary = TRUE;

dcb.fParity = FALSE;

dcb.fOutxCtsFlow = FALSE;

dcb.fOutxDsrFlow = FALSE;

dcb.fDtrControl = DTR_CONTROL_DISABLE;

dcb.fDsrSensitivity = FALSE;

dcb.fTXContinueOnXoff = TRUE;

dcb.fOutX =FALSE;

dcb.fInX = FALSE;

dcb.fErrorChar = FALSE;

dcb.fNull = FALSE;

dcb.fRtsControl = RTS_CONTROL_ENABLE;

dcb.fAbortOnError = FALSE;

dcb.XonLim = 0;

dcb.XoffLim = 0;

dcb.ByteSize = 8;

dcb.Parity = NOPARITY;

dcb.StopBits = ONESTOPBIT;

dcb.XonChar = 0;

dcb.XoffChar = 0;

dcb.ErrorChar = 0;

dcb.EofChar = -1;

dcb.EvtChar = 0;

## USB connection

To be supplied.

## BlueTooth connection

To be supplied.

# Appendix D: Check lists

These checklists need to be updated to reflect changes in the document since version 0.3.

## Checklist 1: Use Cases

| Module | Function | Requirements |  | Meet Requirement |  |  | Remarks (e.g. variations from default |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Ext | Full | Partly | Not | implementation) |
| Framework | Get Manufacturer |  |  |  |  |  |  |
| Framework | Get Phone Model |  |  |  |  |  |  |
| Framework | Get Batter charge |  |  |  |  |  |  |
| Framework | Get Signal quality |  |  |  |  |  |  |
| Framework | Get IMEI |  |  |  |  |  |  |
| Framework | Get battery charge |  |  |  |  |  |  |
| Contacts | Read phone book contact |  |  |  |  |  |  |
| Contacts | Read address book contact |  |  |  |  |  |  |
| Contacts | Write phone book contact |  |  |  |  |  |  |
| Contacts | Write address book contact |  |  |  |  |  |  |
| Contacts | Delete phone book contact |  |  |  |  |  |  |
| Contacts | Delete address book contact |  |  |  |  |  |  |
| Messenger | Read message |  |  |  |  |  |  |
| Messenger | Store message |  |  |  |  |  |  |
| Messenger | Store and send message immediately |  |  |  |  |  |  |
| Messenger | Send stored message |  |  |  |  |  |  |
| Messenger | Delete message |  |  |  |  |  |  |
| Messenger | Set the read flag for a inbox message |  |  |  |  |  |  |
| Messenger | Get the service centre address |  |  |  |  |  |  |
| Messenger | Set the service centre address |  |  |  |  |  |  |
| Messenger | Be informed on incoming messages |  |  |  |  |  |  |
| Sound mgr | Get alert mode |  |  |  |  |  |  |
| Sound mgr | Set alert mode |  |  |  |  |  |  |
| Sound mgr | Get vibrator mode |  |  |  |  |  |  |
| Sound mgr | Set vibrator mode |  |  |  |  |  |  |
| Sound mgr | Select ringing tone |  |  |  |  |  |  |
| Sound mgr | Get selected Ringing tone |  |  |  |  |  |  |
| Sound mgr | Get ringing tone volume |  |  |  |  |  |  |
| Sound mgr | Set ringing tone volume |  |  |  |  |  |  |
| Sound mgr | Start playing ringing tone |  |  |  |  |  |  |
| Sound mgr | Stop playing ringing tone |  |  |  |  |  |  |
| Sound mgr | Read ringing tone |  |  |  |  |  |  |
| Sound mgr | Write ringing tone |  |  |  |  |  |  |
| Sound mgr | File transfer for sound files |  |  |  |  |  |  |
| Photo suite | read bitmap |  |  |  |  |  |  |
| Photo suite | write bitmap |  |  |  |  |  |  |
| Photo suite | File transfer for photos |  |  |  |  |  |  |
| BkupRestore | Phone book |  |  |  |  |  |  |
| BkupRestore | Address book |  |  |  |  |  |  |
| BkupRestore | Appointments |  |  |  |  |  |  |
| BkupRestore | Files and Folders |  |  |  |  |  |  |
| WAP | Import bookmarks |  |  |  |  |  |  |
| WAP | Export bookmarks |  |  |  |  |  |  |
| GPRS MA | Phone detection |  |  |  |  |  |  |
| GPRS MA | Connect/Disconnect |  |  |  |  |  |  |
| Phone Explorer | List the contents of folders |  |  |  |  |  |  |
| Phone Explorer | Retrieve properties of files and folders |  |  |  |  |  |  |
| Phone Explorer | Retrieve volume properties |  |  |  |  |  |  |
| Phone Explorer | Set attributes and rename files and folders |  |  |  |  |  |  |
| Phone Explorer | Copy or move a file or folder to the PC. Read file content. |  |  |  |  |  |  |
| Phone Explorer | Create a new folder |  |  |  |  |  |  |
| Phone Explorer | Remove a file |  |  |  |  |  |  |
| Phone Explorer | Remove an empty folder |  |  |  |  |  |  |
| Phone Explorer | Format a volume |  |  |  |  |  |  |
| Phone Explorer | Copy a file or folder to another location on the phone |  |  |  |  |  |  |
| Phone Explorer | Move a file or folder to another location on the phone |  |  |  |  |  |  |
| Phone Explorer | Create standard folders for a volume |  |  |  |  |  |  |

## Checklist 2: Support of OBEX commands

| Nr. | OBEX-facility | Specification | Support | Remark |
| --- | --- | --- | --- | --- |
| 1 | Folder listing objects | Standard OBEX specification |  |  |
| 2 | GetMMCAttrib header | Siemens OBEX specification |  |  |
| 3 | SetAttributes header | Siemens OBEX specification |  |  |
| 4 | Mapping Windows File attributes to OBEX file permission attributes (and visa versa) according to | Siemens OBEX specification |  |  |
| 5 | PutMMCAttrib header | Siemens OBEX specification |  |  |
| 6 | Generic file transfer from PC to mobile using SETPATH / PUT commands | Standard OBEX specification |  |  |
| 7 | Generic file transfer from mobile to PC using SETPATH / GET commands | Standard OBEX specification |  |  |
| 8 | Delete a File using SETPATH and PUT without a body | Standard OBEX specification |  |  |
| 9 | Remove a Folder using SETPATH and PUT without a body | Standard OBEX specification |  |  |
| 10 | Format header | Siemens OBEX specification |  |  |
| 11 | Move and Copy header triplet | Siemens OBEX specification |  |  |
| 12 | CRC-checksums header (for non-reliable transport layers) | Siemens OBEX specification |  |  |

## Checklist 3: Support of AT Commands

| Nr. | AT description | AT Commands | Remark |
| --- | --- | --- | --- |
| 1 | Get battery charge |  |  |
| 2 | Get manufacturer |  |  |
| 3 | Get mobile type |  |  |
| 4 | Get software version |  |  |
| 5 | Get serial number |  |  |
| 6 | Get network operator |  |  |
| 7 | Read error messages |  |  |
| 8 | Read phone book entry |  |  |
| 9 | Select phone book |  |  |
| 10 | Write phone book entry |  |  |
| 11 | Enter PIN |  |  |
| 12 | Change password protection |  |  |
| 13 | Binary Read |  |  |
| 14 | List SMS (do not change read status) |  |  |
| 15 | Select phone book (including SIEMENS specific) |  |  |
| 16 | Delete SMS from memory |  |  |
| 17 | Configure SMS format |  |  |
| 18 | List SMS (sorted by read, not read, send, not sent) |  |  |
| 19 | Read SMS |  |  |
| 20 | Send SMS immediately |  |  |
| 21 | Write SMS to memory |  |  |
| 22 | Send SMS from memory |  |  |
| 23 | Select preferred SMS memory |  |  |
| 24 | Get/Set Service centre address |  |  |
| 25 | Binary Read |  |  |
| 26 | Get/Set alert mode |  |  |
| 27 | Get/Set vibrator mode |  |  |
| 28 | Set Ringing tone |  |  |
| 29 | Start Playing Ringing tone |  |  |
| 30 | Stop Playing Ringing tone |  |  |
| 31 | Switch of echoing characters |  |  |
| 32 | Reset AT commands parameters to factory settings |  |  |
| 33 | Display acknowledgements as text |  |  |
| 34 | Set the data terminal ready (DTR) behaviour |  |  |
| 35 | Set the received line signal detector (DCD) behaviour |  |  |
| 36 | Display acknowledgements |  |  |
| 37 | Write value 0 to S register 0 |  |  |
| 38 | Quality of Service Profile (Requested) |  |  |
| 39 | Define PDP context |  |  |
| 40 | Select type of Authentication for PPP connection |  |  |
| 41 | Establish a connection via service code 99 |  |  |
| 42 | Release existing connection |  |  |
| 43 | Terminate all active calls and all calls on hold |  |  |
| 44 |  |  |  |