Requirements Specification

L1C / PS / DS for ATC and Mux

| Status | Released |
| --- | --- |
| Version | 1.2_001 |
| Date | 05.07.2005 |

| Responsible | Jens Scholz, COM MD PD SWP 3 Ulm 1 Günter Hofmann, COM MD PD SWP 3 Ulm 1 |
| --- | --- |
| Author(s) | Harald Guter, COM MD PD SWP 3 Ulm 1 |
| Release |  |

Template: Component Requirement Specification

Status: released for piloting, Version: 1.5/001,Date:6.2.03

Copyright (C) Siemens AG 2005

The information contained herein is confidential and the property of Siemens AG and is supplied without liability for errors or omissions. No part may be reproduced or used except as authorised by contract or other written permission. The copyright and the foregoing restriction on reproduction and use extend to all media in which the information may be embodied. Offenders are liable to the payment of damages. All rights reserved in the event of the grant of a patent or the registration of a utility model or design.

Published by COM MD

Siemens AG

Table of Contents

1	Introduction	3

1.1	Purpose	3

1.2	Scope	3

1.3	Definitions, Acronyms and Abbreviations	3

1.4	References	6

1.5	History	6

2	Functional Requirements	8

2.1	Remote Control	8

2.1.1	Requirements from BT Car Kit and Stereo Headset	9

2.1.2	Requirements from Mobile Phone Manager	9

2.1.3	Requirements from AppConnect	12

2.1.4	Requirements for the Multiplexer	13

2.1.5	Requirements from Change Requests	13

2.2	Multiplexer	14

# Introduction

## Purpose

This document gives the requirements on the components for Layer 1 Control, Protocol Stack and Data Services on a feature level. The components shall be integrated into X75 platform products.

## Scope

The scope of this document is limited to products within the X75 platform that are based on SGoldv2.

The specification contains optional and some mandatory features resulting from the 3GPP Technical Specifications. All features that are claimed mandatory for mobile stations in the 3GPP Technical Specifications shall be supported, according to the release given below. Especially, if there is a mandatory feature in the 3GPP Technical Specifications that is not listed below, then the support is required implicitly through the 3GPP compliance.

## Definitions, Acronyms and Abbreviations

In the following, all requirements are marked with a title and identifier for easy referencing. Text without such title and identifier is for informational purpose only.

Several times, the requirements demand a feature (or parameter) to be configurable "at runtime". Here, this means that there shall be a dedicated flag or block contained in the mappings for configuration of the feature. This is the contrary of a configuration via compile switches. The aim of "configuration at runtime" is that the feature set can be varied without booting new software to the device.

| Terms | Definition |  |
| --- | --- | --- |
| ACL | Asynchronous Connection-oriented [logical transport] |  |
| ADPCM | Adaptive Differential Pulse Code Modulation |  |
| AFH | Adaptive Frequency Hopping |  |
| APFC | Address Protocol Field Compression |  |
| API | Application Programming Interface |  |
| BIP | Basic Imaging Profile |  |
| BNF | Backus Naur Form |  |
| BPP | Basic Printing Profile |  |
| CS | Circuit Switched |  |
| CSCF | Call/Session Control Function |  |
| CSD | Circuit Switched Data |  |
| CHAP | Challenge Handshake Authentication Protocol |  |
| DND | Do Not Disturb |  |
| DPOF | Direct Print Order Format |  |
| DPS | Acronym of "Digital Photo Solutions for imaging devices" |  |
| EAP | Extensible Authentication Protocol |  |
| EDGE | Enhanced Data Rates for GSM Evolution |  |
| EPIC | Efficient protocol Independent Compression |  |
| ESP | IP security Encapsulating Security Payload |  |
| FCS | Frame Check Sequence |  |
| FFS | Flash File System |  |
| FIR | Fast Infrared |  |
| GLMS | Group List Management Server |  |
| GPRS | General Packet Radio Service |  |
| GUI | Graphical User Interface |  |
| HCI | Host Controller Interface |  |
| HMAC | Hash based Message Authentication Code |  |
| HTTP | Hyper Text Transfer Protocol |  |
| IAS | Information Access Service |  |
| ICMP | Internet Control Message Protocol |  |
| IMS | IP Multimedia Subsystem |  |
| IMS AKA | IMS Authentication and Key Agreement |  |
| IMS 2.0 | Initial SIEMENS COM MN core network solution for IMS |  |
| IMS x.0 | SIEMENS COM MN IMS core network version x.0 |  |
| IP | Internet Protocol |  |
| IPSec ESP | IP Security Encapsulation |  |
| IR | Incremental Redundancy |  |
| IrDA | Infrared Data Association |  |
| ISIM | IP Multimedia Services Identity Module |  |
| IUT | Implementation under Test |  |
| JSR | Java Specification Request |  |
| LCP | Link Control Protocol |  |
| LMP | Link Manager Protocol |  |
| ME | Mobile Equipment |  |
| MIR | Medium Infrared |  |
| MMC | Multi Media Card |  |
| MMI | Man Machine Interface |  |
| MPM | Mobile Phone Manager |  |
| MRU | Maximum Receive Unit |  |
| MSC | Message Sequence Chart |  |
| MTU | Maximum Transmission Unit |  |
| NAPDEF | Network Access Point Definition |  |
| OBEX | Object Exchange |  |
| OTAP | Over The Air Provisioning |  |
| PAP | Password Authentication Protocol |  |
| PDF | Policy Decision Function |  |
| PDP | Packet Data Protocol |  |
| PEP | Presence Enhance Phonebook |  |
| PFC | Protocol Field Compression |  |
| PICS | Protocol Implementation Conformance Statement |  |
| PIXIT | Protocol Implementation Extra Information for Testing |  |
| PoC | Push to Talk over Cellular |  |
| PoC I | PoC application compliant with MENSA PoC Release 1.0 |  |
| PoC II | PoC application compliant with MENSA PoC Release 2.0 |  |
| PPP | Point to Point Protocol |  |
| PSM | Protocol Service Multiplexer |  |
| PTP | Picture Transfer Protocol |  |
| QoS | Quality of Service |  |
| RCCP | Remote Control Communication Protocol |  |
| RLP | Radio Link Protocol |  |
| ROHC | Robust Header Compression |  |
| RTCP | Real Time Control Protocol |  |
| RTP | Real Time Protocol |  |
| RTSP | Real Time Streaming Protocol |  |
| SAT | SIM Application Toolkit |  |
| SDDB | Service Discovery Database |  |
| SIMPLE | SIP for Instant Messaging and Presence Leveraging Extensions |  |
| SIP | Session Initiation Protocol |  |
| SIR | Slow Infrared |  |
| SSL | Secure Sockets Layer |  |
| TCP | Transport Control Protocol |  |
| TLS | Transport Layer Security |  |
| UDI | Unrestricted Digital Information |  |
| UDP | User Datagram Protocol |  |
| UUID | Universal Unique Identifier |  |
| URC | Unsolicited Response Code |  |
| URI | Uniform Resource Identifier |  |
| URL | Uniform Resource Locator |  |
| USB | Universal Serial Bus |  |
| USIM | Universal Subscriber Identity Module |  |
| UTRAN | UMTS Terrestrial Radio Access Network |  |

## References

3rd Generation Partnership Project; Technical Specifications Release 4

"ODM Support of Mobile Phone Manager - Mobile Phone Manager Interfaces for MPM Support"; William Holohan; Version 1.4

"New 75 MPM Requirements 4 ATC"; Frank Eppink, Sebastian Unruh; Version 3.9

"AT command set for XX Siemens mobile phones and modems"; Comneon GmbH & Co OGH; Version: Master R75, September 29th, 2004

3GPP TS 27.010 Terminal Equipment to User Equipment (TE-UE) multiplexer protocol (Release 5)

"Car Kit Bluetooth Portable (CK P BT) Car Kit Bluetooth 75 (CK BT 75) Car Kit Bluetooth SIM Access (CK SA) AT Command List"; Volker Ebert; Version 1.0

"Bluetooth Stereo Headset (BTSTHS)"; Jens Knapmeyer; Version 0.2

"Car Kit Professional (Bluetooth) " (List of required AT commands_v0_23_.pdf) ; Volker Ebert; Version 0.23

"Encoding / Decoding data for CCM"; R. Heulmanns; Version 2.00

## History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0_005 | 14.12.2004 | Harald Guter | - History added - DS_196: added comment due to review records from Comneon - Added the same comment to DS_203, DS_204, DS_205 - Added new chapter “Requirements from Change Requests” - DS_416: (SVMC) new reqmt because of a CR - DS_417: (SCCM) change for existing command because of a CR and added reference to EncodeDecodeCcm20.pdf - Updated reference of MPM requirements from version 2.6 to 3.2. The review records from Comneon were the reason for this. Affected requirements: DS_174, DS_176, DS_190, DS_191, DS_181 - DS_112: removed term “ad-hoc” and referenced version 0.9 instead of 0.8 of BT car kit requirements - DS_105 & DS_418: (SVRA) new reqmts because of CR and added reference to car kit document - DS_419: new reqmt because of CR - DS_199: removed : “URC shall be available while the mobile is in SIM access mode” |
| 1.0_006 | 25.01.2005 | Harald Guter | Changes from Release Meeting added: - Removed DS_114, DS_106, DS_199 - Updated reference of BT Carkits Specification from version 0.9 to 1.0 - Updated reference of MPM Specification from version 3.2 to 3.4 (difference between the versions is that comments were added) |
| 1.1_001 | 15.03.2005 | Harald Guter | - Removed DS_198: (Car loudspeaker item URC), remove was requested by Comneon, confirmed by accessories team. - Removed Chapter “Only for BT Stereo Headset” and with it DS_203, DS_204, DS_205. These requirements contained the old definitions for the media player information - DS_171: Support for MMS is no longer required (AT^SMMS). - DS_98: removed (AT^SMMS), see also DS_171 - DS_177: removed (event for incoming MMS), see also DS_171 - DS_105 and DS_418: removed (AT^SVRA) as CR-11 was retracted - DS_456: new requirement because of CR-21 (AT^SMPL) - DS_457: new requirement because of CR-23 (play all signal tones) - DS_458: new requirement because of CR-24 (CPBF) - Updated reference of MPM Specification from version 3.4 to 3.9 (difference between the versions is the requirement for ringer settings, chapter 3.10.4, but this information is already available for Comneon – other differences are Siemens internal comments only) - Corrected some typing errors |
| 1.2_001 | 05.07.2005 | Harald Guter | Modifications due to some change requests: - DS_471: AT^SMEL (CR-33) - DS_469: extension of AT^SDBR (CR-42) - DS_470: AT^SGMR (CR-47) - Removed all requirements which were related to PoC (CR-41): DS_107, DS_108, DS_109, DS_110, DS_111, DS_112, DS_113, DS_115, DS_116, DS_118, DS_119, DS_120, DS_121, DS_200, DS_126, DS_117, DS_419 |

# Functional Requirements

## Remote Control

AT command set (DS_38): The mobile shall support Hayes Standard commands, AT command set for GSM mobiles, mobile status and control, call control, phonebook management, bluetooth, SAT (SIM Application Toolkit), file/data/object download, fax, GPRS and Siemens proprietary commands.

Standards: 3GPP TS 07.05
3GPP TS 07.07
ATC_Command_Set_master_75.pdf

AT^STRC (DS_83): The mobile shall support the command AT^STRC with the following functionality:
- The commands activates the trace output
- The syntax shall be: AT^STRC = x,y. The remote control shall call the function TRC_initDataTracer2(x,y).  The range of x,y is integer. 
- If no developer rights exist, the remote control answers with "error".

AT^SCHNULLI (DS_84): The mobile shall support the command AT^SCHNULLI=11,X with the following functionality:
- The commands activates the debug output of the bluetooth stack
- X can have the following values:
  X = 00: Trace output of bluetooth stack is off
  X = 01: Trace output of bluetooth stack is on, level 1 (critical)
  X = 02: Trace output of bluetooth stack is on, level 2 (warn)
  X = 03: Trace output of bluetooth stack is on, level 3 (info)
  X = 04: Trace output of bluetooth stack is on, level 4 (verbose)

AT^SGDI (DS_99): The mobile shall support the command AT^SGDI for getting device infos. The functionality and syntax is described in the 'ODM Support of Mobile Phone Manager' document (since version 0.8).

Standards: ODM Support Requirements for Siemens Mobile Phone Manager.pdf

AT^SCHOBI (DS_102): The mobile shall support the command AT^SCHOBI with the following functionality:
- The command shall affect that modem commands of the netcore are rerouted to a second serial interface. This serial interface can be connected to an adequate analogous HW-modem which simulates the connection to the net.
- The syntax shall be: AT^SCHOBI,,,,,,,,3

AT^SAPO (DS_103): The mobile shall support the command AT^SAPO for control of the test statemachine in C_appim. 
- The syntax shall be: AT^SAPO=<cmd>[,”action;object;arg1;arg2;arg3”] 
- where <cmd> can be 
   0: Close the test statemachine; Response: OK
   1: Open the test statemachine; Response: OK
   2: Send a command to test statemachine; Result: OK, if command is correct, else ERROR
- optional parameters are only used for 2

AT+CKPD (DS_122): The already existing command AT+CKPD shall be extended for PTT keypress. The syntax shall be: AT+CKPD=200.

Standards: ATC_Command_Set_master_75.pdf
HSP (Headset Profile) Specification v1.1

ATC command set document (DS_135): Comneon shall provide a master document "ATC_Command_Set" for each software release which includes and describes all AT commands which are supported by remote control.

Instances (DS_159): It shall be possible to instance Remote Control. A maximum number of 4 instances is required.

### Requirements from BT Car Kit and Stereo Headset

AT+BVRA (DS_101): Remote Control shall support the command AT+BVRA with the following functionality:
- Enable / disable voice recognition (BT specific command). Command is not required if phone does not support voice recognition.

Standards: ATC_Command_Set_x75_Bluetooth_Car_Kits_v1_0_.pdf
ATC_Command_Set_x75_Bluetooth_Stereo_Headset_v0_2_.doc

+BVRA (DS_194): Remote Control shall support the URC +BVRA with the following functionality:
Voice recognition termination URC. Voice recognition handling. URC is not required if phone does not support voice recognition

Standards: ATC_Command_Set_x75_Bluetooth_Car_Kits_v1_0_.pdf
ATC_Command_Set_x75_Bluetooth_Stereo_Headset_v0_2_.doc

+BSIR (DS_100): Remote Control shall support the URC +BSIR with the following functionality:
- In-band ringing tone settings of mobile phone have changed.

Standards: ATC_Command_Set_x75_Bluetooth_Car_Kits_v1_0_.pdf
ATC_Command_Set_x75_Bluetooth_Stereo_Headset_v0_2_.doc

AT+CLCC (DS_104): The already existing command AT+CLCC shall not only return the number of the current call but also the name of the current caller.

Standards: ATC_Command_Set_master_75.pdf
ATC_Command_Set_x75_Bluetooth_Car_Kits_v1_0_.pdf
ATC_Command_Set_x75_Bluetooth_Stereo_Headset_v0_2_.doc

#### Only for BT Car Kit

AT^SABD (DS_196): Remote Control shall support the AT command AT^SABD=2,"ID,02,1" with the following functionality:
Read ID of currently selected audio parameter set.
Note: The functionality of AT^SABD is not up to Comneon. This command is only handed over to Siemens software and handled there.

Standards: ATC_Command_Set_x75_Bluetooth_Car_Kits_v1_0_.pdf

Car loudspeaker (DS_125): Remote Control shall support a command to read/write the setting of the carkit menu item "Car loudspeaker".

Standards: ATC_Command_Set_x75_Bluetooth_Car_Kits_v1_0_.pdf

### Requirements from Mobile Phone Manager

Mp3 and Camera folders (DS_167): Remote Control shall support an AT command as described in the MPM document chapter:
3.1.1 Get current used Mp3 and Camera folders

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Date (DS_168): Remote Control shall support an AT command as described in the MPM document chapter:
3.2.1 Date
- Set date format

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Time (DS_169): Remote Control shall support an AT command as described in the MPM document chapter:
3.2.2 Time
- Set time format

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Speech File Management (AT^SVCL, AT^SVCD, AT^SVCC) (DS_170): Remote Control shall support AT command as described in the MPM document chapter:
3.2.4 Speech File Management
The new commands should be: AT^SVCL, AT^SVCD, AT^SVCC for more details, see referenced documents.

Standards: New 75 MPM Requirements 4 ATC.doc V3.9
003_SIS_MPM_IF_Conventions_V01_003.doc

Message-Sync (DS_171): Remote Control shall support an AT command as described in the MPM document chapter:
3.3.1 Message-Sync
- SMS/EMS read, write, delete
- AT^SMMS is NOT required!

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Install and uninstall Java-Apps (DS_172): Remote Control shall support an AT command as described in the MPM document chapter:
3.4.1 Install and uninstall Java-Apps

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Phone number associated data (DS_174): Remote Control shall support an AT command as described in the MPM document chapter:
3.9.2 Get phone number associated data

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Call duration (DS_175): Remote Control shall support an AT command as described in the MPM document chapter:
3.9.3 Get call duration

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Active profile (DS_176): Remote Control shall support an AT command as described in the MPM document chapter:
3.9.5 Get and set active profile

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Task reminder event (DS_178): Remote Control shall support an AT command/URC as described in the MPM document chapter:
3.9.11 Event when the phone's task reminder gets active

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Appointment reminder event (DS_179): Remote Control shall support an AT command/URC as described in the MPM document chapter:
3.9.12 Event when the phone's appointment reminder gets active

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Alarm clock event (DS_180): Remote Control shall support an AT command/URC as described in the MPM document chapter:
3.9.13 Alarm clock event

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Handsfree (DS_181): Remote Control shall support an AT command as described in the MPM document chapter:
3.9.16 Loudspeaker handsfree

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Theme (DS_182): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.2 Selected theme

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Display Settings (DS_183): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.3 Display Settings

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Ringer Settings (DS_184): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.4 Ringer Settings

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Phone Setup settings (DS_185): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.5 Phone Setup settings

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Call Setup settings (DS_186): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.6 Call Setup settings

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Clock settings (DS_187): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.7 Clock settings

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Accessories Settings (DS_188): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.11 Accessories Settings

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Camera Settings (DS_189): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.12 Camera Settings

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

Alarm Clock (DS_190): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.17 Alarm Clock

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

T9 Wordlists (DS_191): Remote Control shall support an AT command as described in the MPM document chapter:
3.10.28 T9 Wordlists

Standards: New 75 MPM Requirements 4 ATC.doc V3.9

### Requirements from AppConnect

Register/Unregister for indications (DS_87): Remote control shall provide methods to (un)register for indications if a certain command is received in any RC instance. 
Remark: AppConnect is not at all times interested in all commands coming in through the ATC interface. By offering a register/deregister interface AppConnect can decide when it is interested in receiving indications. It is already implemented like this in the 65 series.

Instances and parameters (DS_94): All indication messages shall contain the CepID of the RC instance that received the command (e.g. within the 'Sender' field of the OS message header). RC shall provide a function to translate a RC instance to the corresponding CepID. 
Indications shall also contain the command parameters. 
Remark: This allows AppConnect to assign an ATC command to a certain connection. The registration mechanism and some indication messages are already implemented in the 65 series. Additional indications are necessary for PoC (Session establishment and request RTS) and for Voice Recognition.

No interface from AppConnect (DS_95): RC shall NOT include any header-file of AppConnect, because this would introduce a bi-directional dependency.

Send BT related AT commands (DS_88): The RC interface shall provide functions to send unsolicited commands (e.g +VGS, +BVRA, +BINP, +BSIR, RING, ^SABD)

Standards: ATC_Command_Set_master_75.pdf

HFP - progress in connection management (DS_93): RC shall indicate the progress in the Connection Management as described in the Hands Free Profile (HFP) specification 1.00 paragraph 4.2; 4.3

Standards: HFP (Handsfree Profile) Specification v1.0

Gipsy component (DS_90): RC shall manage the use of the Gipsy component. I.e. any restrictions/limitations on its use shall be guarded and managed by RC.

AT for BT HS and HF (DS_91): RC shall support all AT commands and result codes as described in the BT HS (head set) and HF (hands free) specification.

Standards: HFP (Handsfree Profile) Specification v1.0
HSP (Headset Profile) Specification v1.1

BT HS and HF scenarios (DS_92): RC shall support all AT commands which are needed to handle all scenarios described in the BT HS and HF specification and corresponding Errata Service Release, including 'In-Band ringing' and 'No In-Band ringing'.

Standards: HFP (Handsfree Profile) Specification v1.0
HSP (Headset Profile) Specification v1.1

RING for BT HSP (DS_140): ATC shall send a RING as described in [BT_HSP] paragraph 4.2, except in the case where In-Band ringing is used.

Standards: HSP (Headset Profile) Specification v1.1

Initialization for BT HS and HF (DS_97): After a new BT HS or HF connection is established, RC shall initialize the instance according to the BT profile specification.

Standards: HFP (Handsfree Profile) Specification v1.0
HSP (Headset Profile) Specification v1.1

PC Simulation (DS_123): All Remote Control functionality shall be available in the PC simulation. The link Stack -> SAPI -> ATC shall work like in target with regards to all commands and parameters. 
Note: The full functionality of Remote Control in PC simulation can only be assured if all application interfaces that are used by Remote Control are also available in PC Simu.

### Requirements for the Multiplexer

AT+CMUX=0 (DS_158): Remote Control shall support the command AT+CMUX=0 as defined in 3GPP TS 07.07. If a multiplexer is already enabled and AT+CMUX=0 is called an error shall be returned.

Standards: 3GPP TS 07.07

Baudrate (DS_160): The parameter "port-speed" of the command AT+CMUX is not required.

Start (DS_161): The Remote Control instance which starts the multiplexer shall be closed.

Default state (DS_162): All Remote Control instances shall be started in a reset default state.

### Requirements from Change Requests

AT^SVMC (DS_416): The command AT^SVMC needs not to be supported anymore. (-> CR-5)

AT^SCCM (DS_417): The command AT^SCCM shall be adapted to the new parameters of the CC-monitor. (-> CR-6)

Standards: EncodeDecodeCcm20.pdf

AT^SMPL / ^SMPL (DS_456): Remote Control shall support the URC ^SMPL to inform the bluetooth stereo headset about media player status information.
Additionally Remote Control shall support the AT command AT^SMPL that the bluetooth stereo headset can register/deregister at/from Remote Control to receive this URC (more details are given in CR-21).

Play signal tones (DS_457): Remote control shall support a new AT command to play all signal tones of the mobile (similar to AT^SPST) inclusive level control. 
This is a requirement from the acoustic test team, especially for acoustic shock protection tests (more details and the parameters are given in CR-23).

AT+CPBF (DS_458): Remote Control shall support the AT command AT+CPBF to find phone book entries. This is a requirement from Telefonica (more details are given in CR-24).

Standards: 3GPP TS 07.07

AT^SDBR (DS_469): The already existing AT command AT^SDBR shall be extended with two additional number types (number_type 4 and 5).
The following number types shall be returned:
<number_type>: 0 phone number “HOME”
                              1 phone number “OFFICE”
                              2 phone number “MOBILE”
                              3 phone number “FAX”
                              4 phone number “MOBILE OFFICE”
                              5 phone number “FAX OFFICE”
(See also CR-42)

AT^SGMR (DS_470): Remote Control shall support the AT command AT^SGMR to read out the extended software version. The parameters shall be the same as for the already existing command AT+CGMR.
AT+CGMR returns only the software version number for “official” software (e.g. 01). For local developer software it returns additional information like baseline, buildtime, etc.
The new command AT^SGMR shall behave like AT+CGMR with the difference that if the software version is “official” AND the mobile is in developer state, the additional information shall also be returned like for local software versions.
(See also CR-47)

AT^SMEL (DS_471): Remote Control shall support the AT command AT^SMEL to set and get the melody settings (see also DS_184).
The following Syntax is required:
 
Test command	
AT^SMEL=?
Response
^SMGL:(<purpose>s),(<mode>s), (<volume>s)
OK/ERROR/+CMS ERROR Parameter: 		
<purpose> E_AuMa_Purpose
<mode>	0: ON - Melody for the purpose on
	1: OFF - Melody for the purpose off
	2: BEEP - only beep for the purpose
	3: Ascend - Ascending Melody volume for the purpose
<volume>  manufacturer specific volume from 0 to 5

Read command	
AT^SMEL?
Response
^SMEL:(<purpose>,<mode>,<volume>) <CR><LF>
^SMEL:(<purpose>,<mode>,<volume>) <CR><LF>
[[..]]
OK/ERROR/+CMS ERROR

Write command
AT^SMEL = < purpose >[,<mode>][,<volume>]
Response: 
OK/ERROR/+CME ERROR
Default values for mode is ON and default value for volume is 3

Execute command	
AT^SMEL 	
Response:
^SMEL:(<purpose>,(<mode>s),(<volume>s) )<CR><LF>
^SMEL:(<purpose>,(<mode>s),(<volume>s) )<CR><LF>
[[..]]
OK/ERROR/+CMS ERROR 

(See also CR-33)

## Multiplexer

3GPP compliance (DS_143): The multiplexer shall be compliant to 3GPP TS 27.010 V5.0.0 spec (formerly GSM 07.10) with the
constraints mentioned in: DS_144, DS_145, DS_146, DS_150, DS_156
and exceptions mentioned in: DS_151, DS_152

Standards: 3GPP TS 27.010

Operating options (DS_144): The multiplexer shall support the basic operating option. Other options (advanced with and without error recovery) are not required.

Standards: 3GPP TS 27.010

Number of channels (DS_145): The multiplexer shall support 7 channels (6 data channels and 1 control channel).

Maximum length of the data field (DS_146): The default maximum size of the data field shall be 31 bytes. The limit of the maximum size of the data field shall be 98 bytes.

Standards: 3GPP TS 27.010

Packets with oversize (DS_147): The multiplexer shall discard received data packets if their length of the data field is longer than the maximum size (see DS_146).

Segmentation (DS_148): Segmentation shall be done by the multiplexer. (That means if the multiplexer gets huge data buffers from an application it has to split them into adequate multiplexer data packets)

One multiplexer (DS_149): It shall not be possible to instantiate the multiplexer. Only one multiplexer context shall be supported. That means if the multiplexer is already enabled and someone calls AT+CMUX=0 an error shall be returned.

Type of frame (DS_150): The Multiplexer shall support the following type of frames:
SABM (Set Asynchronous Balanced Mode), UA (Unnumbered Acknowledgement), DM (Disconnected Mode), DISC (Disconnect), UIH (Unnumbered Information with Header check). The support of the optional frame type UI (Unnumbered Information) is not required.

Standards: 3GPP TS 27.010

Priorities (DS_151): The multiplexer shall not support different priorities for different data channels. All supported data channels shall have the same priority.

DLCI mapping (DS_152): The 7 channels of the multiplexer shall be mapped as defined in the following (DLCI = Dynamic Link Channel Index):
DLCI  0: control channel
DLCI  1: Remote Control
DLCI  2: Remote Control
DLCI  3: Remote Control
DLCI  4: reserved
DLCI 58: OBEX
DLCI 59: OBEX
DLCI 60: RIM-OBEX
For DLCI 58-60 it shall only be possible to use the OBEX resp. RIM-OBEX protocol as defined above - no switch to Remote Control (e.g. via escape sequence) is allowed. 
For DLCI 1-3 it is allowed to switch from Remote Control to e.g. Gipsy, OBEX, RIM-OBEX and back with the restriction that there are free instances of these protocols available (e.g. there is only one Gipsy instance, so it is only allowed for one of DLCI 1-3 to switch from Remote Control to Gipsy).
Note 1: DLCI 58-60 are only intended to be used by MPM (Mobile Phone Manager) and will not be published as they are not spec conform.
Note 2: The mapping between multiplexer channels and protocols as defined above will only be possible if enough free instances of these protocols are available when the multiplexer is opened.

Default setting (DS_154): At start up of the mobile the multiplexer shall be disabled. If the multiplexer is disabled the connection handling between physical layers - S-SAPI - OBEX, Remote Control, etc. shall be the same as in a mobile without multiplexer (cp. generation 65 mobiles).

Baudrate (DS_155): The multiplexer shall run at the same speed as the AT command used to start the multiplexer. An AT+IPR to change the baudrate shall be ignored thereafter.

Flow Control (DS_156): The Multiplexer shall support the flow control commands as described in 3GPP TS 27.010 chapter 5.4.6.3.1, 5.4.6.3.3-9 and hardware flow control.

Standards: 3GPP TS 27.010

Escape sequence (DS_163): If a escape sequence (one second silence,"+++",one second silence) is detected over the physical connection the multiplexer shall be closed and switch back to Remote Control mode.