For internal use only

Copyright (C) Siemens AG 2003

The information contained herein is confidential and the property of Siemens AG and is supplied without liability for errors or omissions. No part may be reproduced or used except as authorised by contract or other written permission. The copyright and the foregoing restriction on reproduction and use extend to all media in which the information may be embodied. Offenders are liable to the payment of damages. All rights reserved in the event of the grant of a patent or the registration of a utility model or design.

Published by COM MD PD ST1 KLf

Siemens AG

| COM MD PD ST1 KLf | X75 |
| --- | --- |
| P_X75_SGOLD2.PublicFolders.Projects.System_Test_SW.05.01.ATC.Testplan-001 |  |
| Product Test Plan |  |

| Status | Released |
| --- | --- |
| Version | 3.0_002 |
| Date | 16.11.2004 |

| Responsible | Tobias Kleinmann | COM MD PD ST1 KLF |  |
| --- | --- | --- | --- |
| Author(s) | Tobias Kleinmann | COM MD PD ST1 KLF |  |
|  |  | COM MD PD ST1 KLF |  |
| Release |  |  |  |

Template: Product Test Plan

Status: Reviewed, Version: 1.0_010, Date: 2004-11-16

- History of Change

| Version | Date | Editor | Status | Description |
| --- | --- | --- | --- | --- |
| 1.0 / 001 | 05-10-2004 | Kleinmann | Under construction | Creation |
| 1.0 / 002 | 13-10-2004 | Kleinmann | Under construction | Added limitation |
| 1.0 / 003 | 14-10-2004 | Kleinmann | Under construction | Changed department name |
| 1.0 / 004 | 15-10-2004 | Kleinmann | Under construction | Added AT commands AT^SVCL, AT^SVCD, AT^SVCC |
| 1.0 / 005 | 18-10-2004 | Kleinmann | Under construction | Several error corrections. |
| 1.0 / 006 | 19-10-2004 | Kleinmann | Under construction | Added ‘List of open items’. Improvements. |
| 1.0 / 007 | 28-10-2004 | Kleinmann | Under construction | Changes due to new requirements specification 1.0_004. AT commands that can be /can not be tested described in more detail. |
| 1.0 / 008 | 03-11-2004 | Kleinmann | Ready for review | Several improvements. |
| 1.0 / 009 | 09-11-2004 | Kleinmann | Ready for review | Changed document identifier. BBIT and System test start- and completion criteria changed. |
| 1.0 / 010 | 16-11-2004 | Kleinmann | Reviewed | Modified according to Review Record |
| 1.0 / 011 | 05-01-2005 | Kleinmann | Reviewed | Erroneous status information changed to ‘Reviewed’ |
| 3.0 / 001 | 24-02-2005 | Hennig | Released | Version changed to 3 according to EasyDM version |
| 3.0 / 002 | 25-02-2005 | Kleinmann | Released | AT commands regarding accessory and PoC can not be tested (Section 2.1.2). Test plan modified after clarification. Particular sections cancelled. Added path to test files in Easy DM |

Table of Contents

1 Introduction	6

1.1 Purpose	6

1.2 Scope	6

1.3 References	7

1.4 Terms and Abbreviations	8

1.4.1 Terms	8

1.4.2 Abbreviations	9

1.5 Comments	10

1.6 List of open items	10

2 Test Concept of Product	11

2.1 Requirements for Test	11

2.1.1 Requirements that will be tested	11

2.1.2 Requirements that will NOT be tested	14

2.2 Limitations	18

2.3 Required tools	19

2.4 Test Stages	20

2.4.1 Black-Box Integration Test	20

2.4.1.1 AT Test commands	20

2.4.1.1.1 Objective	20

2.4.1.1.2 Strategy	21

2.4.1.1.3 Test Start Criteria	21

2.4.1.1.4 Completion Criteria	22

2.4.1.2 AT Read commands	23

2.4.1.2.1 Objective	23

2.4.1.2.2 Strategy	23

2.4.1.2.3 Test Start Criteria	23

2.4.1.2.4 Completion Criteria	24

2.4.1.3 AT Write commands	25

2.4.1.3.1 Objective	25

2.4.1.3.2 Strategy	25

2.4.1.3.3 Test Start Criteria	25

2.4.1.3.4 Completion Criteria	25

2.4.1.4 AT Execute commands	26

2.4.1.4.1 Objective	26

2.4.1.4.2 Strategy	26

2.4.1.4.3 Test Start Criteria	26

2.4.1.4.4 Completion Criteria	26

2.4.2 System test	27

2.4.2.1 AT Test commands	27

2.4.2.1.1 Objective	27

2.4.2.1.2 Strategy	27

2.4.2.1.3 Test Start Criteria	27

2.4.2.1.4 Completion Criteria	28

2.4.2.2 AT Read commands	28

2.4.2.2.1 Objective	28

2.4.2.2.2 Strategy	28

2.4.2.2.3 Test Start Criteria	28

2.4.2.2.4 Completion Criteria	28

2.4.2.3 AT Write commands	29

2.4.2.3.1 Objective	29

2.4.2.3.2 Strategy	29

2.4.2.3.3 Test Start Criteria	29

2.4.2.3.4 Completion Criteria	30

2.4.2.4 AT Execute commands	30

2.4.2.4.1 Objective	30

2.4.2.4.2 Strategy	30

2.4.2.4.3 Test Start Criteria	30

2.4.2.4.4 Completion Criteria	30

2.4.3 Field Test	31

- List of Figures

Figure 1: Example of script file	20

Figure 2: Example of html result file	21

- List of Tables

Table 1: Terms	7

Table 2: Abbreviations	8

# Introduction

## Purpose

This document provides the test plan for the ATC command interpreter of the X75 mobile phone platform.

## Scope

Mobile telephones can be operated via remote control using a serial interface (data cable, infrared or bluetooth connection). Remote control is implemented by means of ATC commands according to the ETSI GSM 07.07 and GSM 07.05 specifications, as well as several manufacturer specific AT commands (e.g. AT^SXXX).

Hayes Microcomputer Products developed a language called the AT command set for controlling modems that has become the de facto standard. The majority of modems are Hayes AT compatible. The ATC commands are a set of commands and a further development of the AT commands set designed especially for controlling mobiles.

An AT command generally begins with an ‘AT’ (meaning ‘attention’ ) command prefix. The remainder of the line is interpreted as a sequence of commands. The commands are not case-sensitive. More than one command may be given on a single line, with the semicolon serving as the delimiter between commands. An AT command should start with the character string “AT” and end with “<CR>” (=0x0D). The input of a command is acknowledged by “OK” or “ERROR”.

A command currently in process is interrupted by each additional character entered. This means that you should not enter the next command until you have received the acknowledgement; otherwise the current command is interrupted.

With AT commands it is possible to control the mobile. It is for example possible to read, write and delete entries from the phone book. It is possible to read, write, send and delete SMS etc.

For all tests ATT 2.0, a test tool for AT cellular commands is used. With a data cable the serial port of the PC is connected to the mobile. ATT 2.0 basically functions as a terminal program that sends AT commands to the mobile and receives the mobile’s replies. ATT 2.0 will be used to run sets of commands, called scripts. After each script run, the replies of the mobile are verified. All the results of a test run are recorded into an HTML file. If there is a difference between the received and the expected reply this difference is recorded into the result file. After the script run is completed, the result file has to be checked by the tester. At the end of this file there is a summary about how many AT commands passed and how many didn’t pass the test.

## References

[01] 	Reference manual: AT command set for XX Siemens mobile phones and modems,      Version 	Master. 29th September 2004, Document ID: A30880-A10-A001-XX-D376

[02]		Requirements Specification L1C, PS, DS  for ATC and MUX V1.0_004

[03] 	Bluetooth Stereo Headset AT command list V 0.2

[04]		Car Kit Bluetooth XXX AT command set V 0.8

[05]	Extended command set: Memo-Input-Board specific AT^SACD – commands

[06]	ODM Support of Mobile Phone Manager V1.4

[07]	New 75 MPM Requirements 4 ATC V2.6

[08] 	003_SIS_MPM_IF_Conventions Design Specification Filesystem Architecture/ MPM Interface 	V0.1/003

[09]	ODM Support of Mobile Phone Manager V1.1

[10]	ETSI GSM 07.07: AT command set for GSM mobile equipment, Version 7.5.0,December 	1999, Third edition, Document ID: ETS 100 916

[11]		ETSI GSM 07.05: Use of data terminal equipment – data circuit terminating; Equipment inter	face for short message service and cell broadcast service, Version 5.0.0, Juli1996, Reference 	TS/SMG-040705Q

[12]		ITU-T Draft new Recommendation V.25ter “Serial asynchronous automatic dialing and control”

[13]		User manual ATT2.0 “2nd generation automatic test tool for AT Cellular”; Weiden, 2004. 	http://ezk.klf.siemens.de/Dienststellen/bildergalerie/iut_aufgaben/index.html

[14]		ATT 2.0 –AT Command Test Tool version 2.0 	http://ezk.klf.siemens.de/Dienststellen/bildergalerie/iut_aufgaben/index.html

[15] 	HFP (Handsfree Profile) Specification V1.0

[16]		HSP (Headset Profile) Specification V1.1

[17]	Featurelist X75 (Version 2.0.40.1)

https://ims.icn.siemens.de/livelink/livelink/Open/329894929

All referenced documents can be found in EasyDM:

PublicFolders.Projects.System_Test_SW.05.01.ATC.Test_environment

## Terms and Abbreviations

### Terms

| Term | Description |
| --- | --- |
| test case | For a specific test object a test case specifies preconditions (what is the expected state of the test object before executing this test case?), input values, and expected results (what is the expected state of the test object after the execution of the test case?). Usually, a test case also contains a short description to provide some detail about the test case. Example for a test case: Synopsis: Test read functionality for AT+CVIB Precondition: Mobile connected to Terminal program Action: Send AT+CVIB? to mobile Expected: Mobiles response is according to Specification |
| test cycle | One execution of a complete set of test planning, test preparation, test execution, and test evaluation within one test stage. A test stage might have more than one test cycle. |
| test framework | An environment in which tests could be performed. For example, this includes test applications, test scripts, necessary hardware, etc. Example: PC, Data Cables, Mobiles, SIM cards, ATT 2.0, Test scripts |
| test harness | A subset of the test framework including only test drivers, stubs and tools necessary to execute tests in a development environment. |
| test object | This is an item to be tested. This can range from a single function or method, to a unit, to interfaces (for example between integrated units or components), to a complete software system, or the complete product device. Example: The Remote Control within a specific Software Version on CX65. |
| test stage | Tests executed during a test stage share a common test objective, common test start criteria, as well as common test completion criteria. Tests are applied to a specific set of test objects. |

- Table 1: Terms

### Abbreviations

| Abbreviation | Description |
| --- | --- |
| ATC | Set of AT commands specially designed for controlling mobile phones. |
| ATT 2.0 | Automatic Test Tool |
| BT | Bluetooth |
| HF | Hands free |
| HFP | Hands free profile |
| HS | Headset |
| HSP | Headset profile |
| Project | A collection of scripts that are executed one after the other. |
| Script | A set of AT commands that will be executed in one go. |
| URC | Unsolicited result code |

Table 2: Abbreviations

## Comments

When referring to test-, read-, write- and execute command in this document, the following is meant:

According to 3GPP TS, it is possible to execute an AT command in one or more of the following forms:

| Test command | AT+CXXX=? | The mobile phone or modem returns the list of parameters and value ranges; these can be set using the corresponding write command or by means of internal processes. |
| --- | --- | --- |
| Read command | AT+CXXX? | This command displays the current value setting of the parameter(s). |
| Write command | AT+CXXX=<...> | This command is used to set parameters that are intended to be set. |
| Execute command | AT+CXXX | This command reads non-settable parameters which are influenced by internal processes in the mobile phone or modem. |

## List of open items

# Test Concept of Product

## Requirements for Test

### Requirements that will be tested

The testing is limited to products within the X75 platform that are based on SGoldv2.

The AT commands are tested for test-, read-, write- and execute functionality (if specified) by means of ATC only- tests will not include the functionality using the particular AT commands (use cases) - this is considered to be the responsibility of accessory test group in ULM and MPM test group in Beijing.

The returned responses and result codes from mobile will be checked for specification conformity.

The listing below identifies those items that have been identified as targets for testing.

| Requirement Nr | Requirement. name | Description | Standards |
| --- | --- | --- | --- |
| Remote Control - General |  |  |  |
| DS_38 | AT command set | Hayes Standard commands GSM mobiles Mobile status and control Call control Phonebook management Bluetooth SAT File/data/object download Fax GPRS commands Siemens proprietary commands | [1], [10], [11] |
| DS_83 | AT^SRTC | Activate trace output |  |
| DS_84 | AT^SCHNULLI | Activate debug output of BT Stack |  |
| DS_98 | AT^SMMS | Multimedia Message Service handling | [06] |
| DS_99 | AT^SGDI | Get device information | [06] |
| DS_102 | AT^SCHOBI | Modem commands of the net core are rerouted to a second serial interface |  |
| DS_103 | AT^SAPO | Control of test state machine in C_appim. |  |
| DS_122 | AT+CKPD=200 | Extension for PTT key presses | [01], [16] |
| DS_159 | Instances | It shall be possible to instance Remote Control. A maximum number of 4 instances is required. |  |
| Requirements from BT Car Kit and Stereo Headset |  |  |  |
| DS_101 | AT+BVRA | Enable/disable voice recognition (BT specific command). | [04], [03] |
| DS_194 | URC +BVRA | Voice recognition termination URC. | [04], [03] |
| DS_100 | URC +BSIR | In-band ringing tone settings have changed. | [04], [03] |
| DS_104 | AT+CLCC | The already existing command shall not only return the number of the current call but also the name of the current caller. | [01], [04], [03] |
| DS_88 | Send BT related AT commands. | Functions to send unsolicited commands | [01] |
| DS_93 | Indication of the Connection management process. |  | [15] |
| DS_91 | All AT commands and result codes described in BT HS [16] and BT HF [15] specifications must be supported. |  | [15], [16] |
| DS_92 | All AT commands needed to handle scenarios described in BT HS [16] and BT HF [15] specification and corresponding Errata Service Release, incl. ‘In-band ringing’ and ‘No In-Band ringing’. |  | [15], [16] |
| BT Car Kit only |  |  |  |
| DS_196 | AT^SABD | Read ID of currently selected audio parameter set. | [04] |
| BT Stereo Headset only |  |  |  |
| DS_203 | AT^SABD AT^SACD | Media player status. | [03] |
| DS_204 | AT^SABD AT^SABC | Media player information. | [03] |
| DS_205 | URC for AT^SABD / AT^SABC | Media player URC. | [03] |
| MPM |  |  |  |
| DS_167 | AT^SBNR extension? Mp3 and Camera folders | Get current used Mp3 and Camera folders | [07] |
| DS_168 | AT command to set date format |  | [07] |
| DS_169 | AT command to set time format |  | [07] |
| DS_170 | AT^SVCL, AT^SVCD, AT^SVCC | Speech file management | [07] |
| DS_171 | AT+CMG | SMS/EMS read, write and delete PC<-> Mobile | [07] |
| DS_172 | AT+JAR (proposal) | Install/uninstall Java applications | [07] |
| DS_182 | AT^SSET | Theme activation | [07] |
| DS_186 | AT+CLIP, AT+CCWA, AT+CCFC | Call setup settings | [07] |
| DS_187 | AT+CCLK, AT+CTZR, AT+CTZU | Clock settings | [07] |
| DS_158 | AT+CMUX=0 |  | [10] |

### Requirements that will NOT be tested

| Requirement Nr | Requirement. Name | Description | REASON |
| --- | --- | --- | --- |
| General |  |  |  |
| DS_135 | ATC command set document | COMNEON shall provide a master document… | Pointless. |
| PoC |  |  |  |
| DS_107 | PoC command | Accept incoming session | No name or Syntax available. Testable if further information available. |
| DS_108 | PoC command | Reject incoming session | No name or Syntax available. Testable if further information available. |
| DS_109 | PoC command | Leave / end session | No name or Syntax available. Testable if further information available. |
| DS_110 | PoC command | Request right to speak | No name or Syntax available. Testable if further information available. |
| DS_111 | PoC command | Release right to speak | No name or Syntax available. Testable if further information available. |
| DS_112 | PoC command | Start ad-hoc session with default user / default group | No name or Syntax available. Testable if further information available. |
| DS_113 | PoC command | Get session information (status, name, URI, URI type, ID) | No name or Syntax available. Testable if further information available. |
| DS_114 | PoC command | Read PoC phone book | No name or Syntax available. Testable if further information available. |
| DS_115 | PoC command | Start PoC session with any group / person | No name or Syntax available. Testable if further information available. |
| DS_116 | PoC command | Get PoC service status | No name or Syntax available. Testable if further information available. |
| DS_118 | PoC URC | URC with content: Session invitation | No information which command / event causes this URC |
| DS_119 | PoC URC | URC with content: Right to speak status | No information which command / event causes this URC |
| DS_120 | PoC URC | URC with content: Session information | No information which command / event causes this URC |
| DS_121 | PoC URC | URC with content: PoC service status | No information which command / event causes this URC |
| Due to the fact that PoC needs a specific application but can not be tested by sending pure AT commands, PoC testing is considered to be covered implicitly when performing real-life tests. These tests will be performed by ST in MCH. |  |  |  |
| BT Car Kit |  |  |  |
| DS_125 | Car loudspeaker | Command to read/write the setting of the car kit menu item ‘Car loudspeaker’ | No name or Syntax available. Testable if further information available. |
| DS_106 | Voice recognition result | Enable/disable voice recognition result | No name or Syntax available. Testable if further information available. |
| DS_198 | Car loudspeaker URC | URC with content: Indication that item ‘Car loudspeaker’ in the car kit menu has been changed by the user | No information which command / event causes this URC. |
| DS_199 | Voice recognition URC | URC with content: Result of the voice recognition | No name or Syntax available. Testable if further information available. |
| DS_200 | PoC URC | URC with content: PoC session started | No name or Syntax available. Testable if further information available. |
| DS_126 | PoC URC | URC with content: PoC session suspended | No name or Syntax available. Testable if further information available. |
| DS_117 | PoC URC | URC with content: PoC session stopped | No name or Syntax available. Testable if further information available. |
| BT Headset |  |  |  |
| DS_140 | BT HS and HF scenarios | ATC shall send a RING as described in [BT_HSP] paragraph 4.2, except in the case where In-Band ringing is used. | Considered to be an internal that can not be tested |
| DS_97 | Initialization for BT HS and HF | After a new BT HS or HF connection is established, RC shall initialize the instance according to specification [15] [16] | This requires complete functionality testing. Testing is considered to be the responsibility of accessory test group. |
| Due to the fact that most of the BT AT commands have to be sent within an existing BT connection these commands can not be tested without a connection but will be tested implicitly when performing real-life tests. These tests will be performed by accessory test groups from ST in ULM and MCH. |  |  |  |
| MPM |  |  |  |
| DS_174 | MPM AT command / URC required | Get phone number associated data. | No name or Syntax available. Testable if further information available. |
| DS_175 | MPM AT command / URC required | Get call duration | No name or Syntax available. Testable if further information available. |
| DS_176 | MPM AT command / URC required | Get and set active profile | No name or Syntax available. Testable if further information available. |
| DS_177 | MPM AT command / URC required | Event for incoming MMS | No name or Syntax available. Testable if further information available. |
| DS_178 | MPM AT command / URC required | Event when the phone’s task reminder gets active | No name or Syntax available. Testable if further information available. |
| DS_179 | MPM AT command / URC required | Event when the phone’s appointment reminder gets active | No name or Syntax available. Testable if further information available. |
| DS_180 | MPM AT command / URC required | Alarm clock event | No name or Syntax available. Testable if further information available. |
| DS_181 | MPM AT command / URC required | Loudspeaker hands free | No name or Syntax available. Testable if further information available. |
| DS_183 | MPM AT command required | Display settings | No name or Syntax only partly available. Testable if further information available. |
| DS_184 | MPM AT command required | Ringer settings | No name or Syntax only partly available. Testable if further information available. |
| DS_185 | MPM AT command required | Phone setup settings | No name or Syntax available. Testable if further information available. |
| DS_188 | MPM AT command required | Accessory settings | No name or Syntax available. Testable if further information available. |
| DS_189 | MPM AT command required | Camera settings | No name or Syntax only partly available. Testable if further information available. |
| DS_190 | MPM AT command required | Alarm clock | No name or Syntax only partly available. Testable if further information available. |
| DS_191 | MPM AT command required | T9 wordlists | No name or Syntax only partly available. Testable if further information available. |
| AppConnect |  |  |  |
| DS_87 | Register / Unregister for indications |  | Considered to be an internal that can not be tested |
| DS_94 | Instances and parameters |  | Considered to be an internal that can not be tested |
| DS_95 | No interface from AppConnect |  | Considered to be an internal that can not be tested |
| DS_90 | Gipsy component |  | Considered to be an internal that can not be tested |
| DS_123 | PC Simulation |  | Considered to be an internal that can not be tested |
| Multiplexer |  |  |  |
| DS_161 | Start |  | Considered to be an internal that can not be tested |
| DS_162 | Default state |  | Considered to be an internal that can not be tested |
| DS_143-DS_152, DS_154,155,156, DS_163 |  |  | Considered to be Multiplexer internals |

## Limitations

- The testing is limited to products within the X75 platform that are based on SGoldv2.

- The AT commands are tested for test-, read-, write- and execute functionality by means of ATC only- tests will not include functionality testing (BT Car Kit, BT Head set, MPM etc.). This is considered to be the responsibility of accessory / MPM testing, because they execute the use cases needing the particular AT commands.

Accessory will be tested by Peter Schwer in ULM.

MPM will be tested in Beijing, Manaos and MCH.

- Due to the high number of commands, each particular parameter of a command will be tested in general.

This means, significant values as well as upper- and lower boundaries will be tested.

NOT every settable value for a parameter will be tested.

- Due to the fact that all tests are at least semi- automated (that means automated in interaction with the tester or fully automated), each particular script contains all tests related to one command, e.g. the functional-, boundary- and stress tests for the command AT+CMUT (Mute control) will be performed with the script ‘CMUT.txt’. This has the advantage that test effort is lower and commands are grouped logically, because it is not easy to clearly separate the black box integration- from the system test.

- Testing is based on the specifications mentioned in Section 1.3 (References). The tester has to rely on these specifications and is not responsible for the content and the completeness.

- Since the requirements specification [1] does not contain any special requirements regarding other accessory than BT Car kit and BT headset, it is assumed that all AT commands referred to  in requirements document are sufficient to cover the needed command set for all accessory.

- The field test is considered to be the responsibility of the Field test group (Jörg Langenberg) and will not be tested by ST in KLF.

## Required tools

ATT 2.0

http://ezk.klf.siemens.de/Dienststellen/bildergalerie/iut_aufgaben/index.html

Test scripts for every AT command to test can be found in EasyDM:

http://mchp72ra.mch.sbs.de:8002/sap/bc/bsp/sap/z_easydms/default.htm?sap-cli	ent=400&easydms-dokar=F01&easydms-doknr=102171&easydms-doktl=000&easydms-	dokvr=01&easydms-docfile=&easydms-action=

Serial data cables for all mobiles

At least two mobiles

At least two SIM cards

Two stationary telephones with separate numbers

PC running Windows XP / 2000

## Test Stages

### Black-Box Integration Test

The aim of the black box integration test is to certify that all functionality is implemented as specified. This is a straight-forward test.

The mobile will not be confronted with stress situations.

Boundaries of parameters are also out of focus – this is subject to the system test.

The black box integration test is primarily composed of functional tests. The main objective of the black box integration test is to declare the S25 milestone.

#### AT Test commands

##### Objective

It must be proofed that the test command works for all AT commands where it is specified.

This means the mobile returns the parameters and domains that should be supported for the AT command.

Example:

The command AT+CIND is for mobile equipment control and comes with a set of categories (e.g. battery charge status, signal strength etc.).

The test command AT+CIND=? mandates the mobile to show supported indicator categories and ranges.

Terminal sends:	 AT+CIND=?

Mobile responses:	+CIND:

("battchg",(0-5)),

("signal",(0,5)),

("service",(0,1)),

("message",(0,1)),

("call",(0,1)),

("roam",(0,1)),

("smsfull",(0,1))

OK

The parameter mean, that e.g. for category ‘battery charge’ the level zero to five is implemented and can be used for level indication.

##### Strategy

The general procedure for all test stages is that the commands to test as well as the expected responses have to be coded in a test script that will be pre-compiled by ATT 2.0’s Python pre-compiler.

The result is a python script that will be interpreted by ATT 2.0’s python interpreter and processed by ATT 2.0.

ATT 2.0 sends AT commands to the mobile, waits for the expected responses and tracks all information and received responses into an html result file.

This file has to be checked manually by the tester.

For all tests appropriate scripts are used (see following example).

Figure 1: Example of script file

##### Test Start Criteria

The PL ST 1 will decide if the software has reached the right maturity to start a meaningful BBIT. He will inform the different sites to start. The following points must in addition be fulfilled for this:

-	White box integration tests concerning ATC have been successfully passed

-	The hardware should be functional available

-	ATT 2.0 and the test files are available

-     The mobile is connected to PC via serial data cable

-	The mobile is switched on

-	The Com port is configured with the correct communication parameters

- 	The test function is implemented and specified for the particular AT command

##### Completion Criteria

The tests have been successfully completed when the mobile returns the specified parameters and domains. In the summary of the HTML result file all tests should have been passed.

The defined subset of test cases for BBIT was successfully passed. Project Management (PM) gets Information/Documentation about the functionality of the mobile from the PL ST 1.

COMMENTS in yellow rows

PASSED test cases. The expected response is only part of the received response. (rows in light green)

The AT COMMAND sent to mobile on the mentioned port.

(blue row)

FAILED test cases. The expected response is not received (red rows)

Figure 2: Example of html result file

#### AT Read commands

##### Objective

Objective of the black box integration test for read commands is to ensure that the mobile returns status information for the requested command and that this information is realistic.

##### Strategy

Example:

The command AT+CIND is for mobile equipment control and comes with a set of supported categories (e.g. battery charge status, signal strength etc.)

The read command AT+CIND? requests the current values for all supported indicator categories from mobile.

Terminal sends:	 AT+CIND?

Mobile responses:	+CIND: 5,2,1,0,0,0,0

OK

This response shows the actual values for the supported categories:

Battery charge  		= 	5 means ‘good’

Quality of signal 	= 	2 means ‘less than middle’

Service  		= 	1 means ‘Service available’

Message 		= 	0 means ‘No unread message in memory storage’

…etc.

The test procedure with scripts is used as mentioned before.

##### Test Start Criteria

The PL ST 1 will decide if the software has reached the right maturity to start a meaningful BBIT. He will inform the different sites to start. The following points must in addition be fulfilled for this:

-	White box integration tests concerning ATC have been successfully passed

-	The hardware should be functional available

-	ATT 2.0 and the test files are available

-     The mobile is connected to PC via serial data cable

-	The mobile is switched on

-	The Com port is configured with the correct communication parameters

- 	The read function is implemented and specified for the particular AT command

-	The technical context for the particular AT command has to be given.

This will either be done by the script automatically or the script will inform the tester what has to be done manually

(e.g.: If the entries from call records list have to be read, this list must not be empty. If two instances for remote control must be tested, ATT2.0 will inform the tester to establish the two connections manually.)

##### Completion Criteria

The mobile has to return values for all supported category parameter as specified.

The test cases must be passed as recorded to html file.

#### AT Write commands

##### Objective

Objective of the black box integration test for write commands is to certify that setting parameters has the specified effect.

The responses from mobile must be according to specification.

##### Strategy

Example:

The command AT+CIND is for mobile equipment control and comes with a set of settable categories (e.g. battery charge status, signal strength etc.)

The write command AT+CIND = 1,0,0,1 sends a request to the mobile to switch the indicator for the first, second, third and forth category ON and respectively OFF.

Terminal sends:	 AT+CIND = 1,0,0,1

Mobile responses:	OK

A following read command is used to check whether all values set before are applied.

All this testing is performed as usual with script files.

##### Test Start Criteria

The PL ST 1 will decide if the software has reached the right maturity to start a meaningful BBIT. He will inform the different sites to start. The following points must in addition be fulfilled for this:

-	White box integration tests concerning ATC have been successfully passed

-	The hardware should be functional available

-	ATT 2.0 and the test files are available

-     The mobile is connected to PC via serial data cable

-	The mobile is switched on

-	The Com port is configured with the correct communication parameters

- 	The write function is implemented and specified for the particular AT command

-	The technical context for the particular AT command has to be given.

This will either be done by the script automatically or the script will inform the tester what has to be done manually

(e.g.: If the entries from call records list have to be read, this list must not be empty. If two instances for remote control must be tested, ATT2.0 will inform the tester to establish the two connections manually.)

##### Completion Criteria

The tests have been successfully completed when the mobile responses to the write command are as specified and a following read command confirms the values set before.

In the summary of the HTML result file all tests should have been passed.

#### AT Execute commands

##### Objective

Objective of the black box integration test for execute commands is to proof, that the mobile executes the given command as specified.

##### Strategy

Example:

The execute command AT+CPAS sends an execute request to the mobile to query the telephone status.

Terminal sends:	 AT+CPAS

Mobile responses:	+CPAS: 0

OK

This means that the phone is ready. The response +CPAS: 3 would indicate that the phone is ringing. The response +CPAS: 4 means, that a call is active.

All this testing is performed as usual with script files.

##### Test Start Criteria

The PL ST 1 will decide if the software has reached the right maturity to start a meaningful BBIT. He will inform the different sites to start. The following points must in addition be fulfilled for this:

-	White box integration tests concerning ATC have been successfully passed

-	The hardware should be functional available

-	ATT 2.0 and the test files are available

-     The mobile is connected to PC via serial data cable

-	The mobile is switched on

-	The Com port is configured with the correct communication parameters

- 	The execute function is implemented and specified for the particular AT command

-	The technical context for the particular AT command has to be given.

This will either be done by the script automatically or the script will inform the tester what has to be done manually

(e.g.: If the entries from call records list have to be read, this list must not be empty. If two instances for remote control must be tested, ATT2.0 will inform the tester to establish the two connections manually.)

##### Completion Criteria

The mobile executes the command given in the specified way. In the example above the responses must indicate the actual mobile status. All test cases must be passed successfully.

### System test

The aim of the system test is to certify that

- All functions are implemented (successful black box tests),

- All functions work under stress aspects

  - incoming voice-, fax- or data call

  - incoming message

  - interrupted AT commands

  - arbitrary key presses on mobile’s key pad

  - ……….open for further ideas……………..

- The mobile stays stable when confronted with invalid parameters and gives the specified error messages

The system test is composed of all functional, boundary and stress tests. The main objective of the system test is to declare the DS, S3 and S4 milestones.

#### AT Test commands

##### Objective

The aim of this test is that the mobile stays stable when confronted with many requests.

The mobile responses to the requests have to be according to specification.

##### Strategy

To stress the mobile with commands, one aspect of testing might be a script that includes commands to call a mobile and sends many AT test commands to the ringing mobile.

Also interrupting a still processed command (e.g. a time-consuming network command) by sending another one immediately to the mobile should be tested.

There is no limitation on ideas how to stress the mobile – a realistic compromise has to be found.

All this testing is performed as usual with script files.

##### Test Start Criteria

The PL ST 1 will decide if the software has reached the right maturity to start a meaningful system test. He will inform the different sites to start. The following points must in addition be fulfilled for this:

-	White box integration tests concerning ATC have been successfully passed

-	The hardware should be functional available

-	ATT 2.0 and the test files are available

-     The mobile is connected to PC via serial data cable

-	The mobile is switched on

-	The Com port is configured with the correct communication parameters

- 	The test function is implemented and specified for the particular AT command

##### Completion Criteria

The defined test cases for system test were successfully passed.

Product shows a stable, robust and error-free behavior.

Product handles error situations gracefully.

Project Management (PM) gets Information/Documentation about the functionality of the Mobile from the PL ST 1.

#### AT Read commands

##### Objective

The aim of this test is that the mobile stays stable when confronted with many requests.

The mobile reactions to the requests have to be according to specification.

##### Strategy

To stress the mobile with commands, one aspect of testing might be a script that includes commands to call a mobile and sends many AT test commands to the ringing mobile.

Also interrupting a still processed command (e.g. a time-consuming network command) by sending another one immediately to the mobile should be tested.

There is no limitation on ideas how to stress the mobile – a realistic compromise has to be found.

All this testing is performed as usual with script files.

##### Test Start Criteria

The PL ST 1 will decide if the software has reached the right maturity to start a meaningful system test. He will inform the different sites to start. The following points must in addition be fulfilled for this:

-	White box integration tests concerning ATC have been successfully passed

-	The hardware should be functional available

-	ATT 2.0 and the test files are available

-     The mobile is connected to PC via serial data cable

-	The mobile is switched on

-	The Com port is configured with the correct communication parameters

- 	The read function is implemented and specified for the particular AT command

##### Completion Criteria

The mobile must stay stable and have a well – defined state all the time. The responses to the test commands as specified.

All test cases must be passed successfully.

#### AT Write commands

##### Objective

The aim of this test is that the mobile stays stable when confronted with many write requests and responds to all commands in the way described in the specification.

Furthermore it should be checked that executing many write commands in a short time still changes the mobile state(s) as expected.

It must be ensured that exceeding the specified boundaries with the write command results in the specified error messages and does not have any other effect on the mobile SW.

##### Strategy

A loop containing an AT write command and the corresponding read command might be a good idea for a stress test. This loop should be processed many times very fast. A comparison of the written value with the values returned from mobile ensures that the value was set indeed.

Another example:

The command AT^SSTK=xxx installs or removes an external menu to respectively from the mobile menu tree.

With the appropriate script file it can be tested, whether performing the install/remove action very often has any effect on the stability of the mobile SW.

Try to exceed the boundaries of a command.

Example: The AT write command AT+CALM=x accepts the parameter 0,1 and 2.

The command AT+CALM=3 should result in ERROR. The mobile must stay stable.

This example can be transferred to any other AT command.

##### Test Start Criteria

The PL ST 1 will decide if the software has reached the right maturity to start a meaningful system test. He will inform the different sites to start. The following points must in addition be fulfilled for this:

-	White box integration tests concerning ATC have been successfully passed

-	The hardware should be functional available

-	ATT 2.0 and the test files are available

-     The mobile is connected to PC via serial data cable

-	The mobile is switched on

-	The Com port is configured with the correct communication parameters

- 	The write function is implemented and specified for the particular AT command

-	The technical context for the particular AT command has to be given.

This will either be done by the script automatically or the script will inform the tester what has to be done manually

(e.g.: If the entries from call records list have to be read, this list must not be empty. If two instances for remote control must be tested, ATT2.0 will inform the tester to establish the two connections manually.)

##### Completion Criteria

The tests have been successfully passed when all write commands had the specified effect and the mobile SW stays stable all the time. It must not have been possible to exceed parameter boundaries. When trying so, appropriate error messages must have been given.

In the summary of the HTML result file all tests should have been passed.

#### AT Execute commands

##### Objective

The aim of this test is that the mobile stays stable when confronted with many AT execute commands  and responses to all commands in the way given by the specification.

##### Strategy

As mentioned in sections above, stress testing also will be performed by interrupting currently being processed.

One AT command is sent to mobile that takes time to execute, e.g. a network- related command. Before executing finishes, another time-consuming command could be sent immediately.

##### Test Start Criteria

The PL ST 1 will decide if the software has reached the right maturity to start a meaningful system test. He will inform the different sites to start. The following points must in addition be fulfilled for this:

-	White box integration tests concerning ATC have been successfully passed

-	The hardware should be functional available

-	ATT 2.0 and the test files are available

-     The mobile is connected to PC via serial data cable

-	The mobile is switched on

-	The Com port is configured with the correct communication parameters

- 	The execute function is implemented and specified for the particular AT command

-	The technical context for the particular AT command has to be given.

This will either be done by the script automatically or the script will inform the tester what has to be done manually

(e.g.: If the entries from call records list have to be read, this list must not be empty. If two instances for remote control must be tested, ATT2.0 will inform the tester to establish the two connections manually.)

##### Completion Criteria

The tests have been successfully completed when the AT commands have been executed as expected and the mobile SW always stayed stable.

In the summary of the HTML result file all tests should have been passed.

### Field Test

The field test is considered to be the responsibility of the Field test group ( Mr. Jörg Langenberg ICM MP CCQ QM PV) and will not be tested by ST in KLF.

Responsible person for field test has to create a field test plan.