Siemens C62 Flash tool

Installation:

- Install “flash_programmer_for_siemens_setup.exe”

- Copy “Hd0503-058-m” and “Hd0503-067-m” into “C:\Program Files\microcell\Flash Programmer” folder.

- Install “PL-2303 Driver Installer.exe”

Settings:

- Select Options -> Settings ->C62

- Set Baud rate to 460800 bit/s

- Select Root folder for software releases

- Choose Software release

- Choose Language package

- Choose Customization package

# Phone flashing process

- Switch OFF the phone

- Plug-in flashing cable

- Press Flash button

- Press shortly power button

!!!WARNING!!!

DON'T TOUCH ANYTHING UNTIL THE FLASHING PROCESS COMPLETE