- Dual Mode UMTS Terminal U10

- Product Limitations Description Sheet

| # Date | April 15, 2003 |
| --- | --- |
| Product Name | U10 |
| # Marketing Name | Siemens U10 |
| Software Revision | 59.0E.Prod (.2A) |
| Flex File | SE4267AXXR802B SE4307BXXR800B |
| Language Pack | UMTS_lang0023 |
| KJava | None |
| Model N umber | SE4267AN4R8 SE4307BN4R8 SE4307CN4R8 |

The product limitations are listed in sections 1-5.

| Section Number | Unsatisfied Requirement | Action to Improve | Detailed Enclosed (Yes/No) |
| --- | --- | --- | --- |
| 1 | Hardware | Added Battery | Yes |
| 2 | Software | Production Build | Yes |
| 3 | Field Test Status | Production Build | Yes |
| 4 | Industry Certification Status | Complete | Yes |
| 5 | Government / Safety Compliance | Complete | Yes |

- Table 1: Hardware

Battery performances:

Standby:  The Terminal shall remain in idle mode for a minimum of 100 hours.

Talk time:  When the Terminal is transmitting at +12dBm with a voice activity factor of 40% in W-CDMA mode the Terminal's battery shall last for a minimum of 70 minutes."

- Table 2: Software

Only High Impact issues are reported here. High Impact is defined based on the probability of occurrence and the severity of the problem.

| Tracking No | Description of Defect |
| --- | --- |
| LIBbb73565 | Required emergency numbers not stored on phone |
| LIBbb77277 | Tall image not displayed correctly when inserted in MMS Letter. Instead of being resized to fit the screen, it is just displayed as actual size. This causes the image to overwrite some of the text in the MMS and corrupt it. |
| LIBbb78441 | Panic in History web function |
| LIBbb78544 | FT: SMS class 0 management - When the phone receive a class 0 message, the phone displays that I have 1 or more message in the voicemail. To call back the voicemail, I need to enter in menu and select call back. At this step the phone do not select properly the voicemail number but select the voicemail number + the number of message in the voicemail. |
| LIBbb79182 | Wide object inserted in MMS is not correctly resized. An attempt to insert the attached wide object to an MMS letter causes the object to be incorrectly resized. |
| LIBbb80066 | Wrong Italian prompt - After the Italian language In Facilities menu is set, the menu “Quick Dial” does not translate - "Quick Dial" appears instead of the correct "Selezione Rapida" |
| LIBbb81861 | Cannot receive a call when sending email with an attachment. - The issue is when send the mail with the video or picture attach and the phone start the conversion to 7 bit data from binary. During the time that the phone is converting the video, you cannot receive a call. |
| LIBbb84683 | Display rendering is disabled when display timeout is active. The UIS rendering for animated operator logo in idle is continuing after display timeout is active. Nothing is showing on the display since the display panel is not being refreshed. However there is significant current drain due to the expiration of timers and the execution of the decode and rendering code. |
| LIBbb85732 | KJava - Secure Socket being closed when it was never opened. While KJava is trying, if the user presses End, KJava closes a secure socket that was never actually opened which then causes a panic. |
| LIBbb86161 | Unable to power down the phone while downloading MIDlet over the air. |
| LIBbb86376 | Invalid email address is not flagged. The validity of the mail address should be checked. It is not and the mail is sent to the invalid address. |
| LIBbb86756 | Phone makes beeping sound when hutch key is pressed. While MIDlet(ADD_RECORD) is running and Pressing Hutch key, Phone continuously make beeping sound and only way beeping sound can be stop is by pressing 'END' key or removing battery |
| LIBbb86828 | When open MMS message with (92.5Kb Video file) from INBOX, Phone panics. |
| LIBbb86853 | IT:Panic when played MP3 file after taking MMC card out |
| LIBbb86853 | Panic when played MP3 file after taking MMC card out. Also noticed that the panic does NOT occur if wallpaper is set. If the wallpaper is set to NONE, you get the panic every time. |
| LIBbb87111 | Email password can be copied and pasted. TD # 24589 |
| LIBbb87532 | Phone panics while creating shortcut in Browser. - The user has to go into a web session, press MENU button, go to New Shortcuts, and try to create shortcut on the New Shortcut item 4 times to get this panic. |
| LIBbb87728 | Screen save set but does not activate. TD # 24524 - When PDP context is ON, the screensaver is not activated after 1 minute. |
| LIBbb89097 | FT: PANIC After Attempting to Store Email Address Embedded In EMS |
| LIBbb89099 | FT: Cannot Store Phone Numbers In EMS With Inserted Pictures |
| LIBbb89427 | No emergency call in wcdma without SIM - If I set in Network Setup Band to Automatic and (with out SIM) in a zone with only GSM signal I can make the call.(log no_sim_auto_netw.zip) If I set in Network Setup Band to WCDMA and (without sim)in a zone with only GSM signal I can not make the call on the display I can see call fail (log no_sim_wcdma_setting.zip) |
| LIBbb89540 | Panic when MMS video attach is played during MIDlet download; TD #25295 |
| LIBbb90007 | Phone panics during download of java midlet. |
| LIBbb90411 | Supported formats list attribute incorrectly formatted in SAM - This has originated because of incorrect interpretation of an ambiguous statement in the BT OBEX profile specification. This problem was not found earlier because it is handled by all the PC BT cards and PDAs that we have tested with so far. The Brain box PC card and Palm m515 are the first devices that could not handle this. |
| LIBbb90540 | PANIC (10A00) when FTP uploading 1 MB file. 100% Repeatable |
| LIBbb90574 | FTA test 44.2.3.2.4 fails - GCF test Failure fail 44.2.3.2.4 (Routing area updating procedure/ Combined routing area updating /Combined routing area updating / rejected / PLMN not allowed). The phone powers down/freezes around step 15. |
| LIBbb90726 | IT: Phone did a panic when trying to make a SHO call on WCDMA. |
| LIBbb90841 | Core DUMP (10304) occurred after power-up when doing automatic power-up and network search. |
| LIBbb90848 | UE needs 30&#180; to come back to HPLMN after attempt to register to forb - UE camped and registered in its HPLMN and Manual Available Networks is executed. After the UE showed the list of available networks, a forbidden UMTS Network is selected (Xfera in this particular case) and tried to register in it. The UE comes back to its HPLMN only after 30 minutes of this attempt. |
| LIBbb91177 | UE losses coverage when accessing items in Call Forward menu. The time that the UE losses coverage/service is about 15-20 seconds. This is verified by the coverage icon in the screen but also because there is no service during this time (no MO or MT calls for example can be received ("No service" at screen). |
| LIBbb91907 | UE Panic when Java application is running and data call is received. |
| LIBbb92113 | Web session terminated by timeout. |
| LIBbb92412 | Phone panic in standby |
| LIBbb93230 | CORE DUMP experienced after power up the UE. The UE registered properly in the NW (Telefonica) and showed service but after seconds it core dumped. |
| LIBbb93619 | UE losses coverage when accessing Call Barring menu for about 20-30 seconds. Go to Main menu -> Settings -> Security -> Call Barring As soon as either "outgoing calls" or "incoming calls" is pressed the UE losses coverage. This can be noticed by the beep from the service tone previously enabled and that the coverage disappears at UE screen. |
| LIBbb93559 | PANIC experienced after a successful data session: non-transparent with a 50Kb size file. After this data session finished. The data session was reconfigured from non-transparent to transparent at hyper-terminal, the UE accepted the change but after a short time it panicked. |

- Table 3: Field Test Status

The EMEA Field Test team has completed 50% of the planned test cases for this software load. Table 2 includes the High Impact CRs resulted from this testing to date. Furthermore, Phase 1 features are not tested at all in the field.

- Table 4: Industry Certification Status

- The Software 59.0E.2A passed the GCF tests.

- Table 5: Government / Safety Compliance

- The Software 59.0E.2A passed Safety Compliance tests.