M81_Hawk

Certifications Requirement Specification

References

[1] ST03 Compliance Assessment Process

Enclosures

None

Revision Information

| Created: | Revision | Name | Comments |
| --- | --- | --- | --- |
| 03.04.2006 | 0.1 – based on Philippe CRS | Oluf Kristensen | Initial revision |
| Modified |  |  |  |

Contents

1	Introduction	2

2	Abbreviations and Definitions	2

3	Regulatory requirements - Overview	3

3.1	European Community	3

3.2	Outside Europe	3

4	Voluntary requirements - Overview	3

4.1	GCF	3

4.2	PTCRB	3

4.3	Additional voluntary requirements	3

5	Requirements Matrix	4

5.1	Regulatory requirements	4

5.2	Voluntary requirements	5

6	Regulatory requirements - detailed	5

6.1	Health aspect	5

6.2	Safety aspect	6

6.3	EMC aspect	6

6.3.1	Harmonised Standards for EMC aspects	7

6.4	Radio spectrum usage aspect	7

6.4.1	Harmonized standards for GSM	8

6.4.2	Harmonized standards for Bluetooth	8

6.5	Special requirements	8

6.5.1	European Community	8

7	Voluntary requirements - detailed	9

7.1	Global Certification Forum	9

7.2	PCS Type Certification Review Board	10

7.3	Bluetooth functionality	10

8	Marking, labelling and User Guide	10

8.1	Terminal marking and labelling	10

8.1.1	CE marking layout and requirements	11

8.1.2	Notified Body Identifiers	11

8.2	User Guide	11

# Introduction

This document describes certification requirements for M81_Hawk supporting GSM/GPRS/EDGE in terms of regulatory requirements within EC and voluntary requirements within EC and US.

Therefore, this document covers aspects of R&TTE and GCF certifications. M81 is not subject to PTCRB certification.

The document provides additional information of relevance to FCC certification, but does not intend to cover all aspects of FCC certification.

The FCC information contained in this document is for ST internal purposes only. The full information related to FCC is in the responsibility of EA as defined in [1].

This document does intentionally not handle requirements for APAC, in particular China.

In the following, GSM/GPRS/EDGE terminals are referred to as GSM terminals.

# Abbreviations and Definitions

| Abbreviation | Definition |
| --- | --- |
| Bluetooth SIG | Bluetooth Special Interests Group |
| EA | EMC/Antenna; i.e. a sub-department within RF department |
| EC | The European Community |
| EMC | Electro-Magnetic Compatibility |
| FCC | Federal Communications Commission |
| GCF | Global Certification Forum |
| GSMA | GSM Association. The Association of GSM Network Operators worldwide. |
| GSMNA | GSM North America. The North American Regional Interest Group of the GSM Association. |
| HAC | Hearing Aid Compatibility |
| IEC | International Electro-technical Committee |
| IC | Industry Canada |
| PTCRB | PCS Type Certification Review Board |
| R&TTE | Directive 1999/5/EC – Radio Equipment and Telecommunications Terminal Equipment Directive |
| SAR | Specific Absorption Rate |
| (U)SIM | (UMTS) Subscriber Identity Module |
| SAT | SIM Application Toolkit |
| SDO | Standards Development Organisation |
| UMTS | Universal Mobile Telecommunications System |
| 3GPP | Third Generation Partnership Project. A project of SDOs in charge of elaboration of specifications for a 3. Generation mobile radio system (UMTS). |

# Regulatory requirements - Overview

## European Community

GSM and UMTS terminals are under the scope of Directive 1999/5/EC, Radio Equipment & Telecommunications Terminal Equipment (R&TTE). Directive 1999/5/EC is the overall framework for radio equipment and telecommunications terminal equipment for all countries in the European Community (EC) and defines requirements for:

- Health and safety aspects

- EMC aspects

- Spectrum usage aspects

## Outside Europe

For North America and other countries outside the EC, the technical requirements are almost identical to those applied in the EC. Additional requirements are listed in the requirements matrix.

# Voluntary requirements - Overview

## GCF

The scope of GCF is terminal   network interoperability + additional Application Enablers under running introduction as controlled by GCF SG.

## PTCRB

The scope of PTCRB is terminal network interoperability + additional Application Enablers under running introduction as controlled by PTCRB.

## Additional voluntary requirements

Besides regulatory requirements and GCF/PTCRB requirements, the terminal shall comply with some further supplementary, “voluntary” certification schemes e.g. Bluetooth Qualification as described in the requirements matrix.

# Requirements Matrix

## Regulatory requirements

| Aspect | Region |  |  |  |
| --- | --- | --- | --- | --- |
|  | Europe |  | US (& Canada) |  |
|  | Mandated by | Technical requirement | Mandated by | Technical requirement |
| Health and safety | R&TTE Art 3.1.a | Health aspect: Specific Absorption Rate (SAR) EN 50 360/361: 2001 | FCC | FCC SAR Regulation OET 65 |
|  |  | Safety aspect: Electrical safety EN 60 950-1: 2001 | - | - |
| EMC | R&TTE Art. 3.1.b | EN 301 489-1: V1.5.1 EN 301 489-7: V1.2.1 EN 55013:2001 EN 55020:2002 EN 55022:1998 IMT2000 UTRA FDD: EN 301 489-24: V1.2.1 Bluetooth: EN 301 489-17: V1.2.1 | FCC | FCC Part 2 FCC Part 15 FCC part 22 FCC Part 24 |
| Radio Spectrum | R&TTE Art 3.2 | GSM: EN 301 511:V9.0.2 IMT 2000 UTRA FDD UE: EN 301 908-1: V2.2.1 EN 301 908-2: V2.2.1 |  |  |
|  |  | Bluetooth: EN 300 328: V1.6.1 (EN 300 440-2) : V1.1.1 |  |  |
| Special requirements | R&TTE Art. 3.3 | None specified for Europe at the moment. IMEI security is discussed | FCC | TTY: Cellular text Telephone Modem according to 3GPP TS 26.226 TS 26.230 TS 26.231 |
|  |  |  |  | 5-key nob |
|  |  |  |  | USA FCC Wireless E911 Rules |
|  |  |  |  | HAC: ANSI C63.19 |

Approval for Canada (Industry Canada, IC) will be done jointly with FCC approval.

## Voluntary requirements

| Aspect | Specified by | Technical requirements |
| --- | --- | --- |
| Terminal <-> network interoperability and Application Enablers | GSM and UMTS: Global Certification Forum (GCF) | GCF-CC Database Field Trials IOP testing |
| Terminal <-> network interoperability and Application Enablers | GSMNA | PTCRB NAPRD.03 |
| Bluetooth functionality | Bluetooth Special Interest Group | Bluetooth Qualification Program Reference Document |

# Regulatory requirements - detailed

## Health aspect

Health is an integral aspect of Article 3.1.a of EU Directive 1999/5/EC.

Specific Absorption Rate is subject to regulatory approval. Harmonised Standards for SAR; i.e. EN 50 360 is available and can be used for demonstrating compliance to R&TTE.

| Relevant harmonized standard for article 3.1a) | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 50360 | 2001 | Requirements for mobile phones for human expose to EM fields | --- |

In addition to demonstrating technical compliance it is required to disclose actual SAR values to the public by means of a company WEB page.

Health aspects are handled by FCC in US.

The FCC has continuously further developed SAR limits to include requirements for e.g. Body-worn configurations reflected in FCC OET65. Within EU, standardisation of requirements for Body-Worn configuration is still ongoing as of Dec 2005. International standardisation is focused on IEC 62 209. SAR values for US are made public available at FCC Website. In addition, both FCC require detailed wordings to be included in user manuals.

## Safety aspect

Safety for stand-alone terminals operating at a few Volts is a new aspect introduced by Article 3.1.a of Directive 1999/5/EC in EC.

EN 60 950 is the corresponding Harmonised Standard applicable within the EC.

EN 60 950 concerns:

- Protection against electrical shock and hazards

- Protection against thermal rises

- Resistance to heat and fire

| Relevant harmonized standard for article 3.1a) | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 60 950 | 2000 | Safety of information technology equipment | 1.7.2006 |
| EN 60 950-1 | 2001 | Safety of information technology equipment | --- |

National standards for other regions e.g. AS 3260 for Australia and UL60950 for US are all based on IEC 950, from which also EN 60 950 is derived.

Compliance to safety regulations includes flammability requirements for plastic materials documented by corresponding Underwriters Laboratories (UL) File Listings (Yellow Cards) and Certificates of Conformity with all deliveries. PCBs as well as batteries need to be UL recognised.

## EMC aspect

EMC is an integrated requirement of Directive 1999/5/EC, identified as an essential requirement in Article 3.1.b.

### Harmonised Standards for EMC aspects

Updated information’s on Harmonised Standards for EMC can always be found at the European Commissions website: http://europa.eu.int/comm/enterprise/newapproach/standardization/harmstds/reflist/emc.html

| Relevant harmonized standard for article 3.1b) | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 301 489-1 | V1.4.1 | common requirements for a large range of product types | 11.08.2008 |
| EN 301 489-1 | V1.5.1 | common requirements for a large range of product types | --- |
| EN 301 489-7 | V1.2.1 | specific requirements for GSM terminals operating in the 900 MHz and 1800 MHz frequency range | --- |
| EN 301 489-17 | V1.2.1 | specific requirements for short-range devices; i.e. Bluetooth functionality | --- |

For US, the FCC, who is in charge of radio spectrum management, handles the EMC aspect. The corresponding technical requirements are defined in FCC Part 2, Part 15, Part 22 and Part 24. Separate EMC testing and certification is mandatory in US.

The FCC EMC requirements only concern emission. No immunity requirements are applied in US.

HAC – “Hearing Aid Compatibility”: With order from August 2003, the FCC requires major handset manufactures to bring at least two phones supporting HAC on the US market. The transition period exceeded by August 2005 for EMC requirements (reduced emissions,  ANSI C63.19 rating “U3”) and one year later for additional telecoil-coupling ("UT3"). Detailed information is available within EA and at the FCC website.

## Radio spectrum usage aspect

Directive 1999/5/EC identifies in Article 3.2 radio spectrum usage as an essential requirement because radio spectrum is a limited resource.

Information of ETSI about harmonized standards:

http://portal.etsi.org/erm/kta/R&TTE/rtte.asp

Updated information’s on Harmonised Standards for R&TTE can always be found at the European Commissions website:

http://europa.eu.int/comm/enterprise/newapproach/standardization/harmstds/reflist/radiotte.html

### Harmonized standards for GSM

| Relevant harmonized standard for GSM article 3.2 | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 301 511 | V9.0.2 | Specific requirements for GSM terminals operating in the 900 MHz and 1800 MHz frequency range including EGPRS and pointing to TS 151 010. | --- |

### Harmonized standards for Bluetooth

| Relevant harmonized standard for Bluetooth for article 3.2 | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 300 328 | V1.6.1 | Short range device ISM 2.4 GHz | --- |
| EN 300 440-2 | V1.1.1 | Short Range 1-40GHz Conformity to EN 300 440-2 is not mandatory but helps to prevent extra declarations for France. | --- |

For US, the FCC handles radio spectrum aspects. See EMC section above.

## Special requirements

### European Community

Directive 1999/5/EC provides in Article 3.3 several options to include further special requirements concerning:

- Network inter-operability / support for certain network interface connecting points.

- Avoiding harm to the network and degradation of service.

- Safeguards to ensure personal data and privacy of users and subscribers.

- Support of certain features to avoid fraud.

- Support of certain features ensuring access to emergency features.

- Support of certain features to facilitate terminal usage by users with a disability.

By Dec 2005, no such requirements are defined under Article 3.3 in EC, but inclusion of IMEI security is under discussion.

# Voluntary requirements - detailed

## Global Certification Forum

The Global Certification Forum (GCF) is dealing with the verification of terminals against the GCF’s technical requirements with global recognition and acceptance of results to ensure interoperability between terminals and networks as well as interoperability of application enablers.

In the scope of GCF are GSM terminals operating in the 900/1800 MHz bands and UMTS FDD terminals operating in UMTS FDD band I.

GCF is providing a common set of certification criteria for the following areas:

- RF

- Protocol,

- SIM/SAT/USIM,

- Audio

- Application enabler e.g. Multimedia Messaging Service (MMS), Push-to-talk over Cellular (PoC).

- Field Trials

Certification criteria for further application enablers Video Telephony (VT) and Instant Messaging (IMPS) 1.2.1 are likely to be included into GCF early 2006.

The corresponding technical requirements are defined in:

- GCF–CC: Certification Criteria

- CC data base at http://gcftech.org

Note: Registration is needed to access the GCF–CC data base.

To obtain maximum synergies from C81 Philippe, the M81 Hawk shall be compliant to and partly tested against GCF-CC V3.21.x. The certification shall be finalised before August 1, 2006. Only testing of Acoustics (51.010-1 section 30) and Radiated Spurious Emission (51.010-1 section 12.2*) is foreseen. All other test results to be reused from C81 Philippe.

New certification criteria are continuously added as new features and services are introduced in the GSM networks. Compliance can be demonstrated by conducting tests in test laboratories using validated test cases running on commercially available test equipment.

In addition there are requirements for Field Trials. Field Trials have to be performed on live networks and are intended to cover dynamic scenarios, which can not be performed in laboratory environments. Field Trials shall be performed in at least five network configurations per supported band, representing infrastructure implementations from all major suppliers.

During spring 2006 GCF defines specific applicable test cases for Field Trials.

Furthermore, a GCF certification declaration shall be accompanied by a list of 3GPP core specifications and its corresponding versions which have been the basis for the implementation of the terminal. Thus, GCF requires that the terminals design and its implementation shall be based on core specifications.

A detailed description of the principles and procedures of GCF is given in

- GCF–PD: 	Principles Document

- GCF–AP: 	Application Procedures

- GCF–AD: 	Abbreviations and Definitions

- GCF–OP: 	Operating Procedures

- GCF–OB: 	Operating Budget

The latest versions of the above mentioned documents are available at the GCF web site at http://gcf.gsm.org.

Note: Registration is needed to access the GCF–CC data base.

## PCS Type Certification Review Board

M81 is not subject to PTCRB certification as decided by the project management.

## Bluetooth functionality

Bluetooth is standardised by Bluetooth Special Interest Group (SIG). SIG also specified the Bluetooth Qualification Process. Bluetooth functionality of terminals shall be qualified following the provisions of Bluetooth Qualification Program Reference Document.

The PRD specifies the framework of the qualification process. The details, such as the forms, templates, and checklists used to support this process, are provided in companion documents, which are referred to by the PRD, and/or are accessible using hyperlinks imbedded inside the PRD. Apart from these resources, supplemental information is available at the Bluetooth Web Site, including answers to frequently asked questions (FAQ).

A number of Profiles exists for various Bluetooth functionalities. Specific test suites apply to Profiles.

# Marking, labelling and User Guide

## Terminal marking and labelling

With reference to 1999/5/EC Article 12 and Annex VII a range of mandatory marking and labelling requirements are hereby defined.

Markings shall be visible for inspection without the need for tools.

The size of the IMEI label shall be large enough for all needed signs, numbers, etc. (e.g. IMEI, CE sign, FCC ID).

| Requirement basis | Corresponding marking |
| --- | --- |
| R&TTE Directive 1999/5/EC | Terminals using harmonised frequency bands only: CE + Notified Body Identifier Terminals using non-harmonised frequency bands: CE + Notified Body Identifier + Alert symbol (!) |
|  | Siemens/BenQ Type Designation |
|  | Serial Number / IMEI in bar-code format and writing |
|  | Manufacturer Name |
| FCC | FCC-ID number Country of Origin |
| IC | Canada ID |
| Additional | Siemens/BenQ stock number |

### CE marking layout and requirements

The CE marking must have a height of at least 5 mm except where this is not possible on account of the nature of the apparatus.

The CE marking must be affixed to the terminalor to its data plate. Additionally it must be affixed to the packaging, if any, and to the accompanying documents.

The CE marking must be affixed visibly, legibly and indelibly.

Notified body Identifier and the equipment class identifier, (e.g. !) if applicable being part of the CE marking need to be put on the packaging and in the manual.

### Notified Body Identifiers

M81 Hawk shall use BABT as Notified Body.

Corresponding NB identifier: 0168

## User Guide

With reference to 1999/5/EC Article 6.3:

The User Guide shall contain information on the intended use of the terminal e.g. supported GSM frequency bands. Where non-harmonised frequency bands are used, any geographical restrictions for its use shall be clearly identified within the User Guide.

The User Guide shall furthermore address a range of FCC and US legal requirements, in particular warnings.

Conformity to this requirement is not a must but helps to prevent extra declarations for France.

Requirements applicable to handsets marketed in US, not applicable for handsets roaming into US

R&TTE aspects regarding Bluetooth are mandatory

Bluetooth transmitters below 10mW transmitted power are handled as harmonised frequency bands.