Chapter III 2

Accessory-

Tests

Cordinators:		Ahmet Akdeniz	 	QM PL AC

Wen Hsin Lin 		 Y2Q (QRE)

Menzi Liang		WS0 (RD)

WT Liu			W10 (RD)

Barry Dai			WW0 (PM)

Introduction

This document contains all general test specifications, including all available test parameters, required for various accessory products. The general test specifications provide a common basis for definition of product-specific tests as they appear in the resulting TTD (Technical Terms of Delivery) and/or PD / LB (Product Description / Leistungsbeschreibung); following the practice of ISO standards, the component or product examined is called “specimen”.

In most cases, the test specifications are based on various common standards (e.g., EN 60068-2-2) and if no modifications or additions are needed, the standard applies “as is”. On the other hand, if a product-specific test variant is required, the deviating and/or narrowing conditions and parameters are stated explicitly and they override the corresponding specification of the standard.

The concept of "severity" as employed in international standards is applied. The complete test specification is always understood as the basis given by the standard with a particular severity, optionally altered by additional information. Consequently, all circumstances of conditioning and all applicable parameters must be looked up directly in the standard and first then modified by the optional explicit data contained in the respective sections of the test specification table. In the resulting product-specific tests the section "Product-specific Information" may contain additional statements pertaining to the given product and the section "Additional Agreements" may reflect modifications to the test specification negotiated in the course of the development project.

The applicable standard itself (if any) is displayed in the upper-right corner of each test specification table and denoted by "Ref" for brevity of reference. For example, in the test table for TAD_A001 the word "Ref" stands for "EN 60068-2-2" and, consequently the reference "Ref, Clause 12 and 13" points to Clause 12 and 13 of standard EN 60068-2-2.

Tests which are not based on a common standard are fully specified by the description provided.

As a rule (and if applicable), the specimens of accessory products are tested in switched-off state. For specimens with mechanical interfaces it may be meaningful to perform the tests on one half of units with the interface component attached (e.g., plug to connector, phone to holder) and the other half with an open (disconnected) interface. Additional details and possible modifications of these rules may be specified in section "Product-specific Information".

Each test is uniquely identified by its “Test Number” which consists of two parts: the first part (e.g., “TAD_A001”) points to the common general test specification while the second part reflects the code of the product-specific test variant; the second part is set to (GQR) for all tests in this General Quality Requirements document. Each page footer shows the common  “Release”  identifier and the corresponding  “Release Date” indicating the currency of the document.

A flag scheme marking the applicability of various items by a full "●" or an empty "○" circle for “Yes” or “No”, resp., is employed throughout the test specifications.

There are two categories of test specifications:

- Test Series may optionally be used to define sequences of single tests which are specified by the referenced Test Modules (see below). The main purpose of introducing Test Series is to exactly describe transitions (Pre-Test Actions, Post-Test Actions) between execution of single tests (Test Execution). A Test Series may combine tests of the same or different kind, e.g., mechanical, climatic, etc.

- Test Modules specify the single tests in detail, including all options for test parameters. The flags in the left-hand and right-hand columns of the test table indicate the checks to be performed before and after the test (Pre-Test Examinations, Post-Test Examinations). Test Modules are sorted in groups according to their type, e.g., Mechanical Tests, Climatic Tests, etc.

Table of Contents

Test Series	4

TAD_S001: Vibration & Shock at Room Temperature	4

TAD_S002: Vibration & Shock at Low and High Temperatures	5

TAD_S003: Nickel Resistance after Sweat Exposure	7

Test Modules: Ageing / Material	8

TAD_A001: Cyclic Dry Heat	8

TAD_A002: Steady-State Damp Heat	10

TAD_A003: Dust and Sand	13

TAD_A004: Gas Mixture	15

TAD_A005: Steady-State Solar Radiation	17

TAD_A008: Sweat	21

TAD_A009: Nickel	24

TAD_A010: Cream, Oil, Vaseline	26

Test Modules: Climatic	29

TAD_C001: Cyclic Cold and Heat	29

TAD_C002: Cyclic Damp Heat	31

TAD_C003: Cyclic Solar Radiation	33

Test Modules: Mechanical	36

TAD_M001: Vibration	36

TAD_M002: Shock	39

TAD_M003: Free Fall	42

TAD_M004: Abrasion	46

TAD_M005: Static Load	49

TAD_M006: Twist	52

TAD_M007: Temperature Difference between Phone and Holder	54

TAD_M101: Straight-Pull Static Load on Assembled Cable	56

TAD_M102: Periodic Bending under Load of Assembled Cable	59

TAD_M103: 90-Degree-Bend Static Load on Assembled Cable	63

TAD_M104: Straight-Pull Static Load on Stand-Alone Cable	66

TAD_M105: Periodic Bending under Load of Stand-Alone Cable	69

TAD_M106: Straight-Pull Dynamic Load on Helical Cable	73

TAD_M801: IP Class Protection	76

TAD_M901: Lifetime of Mechanical Operating Elements	78

TAD_M902: Periodic Bending of Belt Clip	81

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Series | to Test |  |  |
| TAD_S001 (GQR) | Vibration & Shock at Room Temperature |  |  |  |
| Test Series TAD_S001: Vibration & Shock at Room Temperature Test Series Purpose  This test series reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to Vibration & Shock at Room Temperature.  This test series may be used to study structural integrity, mechanical robustness, and dynamic behaviour of a product/component.. Procedure & Conditions  The procedure of this test series is combining the vibration and shock examinations at room temperature and consists of two steps. Step_01; Vibration. Pre-Test Actions.  None. Test Execution.  Perform test according to TAD_M001.. Post-Test Actions.  Perform checks according to TAD_M001 evaluation criteria.  Keep the specimen mounted on the fixture tool and continue with the next step; the same parts are being used in TAD_M001 and TAD_M002 (see next step). Consequently, the specimen quantity needed for this test series equals the quantity for a single test module... Step_02; Shock. Pre-Test Actions.  None. Test Execution.  Perform test according to TAD_M002.. Post-Test Actions.  Perform checks according to TAD_M002 evaluation criteria.... Evaluation Criteria  None in addition to specific test evaluations.. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Series | to Test |  |  |
| TAD_S002 (GQR) | Vibration & Shock at Low and High Temperatures |  |  |  |
| Test Series TAD_S002: Vibration & Shock at Low and High Temperatures Test Series Purpose  This test series reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to Vibration & Shock at Low and High Temperatures.  This test series may be used to study structural integrity, mechanical robustness, and dynamic behaviour of a product/component.. Procedure & Conditions  The procedure of this test series is combining the vibration and shock examinations at two temperatures (low and high) and consists of four steps. Step_01; Vibration at Low Temperature. Pre-Test Actions.  Cool down at rate of 5°C per minute to the low temperature specified in TAD_M001 and wait at this temperature long enough to achieve thermal equilibrium.. Test Execution.  Perform test according to TAD_M001 at low temperature.. Post-Test Actions.  Perform checks according to TAD_M001 evaluation criteria.  Keep the specimen mounted on the fixture tool and continue with the next step; the same parts are being used in TAD_M001 and TAD_M002 (see next step). Consequently, the specimen quantity needed for this test series equals the quantity for a single test module... Step_02; Shock at Low Temperature. Pre-Test Actions.  None. Test Execution.  Perform test according to TAD_M002 at low temperature.. Post-Test Actions.  Perform checks according to TAD_M002 evaluation criteria.  Keep the specimen mounted on the fixture tool and continue with the next step... Step_03; Vibration at High Temperature. Pre-Test Actions.  Warm up at rate of 5°C per minute to the high temperature specified in TAD_M001 and wait at this temperature long enough to achieve thermal equilibrium.. Test Execution.  Perform test according to TAD_M001 at high temperature.. Post-Test Actions.  Perform checks according to TAD_M001 evaluation criteria.  Keep the specimen mounted on the fixture tool and continue with the next step; the same parts are being used in TAD_M001 and TAD_M002 (see next step). Consequently, the specimen quantity needed for this test series equals the quantity for a single test module... Step_04; Shock at High Temperature. Pre-Test Actions.  None. Test Execution.  Perform test according to TAD_M002 at high temperature.. Post-Test Actions.  Perform checks according to TAD_M002 evaluation criteria.... Evaluation Criteria  None in addition to specific test evaluations.. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Series | to Test |  |  |
| TAD_S003 (GQR) | Nickel Resistance after Sweat Exposure |  |  |  |
| Test Series TAD_S003: Nickel Resistance after Sweat Exposure Test Series Purpose  This test reveals the ability of a product/component to withstand some common substances of real-life situations. Specifically, it examines effects due to application of Sweat followed by examination of Nickel resistance on the same specimen.. Procedure & Conditions  The procedure of this test series is combining the Sweat and Nickel examinations on the same specimen and consists of two steps. Step_01; Sweat. Pre-Test Actions.  None. Test Execution.  Perform test according to TAD_A008.. Post-Test Actions.  Perform checks according to TAD_A008 evaluation criteria.  Use the same place on the surface of the specimen for the next step... Step_02; Nickel. Pre-Test Actions.  None. Test Execution.  Perform test according to TAD_A009.. Post-Test Actions.  Perform checks according to TAD_A009 evaluation criteria.... Evaluation Criteria  None in addition to specific test evaluations.. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Ageing / Material | to Test |  | Ref: EN 60068-2-2 |
| TAD_A001 (GQR) | Cyclic Dry Heat |  |  |  |
|  | see Ref, Test Bb: Dry heat for non heat-dissipating specimen with gradual change of temperature |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Ageing / Material TAD_A001: Steady-State Dry Heat Test Purpose  This test reveals the ability of a product/component to withstand certain ambient conditions without damage or degradation in the specified performance. Specifically, it examines effects due to application of Steady-State Dry Heat.  This test may be used to simulate product/component storage or transportation where no special measures have been taken with respect of maintaining pre-defined ambient conditions.  see Ref, Introduction; see Ref, Clause 12 and 13. Scope of Testing  Structure, behaviour, and appearance of (electro-)mechanical parts made of plastic or metal; special attention to mechanical stability (plastic moldings, composite materials), surface ageing  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 14. Procedure & Conditions  The test procedure described in Ref, Clause 16, 17, 18, 19, 20, and 21, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity; if the duration of the test exceeds 7 days, this number may be reduced by 3 units every 7 days.. Parameters & Severity  see Ref, Clause 14, 18, and 20 Severity Variable; Severity Value. Temperature Duration Minimum QUANTITY to test; (+85 +/-2) °C 4 days 6 units [products without electronic components]. Temperature Duration Minimum QUANTITY to test; Step 1: 16 h at 85 ºC, including an initial time for temperature stabilization Step 2: temperature change to 40°C within 15 min Step 3: 8 h at 40 ºC including an initial time for temperature stabilization Step 4: temperature change to 85ºC within 15 min 21 days (repetitions from step 1 to 4 for 21 d) 9 units (reduced by 3 units every 7 days) [products with electronic components].. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, cracks or permanent deformation)  No change in appearance (especially, signs of ageing of surface and/or in colours). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Ageing / Material | to Test |  | Ref: EN 60068-2-78 |
| TAD_A002 (GQR) | Steady-State Damp Heat |  |  |  |
|  | see Ref, Test Cab: Damp heat, steady state |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Ageing / Material TAD_A002: Steady-State Damp Heat Test Purpose  This test reveals the ability of a product/component to withstand certain ambient conditions without damage or degradation in the specified performance. Specifically, it examines effects due to application of Steady-State Damp Heat.  This test may be used to simulate product/component storage or transportation where no special measures have been taken with respect of maintaining pre-defined ambient conditions.  see Ref, Clause 1. Scope of Testing  Structure, behaviour, and appearance of (electro-)mechanical parts made of plastic or metal; special attention to mechanical stability (plastic moldings, composite materials), surface ageing, connections, solderings, bonded components, material swelling  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 3 and 4. Procedure & Conditions  The test procedure described in Ref, Clause 6, 7, 8, 9, and 10, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity; if the duration of the test exceeds 7 days, this number may be reduced by 3 units every 7 days.. Parameters & Severity  see Ref, Clause 4, 5, 7, and 9 Severity Variable; Severity Value. Temperature Relative humidity Duration Minimum QUANTITY to test; (+85 +/-2) °C (85 +/-3) % 4 days 3 units [carrying cases]. Temperature Relative humidity Duration Minimum QUANTITY to test; (+40 +/-2) °C (93 +/-3) % 4 days 3 units [indoor, outdoor, and car products (except carrying cases) without electronic components (unproblematic material data sheet)]. Temperature Relative humidity Duration Minimum QUANTITY to test; (+40 +/-2) °C (93 +/-3) % 10 days 6 units (reduced by 3 units after 7 days) [indoor products (except carrying cases) with electronic components; for indoor, outdoor, and car products (except carrying cases) without electronic components (problemat. Temperature Relative humidity Duration Minimum QUANTITY to test; (+40 +/-2) °C (93 +/-3) % 21 days 9 units (reduced by 3 units every 7 days) [outdoor and car products (except carrying cases) with electronic components].. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, cracks or permanent deformation)  No change in appearance (especially, signs of ageing of surface and/or in colours). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Ageing / Material | to Test |  | Ref: EN 60068-2-68 |
| TAD_A003 (GQR) | Dust and Sand |  |  |  |
|  | see Ref, Test Lb: free settling dust |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Ageing / Material TAD_A003: Dust and Sand Test Purpose  This test reveals the ability of a product/component to withstand certain ambient conditions without damage or degradation in the specified performance. Specifically, it examines effects due to application of Dust and Sand.  This test may be used to simulate product/component storage and operation under real-life situation.  see Ref, Introduction; see Ref, Clause 1, 3, 5.1, 5.2.1. Scope of Testing  Behaviour and appearance of (electro-)mechanical parts made of plastic or metal; special attention to movable parts, dust protection parts. Equipment Requirements  see Ref, Clause 5.2.2 and 5.2.3  Equipment Specifics & Exceptions - Contrary to Ref, Clause 5.2.3 (pointing to Clause 6.1.4.1, Variant 1), the dust used for this test shall be the so-called 'Arizona test dust, Category fine', designated as ISO 12103-A2 in international standard ISO 12103-1.. Procedure & Conditions  The test procedure described in Ref, Clause 5.2.5, 5.2.6, 5.2.7, 5.2.8, 5.2.9, and 5.2.10 shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics & Exceptions In agreement with SIEMENS, determine the critical surfaces of the specimen (e.g., those with holes, opennings, slits, etc.) to be stressed and evaluated.  Measurements Specifics & Exceptions - AFTER test: Knock the specimen on a hard surface and blow away extraneous dust. Check the tendency of any moving part (especially keys) to stick by pressing it 20 times.. Parameters & Severity  see Ref, Table 1 (Test Lb), Clause 5.2.2, replace 5.2.3 by ISO 12103-A2, 5.2.7, and 5.2.9 Severity Variable; Severity Value. Duration Minimum QUANTITY to test; 4 days 3 units.. Evaluation Criteria  No impairment of functionality (especially, degradation in function of moving parts, e.g., key sticking)  No impairment of accoustic properties (microphone, loudspeaker, ear piece)  No degradation in function of dust protection parts. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Ageing / Material | to Test |  | Ref: EN 60068-2-60 |
| TAD_A004 (GQR) | Gas Mixture |  |  |  |
|  | see Ref, Test Ke: Flowing mixed gas corrosion test |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Ageing / Material TAD_A004: Gas Mixture Test Purpose  This test reveals the ability of a product/component to withstand certain ambient conditions without damage or degradation in the specified performance. Specifically, it examines effects due to application of Gas Mixture.  This test may be used to simulate product/component storage and operation under real-life situation.  see Ref, Clause 1.1. Scope of Testing  Behaviour and appearance of (electro-)mechanical parts made of plastic or metal; special attention to surface ageing, connections, solderings, metal parts, coated surfaces. Equipment Requirements  see Ref, Clause 2. Procedure & Conditions  The test procedure described in Ref, Clause 4, 5, 6 (use Test procedure 2), 7, and 8, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics & Exceptions - DURING test: After 4 days of stressing, examine the specimens; if any corrosion found, reject the specimens and terminate the test.. Parameters & Severity  see Ref, Clause 2, 6.3 (Test procedure 2) Severity Variable; Severity Value. Method Duration Minimum QUANTITY to test; Method 4 10 days 3 units.. Evaluation Criteria  No impairment of functionality (especially, change in electrical parameters, e.g., contact resistance)  No severe corrosion  No change in appearance (especially, signs of ageing of surface and/or in colours). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Ageing / Material | to Test |  | Ref: EN 60068-2-5 |
| TAD_A005 (GQR) | Steady-State Solar Radiation |  |  |  |
|  | see Ref, Test Sa: Simulated solar radiation at ground level |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Ageing / Material TAD_A005: Steady-State Solar Radiation Test Purpose  This test reveals the ability of a product/component to properly operate under earth-typical environmental conditions. Specifically, it examines effects due to application of Steady-State Solar Radiation.  This test may be used to simulate product/component storage and operation under real-life situation.  see Ref, Clause 1. Scope of Testing  Structure, behaviour, and appearance of (electro-)mechanical parts made of plastic or metal; special attention to mechanical stability (plastic moldings, composite materials), surface ageing  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 3  Equipment Specifics & Exceptions - Lamp providing natural sun light spectrum, e.g., Fa. Hoenle SOL 1200, recommended.. Procedure & Conditions  The test procedure described in Ref, Clause 2 and 4, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  All surfaces of the specimen which are exposed to light and visible during normal use of the product shall be tested.  Conditioning Specifics & Exceptions - DURING test: Contrary to Ref, Clause 4.3 and 4.4, the test procedure following a special time dependence of temperature-radiation profile shall be applied. Surface temperature measurement sensors shall be used for a minimum of one unit and no forced air cooling shall be be applied.. Time dependence of temperature-radiation profile. Parameters & Severity  see Ref, Clause 3  Parameters Specifics & Exceptions - Room temperature = (+25 +/-2) °C Severity Variable; Severity Value. Irradiance High temperature Exposure time Minimum QUANTITY to test; 1120 W/m² (+55 +/-2) °C 16 hours 2 to 12 units (2 units per irradiated surface with up to 6 surfaces) [depending on material]. Irradiance High temperature Exposure time Minimum QUANTITY to test; 1120 W/m² (+55 +/-2) °C 24 hours 2 to 12 units (2 units per irradiated surface with up to 6 surfaces) [depending on material]. Irradiance High temperature Exposure time Minimum QUANTITY to test; 1120 W/m² (+55 +/-2) °C 48 hours 2 to 12 units (2 units per irradiated surface with up to 6 surfaces) [depending on material]. Irradiance High temperature Exposure time Minimum QUANTITY to test; 1120 W/m² (+55 +/-2) °C 72 hours 2 to 12 units (2 units per irradiated surface with up to 6 surfaces) [depending on material].. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, cracks or permanent deformation)  No change in appearance (especially, signs of ageing of surface and/or in colours). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Ageing / Material | to Test |  |  |
| TAD_A008 (GQR) | Sweat |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. | Test Modules: Ageing / Material TAD_A008: Sweat Test Purpose  The test is intended to see, if painted surfaces, plated surfaces and metal surfaces can withstand sweat. Preferentially, parts in delivery quality have to be provided directly by the supplier. In addition to that it is necessary to avoid surfaces with scratches or cutting edges which are especially vulnerable to corrosion.. Scope of Testing  Appearance of (electro-)mechanical parts made of plastic or metal; special attention to surface ageing, coated surfaces. Equipment Requirements  Test box with holder containing sweat-like liquid (test agent 1, test agent 2); for a correct partial concentration of test agent vapour, use a ratio of about 1:15 between the volume of liquid and the volume of test box (e.g., 0.3 l liquid in 4.5 l test box); temperature-controlled oven  Test agent 1 (alkaline sweat): 100 ml de-ionized or distilled water 5 g NaCl 5 g di sodium hydrogen phosphate (Na2HPO4 x 12 H2O) Add NaOH until reaching pH 9.5  Test agent 2 (acid sweat): 100 ml de-ionized or distilled water 5 g NaCl 5 g di sodium hydrogen phosphate (Na2HPO4 x 12 H2O) 2 ml of 99% acetic acid Add NaOH until reaching pH 4.7. Example of a test box. Procedure & Conditions  The test procedure described in the following points shall be successively executed with both test agents (alkaline sweat, acid sweat) for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics - BEFORE test: Position the specimen on the holder in the test box and cover it with blotting paper (see illustration); then place the test box in a temperature-controlled oven. - DURING test: Expose the text box with the specimen to high temperature for duration, as given by the specified severity.  Measurements Specifics - AFTER test: Examine the specimen and evaluate its condition.. Parameters & Severity Severity Variable; Severity Value. High temperature Duration Minimum QUANTITY to test; (+45 +/-2) °C 3 days 6 units (3 units per test agent with 2 test agents: alkaline sweat, acid sweat).. Evaluation Criteria   No mechanical damage (especially, cracks or permanent deformation)  No change in appearance (especially, no signs of corrosion of surface, no discolouration, no impact on the adhesion of glued parts). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Ageing / Material | to Test |  |  |
| TAD_A009 (GQR) | Nickel |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. | Test Modules: Ageing / Material TAD_A009: Nickel Test Purpose  According to EN 60950 and UL 1950 the vendor has to minimize the risk for nickel allergy. Therefore the nickel test is to be done in case of plated surfaces/galvanization. Preferentially, parts in delivery quality have to be provided directly by the supplier. The oxidation of nickel is achieved by performing the sweat test before the nickel test. (The nickel test is based on the formation of a red complex in the presence of Ni2+ ions and dimethylglyoxim. Therefore the Ni metal has to be oxidized at first to Ni2+ ions so that the evidence of Ni can be provided.). In addition to that it is necessary to avoid surfaces with scratches or cutting edges which are especially vulnerable to corrosion.. Scope of Testing  Release of nickel from plated and metal surfaces. Equipment Requirements  Cotton swab; components of test agent must be stored separately  Test agent: 5 mol/l ammonia in de-ionized or distilled water (component 1) 1% dimethylglyoxim in ethanol (component 2) (formula: C4H8N2O2; trade names: dimethylglyoxim or 2,3-butandiondioxim or diacetyldioxim). Procedure & Conditions  The test procedure described in the following points shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics - BEFORE test: Prepare the test agent by mixing both components immediately before testing (e.g., put two drops of each component on a cotton swab). - DURING test: Rub the surface of the specimen with a cotton swab moistured by the test agent for duration, as given by the specified severity.  Measurements Specifics - AFTER test: Examine the specimen surface and the cotton swab for any sign of red colour (from rose pink to heavy cherry).. Parameters & Severity Severity Variable; Severity Value. Duration Minimum QUANTITY to test; 30 sec 3 units.. Evaluation Criteria  The nickel liberation must not occur throughout the whole surface tested; no red color throughout the whole surface tested is accepted. The release of Ni2+ ions should be in the commonly observed range. The part yielding the best nickel test result is to be selected if comparable parts of several suppliers are available.. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Ageing / Material | to Test |  |  |
| TAD_A010 (GQR) | Cream, Oil, Vaseline |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. | Test Modules: Ageing / Material TAD_A010: Cream, Oil, Vaseline Test Purpose  This test reveals the ability of a product/component to withstand some common substances of real-life situations. Specifically, it examines effects due to application of Cream, Oil, Vaseline.. Scope of Testing  Appearance of (electro-)mechanical parts made of plastic or metal; special attention to surface ageing, coated surfaces. Equipment Requirements  Temperature-controlled oven  Test agent: skin cream (as representative for cosmetic substances, e.g., moisturizing cream, sun cream)  Test agent: cooking oil (as representative for domestic fats, e.g., sunflower oil)  Test agent: vaseline (as representative for petroleum products, e.g., petroleum jelly, vaseline). Procedure & Conditions  The test procedure described in the following points shall be executed for a number of new specimens (see Minimum QUANTITY to test) and all test agents , as given by the specified severity.  Conditioning Specifics - BEFORE test: Smear one half of the specimen surface with the test agent; the other half must remain clean for comparison purposes. Place the specimen in a temperature-controlled oven. - DURING test: Expose the specimen to high temperature for duration, as given by the specified severity.  Measurements Specifics - AFTER test: Examine the smeared half of the specimen surface and compare it with the clean part.. Parameters & Severity Severity Variable; Severity Value. Test agent High temperature Duration Minimum QUANTITY to test; Skin cream (+45 +/-2) °C 8 hours 3 units. Test agent High temperature Duration Minimum QUANTITY to test; Cooking oil (+45 +/-2) °C 8 hours 3 units. Test agent High temperature Duration Minimum QUANTITY to test; Vaseline (+45 +/-2) °C 8 hours 3 units.. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, cracks or permanent deformation)  No change in appearance (especially, signs of ageing of surface and/or in colours). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Climatic | to Test |  | Ref: EN 60068-2-14 |
| TAD_C001 (GQR) | Cyclic Cold and Heat |  |  |  |
|  | see Ref, Test Nb: Change of temperature with specified rate of change |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Climatic TAD_C001: Cyclic Cold and Heat Test Purpose  This test reveals the ability of a product/component to properly operate under earth-typical environmental conditions. Specifically, it examines effects due to application of Cyclic Cold and Heat.  This test may be used to simulate product/component storage and operation under real-life situation.  see Ref, Introduction; see Ref, Clause 2.1 and 2.2. Scope of Testing  Structure, behaviour, and appearance of (electro-)mechanical parts made of plastic or metal; special attention to mechanical stability (plastic moldings, composite materials), surface ageing  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 2.3. Procedure & Conditions  The test procedure described in Ref, Clause 2.5, 2.6, 2.7, and 2.8, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics & Exceptions - AFTER test: Post treatment 1 hour according to EN 60068-2-2, Clause 18.6 and 20. Parameters & Severity  see Ref, Clause 2.3, 2.4, and 2.6 Severity Variable; Severity Value. Low temperature T(A) High temperature T(B) Rate of temperature change Exposure time t(1) Number of cycles Minimum QUANTITY to test; (-40 +/-2) °C (+85 +/-2) °C (1 +/-0.2) °C/min 2 hours 10 3 units.. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, cracks or permanent deformation)  No change in appearance (especially, signs of ageing of surface and/or in colours). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Climatic | to Test |  | Ref: EN 60068-2-30 |
| TAD_C002 (GQR) | Cyclic Damp Heat |  |  |  |
|  | see Ref, Test Db: Damp heat, cyclic (12+12-hour cycle) |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Climatic TAD_C002: Cyclic Damp Heat Test Purpose  This test reveals the ability of a product/component to properly operate under earth-typical environmental conditions. Specifically, it examines effects due to application of Cyclic Damp Heat.  This test may be used to simulate product/component storage and operation under real-life situation.  see Ref, Clause 1 and 2. Scope of Testing  Structure, behaviour, and appearance of (electro-)mechanical parts made of plastic or metal; special attention to mechanical stability (plastic moldings, composite materials), surface ageing, glueing properties, humidity condensation, electrolytic corrosion, material migration, surface porosity, capillarity attraction  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 3. Procedure & Conditions  The test procedure described in Ref, Clause 5, 6 (use Variant 1), 7, 8, and 9, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.. Parameters & Severity  see Ref, Clause 3, 6, 8, and 9 Severity Variable; Severity Value. Upper temperature Number of cycles Minimum QUANTITY to test; (+55 +/-2) °C 6 3 unit.. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, cracks or permanent deformation)  No change in appearance (especially, signs of ageing of surface and/or in colours). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Climatic | to Test |  | Ref: EN 60068-2-5 |
| TAD_C003 (GQR) | Cyclic Solar Radiation |  |  |  |
|  | see Ref, Test Sa: Simulated solar radiation at ground level |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Climatic TAD_C003: Cyclic Solar Radiation Test Purpose  This test reveals the ability of a product/component to properly operate under earth-typical environmental conditions. Specifically, it examines effects due to application of Cyclic Solar Radiation.  This test may be used to simulate product/component storage and operation under real-life situation.  see Ref, Clause 1. Scope of Testing  Structure, behaviour, and appearance of (electro-)mechanical parts made of plastic or metal; special attention to mechanical stability (plastic moldings, composite materials), surface ageing  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 3  Equipment Specifics & Exceptions - Lamp providing natural sun light spectrum, e.g., Fa. Hoenle SOL 1200, recommended.. Procedure & Conditions  The test procedure described in Ref, Clause 2 and 4, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  All surfaces of the specimen which are exposed to light and visible during normal use of the product shall be tested.  Conditioning Specifics & Exceptions - DURING test: Contrary to Ref, Clause 4.3 and 4.4, the test procedure following a special time dependence of temperature-radiation profile shall be applied. Surface temperature measurement sensors shall be used for a minimum of one unit and no forced air cooling shall be be applied.. Time dependence of temperature-radiation profile. Parameters & Severity  see Ref, Clause 3  Parameters Specifics & Exceptions - Room temperature = (+25 +/-2) °C Severity Variable; Severity Value. Irradiance High temperature Number of cycles Minimum QUANTITY to test; 1120 W/m² (+55 +/-2) °C 3 2 to 12 units (2 units per irradiated surface with up to 6 surfaces) [carrying cases]. Irradiance High temperature Number of cycles Minimum QUANTITY to test; 1120 W/m² (+55 +/-2) °C 1 2 to 12 units (2 units per irradiated surface with up to 6 surfaces) [outdoor and car products (except carrying cases)]. Irradiance High temperature Number of cycles Minimum QUANTITY to test; 700 W/m² (+40 +/-2) °C 1 2 to 12 units (2 units per irradiated surface with up to 6 surfaces) [indoor products (except carrying cases)].. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, cracks or permanent deformation)  No change in appearance (especially, signs of ageing of surface and/or in colours). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  | Ref: EN 60068-2-6 |
| TAD_M001 (GQR) | Vibration |  |  |  |
|  | see Ref, Test Fc: Vibration (sinusoidal) |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M001: Vibration Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to application of Vibration.  This test may be used to study structural integrity, mechanical robustness, and dynamic behaviour of a product/component; the effects of transportation may be simulated.  see Ref, Clause 1. Scope of Testing  Mechanical behaviour and electrical characteristics of (electro-)mechanical parts; special attention to structural stability, interface connections, solderings  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 4 (definitions in Clause 3)  Equipment Specifics & Exceptions - Shaker suitable to test the specimen - Head expander for fixation of the fixture plate - Specimen-specific fixture tool made of aluminium. Procedure & Conditions  The test procedure described in Ref, Clause 6, 7, 8 (use Endurance by sweeping), 9, 10, and 11, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity (1 unit per axis +X, +Y, +Z; 1 unit per axis -X, -Y, -Z); the steps described below can be used for specification of a test series at a selected temperature, or as a stand-alone test.  Conditioning Specifics & Exceptions - BEFORE test: Mount all housing parts of the specimen firmly on the fixture tool, if possible in the same way as in the intended product use (no movement of the unit is allowed).  Measurements Specifics & Exceptions Determine electrical characteristics meaningful to the performance of the specimen and prepare measurement setup. - BEFORE test: Record initial characteristics. - DURING test: Monitor and record transitional characteristics under stress. - AFTER test: Record final characteristics.. Parameters & Severity  see Ref, Clause 4 (definitions in Clause 3)  Parameters Specifics & Exceptions - Low temperature = (-25 +/-2) °C - Room temperature = (+25 +/-2) °C - High temperature = (+65 +/-2) °C Severity Variable; Severity Value. Frequency range Vibration amplitude: - displacement for 5 Hz - 20 Hz - acceleration for 20 Hz - 500 Hz Duration in sweep cycles per axis Minimum QUANTITY to test; 5 Hz - 500 Hz 1.8 mm 3g (approx. 30 m/s²) 10 6 units (1 unit per axis with 6 axes: +X, +Y, +Z, -X, -Y, -Z ).. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, loose or broken parts, cracks or permanent deformation)  No degradation in function of locking and catching parts (especially, they must not open while testing). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  | Ref: EN 60068-2-27 |
| TAD_M002 (GQR) | Shock |  |  |  |
|  | see Ref, Test Ea and guidance: Shock |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M002: Shock Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to application of Shock.  This test may be used to study structural integrity, mechanical robustness, and dynamic behaviour of a product/component; the effects of transportation may be simulated.  see Ref, Introduction; see Ref, Clause 1 and 2. Scope of Testing  Mechanical behaviour and electrical characteristics of (electro-)mechanical parts; special attention to structural stability, interface connections, solderings  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 4 (definitions in Clause 3)  Equipment Specifics & Exceptions - Shaker suitable to test the specimen - Head expander for fixation of the fixture plate - Specimen-specific fixture tool made of aluminium. Procedure & Conditions  The test procedure described in Ref, Clause 6, 7, 8 (use Clause 4.1.1: Half-sine pulse shape), 9, and 10, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity (1 unit per axis +X, +Y, +Z; 1 unit per axis -X, -Y, -Z); the steps described below can be used for specification of a test series at a selected temperature, or as a stand-alone test.  Conditioning Specifics & Exceptions - BEFORE test: Mount all housing parts of the specimen firmly on the fixture tool, if possible in the same way as in the intended product use (no movement of the unit is allowed). - DURING test: Contrary to Clause 8.1, apply only 1 shock. Parameters & Severity  see Ref, Clause 4 (definitions in Clause 3)  Parameters Specifics & Exceptions - Low temperature = (-25 +/-2) °C - Room temperature = (+25 +/-2) °C - High temperature = (+65 +/-2) °C Severity Variable; Severity Value. Peak acceleration Duration of nominal pulse Minimum QUANTITY to test; 50 g 11 ms 6 units (1 unit per axis with 6 axes: +X, +Y, +Z, -X, -Y, -Z ) [products mounted in cross-country vehicles]. Peak acceleration Duration of nominal pulse Minimum QUANTITY to test; 30 g 18 ms 6 units (1 unit per axis with 6 axes: +X, +Y, +Z, -X, -Y, -Z ) [other products ( except mounted in cross-country vehicles)].. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, loose or broken parts, cracks or permanent deformation)  No degradation in function of locking and catching parts (especially, they must not open while testing). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  | Ref: EN 60068-2-32 |
| TAD_M003 (GQR) | Free Fall |  |  |  |
|  | see Ref, Test Ed: Free fall |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M003: Free Fall Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to Free Fall.  This test may be used to study effects of rough handling of a product/component and to demonstrate a minimum degree of robustness for assessment of safety requirements.  see Ref, Introduction; see Ref, Clause 1. Scope of Testing  Mechanical behaviour and electrical characteristics of (electro-)mechanical parts; special attention to structural stability, interface connections, solderings  Capability of electronic components to survive conditioning. Equipment Requirements  see Ref, Clause 2  Equipment Specifics & Exceptions - Drop tester capable of changing the drop height and attitude (i.e., positioning of the specimen according to the required drop orientation) - Concrete floor plate (minimum weight 20 kg, minimum thickness 50 mm) placed on a solid underground - Optionally, high-speed camera system (minimum of 1000 pictures/second) for analysis of mechanical damages.. Example of a drop tester. Procedure & Conditions  The test procedure described in Ref, Procedure 1, Clause 4, 5, and 6, shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics & Exceptions In agreement with SIEMENS, determine the critical corners on the specimen (and corresponding fall attitudes for each corner, adjacent edges, adjacent surfaces) to be stressed and evaluated. - DURING test: Contrary to Ref, Clause 5.2, apply only 1 fall per attitude. The specimen must stay within the test area after the impact. - AFTER test: The specimen must be shaken shortly by hand between the single falls to detect loose parts.. Scheme specifying drop attitude. Parameters & Severity  see Ref, Clause 2  Parameters Specifics & Exceptions - Number of attitudes = 7 (1 corner, 3 adjacent edges, 3 adjacent surfaces) - Number of falls per attitude = 1 Severity Variable; Severity Value. Height of fall Minimum QUANTITY to test; 800 mm 3 to 9 units (3 units per critical corner with up to 3 corners) [charger and products mounted in the car (drop at installation)]. Height of fall Minimum QUANTITY to test; 1000 mm 3 to 9 units (3 units per critical corner with up to 3 corners) [products portable in the car and those used on a desk]. Height of fall Minimum QUANTITY to test; 1500 mm 3 to 9 units (3 units per critical corner with up to 3 corners) [products used at breast level]. Height of fall Minimum QUANTITY to test; 1800 mm 3 to 9 units (3 units per critical corner with up to 3 corners) [products used at head level].. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, loose or broken parts, cracks or permanent deformation; minor surface impairment allowed)  No conductive parts accessible. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  | Ref: SN 27650 |
| TAD_M004 (GQR) | Abrasion |  |  |  |
|  | see Ref, Test for abrasive strength |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. | Test Modules: Mechanical TAD_M004: Abrasion Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects on coated surfaces due to Abrasion.  This test may be used to study effects of normal and rough handling of a product/component.  see Ref, Clause 1. Scope of Testing  Abrasion resistance of coatings and imprints on surfaces; special attention to exposed surfaces, operating elements. Equipment Requirements  see Ref, Clause 5. Example of an abrasion tester. Procedure & Conditions  The test procedure described in Ref, Clause 4 and 6 shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics & Exceptions - BEFORE test: Depending on test equipment, small pieces may be cut out from the specimen for testing in the machine. - DURING test: Ambient temperature shall be between 20°C and 25°C at relative humidity between 40% and 60%.  Measurements Specifics & Exceptions - BEFORE test: Since the evaluation of the test results deviates from Ref, Clause 7, no measurement of the actual layer thickness is needed.. Parameters & Severity  see Ref, Clause 4 and 6  Parameters Specifics & Exceptions - Pressure exerted = 2.5 N Severity Variable; Severity Value. Minimum abrasion path Minimum QUANTITY to test (general surfaces: to be used if none of the below surface categories applies); 1000 cm 3 units [general surfaces: to be used if none of the below surface categories applies]. Minimum abrasion path Minimum QUANTITY to test (painted surfaces); 700 cm 3 units [painted surfaces]. Minimum abrasion path Minimum QUANTITY to test (plated surfaces); 1000 cm 3 units [plated surfaces]. Minimum abrasion path Minimum QUANTITY to test (metal surfaces); 1000 cm 3 units [metal surfaces]. Minimum abrasion path Minimum QUANTITY to test (imprints on functional elements); 700 cm 3 units [imprints on functional elements]. Minimum abrasion path Minimum QUANTITY to test (imprints on non-functional elements touched occasionally); 60 cm 3 units [imprints on non-functional elements touched frequently]. Minimum abrasion path Minimum QUANTITY to test (imprints on non-functional elements normally not touched); 20 cm 3 units [imprints on non-functional elements touched rarely]. Minimum abrasion path Minimum QUANTITY to test (approval imprints); 200 cm 3 units [approval imprints]. Minimum abrasion path Minimum QUANTITY to test (labels); 60 cm 3 units [labels].. Evaluation Criteria  The actual abrasion path resulting in the abrasion area of 0.25 mm² must be greater or equal the minimum abrasion path, as given by the severity.  Up to the minimum abrasion path the prints must stay clearly legible and no substantial damage to coated and/or imprinted surfaces must occur.. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M005 (GQR) | Static Load |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M005: Static Load Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to application of Static Load.  This test may be used to study effects of rough handling of a product/component and to demonstrate a minimum degree of robustness for assessment of safety requirements.. Scope of Testing  Mechanical behaviour of (electro-)mechanical parts; special attention to structural stability of housings and of operating elements. Equipment Requirements  Universal tester using spherical stamp with rod diameter of 8 mm and sphere radius of 10 mm; specimen-specific fixture tool. Procedure & Conditions  The test procedure described in the following points shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics In agreement with SIEMENS, determine meaningful pressure points on the specimen (e.g., middle, top, bottom, left, right) to be stressed and evaluated. - BEFORE test: Mount the specimen in the universal tester. - DURING test: Apply gradually (force increase 10 N/sec) the load given by the specified severity at all defined pressure points. - AFTER test: Dismount the specimen.  Measurements Specifics - AFTER test: Examine the specimen and evaluate its condition.. Parameters & Severity Severity Variable; Severity Value. Load Minimum QUANTITY to test (complete housing); 200 N 3 units [complete housing]. Load Minimum QUANTITY to test (display window, outside to inside); 120 N 3 units [display window, outside to inside]. Load Minimum QUANTITY to test (display window, inside to outside); 50 N 3 units [display window, inside to outside]. Load Minimum QUANTITY to test (keys); 200 N 3 units [keys].. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, loose or broken parts, cracks or permanent deformation). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M006 (GQR) | Twist |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ○. Electric Characteristics. ○. | Test Modules: Mechanical TAD_M006: Twist Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to application of Twist.  This test may be used to study effects of rough handling of a product/component and to demonstrate a minimum degree of robustness for assessment of safety requirements.. Scope of Testing  Mechanical behaviour of (electro-)mechanical parts; special attention to structural stability of housings, surface coatings, inlay parts. Equipment Requirements  Push-pull twist tester and specimen-specific fixture tool.. Example of a push-pull twist tester. Procedure & Conditions  The test procedure described in the following points shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics - BEFORE test: Mount the specimen (axis in horizontal direction) in the fixture tool. - DURING test: Successively apply the torque (force times distance from axis) given by the specified severity (e.g., 0.5 Nm = 5 N x 0.1 m) in the positive and negative sense with regard to the specimen axis; repeat this basic cycle at a rate of 5 cycles per minute for a number of times (number of cycles) given by the specified severity. - AFTER test: Dismount the specimen.. Parameters & Severity Severity Variable; Severity Value. Torque Number of cycles Minimum QUANTITY to test; 0.5 Nm 50 3 units.. Evaluation Criteria  No impairment of functionality or optical appearance  No mechanical damage (especially, loose or broken parts, cracks or permanent deformation). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ○. Electric Characteristics. ○. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M007 (GQR) | Temperature Difference between Phone and Holder |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ○. Electric Characteristics. ○. | Test Modules: Mechanical TAD_M007: Temperature Difference between Phone and Holder Test Purpose  This test reveals the ability of a product/component to properly operate under earth-typical environmental conditions. Specifically, it examines effects due to Temperature Difference between Phone and Holder.  This test may be used to simulate product/component operation under winter conditions when mobile phone at room temperature is inserted into cold holder in the car.  CAUTION: This test does not make sense before the relevant mechanical tolerances have been finalized. Before executing this test, the test TAD_S002: Vibration & Shock at Low and High Temperature must have been executed and passed.. Scope of Testing  Mechanical tolerances between mobile phone and holder; special attention to interface connections, locking mechanism. Equipment Requirements  Mobile phone (3 units) at room temperature. Procedure & Conditions  The test procedure described in the following points shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics - BEFORE test: Cool down the holder to the low temperature and wait at this temperature long enough to achieve thermal equilibrium. - DURING test: Insert the mobile phone at room temperature into the holder.  Measurements Specifics - DURING test: Check mechanical tolerances and functions.. Parameters & Severity  Parameters Specifics - Room temperature = (+25 +/-2) °C Severity Variable; Severity Value. Low temperature Minimum QUANTITY to test; (-10 +/-2) °C 3 units.. Evaluation Criteria  No impairment of functionality (especially, phone and holder must fit together; insert/remove and lock/unlock must work). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ○. Electric Characteristics. ○. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M101 (GQR) | Straight-Pull Static Load on Assembled Cable |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M101: Straight-Pull Static Load on Assembled Cable Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a cable at its mechanical interface to another component. Specifically, it examines effects due to application of Straight-Pull Static Load on Assembled Cable.  This test may be used to study effects of normal and rough handling of a product/component.. Scope of Testing  Mechanical stability and electrical characteristics of a cable and its wires; cable is assembled permanently with another component (e.g., cable to housing). Equipment Requirements  Specimen-specific fixture tool and load  Specimen usually consists of a single cable assembled with a plug, a housing, etc.; if more parallel cables are used in such an assembly, they shall be tested together and considered as a single cable for all purposes of this specification.. Procedure & Conditions  The test procedure described in the following points shall be successively executed for a number of new specimens (see Minimum QUANTITY to test) and at temperatures (low, room, high), as given by the specified severity.  Conditioning Specifics - BEFORE test: Mount the specimen (axis in vertical direction) in the fixture tool with the cable hanging down and apply a selected temperature (be sure to achieve thermal equilibrium before testing). - DURING test: Apply slowly (without dropping) the load given by the specified severity and stress the specimen for 1 minute. - AFTER test: Remove the load and dismount the specimen.  Measurements Specifics Determine electrical characteristics meaningful to the performance of each conductor/wire of the specimen (e.g., ohmic resistance, wave impedance) and prepare measurement setup. - BEFORE test: Record initial characteristics. - DURING test: Monitor and record transitional characteristics under stress. Observe the stability of the cable sleeve and of the cable shape. - AFTER test: Record final characteristics.. Scheme of testing setup. Parameters & Severity Severity Variable; Severity Value. Load Low temperature Room temperature High temperature Minimum QUANTITY to test; 30 N (0 +/-2) °C (+25 +/-2) °C (+45 +/-2) °C 9 units (3 units per temperature with 3 temperatures: low, room, high) [indoor products]. Load Low temperature Room temperature High temperature Minimum QUANTITY to test; 30 N (-25 +/-2) °C (+25 +/-2) °C (+70 +/-2) °C 9 units (3 units per temperature with 3 temperatures: low, room, high) [outdoor and car products].. Evaluation Criteria  No impairment of functionality (especially, connectivity of each conductor/wire and mutual insulation); the difference between the initial and the final characteristics should not exceed 1 per cent.  No damage to cable sleeve and/or cable shape. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M102 (GQR) | Periodic Bending under Load of Assembled Cable |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M102: Periodic Bending under Load of Assembled Cable Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a cable at its mechanical interface to another component. Specifically, it examines effects due to application of Periodic Bending under Load of Assembled Cable.  This test may be used to study effects of normal and rough handling of a product/component.. Scope of Testing  Mechanical stability and electrical characteristics of a cable and its wires; cable is assembled permanently with another component (e.g., cable to housing). Equipment Requirements  Specimen-specific fixture tool and load  Specimen usually consists of a single cable assembled with a plug, a housing, etc.; if more parallel cables are used in such an assembly, they shall be tested together and considered as a single cable for all purposes of this specification.  The cable bending relative to specimen axis is usually implemented in a testing setup with a moving fixture tool and a loaded cable kept in vertical position.  The testing procedure described below assumes a testing setup capable of double-sided bending from 0° (vertical axis) to +65° and -65° which can be applied to a specimen in any axial orientation (e.g., initial at 0° and rotated by 90°).. Procedure & Conditions  The test procedure described in the following points shall be executed at room temperature for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics - BEFORE test: Mount the specimen (axis in vertical direction) in the fixture tool with the cable hanging down. - DURING test: Apply slowly (without dropping) the load given by the specified severity. Execute the first cycle in the initial orientation by bending the cable away from the specimen axis by +65° (e.g., to the right), then by -65° (e.g., to the left) in the opposite direction, and then returning to the starting position in vertical direction; repeat this basic cycle at a rate of not less than 60 cycles per minute for a number of times (number of cycles) given by the specified severity. Remove the load, rotate the specimen in its vertical axis by 90°, and execute the same procedure in this orientation. - AFTER test: Remove the load and dismount the specimen.  Measurements Specifics Determine electrical characteristics meaningful to the performance of each conductor/wire of the specimen (e.g., ohmic resistance, wave impedance) and prepare measurement setup. - BEFORE test: Record initial characteristics. - DURING test: Monitor and record transitional characteristics under stress. Observe the stability of the cable sleeve and of the cable shape. - AFTER test: Record final characteristics.. Scheme of testing setup. Parameters & Severity Severity Variable; Severity Value. Load Room temperature Number of cycles Minimum QUANTITY to test (AF cable); 3 N (+25 +/-2) °C 5000 3 units [AF cable movable in normal product usage]. Load Room temperature Number of cycles Minimum QUANTITY to test (AF cable); 3 N (+25 +/-2) °C 200 3 units [AF cable fixed in normal product usage]. Load Room temperature Number of cycles Minimum QUANTITY to test (RF cable); 2 N (+25 +/-2) °C 5000 3 units [RF cable movable in normal product usage]. Load Room temperature Number of cycles Minimum QUANTITY to test (RF cable); 2 N (+25 +/-2) °C 200 3 units [RF cable fixed in normal product usage].. Evaluation Criteria  No impairment of functionality (especially, connectivity of each conductor/wire and mutual insulation); the difference between the initial and the final characteristics should not exceed 1 per cent.  No damage to cable sleeve and/or cable shape. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M103 (GQR) | 90-Degree-Bend Static Load on Assembled Cable |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M103: 90-Degree-Bend Static Load on Assembled Cable Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a cable at its mechanical interface to another component. Specifically, it examines effects due to application of 90-Degree-Bend Static Load on Assembled Cable.  This test may be used to study effects of normal and rough handling of a product/component.. Scope of Testing  Mechanical stability and electrical characteristics of a cable and its wires; cable is assembled permanently with another component (e.g., cable to housing). Equipment Requirements  Specimen-specific fixture tool and load  Specimen usually consists of a single cable assembled with a plug, a housing, etc.; if more parallel cables are used in such an assembly, they shall be tested together and considered as a single cable for all purposes of this specification.. Procedure & Conditions  The test procedure described in the following points shall be successively executed for a number of new specimens (see Minimum QUANTITY to test) and at temperatures (low, room, high), as given by the specified severity.  Conditioning Specifics - BEFORE test: Mount the specimen (axis in horizontal direction) in the fixture tool with the cable hanging down at 90° to specimen axis and apply a selected temperature (be sure to achieve thermal equilibrium before testing). - DURING test: Apply slowly (without dropping) the load given by the specified severity and stress the specimen for 1 minute. Remove the load, rotate the specimen in its horizontal axis by 90°, and re-apply the load for another 1 minute. Repeate this procedure also for rotations of 180° and 270°, thus successively stressing the specimen in all 4 basic orientations. - AFTER test: Remove the load and dismount the specimen.  Measurements Specifics Determine electrical characteristics meaningful to the performance of each conductor/wire of the specimen (e.g., ohmic resistance, wave impedance) and prepare measurement setup. - BEFORE test: Record initial characteristics. - DURING test: Monitor and record transitional characteristics under stress. Observe the stability of the cable sleeve and of the cable shape. - AFTER test: Record final characteristics.. Scheme of testing setup. Parameters & Severity Severity Variable; Severity Value. Load Low temperature Room temperature High temperature Minimum QUANTITY to test; 10 N (0 +/-2) °C (+25 +/-2) °C (+45 +/-2) °C 9 units (3 units per temperature with 3 temperatures: low, room, high) [indoor products]. Load Low temperature Room temperature High temperature Minimum QUANTITY to test; 10 N (-25 +/-2) °C (+25 +/-2) °C (+70 +/-2) °C 9 units (3 units per temperature with 3 temperatures: low, room, high) [outdoor and car products].. Evaluation Criteria  No impairment of functionality (especially, connectivity of each conductor/wire and mutual insulation); the difference between the initial and the final characteristics should not exceed 1 per cent.  No damage to cable sleeve and/or cable shape. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M104 (GQR) | Straight-Pull Static Load on Stand-Alone Cable |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M104: Straight-Pull Static Load on Stand-Alone Cable Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a cable. Specifically, it examines effects due to application of Straight-Pull Static Load on Stand-Alone Cable.  This test may be used to study effects of normal and rough handling of a product/component.. Scope of Testing  Mechanical stability and electrical characteristics of a cable and its wires. Equipment Requirements  Specimen-specific fixture tool (edge radius 0.3 mm) and load  Specimen consists of a cable.. Procedure & Conditions  The test procedure described in the following points shall be successively executed for a number of new specimens (see Minimum QUANTITY to test) and at temperatures (low, room, high), as given by the specified severity.  Conditioning Specifics - BEFORE test: Mount the specimen (axis in vertical direction) in the fixture tool with the cable hanging down and apply a selected temperature (be sure to achieve thermal equilibrium before testing). - DURING test: Apply slowly (without dropping) the load given by the specified severity and stress the specimen for 1 minute. - AFTER test: Remove the load and dismount the specimen.  Measurements Specifics Determine electrical characteristics meaningful to the performance of each conductor/wire of the specimen (e.g., ohmic resistance, wave impedance) and prepare measurement setup. - BEFORE test: Record initial characteristics. - DURING test: Monitor and record transitional characteristics under stress. Observe the stability of the cable sleeve and of the cable shape. - AFTER test: Record final characteristics.. Scheme of testing setup. Parameters & Severity Severity Variable; Severity Value. Load Low temperature Room temperature High temperature Minimum QUANTITY to test; 50 N (-25 +/-2) °C (+25 +/-2) °C (+70 +/-2) °C 9 units (3 units per temperature with 3 temperatures: low, room, high).. Evaluation Criteria  No impairment of functionality (especially, connectivity of each conductor/wire and mutual insulation); the difference between the initial and the final characteristics should not exceed 1 per cent.  No damage to cable sleeve and/or cable shape. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M105 (GQR) | Periodic Bending under Load of Stand-Alone Cable |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M105: Periodic Bending under Load of Stand-Alone Cable Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a cable. Specifically, it examines effects due to application of Periodic Bending under Load of Stand-Alone Cable.  This test may be used to study effects of normal and rough handling of a product/component.. Scope of Testing  Mechanical stability and electrical characteristics of a cable and its wires. Equipment Requirements  Specimen-specific fixture tool (edge radius 0.3 mm) and load  Specimen consists of a cable.  The cable bending relative to specimen axis is usually implemented in a testing setup with a moving fixture tool and a loaded cable kept in vertical position.  The testing procedure described below assumes a testing setup capable of double-sided bending from 0° (vertical axis) to +90° and -90° which can be applied to a specimen in any axial orientation (e.g., initial at 0° and rotated by 90°).. Procedure & Conditions  The test procedure described in the following points shall be successively executed for a number of new specimens (see Minimum QUANTITY to test) and at temperatures (low, room, high), as given by the specified severity.  Conditioning Specifics - BEFORE test: Mount the specimen (axis in vertical direction) in the fixture tool with the cable hanging down. - DURING test: Apply slowly (without dropping) the load given by the specified severity. Execute the first cycle in the initial orientation by bending the cable away from the specimen axis by +90° (e.g., to the right), then by -90° (e.g., to the left) in the opposite direction, and then returning to the starting position in vertical direction; repeat this basic cycle at a rate of not less than 60 cycles per minute for a number of times (number of cycles) given by the specified severity. Remove the load, rotate the specimen in its vertical axis by 90°, and execute the same procedure in this orientation. - AFTER test: Remove the load and dismount the specimen.  Measurements Specifics Determine electrical characteristics meaningful to the performance of each conductor/wire of the specimen (e.g., ohmic resistance, wave impedance) and prepare measurement setup. - BEFORE test: Record initial characteristics. - DURING test: Monitor and record transitional characteristics under stress. Observe the stability of the cable sleeve and of the cable shape. - AFTER test: Record final characteristics.. Scheme of testing setup. Parameters & Severity Severity Variable; Severity Value. Load Low temperature Room temperature High temperature Number of cycles Minimum QUANTITY to test (AF cable); 3 N (-25 +/-2) °C (+25 +/-2) °C (+70 +/-2) °C 5000 9 units (3 units per temperature with 3 temperatures: low, room, high) [AF cable movable in normal product usage]. Load Low temperature Room temperature High temperature Number of cycles Minimum QUANTITY to test (AF cable); 3 N (-25 +/-2) °C (+25 +/-2) °C (+70 +/-2) °C 200 9 units (3 units per temperature with 3 temperatures: low, room, high) [AF cable fixed in normal product usage]. Load Low temperature Room temperature High temperature Number of cycles Minimum QUANTITY to test (RF cable); 2 N (-25 +/-2) °C (+25 +/-2) °C (+70 +/-2) °C 5000 9 units (3 units per temperature with 3 temperatures: low, room, high) [RF cable movable in normal product usage]. Load Low temperature Room temperature High temperature Number of cycles Minimum QUANTITY to test (RF cable); 2 N (-25 +/-2) °C (+25 +/-2) °C (+70 +/-2) °C 200 9 units (3 units per temperature with 3 temperatures: low, room, high) [RF cable fixed in normal product usage].. Evaluation Criteria  No impairment of functionality (especially, connectivity of each conductor/wire and mutual insulation); the difference between the initial and the final characteristics should not exceed 1 per cent.  No damage to cable sleeve and/or cable shape. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M106 (GQR) | Straight-Pull Dynamic Load on Helical Cable |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M106: Straight-Pull Dynamic Load on Helical Cable Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a cable. Specifically, it examines effects due to application of Straight-Pull Dynamic Load on Helical Cable.  This test may be used to study effects of normal and rough handling of a product/component.. Scope of Testing  Mechanical stability and electrical characteristics of a cable and its wires. Equipment Requirements  Specimen-specific fixture tools (fixed and moving; edge radius 0.3 mm) and load  Specimen consists of a helical cable.. Procedure & Conditions  The test procedure described in the following points shall be successively executed for a number of new specimens (see Minimum QUANTITY to test) and at temperatures (low, room, high), as given by the specified severity (note that the number of cycles for low & high temperatures may be different from the value for room temperature).  Conditioning Specifics - BEFORE test: Mount the specimen in horizontal direction between the fixed and moving parts of the fixture tool in such a way that the cable is loosely contracted (i.e., without any force applied) and its marked length comprises 30 windings. - DURING test: Execute the first cycle by pulling on the cable in horizontal direction up to the given elongation length (see below) and then return back to the initial loosely contracted position; repeat this basic cycle at a rate of not less than 60 cycles per minute for a number of times (number of cycles) given by the specified severity. - AFTER test: Dismount the specimen and keep it in loosely contracted state for a recovery period of 24 hours before measuring its marked length.  Measurements Specifics Determine electrical characteristics meaningful to the performance of each conductor/wire of the specimen (e.g., ohmic resistance, wave impedance) and prepare measurement setup. - BEFORE test: Record initial characteristics; measure loosely contracted cable length of those 30 windings which will be subjected to stressing. - DURING test: Monitor and record transitional characteristics under stress. Observe the stability of the cable sleeve and of the cable shape. - AFTER test: Record final characteristics; measure loosely contracted cable length of those 30 windings which have been subjected to stressing.. Scheme of testing setup. Parameters & Severity  Parameters Specifics - Elongation length = 90 x diameter of windings Severity Variable; Severity Value. Low temperature Room temperature High temperature Number of cycles (low & high) Number of cycles (room) Minimum QUANTITY to test; (-25 +/-2) °C (+25 +/-2) °C (+70 +/-2) °C 5000 15000 9 units (3 units per temperature with 3 temperatures: low, room, high).. Evaluation Criteria  No impairment of functionality (especially, connectivity of each conductor/wire and mutual insulation); the difference between the initial and the final characteristics should not exceed 1 per cent.  No damage to cable sleeve and/or cable shape  The marked length of the loosely contracted cable must not exceed 1.25 times its original length.. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  | Ref: EN 60529 |
| TAD_M801 (GQR) | IP Class Protection |  |  |  |
|  | see Ref, Degrees of protection provided by enclosures (IP Code) |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. | Test Modules: Mechanical TAD_M801: IP Class Protection Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to penetration of foreign bodies and water inside the housing according to test specifications for IP Class Protection.  This test may be used to study effects of normal and rough handling of a product/component.  see Ref, Introduction; see Ref, Clause 1 through 10. Scope of Testing  Protection of inner parts of the product/component against accidental contact, insertion of foreign bodies, water intrusion. Equipment Requirements  see Ref, Clause 11 through 15. Procedure & Conditions  The test procedure described in Ref, Clause 11 through 15 shall be executed for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics & Exceptions - DURING test: The specimen is tested in an operational state as specified in section 'Product-specific Comments' below.. Parameters & Severity  see Ref, Clause 11 through 15 Severity Variable; Severity Value. IP Code Minimum QUANTITY to test; IP 30 1 unit [protection class given by Product Document and/or TTD]. IP Code Minimum QUANTITY to test; IP 40 2 units [protection class given by Product Document and/or TTD]. IP Code Minimum QUANTITY to test; IP 54 3 units [protection class given by Product Document and/or TTD].. Evaluation Criteria  IP Class protection must satisfy the standard for given IP Code.. | Post-Test Examinations. Visual Inspection. ●. Functional Check. ○. Acoustic Characteristics. ○. Electric Characteristics. ○. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M901 (GQR) | Lifetime of Mechanical Operating Elements |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. | Test Modules: Mechanical TAD_M901: Lifetime of Mechanical Operating Elements Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a product/component. Specifically, it examines effects due to repeated usage or triggering, thus indicating Lifetime of Mechanical Operating Elements.  This test may be used to study effects of normal handling of a product/component over its entire lifetime.. Scope of Testing  Mechanical behaviour and electrical characteristics of (movable) operating elements (e.g., keys) of the product/component during a simulated long-term usage. Equipment Requirements  Mechanical equipment capable of repeatedly actuating the operating element under test, e.g., universal tester; specimen-specific fixture tool. Procedure & Conditions  The test procedure described in the following points shall be executed at room temperature for a number of new specimens (see Minimum QUANTITY to test), as given by the specified severity.  Conditioning Specifics All applicable operating elements of the specimen indicated in the severity table shall be successively subjected to mechanical stressing which simulates a normal handling. - BEFORE test: Mount the specimen in the fixture tool of universal tester. - DURING test: Actuate each operating element for a number of times (number of cycles) given by the specified severity. - AFTER test: Dismount the specimen.. Parameters & Severity  Parameters Specifics - Room temperature = (+25 +/-2) °C Severity Variable; Severity Value. Number of cycles Minimum QUANTITY to test (key used frequently); 10000 3 units [key used frequently]. Number of cycles Minimum QUANTITY to test (key used rarely); 3000 3 units [key used rarely]. Number of cycles Minimum QUANTITY to test (quiver/button/latch); 3000 3 units [quiver/button/latch]. Number of cycles Minimum QUANTITY to test (RF connector used frequently); 3000 3 units [RF connector used frequently]. Number of cycles Minimum QUANTITY to test (RF connector used rarely); 100 3 units [RF connector used rarely]. Number of cycles Minimum QUANTITY to test (I/O connector used frequently); 10000 3 units [I/O connector used frequently]. Number of cycles Minimum QUANTITY to test (I/O connector used rarely); 3000 3 units [I/O connector used rarely]. Number of cycles Minimum QUANTITY to test (clip); 500 3 units [clip].. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, loose or broken parts, cracks or permanent deformation). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ●. Electric Characteristics. ●. |

| Test Number | __________________________Description | Specimens | 0 | Standard |
| --- | --- | --- | --- | --- |
|  | Test Modules: Mechanical | to Test |  |  |
| TAD_M902 (GQR) | Periodic Bending of Belt Clip |  |  |  |
| Pre-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ○. Electric Characteristics. ○. | Test Modules: Mechanical TAD_M902: Periodic Bending of Belt Clip Test Purpose  This test reveals mechanical weakness and/or degradation in the specified performance of a belt clip. Specifically, it examines effects due to repeated usage by application of Periodic Bending of Belt Clip.  This test may be used to study effects of normal and rough handling of a belt clip over its entire lifetime.. Scope of Testing  Mechanical behaviour of belt clip during a simulated long-term usage. Equipment Requirements  Mechanical equipment capable of repeatedly actuating the belt clip, e.g., universal tester; specimen-specific fixture tool.  The testing procedure described below assumes a testing setup capable of changing the stress angle from 0° to an upper value (high, low) and then to 0° again within a given time (stress time); this is followed by a given relaxation period at 0° (relaxation time).. Example of a clip bend tester. Procedure & Conditions  The test procedure described in the following points shall be successively executed at room temperature for a number of new specimens (see Minimum QUANTITY to test) and both stress angles (high, low), as given by the specified severity.  Conditioning Specifics The specimen (belt clip) shall be subjected to mechanical testing with high stress angle (rough handling) first, followed then by low stress angle (normal handling). - BEFORE test: Mount the specimen in the fixture tool of clip bend tester. - DURING test: Actuate the specimen repeatedly according to number of cycles and associated parameters in the severity table (high stress angle, low stress angle). - AFTER test: Dismount the specimen.  Measurements Specifics - DURING test: Check mechanical integrity and function of the specimen repeatedly after a number of cycles (check after cycles) given by the specified severity.. Parameters & Severity  Parameters Specifics - Room temperature = (+25 +/-2) °C Severity Variable; Severity Value. High stress angle Stress time Relaxation time Check after cycles Number of cycles Minimum QUANTITY to test; 45° 0.5 seconds 5.5 seconds 10 100 3 units (used for both stress levels). Low stress angle Stress time Relaxation time Check after cycles Number of cycles Minimum QUANTITY to test; 20° 1 seconds 5 seconds 1000 5000 3 units (used for both stress levels).. Evaluation Criteria  No impairment of functionality  No mechanical damage (especially, permanent deformation or damages at bending radius). | Post-Test Examinations. Visual Inspection. ●. Functional Check. ●. Acoustic Characteristics. ○. Electric Characteristics. ○. |