Chapter VI

Quality Requirements for Packaging

Coordinators:	Hans-Jürgen Lindau		QM PL PPM

Antonia Wüstenberg		PBM SP

Wen Hsin Lin			Y2Q (QRE)

JC Pan				BM2 (LDC)

1	Quality requirements for packaging	3

1.1	Packing SLT- test for the over-box used for the packaging manner B and C	3

1.1.1	Mechanical function check	4

1.1.2	Storage test	4

1.1.3	Drop test	4

1.1.4	Vibration test (random)	5

1.1.5	Drop test for the retail box	6

1.2	Packing SLT- test for the retail-box used for the packaging manner A	7

1.2.1	Mechanical function check	7

1.2.2	Storage test	7

1.2.3	Drop test	7

1.2.4	Vibration test (random)	7

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
|  | # Quality requirements for packaging The packaging is to be designed in such a way that it is resistant to all usual storage and transport conditions (truck freight, air cargo, sea freight) for protecting its content from mechanical and climatic influences. Damaged packages (e.g. after drop) may not lead to injuries (sharp edges). Based on the three different packaging manners (A: Retail-box in pallet carton, B: Over-box only, C: Over-box in pallet carton see Fig. 8.1) the over-box and the retail-box are subjected to environmental testing. Fig.8.1 ## Packing SLT- test for the over-box used for the packaging manner B and C Mechanical Function Check Storage Test Drop Test Vibration Test Mechanical Function Check SLT test flow for packaging Quantity to be tested: 1 over-box 1) The quantity to be tested is one over-box; the handset quantity to be tested varies with the capacity of the over-box. Since non-functional handsets are used, only the mechanical functions are to be checked. 1) After the storage test the box is stored for at least 3 hours at room temperature in order to remove humidity; thereafter the box is subjected to the drop test. |  |  |
| Quantity to test: | 1 over-box |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
|  | ### Mechanical function check Conditions: Temperature: RT Test Items: - Handset: Fitting accuracy of plugs, battery covers, sliders, clamp shells / hinge etc. - Packaging: No broken material is accepted. Evaluation criteria: - No material deformation or crack is allowed. ### Storage test Step 1: Storage high temp./humidity exposure test (e. g. boat) 25 °C/60% for 1hr60 °C/90% for 120hrs25 °C/60% for 1hr Step 2: Storage / transportation test (e. g. railway) 25 °C/60% for 1hr-20 °C for 24hrs25 °C/60% for 1hr 60 °C/90% for 24hrs 25 °C/60% for 1 hr (= 1 cycle) Number of repetitions: 2 cycles After finishing the two steps the mechanical function /visual check has to be done. Evaluation criteria: - Any change or damage to the hand set is not allowed. - No crack in the packing material is allowed. - Handsets and accessories must not show any signs of condensation water. ### Drop test Method & Conditions: Surface (floor): concrete or steel Height: 106 cm in case of gross weight < 9 kg 91 cm in case of gross weight > 9 kg Number of drops: 10 Drop positions: see Fig. 8.2 temperature: RT |  |  |
| Quantity to test: | 1 over-box see Packaging SLT test flow |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
|  | Fig. 8.2 Evaluation criteria: - No damage to the handset and accessories is allowed: no material deformation, crack or broken parts - Neither cracks nor broken parts of the over-box are allowed. - The over-box shall not get open so that the retail-boxes get out of the over-box. ### Vibration test (random) Method & Conditions: Frequency range: from 5 Hz to maximum 200 Hz; Grms: 1.47 G Vibration: all 3 axes (X, Y and Z) Test duration: 30 min per axis (= 1.5 hrs) Frequency (Hz); PSD. 5; 0.015. 100; 0.015. 200; 0.0038. Evaluation criteria: - No damage to the handset and accessories is allowed: no material deformation, crack or broken parts - Neither cracks nor broken parts of the over-box are allowed. - No scuffing at any position is accepted. |  |
| Quantity to test: | 1 over-box see Packaging SLT test flow |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
|  | ### Drop test for the retail box After finishing the SLT packaging test (including evaluation of packaging, handset and accessories), two of the retail boxes, which have survived the SLT packaging test with the over-box best, are taken for the drop test under the following conditions: Method & Conditions: Surface (floor): concrete or steel Height: 80 cm Number of drops: 7 Drop positions: see Fig. 3 temperature: RT Fig.8.3 : drop positions: 1 corner 3 adjacent edges 1-3 3 adjacent surfaces A-C Evaluation criteria: - No damage to the handset and accessories is allowed: no material deformation, crack or broken parts - Neither cracks nor broken parts of the over-box are allowed. - The retail-box shall not get open so that the handset or accessories get out of the over-box. |  |  |
| Quantity to test: | 2 retail-boxes |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
|  | ## Packing SLT- test for the retail-box used for the packaging manner A ### Mechanical function check Method & Conditions and evaluation criteria: The same as for the over-box ### Storage test Method & Conditions and Evaluation criteria: The same as for the over-box ### Drop test Method & Conditions and evaluation criteria: As described in section 1.1.5 Drop test for the retail-box ### Vibration test (random) Method & Conditions: The same as for the over-box Evaluation criteria: - No damage to the handset and accessories is allowed: no material deformation, crack or broken parts - Neither cracks nor broken parts of the retail-box are allowed. - No scuffing at any position is accepted. |  |  |
| Quantity to test | 5 retail-boxes |  |  |