Chapter III 1

Accessory-

Tests

Cordinators:		Ahmet Akdeniz	 	QM PL AC

Wen Hsin Lin 		 Y2Q (QRE)

Menzi Liang		WS0 (RD)

WT Liu			W10 (RD)

Barry Dai			WW0 (PM)

General Quality Requirements (Com MD - Accessory Devices)

This chapter defines the general quality requirements by referencing two associated documents, by stating the targets for accessory devices, and by providing an overview of environmental testing.

These two documents provide a common basis for definition of product-specific quality requirements and environmental tests, and they become an integral part of the contractual agreement with SUPPLIER.

Development of accessory devices follows generally acknowledged design principles (state-of-the-art), as well as international and SIEMENS standards for product safety, recycling, and environmental compatibility.

Targets for Reliability and Quality

The quality of the product, as manifested by the number of product returns from CUSTOMER to SIEMENS, is measured by the return rate (see definition in the above-referenced document "GQR QM Annex"). Because of our goal to keep the quality level as high as possible, the return rate will be negotiated with SUPPLIER on a case by case basis.

Following values for the complete product are targeted by the management of Com MD - Accessory Devices:

| Usability Period | Target 3 years |  |
| --- | --- | --- |

Targets for Climatic and Environmental Influences

The product document (M0, S0, M1) will define a particular, product-specific value from the following general ranges of targets for accessory products.

| Temperature Ranges |  |  |  |
| --- | --- | --- | --- |
| Storage & Transportation |  |  |  |
|  | All Products |  | -40°C to +85°C |
| Full Functionality |  |  |  |
|  | Car & Outdoor Products |  |  |
|  |  | power source (external) | -20°C to +65°C |
|  |  | battery operation | -10°C to +55°C |
|  | Office & Indoor Products |  | 0°C to +45°C |
| Limited Functionality |  |  |  |
|  | Car & Outdoor Products |  |  |
|  |  | low temperature range: battery operation external power source | -25°C to -10°C -25°C to -20°C |
|  |  | high temperature range: battery operation external power source | +55°C to +70°C +65°C to +70°C |
|  | Office & Indoor Products |  |  |
|  |  | low temperature range | -10°C to 0°C |
|  |  | high temperature range | +45°C to +60°C |
| Sun Radiation |  |  |  |
| Irradiance (device switched off) |  |  |  |
|  | Car & Outdoor Products |  | 1120 W/m² |
|  | Office & Indoor Products |  | 700 W/m² |
| Other Ambient Influences |  |  |  |
| Contact - Foreign Body - Water Intrusion |  |  |  |
|  | IP Class |  | IP 30 or IP 40 or IP 54 |

Range regarding Humidity ?

Test Plan Overview

Each product/component must be tested according to an environmental test plan which is based on test specifications of the above-referenced document "GQR QM Tests". The product-specific test plan is prepared by Product Development and agreed with Quality Management. The Product Development is responsible for execution of all tests; the results are presented to Quality Management.

The table on the next page summarizes all product/component groups (indoor, outdoor, etc.) and all test specifications provided today; whenever a test is applicable, it is defined as mandatory (M), provisionally mandatory (m, a, c), or optional (O); the cable tests (c) are mandatory for all cable products and all other products/components with cables permanently assembled. In addition, the tests marked by (s) are understood as specifications for a single step of a Test Series, i.e., they are not executed stand-alone.

| General Quality Requirements ( Com MD - Accessory Devices) TEST PLAN OVERVIEW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | INDOOR Usage |  |  |  | OUTDOOR Usage |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Legend |  |  |  |  | CAR Accessory |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | M mandatory m mandatory (if applicable component present) a mandatory (if coated or metal surfaces present) c mandatory (if cables permanently assembled) O optional (agreement with QM) s specification for Test Series (not stand-alone) | Indoor Accessory |  |  | Indoor Cable | Carkit E Box | Carkit Display | Active Holder | Passive Holder | E2i Holder | Cig Lighter Appl |  |  | Outdoor Accessory | Carrying Case | Headset |  |  | Outdoor Cable |
|  | Test Series |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TAD_S001 | Vibration & Shock at Room Temperature | O |  |  | O |  |  |  |  |  |  |  |  | O | O | M |  |  | O |
| TAD_S002 | Vibration & Shock at Low and High Temperatures |  |  |  |  | M | M | M | M | M | M |  |  |  |  |  |  |  |  |
| TAD_S003 | Nickel Resistance after Sweat Exposure | O |  |  |  |  | O | O | O |  | O |  |  | O | M | M |  |  |  |
|  | Test Modules: Ageing / Material |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TAD_A001 | Cyclic Dry Heat | O |  |  |  | M | M | O | O | O | M |  |  | M |  | M |  |  | O |
| TAD_A002 | Steady-State Damp Heat |  |  |  |  | M | M | O |  |  | M |  |  | M | O | M |  |  | O |
| TAD_A003 | Dust and Sand | O |  |  |  | M | M | O | O | O | M |  |  | M |  | M |  |  | O |
| TAD_A004 | Gas Mixture | O |  |  |  | M | M | O |  |  | M |  |  | O |  | M |  |  | O |
| TAD_A005 | Steady-State Solar Radiation | O |  |  | O |  | O | O | O | O | O |  |  | O | O | O |  |  | O |
| TAD_A008 | Sweat | O |  |  |  |  | O | O | O |  | O |  |  | M | M | M |  |  |  |
| TAD_A009 | Nickel | s |  |  |  |  | s | s | s |  | s |  |  | s | s | s |  |  |  |
| TAD_A010 | Cream, Oil, Vaseline | O |  |  |  |  | O | O | O |  | O |  |  | M | M | M |  |  |  |
|  | Test Modules: Climatic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TAD_C001 | Cyclic Cold and Heat | M |  |  |  | M | M | M | M | O | M |  |  | M | M | M |  |  | M |
| TAD_C002 | Cyclic Damp Heat | M |  |  |  | M | M | M | M | O | M |  |  | M |  | M |  |  | M |
| TAD_C003 | Cyclic Solar Radiation | M |  |  | O |  | O | O | O | O | O |  |  | O | O | O |  |  | O |
|  | Test Modules: Mechanical |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TAD_M001 | Vibration | s |  |  | s | s | s | s | s | s | s |  |  | s | s | s |  |  | s |
| TAD_M002 | Shock | s |  |  | s | s | s | s | s | s | s |  |  | s | s | s |  |  | s |
| TAD_M003 | Free Fall | M |  |  |  | M | M | M | M | O | M |  |  | M |  | M |  |  |  |
| TAD_M004 | Abrasion | a |  |  | a | a | a | a | a | a | a |  |  | a | a | a |  |  | a |
| TAD_M005 | Static Load | M |  |  |  | M | O | M | M | O | M |  |  | M |  |  |  |  |  |
| TAD_M006 | Twist | O |  |  |  |  | O |  |  |  |  |  |  | O |  |  |  |  |  |
| TAD_M007 | Temperature Difference between Phone and Holder |  |  |  |  |  |  | M | M | M |  |  |  |  |  |  |  |  |  |
| TAD_M101 | Straight-Pull Static Load on Assembled Cable | c |  |  | M | c | c | c | c | c | c |  |  | c |  | c |  |  | M |
| TAD_M102 | Periodic Bending under Load of Assembled Cable | c |  |  | M | c | c | c | c | c | c |  |  | c |  | c |  |  | M |
| TAD_M103 | 90-Degree-Bend Static Load on Assembled Cable | c |  |  | M | c | c | c | c | c | c |  |  | c |  | c |  |  | M |
| TAD_M104 | Straight-Pull Static Load on Stand-Alone Cable | c |  |  | M | c | c | c | c | c | c |  |  | c |  | c |  |  | M |
| TAD_M105 | Periodic Bending under Load of Stand-Alone Cable | c |  |  | M | c | c | c | c | c | c |  |  | c |  | c |  |  | M |
| TAD_M106 | Straight-Pull Dynamic Load on Helical Cable | c |  |  | m | c | c | c | c | c | c |  |  | c |  | c |  |  | m |
| TAD_M801 | IP Class Protection | O |  |  |  | O | O |  |  |  | O |  |  | O |  |  |  |  |  |
| TAD_M901 | Lifetime of Mechanical Operating Elements | O |  |  | M | O | O | M | M | O | O |  |  | O | O | O |  |  | M |
| TAD_M902 | Periodic Bending of Belt Clip |  |  |  |  |  |  |  |  |  |  |  |  |  | m |  |  |  |  |