Chapter V

Charger

Coordinators:	Andreas Wiltink			SCM QM SQM SQA MC

Wen Hsin Lin			Y2Q (QRE)

WB Liu				WL1 (RD)

Barry Dai				WW0 (PM)

1	Quality Requirements and Environmental Conditions for Chargers	3

1.1	Criteria for Acceptance	3

1.2	Agreed Quality Characteristics for Reliability	3

1.3	Agreed quality characteristics for time and consumption	3

1.3.1	Labeling:	3

1.4	Mechanical Environmental Conditions	4

1.4.1	Static load	4

1.4.2	Rip-off test	4

1.5	Simulated life test (SLT) for Chargers	5

1.5.1	Function test	6

1.5.2	Temperature shock	6

1.5.3	Drop test	7

1.5.4	Temperature cycling test	8

1.5.5	Temperature / humidity test	8

1.5.6	ESD test	9

1.5.7	Vibration random	10

1.6	Cable and Connector tests	11

1.6.1	Straight pull static load on assembled cable	11

1.6.2	Periodic bending under load of assembled cable	11

1.6.3	90° bending static load on assembled cable	12

1.6.4	Concluding electrical test	12

1.6.5	General cable and connector tests	12

1.6.6	Cable and plug design	12

1.7	Climatic Environmental Conditions	13

1.7.1	Temperature range - requirement for chargers	13

1.7.2	Humidity range - requirement for chargers	13

1.7.3	Damp Heat Test, Constant exposure	13

1.7.4	Heat with solar radiation	13

1.7.5	Mixed Gas Test / Salt fog test	13

1.7.6	High Temperature / cyclic	14

1.7.7	Dust Test	14

1.7.8	Cosmetic Test	14

1.7.9	Cleaning resistance Test	15

1.8	Mechanical Conditions for the single charger packaging	15

1.8.1	Packaging test series:	15

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
|  | # Quality Requirements and Environmental Conditions for Chargers ## Criteria for Acceptance See quality assurance agreement document. The acceptance criteria for a delivery lot, test criteria’s, test procedures and test reports, are fixed in a separate quality assurance agreement (QAA) ## Agreed Quality Characteristics for Reliability Housing: The housing of the equipment must be torsional rigid in installed condition. There must be no tangible play between the housing shells. Plug-in System: Sturdy plug-in connection with locking function. Appearance: Uniform housing surfaces free of paint streaks. Fixed by marginal product. Dirt-repellent surface resistant to fingerprints. Heating: The temperature on the housing of the charger is not allowed to exceed 85 °C under worst case condition and at an ambient temperature of 45 °C and after 30 min at full load. Resistance against cosmetics and oils: After contact with cosmetics/oils it must be guaranteed that a changing of the housing material behavior does not result in dangerous situations to the customer (see cosmetic test). ## Agreed quality characteristics for time and consumption There must be no clearly visible signs of scuffing through normal handling of the charger. ### Labeling: Inspection: The labeling must not exhibit any signs of breaks or pores when inspected from a distance of 25 cm. |  | EN 60950-1 |
| Test No. | Description |  | Standard |
| CH-1 CH-2 | Wipe resistance: The ink must not be rubbed off after being rubbed backwards and forwards ten times with a cloth soaked in white spirit. Slight transfer of colour to the cloth is permissible (see cleaning resistance test). Environmental Testing: The quality characteristics for printing quality, adhesive strength and wipe resistance must also be satisfied after climatic testing set out in section 1.7. The functions of the device must be assured over its entire service life, with the device remaining free of visible signs of wear. ## Mechanical Environmental Conditions ### Static load The housing of the charger must be able to withstand a partial pressure exerted by a spherical stamp with Ǿ 8mm and r = 10 mm without evidencing signs of damage when static load is applied in the middle of the biggest supporting surface of the charger. There may be no permanent deforming of the material. Method & Conditions: Force (stamp see above): 200 N Stamp motion in: 10 N / sec The stamp moves back after reaching the 200 N. Requirement: 100% functional after test, no physical change ### Rip-off test The purpose of this test is to control the welding process of the supplier. Method & Conditions: - Fix one of the two welding housing parts (e.g. plug-side) and pull off the other side (e.g.cover-side) by clamp-fixture until it is ripped off. - The clamp is only allowed to touch one of the housing parts. - Maximum force must be recorded. Pull speed: 10 mm / min_ Test temperature: ambient temperature: 23 °C ±5 °C Requirement: Recorded force maximum: > 600 N |  | Basic test Basic test |
| Quantity to Test | 3 units per test |  |  |
| Test No. | Description |  | Standard |
| SLT | ## Simulated life test (SLT) for Chargers Function test Voltage check Mechanical func. 14 units: 1 - 14 Vibration test 5 Hz to 500 Hz; 3g 1 h per axis (x, y, z) 14 units: 1 - 14 Function test Voltage check Mechanical func. 14 units: 1 - 14 Temperature shock -40/85 °C; 27 cycles 14 units: 1 - 14 Temp. Cycling -30 / 75 °C 48 hrs 4 units: 7 – 10 Drop at RT 100 cm 6 units: 1 - 6 Temp. Humidity 65 °C / 90% rel. H. 4 units: 11 – 14 ESD Air ± 10 kV 3 units: 4 - 6 ESD contact ± 6 kV 3 units: 1 - 3 Drop at RT 100 cm 8 units: 7 - 14 |  | Basic test |
| Quantity to Test | 14 units |  |  |
| Test No. | Description |  | Standard |
| SLT-CH1 SLT-CH2 | ### Function test Method & Conditions: Test temperature: ambient temperature: 23 °C ±5 °C Test items: - Voltage check - mechanical function - appearance of plastic housing and cable Evaluation criteria: - Mechanical function must pass specification. - No material deformation or crack is allowed ### Temperature shock Method & Conditions: Step 1: 45min1) at 85ºC, including an initial time for temperature stabilization Step 2: temperature change to -40ºC (temperature stabilization is to be reached within 5 min. Step 3: 45 min1) at -40ºC including an initial time for temperature stabilization Step 4: temperature change to 85ºC (temperature stabilization is to be reached within 5 min). Number of repetitions (from step 1 to step 4): 27 cycles - The exposure time is measured from the moment of insertion of the specimen into the test chamber. Evaluation criteria: - Mechanical function must pass specification. - No material deformation or crack is allowed |  | Basic test |
| Quantity to Test | 14 units see SLT test flow |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
| SLT-CH3 | ### Drop test Method & Conditions: Height of Drop: 1 m Type of Surface: concrete or steel Number of drops: 7 Drop direction: 1 corner, 3 adjacent edges, 3 adjacent surfaces Evaluation criteria: - Mechanical function must pass specification. - No material deformation or crack is allowed. - There shall be no mechanical damage to the test device (no loose parts inside). |  | Basic test |
| Quantity to Test | 6 units see SLT test flow |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
| SLT-CH4 SLT-CH5 | ### Temperature cycling test Method & Conditions: Step 1: 60 min1) at -30 ºC, including an initial time for temperature stabilization Step 2: temperature change to 75 °C within 60 min Step 3: 60 min1) at 75 ºC including an initial time for temperature stabilization Step 4: temperature change to at -30 ºC within 60 min Test duration: 48 h (repetitions from step 1 to step 4 for 48 h) 1) The exposure time is measured from the moment of insertion of the specimen into the test chamber. Evaluation criteria: - Mechanical function must pass specification. - No material deformation or crack is allowed. ### Temperature / humidity test Method & Conditions: Temperature: 65 °C Relative humidity: 90 ±3 % Test duration: 48 h Evaluation criteria: - Mechanical function must pass specification. - No material deformation or crack is allowed. |  | Basic test EN 60068-2-14 |
| Quantity to Test | 4 units acc. SLT test flow |  |  |
| Test No. | Description |  | Standard |
| SLT-CH6 | ### ESD test Method & Conditions: The test points include all openings. Charger has to be tested at 25°C and 30% ~ 60% rel. humidity. Evaluation criteria: - No SW/HW failure is allowed. For further detailed requirements see Chapter VII EMC+ EMF under section 1.4.1 ESD-requirements |  | Basic test |
| Quantity to Test | 6 units acc. SLT test flow |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
| SLT-CH7 | ### Vibration random Method & Conditions: Frequency range: from 5Hz to maximum 500Hz; Grms=3g Frequency (Hz); Acceleration (m/s2); Velocity; Way. 5; 0,296; 0,009; 0,300. 50; 30,000; 0,095; 0,304. 500; 30,000; 0,010; 0,003. Vibration: all 3 axes (X, Y and Z) Test duration: 1 h per axis (= 3 hrs) All handsets are to be tested in “power on” mode; 2 units with test SIM inserted. Evaluation criteria: - The functions acc. SLT-1 may not be affected or impaired in any way. - Neither material deformation nor a crack is allowed. - There shall be no mechanical damage to the test device (no loose parts inside). | Basic test IEC-68-2-36 |
| Quantity to Test | 14 units acc. SLT test flow |  |  |
| Test No. | Description |  | Standard |
| CH-Ca-1 CH-Ca-2 | ## Cable and Connector tests ### Straight pull static load on assembled cable - Clamp housing - Cable hanging vertically - Test temperature 0°, +25° and +45° - Load 30N (do not drop load into the cable) - Duration of load: 1 minute - 1 cycle - Electrical test as described in section 1.6.4 - Visual assessment of sleeve stability and shape retention ### Periodic bending under load of assembled cable - Plug positioning and cable displacement as indicated in drawing - 2000 cycles per 130° displacement position - Room temperature - Load cable with 3N - Electrical test as described in section 1.6.4 - Visual assessment of sleeve stability and shape retention 130° |  | Basic test |
| Quantity to Test | 3 units per test |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
| CH-Ca-3 CH-Ca-4 CH-Ca-5 CH-Ca-6 | ### 90° bending static load on assembled cable - Housing fixture and cable displacement as indicated in the drawing - Load 1 time in all four directions with 10N per cable for 1 minute - Apply load at 0°, +25° and +45° - Electrical test as described in section 1.6.4 - Visual assessment of sleeve stability and shape retention ### Concluding electrical test Test all individual wires for continuity singly and test inter-wire insulation. ### General cable and connector tests AC Connector: Plug-in cycles: 5000 actuations Requirement: No visible signs of wear Output cable: Plug-in cycles: 5000 Actuation Requirement (max. resistance): 40 m per contact No visible signs of wear Twisting cable: The cable should be twisted 1080 degrees (3 turns) to the left, back to relax situation and then 1080 degrees (3 turns) to the right. This should be done 15 cm behind the strain-relief from the housing. No malfunction of the product should occur during or after the procedure. The mechanical cable test must be performed according to A24851-A1-A27-*-7624 and A24851-A1-A28-*-7624. ### Cable and plug design The cable and plug design must be in accordance with technical delivery conditions and specifications for these parts. |  | Basic test |
| Quantity to Test | 3 units per test |  |  |
| Test No. | Description |  | Standard |
| CH-3 CH-4 CH-5 | ## Climatic Environmental Conditions ### Temperature range - requirement for chargers Storage Temp. Range (Package condition): -30°C to +85°C Fully Functioning: 0°C to +45°C ### Humidity range - requirement for chargers Storage Humidity (Package condition): 10 % to 90 % Fully Functioning: 15 % to 85 % ### Damp Heat Test, Constant exposure High Temperature: 40°C ± 2°C Relative humidity: 93% ± 2% Test duration: 10 days Requirements: 100% function; no physical change ### Heat with solar radiation This test has to be done in case of new housing material. Ambient Temperature: room temperature Irradiation: 700W/m² Duration: 8 hours Humidity: 50% Requirements: no physical change ### Mixed Gas Test / Salt fog test Composition of Gas: SO2 0.2 cm3/m3 H2S 0.01 cm3/m3 O2 0.2 cm3/m3 Cl2 0.01 cm3/m3 Test climate: 25°C/75% relative humidity Test duration: 10 days Requirements: Alternative: salt fog test Salt solution: 5 % NaCl (sodium chloride) Ambient temperature: pH value 6.5 - 7.2 (at 20°C ±2°C) Storage temperature: 20 °C Duration of spraying: 2 h Storage time: 22 h Number of repetitions: 1 Requirements: 100% function; no Increasing of the contact resistance; no corrosion |  | Basic test EN 60068-2-56 Optional test EN 60068-2-5/SaC Basic test EN 60068-2-60 DIN EN 60068-2-52 Strength 2 |
| Quantity to Test | 3 units per test |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
| CH-6 CH-7 CH-8 | ### High Temperature / cyclic Method & Conditions: Step 1: 16 h1) at 85 ºC, including an initial time for temperature stabilization Step 2: temperature change to 40 °C within 15 min Step 3: 8 h1) at 40 ºC including an initial time for temperature stabilization Step 4: temperature change to at 85 ºC within 15 min Test duration: 21 days (repetitions from step 1 to 4 for 21 d) Requirements: 100% Function; no physical change. ### Dust Test This test has to be done in case of an open design. Type of test: Ed Duration: 4 days Dust concentration: 6 ± 1g per day and m2 Dust composition: <5µm: 39% mineral dust 5µm to 10µm: 18% mineral dust 10µm to 20µm: 16% mineral dust 20µm to 40µm: 18% mineral dust 40µm to 80µm: 9% mineral dust Operation mode: switched off Requirement: 100% functional after test without additional heating of the device ### Cosmetic Test Vaseline and hand cream are to be applied as representatives for the cosmetic agents. Temperature: 45 °C Test duration: 8 h A glove is greased with the cream. Half of the test device is to be smeared using the greased glove. The test device is exposed to an oven temperature of 45 °C for 8 h. The exposed and unexposed surfaces of the product are inspected and compared. Requirement: no physical change of the housing material leading to a dangerous situation to the customer |  | Basic test Optional test acc. EN 60068-2-68 Basic test |
| Quantity to Test | 3 units per test |  |  |

| Test No. | Description |  | Standard |
| --- | --- | --- | --- |
| CH-9 | ### Cleaning resistance Test The following cleaning agents are to be applied: A: Ethanol 93% B: AJAX expert cleanser C: White spirit A soft cloth is wetted using one of the cleansers. 10 back and forth movements are to be made on the test device like normal cleaning (no hard rubbing). After drying the test device is subject to visual inspection. Requirement: The ink must not be rubbed off the label. ## Mechanical Conditions for the single charger packaging If necessary the single charger packaging (in case of carton) is tested according to the following test series: ### Packaging test series: - Storage test acc. packaging (for details see chapter VI Quality Requirements for Packaging) - Drop test Height of drop: 0.80 m Number of drops: 7 Drop direction: 1 corner; 3 adjacent edges; 3 adjacent faces) Requirements: packaging and charger shall not be damaged - Vibration test acc. packaging (for details see chapter VI Quality Requirements for Packaging) |  | Basic test |