- Performance Description (LB)

- Product/Version/Country:		Triple Band Mobile Phone Minos

- CX75 (EMEA, APAC)

- Document Title:	input.doc

Name	Department	Signature, Date

Feldhoff Markus	ICM MP PD PGG

Barner Ulrich	ICM MP PD PGG

Hombeck Jan	ICM MP PBM PG75

Olivan Lopez Javier	ICM MP PBM PG75

Teistler Thomas	ICM MP PBM BA PG75

Distribution:

| Name | Department | Location |
| --- | --- | --- |
| PD/REA-Team X75 Minos |  |  |

This version of the document is saved on the KLf network under:

/tmp/in/input.doc

Contents

1	List of References	5

1.1	Project specific references	5

1.2	General References and Standards	5

2	General Data	7

2.1	Design	7

2.2	Key Features X75 Minos	8

2.3	Comparison with Previous Products	10

3	Mechanics	11

3.1	Unit Description X75 Minos	11

3.2	Interfaces X75 Minos to accessories	11

3.3	Key-Data	14

3.4	Housing Colours	14

3.5	Mechanical Partlist:	15

3.6	Assembly drawing X75 Minos	17

3.7	PCB top-side	18

3.8	PCB bottom-side	18

4	Electronic	19

4.1	Radio part	19

4.1.1	Compliance	19

4.1.2	Triple band radio HIT (Renesas Solution)	20

4.1.3	EGPRS Modem	22

4.2	Digital Hardware and Electro Acoustic	23

4.2.1	Overview of Hardware Structure	23

4.2.2	Digital Baseband	24

4.2.3	Camera	24

4.2.4	Display Module	24

4.2.5	Keyboard	24

4.2.6	Illumination	24

4.2.7	SIM	25

4.2.8	IRDA	25

4.2.9	Bluetooth	25

4.2.10	Vibration Motor	25

4.2.11	Electro Acoustic	25

4.2.12	LED-Flash	25

4.2.13	Stepup for LED-Flash	25

4.2.14	Power Supply, Battery, Charging	25

4.2.15	Accessory Interface	27

4.2.16	Gimmick-Chip	27

4.3	Printed Circuit Board	28

4.4	EMC/ESD-Concept	31

4.4.1	EMC	31

4.4.2	ESD	31

4.4.3	Keypad	31

4.4.4	SIM + MMC	31

4.4.5	Camera	31

4.4.6	Display	31

4.4.7	Electromagnetic Compatibility	31

4.4.8	Antenna performance	31

4.4.9	SAR	32

4.4.10	General	32

5	Software	33

5.1	Features	33

5.2	Requirements to SW	33

5.3	User Interface	33

5.4	Availability	33

5.5	Customization	33

6	Accessories	34

7	Manufacturing Concept	35

7.1	Overview	35

7.2	Modules	35

7.3	Components Spectrum	35

7.3.1	SMD Component Number and Number of Types (as of 15.110.2004)	35

7.3.2	SMD Spectrum (as of 15.11.2003)	35

7.3.3	Manual Soldering	36

7.3.4	Delivered Form of the Components	36

7.4	Production Means and Stages	36

7.5	Basic Assembly Concept	36

7.6	New production technology	36

7.7	Quality Targets GJ03/04	37

See Chapter:   8.2  NPI Quality requirements	37

7.8	Expected Production Quantities/Production Capacity (as of 14.11.2004)	37

8	Test Rig Planning	38

8.1	General Requirements	38

8.2	NPI Quality requirements	38

9	Customer Care	39

9.1	Customer Care strategy	39

9.2	Service Objectives	39

9.3	Repair level definition	39

9.4	World-wide distribution of service level	40

9.5	Roll out plan for the service concept	40

9.6	Service parts	40

9.7	Technical Service Requirements	41

9.7.1	Test equipment	41

9.7.2	Technical Service Requirements	41

10	Quality	42

10.1	General Quality Requirements	42

10.2	Quality Requirements for Visual Appearance of Mobile Phones	44

10.3	Environmental Protection	45

10.4	Quality Plan	46

10.5	Department PSQA-plans	46

10.6	Product Safety and Technical Risk Assessment	46

10.7	Software Quality	47

10.8	Field Trials	48

10.9	Requirements for Product Audit	50

10.10	Requirements for Outgoing Inspection	50

10.11	Robust Development	50

10.12	Field Return Rate	50

10.13	Quality of Suppliers and Components	51

10.14	Quality in Production	51

10.15	Deviations from Agreed Quality Level	52

11	System Test	53

11.1	Introduction	53

11.2	Abbreviations and Definitions	53

11.3	Type Approval (TA)	53

11.4	Regulatory requirements - Overview	53

11.4.1	European Community	53

11.4.2	Outside Europe	54

11.5	Voluntary requirements - Overview	54

11.5.1	GCF	54

11.5.2	PTCRB	54

11.5.3	Additional voluntary requirements	54

11.6	Requirements Matrix	54

11.6.1	Regulatory requirements	54

11.6.2	“Voluntary” requirements	55

11.7	Regulatory requirements - detailed	56

11.7.1	Health aspect	56

11.7.2	Safety aspect	57

11.7.3	EMC aspect	57

11.7.4	Radio spectrum usage aspect	58

11.7.5	Special requirements	59

11.7.6	Inter-operability	60

11.8	Voluntary requirements	60

11.8.1	Global Certification Forum - Inter-operability	60

11.8.2	Bluetooth functionality	61

11.8.3	WAP functionality	61

11.8.4	IrDA functionality	62

11.8.5	USB functionality	62

11.8.6	Network Selection from SIM Card Preferred List	62

11.9	Marking and Labeling	62

11.9.1	China / APAC	64

11.9.2	Variant Overview	64

11.9.3	Additional System Test requirements	65

11.9.4	New type approval and certification requirements	65

12	Milestones	67

13	Miscellaneous	68

13.1	Technical Risks	68

13.1.1	EA	68

13.1.2	ST2	68

13.1.3	BB	68

13.1.4	NPI32	68

13.1.5	NPI31	69

13.1.6	MD	69

13.1.7	SW	69

13.2	Packaging/User Manual	69

13.3	Economic Product Plan	70

# List of References

## Project specific references

The appendices and enclosures (if present as files) are saved separately on the KLf network under /tmp/in/input.doc

M1 enclosed documents Minos

| No. | Document | Date | File Name |
| --- | --- | --- | --- |
|  | Product description X75 Minos (EMEA, APAC) (this document) |  | input.doc |
|  | X75 Minos Product Contract |  | Minos+M1+document+-+conf+-+041126.ppt |
|  | X75 Minos M0 Document |  | M0 Document Minos V.1.pdf |
|  | Software Product Description X75 (EMEA, APAC) |  | X75_SW_Feature_List_V2.0_040_02.xls |
|  | X75 UI-Specification |  | X75_MINOS.PDF |
|  | Accessories for the Mobile Phones X75 Minos (EMEA, APAC) |  | Compatibility_list_Series65.xls |
|  | Project schedule X75 Minos (EMEA, APAC) |  | Final Schedule_X75_Minos_M1.xls |
|  | X75 Minos (EMEA/APAC) RF-BB Interface |  | Penelope rf baseband interface spec.xls |

The X75 Minos M1 milestone will be declared according to “QMS Procedure No.: T008-MP Milestone Results in Product Development” (Issue 6 Published: 28.05.2004).

## General References and Standards

EUROPE

- Product Safety as per 1999 / 5 / EEC

- CENELC, SC211/B, WGMTE (2nd draft, Rome, Feb. 1996, 3rd draft, Zurich, April 1996)

- ETS 300 019-1-0 / ETS 300 019-2-7 (Environmental Requirements)

- ETR 051 (Human Factors - Basic Requirements) and ETR 166 (Evaluation Methods)

- IEC 68 ff. (Environmental Requirements)

- DIN 40050 (Climatic Conditions and Protection Classes)

- EN 1811 Reference Testing Procedure for Nickel releasing Products in Prolonged Direct Contact with Human Skin

- Laser Guidelines IEC 60825

- DIN EN 22248 Packaging; complete, filled transport packages; vertical impact test by dropping

- SN 47030-1 Molded plastics; thermoplastic molding materials; Siemens-item-numbers

- SN 27650 Testing of Paint Coatings and Similar Coatings; Test for Abrasive Strength

- EN ISO 2233 Packaging. Complete, filled transport packages. Conditioning for testing

- SN 36350-1 Environment Compatible Products; Part 1: Guide for product development

- Testing technical standards for mobile phones (Version 3.1+; 24.04.2003)

- Siemens General Quality Requirements (Rev. 3.1, 2002-01-23)

- Hardware Interface Specification R65 V0.1.pdf

- Clip_on_Flash_Specification0V5.pdf (plus modification: photodiode for light intensity measurement)

- R&TTE 99/5/EC

- EMC-directive 89/336/EC

- e-approval 95/54/EC

- customer requirements

US / CANADA

- FCC-rules

- IEEE-standards

- ANSI-standards

- RSS-regulations

- EIA / TIA 571 Environmental Aspects for Telecommunications Equipment

- EIA / TIA 136, 136A, 136-270B (TDMA)

- customer requirements

APAC

- YD 1032 (China)

- ICNIRP

- customer requirements

# General Data

## Design

The X75 Minos (EMEA, APAC) standard variant shall have one design with one colour:

DESIGN skizze tbd.

The X75 Minos (EMEA, APAC) design addresses the following requirements:

- CX75 (emotion class)

- Unisex ; Age Focus 20-45

- (low)/mid income, community centric

- Private & business usage

The target group prefer a timeless, classic styled design, a bit exclusive with an emotional and innovative touch – unique but not outstanding.

The design has to create an impression of convenient usability and reliability.
 
Main issues:

- Keypad: usability on focus

- Balanced proportions between keypad and display

- Finishing: high value

- Dimensions: small and ergonomic, as thin as possible

Minos is positioned at the high end of the emotion class. It is addressing a unisex, emotional and community centric target group, leading a busy and multi-facetted life with many friends, tasks & appointments at the same time.

Relaxing, enjoying life, and close networking with friends are their key needs. Therefore they are looking for features supporting spontaneous relaxation, enjoyment and socializing. They are also highly interested in intelligent technologies that enable smooth networking of business and private life and help them to balance their turbulent lifestyle in an engaging & warm way. Valuable & innovative design is expected.

Therefore Minos offers enjoyment on the move via state-of-the-art music capability and entertainment functions in combination with efficient life management functionalities for generating personal time slots for joyful leisure activities

## Key Features X75 Minos

General: 	Design:	Bar phone with high value material & color finishing,

e.g. galvanized plastic

- Key Features:	Integrated 1.3 Mega Pixel camera

integrated camera flash light

RS-MMC-slot

Bluetooth

EDGE/GPRS Class 10

Video decoding & capturing and streaming

- Display:	132x176, 262K color, TFT, 1.8 inch

- Applications & Accessories:	3D Java Games / Multiplayer Games via BT

- Audio	Music player, ring tones (MP3/AAC/AAC+), (AAC ++)

Battery:	Lition Battery Pack

Nominal Capacity:	780mAh@0.2CA

GSM Capacity:	750 mAh

- normal talk

- Power Input: 2.0 A (0.6 ms) / 0.25 A (0.4 ms)

- Cut-off Threshold 3.2 V

Stand-by Time:	≥ 250h ( detailed measurements need to be done)

Talk Time:	≥ 5 h (detailed measurements need to be done)

SIM Card:	Small (=”Plug In“) 1.8 V or 3 V-SIM card

Speech Coder:	Half Rate, Full Rate, Enhanced Full Rate and Adaptive Multi Rate speech coders are available as standard.

Temperature Range:	-30°C to + 55°C (Normal operation)

-1000C to + 850C (Storage capability)

Display:

| Resolution : | 132x176 |
| --- | --- |
| Dimensions : | panel dimensions: 32.3 mm x 47.1 mm active area : 27.7 mm x 37.0 mm |
| Pixel Size : | 0.21 mm x 0.21 mm |
| Technology : | TFT |
| Operating Temperature: | -20°C....+55°C |
| Backlight : | - Lightguide + 4 white side-shooter LEDs / mounted on the flexfoil inside the module |
| Resolution : | 132x176 |
| Dimensions : | panel dimensions: 32.3 mm x 47.1 mm active area : 27.7 mm x 37.0 mm |

Keypad:		hardcaps in IMF-technology

- 12-key-block (0-9, #, *)

- two function keys (SEND, END)

- ON/OFF key combined with the END key; the symbol  (I inside O) is
	used as a symbol for ON/OFF.

- 5 way-navi-key

- 2 soft-keys for different SW-enabled functions

- 1 dedicated hardkey (provider key)

- white as illumination colour

- tactile finder on key “5“

- 6 white LEDs for keypad

Camera:		Mega-Pixel Camera

Multimedia/Audio:		Three-in-one-earpiece for handset, handsfree and ringing tones

- uni-directional microphone

- Loud signal emitter (soundringer) (>100dB(A) SPL @5cm, 'Hongkong-Spec.') only for rectangular sound signals (NOT POSSIBLE for all Soundringer melodies)

- Polyphonic ringer tones 64 voices

- Hands free mode

- different selectable volume levels for handsfree, handset  and ringer mode (for the amount see SW product description)

- Speaker Dependant (SD) Voice Name Dialling (available within 2nd SW release);
Speaker Dependant (SD) Voice Control

- Audio decoding: MP3 (ringtones and music player), Real Audio 8 (available within 2nd SW release), MPEG4 AAC/AAC+/AAC++ (ring tones and music player), AMR-NB,encoding: AMR-NB,

- Audio streaming: AMR-NB, AAC, AAC++, Real Audio 8 (available within 2nd SW release,);

- 64 chords polyphonic ringtones

- Video resolution:QCIF, subQCIF;decoding (15fps): H263, MPEG4, Real Video 8 (available within 2nd SW release) ;encoding (15 fps): H263; streaming: Real Video 8 (available within 2nd SW release), 3GPP (H.263, MPEG4)

## Comparison with Previous Products

| Feature | CX65/CX66 Ulysses | X75 Minos | Delta |
| --- | --- | --- | --- |
| Supported Systems | Triple Band EGSM 900/GSM1800/ GSM1900 (EMEA, APAC) GSM 850/GSM1800/ GSM1900 (NAFTA) | Triple Band EGSM 900/GSM1800/ GSM1900 (EMEA, APAC) | No GSM 850 |
| Stand-by Time | ≥ 250h | ≥ 250h |  |
| Talk Time | ≥ 5 h | ≥ 5 h |  |
| Battery Technology Battery Capacity | Li-Ion Battery Pack Nominal Cap.: 780 mAh | Li-Ion Battery Pack Nominal Cap.: 780 mAh |  |
| Weight | Approx. 90 g | approx. 89 g |  |
| Volume | Approx. 78 cm3 | approx. 84 cm3 | Higher volume |
| Length | 108 mm | 102.9 mm | 6.1mm shorter |
| Width | 44…47 mm | 47.0 mm (Sidekeys) |  |
| Thickness | 17…18 mm | 18.7 mm (Display) | 0.7 mm thicker |
| SIM | Plug-In 1.8V/3V | Plug-In 1.8V/3V |  |
| Antenna | Integrated | Integrated |  |
| Antenna Performance in comparison | -0,8 dB @ 900 MHz -0,5 dB @ 1800 MHz | ~28 dBm @ 900 MHz ~25 dBm @ 1800 MHz ~24 dBm @ 1900 MHz (2,7-Band-Antenna) |  |
| Half Rate | Yes | Yes |  |
| Enhanced Full Rate | Yes | Yes |  |
| AMR | Yes | Yes |  |
| Fax/Data | Yes | Yes |  |
| GPRS | Yes (Class 10) | EDGE/Class 10 | Higher data transfer rate |
| Keypad Illumination | Yes (white) | White LED |  |
| Display / Display Illumination | TFT/TFD 132 x 176 Pixel 2,0” 65k colour | TFT/TFD 132 x 176 Pixel 1,8” 262k colour | Higher Resolution Colour |
| User Memory | Approx. 11 MB | 20 MB internal |  |
| Exchangeable Memory | no | Yes, RS-MMC reader available | Possibility for higher Memory |
| Camera | VGA | Mega pixel camera (integrated) | Higher Resolution New feature in CX-class |
| BT | No | Yes |  |
| Flash light | No | Yes |  |
| Video | decoding (15fps): H263 encoding (15 fps): H263 | decoding (15fps): H263, MPEG4, Real Video 8 encoding (15 fps): H263 streaming: Real Video 8, 3GPP (H.263, MPEG4) |  |
| Audio / Music | 32 chords polyphonic ringtones | decoding: MP3,Real Audio 8, MPEG4, AAC/AAC+ / AAC++ , AMR-NB encoding: AMR-NB streaming: AMR-NB, AAC, AAC++, Real Audio 8 64 chords polyphonic |  |
| Voice/ speech function | No | Speaker Dependant (SD) Voice Name Dialling Speaker Dependant (SD) Voice Control |  |
| Sidekey | No | Yes |  |

# Mechanics

## Unit Description X75 Minos

The X75 Minos is designed as a Bar phone with two PCBs. The whole unit consists of a NoID-Block and parts for the customization. The NoID-Block itself is made of the soldered PCBs and the preassembled upper and lower cases. It is full functional and testable. The parts for the customization are the keypad, the front cover, the rear cover and the battery cover, but only the battery cover will be exchangeable for the end user.

Except of the upper case which is a 2 shot molded plastic part all other cases and covers are 1 shot molded plastic parts. The upper case is galvanized (1 colour) and the rear and battery covers are lacquered (1 colour). The decoration of the front cover is done by IMD and the decoration of the keypad by IMF. The lower case is without any additional surface decoration (1 colour).

## Interfaces X75 Minos to accessories

The phone has got a full compatible interface to accessories. All shown interfaces are for car-cradle. Interfaces for Belt-Clip will not be necessary.

Accessory-Interface

Accessory-Interfaces

Drawings of accessory-interfaces

Drawing of accessory-interfaces

## Key-Data

| Volume | approx. 84 cm3 |
| --- | --- |
| Weight | approx. 89 g |
| Length | 102.9 mm |
| Width | 47.0 mm (Sidekeys) |
| Thickness | 18.7 mm (Display) |

## Housing Colours

| Part | Material | Colour | Surface Finish |
| --- | --- | --- | --- |
| Front cover (1 shot molded part) | PC/ABS | Nautic Blue (Siemens St) Moonlight silver (VF) Black Stone (TMO) | IMD: Scratch resistance and decoration on top side |
| Upper Case (2 shot molded part) | PC/ABS PC | silver | galvanized |
| Display lens | PMMA | crystal clear / black | IMD: Scratch resistance on top side Tampon-/ silk screen printing: Decoration on bottom side |
| Lower Case (1 shot molded part) | PC/ABS | black |  |
| Rear Cover (1 shot molded part) | PC/ABS | silver | lacquered |
| Battery-Cover (1 shot molded part) | PC/ABS | silver | lacquered |
| Keypad (molded hardcaps glued on silicon carrier) | PC/ABS Silicone | Nautic Blue (Siemens St) Moonlight silver (VF) Black Stone (TMO) | IMF: Decoration on top side |

## Mechanical Partlist:

| Assembly | Part | Part-Nr. | # | Supplier |
| --- | --- | --- | --- | --- |
| Front cover ass. |  | C39158-A131-A50 | 1 | Nolato |
|  | Front cover decorated X75M | C39158-A131-B50 | 1 | Nolato |
|  | Front cover adhesive tape X75M | C39158-A131-C50 | 1 | Nolato |
| Keypad compl. | “LATIN” | C39158-A131-B600 | 1 | TopBound/Silitech |
| Upper case ass. |  | C39158-A131-A1 | 1 | Nolato |
|  | DPL protection foil X75M | C39158-A131-C340 | 1 | Nolato |
|  | Display lens X75M | C39158-A131-B300 | 1 | Nolato |
|  | DPL adhesive tape X75M | C39158-A131-C41 | 1 | Nolato |
|  | Upper case decorated X75M | C39158-A131-B1 | 1 | Nolato |
|  | Mesh X75M | C39158-A131-C93 | 1 | Nolato |
|  | LCD-Cushion X75M | C39158-A131-C32 | 1 | Nolato |
|  | Speaker Cushion X75M | C39158-A131-C345 | 1 | Nolato |
|  | MMI sheetmetal X75M | C39158-A131-C65 | 1 | Nolato |
| MMI ass. |  | S30880-Q2500-Q98 | 1 | Siemens LPZ |
|  | MMI |  | 1 |  |
|  | MMI Metaldomefoil X75M | C39158-A131-C61 | 1 | TopBound/Panasonic |
| Lower case ass. |  | C39158-A131-A200 | 1 | Nolato |
|  | Lower case X75M | C39158-A131-B200 | 1 | Nolato |
|  | IrDa window X75M | C39158-A131-C91 | 1 | Nolato |
|  | Sidekey Button volume X75M | C39158-A131-B720 | 1 | Nolato |
|  | Sidekey Button PoC X75M | C39158-A131-B721 | 1 | Nolato |
|  | Sidekey switch (2pcs) | C39315-Z77-C223 | 2 | ITT |
|  | Antenna X75M | C39158-A131-C80 | 1 | Nolato |
|  | Bluetooth Antenna X75M | C39158-A131-C81 | 1 | Nolato |
|  | Speaker X75M | C39212-Z3-C54 | 1 | Hosiden |
|  | Micro 4mm | C39254-Z6-C104 | 1 | Hosiden |
|  | Vibra | C39453-Z5-C205 | 1 | Shicoh/Melco/Matsushita |
|  | Flash light reflector | C39158-A131-C260 | 1 | Nolato |
| Rear cover ass. |  | C39158-A131-A550 | 1 | Nolato |
|  | Rear cover dec. X75M | C39158-A131-B550 | 1 | Nolato |
|  | Flash light lens | C39158-A131-B900 | 1 | Nolato |
| Battery cover ass |  | C39158-A131-A500 | 1 | Nolato |
|  | Battery cover dec. X75M | C39158-A131-B500 | 1 | Nolato |
|  | Mirror | C39158-A131-B910 | 1 | Nolato |
| others | Screws NoID-block/Front cover | C39158-A109-B900 | 6 | Ejot |
|  | Main-PCB |  | 1 |  |
|  | Shielding RF X75M | C39158-A131-A900-1 | 1 | Härter |
|  | Shielding BB top X75M | C39158-A131-A905-1 | 1 | Härter |
|  | Shielding BB bottom X75M | C39158-A131-A910-1 | 1 | Härter |
|  | Shielding BT X75M | C39158-A131-A915-1 | 1 | Härter |
|  | LCD-module | V24851-Z1508-A112 | 1 | Sharp/Philips/Alps |
|  | LCD frame X75M | C39158-A131-C90-1 | 1 | Stansomatic |
|  | LCD-connector 20pin | C39334-Z97-C380 | 1 | FCI |
|  | Camera-module 1.3 Mega-Pixel | C39453-Z5-C291 | 1 | Premier/Semco/Toshiba |
|  | Camera socket | C39334-Z97-C379 | 1 | SMK |
|  | MMC-reader ejector-tray | C39334-Z97-C348 | 1 | Alps |
|  | MMC-tray | C39158-A131-B750 | 1 | Alps |
|  | Batterypack R65/750mAh | V30145-K1310-X277 | 1 | Varta/Samsung/NEC |
|  | Battery connector | C39334-Z97-C213 | 1 | Lumberg/FCI |
|  | BtoB-connector 14pin | C39334-Z97-C363 | 1 | FCI |
|  | ESD connector | C39334-Z97-C361 | 3 | Gore |
|  | RF-connector (6mm) | C39334-Z93-C297 | 1 | Tyco |
|  | SIM-reader K1 | C39334-Z97-C337 | 1 | Lumberg |
|  | I/O-connector | C39334-Z93-C303 | 1 | Lumberg |
|  | IRDA module | V39197-F5008-F492 | 1 | Citizen/Agilent |

## Assembly drawing X75 Minos

Front cover ass.

Upper case ass.

LCD-Module

PCB

Camera-Module

Lower case ass.

Rear cover ass.

Battery cover ass.

Keypad

MMI

MMC-Tray

Screws
- 4 for NoID-Block
- 2 for Front cover

Battery pack

## PCB top-side

LCD-Frame

LCD-Connector

Shielding BB top

BtoB-Connector

RS-MMC-Reader

ESD-Connector

## PCB bottom-side

Camera-Connector

Flash light LEDShielding RF

SIM-Reader

Shielding BT

Irda-Module

RF-Connector

Battery-Connector

Shielding BB bottom

# Electronic

Following chapters give an overview about the hardware of the S65 (EMEA, APAC) Penelope.

## Radio part

The radio part realizes the conversion of the GMSK/8PSK RF-signals from the antenna to the baseband and vice versa. In the receiving direction, the signals are split in the I- and Q-component and led to the A/D-converter of the logic part. In the transmission direction, the GMSK/8PSK signals are generated digitally by the baseband section. After D/A conversion the analogue signals are fed into an IQ modulator and upconverted to radio frequencies by the polar loop system. After that the signals are amplified in the power amplifier.

Transmitter and Receiver are never active at the same time. Therefore full duplex operation is not possible. However the monitoring band (monitoring timeslot) in the TDMA-frame can be chosen independently of the receiving respectively the transmitting band (RX- and TX timeslot of the band).

The RF part is dimensioned for triple band operation in the frequency ranges EGSM900/DCS1800/ PCS1900 respectively GSM850/DCS1800/PCS1900 supporting EGPRS functionality up to multiclass 10.

As depicted in the table, there is only one solution implemented.

RENESAS (single solution)

Renesas Bright 5PL chip set (HD155153NP) - PLL for local oscillator LO1 and LO2 and TxVCO - Integrated local oscillators LO1, LO2 (without loop filter) - Integrated TxVCO (without loop filter and core inductors for GSM) - Direct conversion receiver including LNA, DC-mixer, channel filtering and PGC-amplifier - Active part of 26 MHz reference oscillator - Integrated Polar Loop, phase and amplitude control of transmitted output power

Renesas LTCC transmit PA PF09026B (incl. integrated power control circuitry for GMSK mode)

Hitachi Frontend-Module HWXQ411 including RX-/TX-switch and EGSM900 / DCS1800 / PCS 1900 receiver SAW-filters or HWXQ413 including RX-/TX-switch and GSM850 / DCS1800 / PCS 1900 receiver SAW-filters

Crystal and passive circuitry of the 26MHz VCXO reference oscillator

### Compliance

The RF part is compliant to GSM 11.10-1 release 1999 version 8.30. The transmitter output power is compliant to following power classes:

For GMSK Mode:
GSM850 / EGSM900: 	power class 4

GSM1800 / GSM1900: 	power class 1

For 8PSK Mode:
GSM850 / EGSM900: 	power class E2
GSM1800 / GSM1900: 	power class E2

### Triple band radio HIT (Renesas Solution)

#### Renesas Bright 5PL chipset (HD155153NP)

##### First local oscillator  (~3.6GHz)

The first local oscillator (LO1) consists of a PLL and VCO inside Bright 5PL and an external loop filter.

##### RF-PLL

The minimum frequency step is 400 kHz in DCS1800/PCS1900 bands and 800kHz in GSM850/EGSM900 bands due to the internal divider by two for DCS1800/PCS1900 and divider by four for GSM850/EGSM900. The PLL is controlled by the internal state machine.

##### RFVCO (LO1)

The full oscillation range is divided into 16 sub bands and covers 3476 to 3980MHz. The choice of the appropriate band and the complete operation is controlled internally by the Bright state machine.

##### Second local oscillator (~640MHz)

The second local oscillator (LO2) consists of a PLL and a VCO which are integrated in Bright 5PL and a second order loopfilter which is realized external.  Due to the direct conversion receiver architecture, the LO2 is only required for transmit operation in order to generate the transmit IF. To avoid inband spurious in the transmit signal the TX IF frequency is not fixed for the whole band. The LO2 covers a frequency range from 640 to 656MHz.

Before the LO2 signal enters the modulator it is divided by 8. So the resulting TX IF frequencies are 80/82 MHz. The complete LO2 operation is controlled by the Bright internal state machine.

##### Receiver

The Bright 5PL incorporates three RF LNAs for GSM850/EGSM900, DCS1800 and PCS1900 operation followed by direct conversion mixers which are IQ demodulators. The LNA/mixer can be switched in normal-, low- and lower-gain mode. For the “normal gain“ state the mixers are optimised in terms of conversion gain and noise figure, in the “low gain“ state the mixers are optimised for large-signal behaviour for operation at a high input power levels. The "lower gain mode" reduces the RF-level by activating a differential impedance in front of the LNA to improve the large signal performance.

Furthermore the IC includes a programmable gain baseband amplifier PGA (90dB control range, 2dB steps) with automatic DC offset calibration. The channel filtering is realized inside the chip with a four stage baseband filter for both IQ chains. Only two capacitors which are part of the first passive RC-filters are external. The second, third and fourth filters are active filters and are fully integrated. The distributed channel filter is necessary to suppress adjacent channel and inband-blocking interferer to avoid any compression in each amplifier stage.

The downconverted IQ signals are fed into the A/D converters inside the SGOLD2. By a special algorythm the level of the IQ signals is kept constant on a defined level by varying the PGA gain and selecting the appropriate LNA gains.

##### Transmitter

Polar Loop

The generation of the modulated RF signal in Bright 5PL is based on the principle of the polar loop architecture. The IQ signals generated by the SGOLD2 baseband are modulated to the transmit IF of 80MHz by an IQ modulator. The required carrier frequency is provided by the LO2.

As in an conventional upconversion loop the IF signal is upconverted to the transmit frequency. Therefore a downconverter and a phase detector are used in order to compare the IF signal to the downconverted transmit signal. This loop is called PM loop and is used for GMSK operation.

In 8PSK mode the 80MHz IF signal is split into PM and AM components. A second loop is then used to control the AM components. Therfore an AM detector is implemented which compares the 80MHz IF signal and the downconverted transmit signal. The AM loop is also used for power ramping. Inside the AM loop there are two analogue gain controlled amplifiers. These are used for setting the output power level and to keep the loop bandwidth constant.

The seperated AM and PM components of an 8PSK signal are finally fit together inside the PA. The PA is driven by the TXVCO signals containing the PM components. The AM component is added by an amplitude control input pin, which controls the supply voltage of the RF transistors and therefore the PA output power level.

TxVCO

The TxVCO is integrated inside Bright 5PL. It consists of 16 VCO bands for GSM850/EGSM900 and 16 bands for DCS1800/PCS1900. Two independant output buffers, one for each band, deliver the output signal to the PA module input. The choice of the appropriate band is done automatically and controlled by the internal state machine.

#### Renesas LTCC transmitter power amplifier PF09026B

##### PA Module

The power amplifier is a LTCC (Aluminia-oxyd-ceramic) PA module from Renesas, matched to 50  at all signal ports. It contains two separate 3-stage amplifier chains for GSM850/EGSM900 and DCS1800/PCS1900 operation. It is possible to control the output power level of both bands via one VRAMP port. The appropriate amplifier chain is activated by a logic signal, which is provided by the SGOLD2 (RF_BAND_SW). The module is switched on by a control signal generated by the B5PL internal state machine. The RF transistor bias voltage is generated inside the PA module.
The PA module consist also of two additional directional coupler to provide the needed RF feedback signal for the AM control in 8PSK Mode.

##### Power control

In GMSK operation the output power is directly controlled by a control voltage coming from SGOLD2. This voltage is fed through B5PL and connected to the PA module (VRAMP). The 3GPP specification can be fulfilled without power sensing or any type of feedback.

In 8PSK operation the AM loop controls the output power level. Therefore the same SGOLD2 signal is used but connected to the variable gain amplifiers inside B5PL. The AM control voltage finally controls the PA module (VRAMP). All switching between GMSK and 8PSK modes is controlled by the B5PL internal state machine.

#### Hitachi Frontend-Module (FEM)

The FEM includes the RX/TX- and bandswitch function based on a combined PIN diode and diplexer-circuit. In the transmit paths a harmonic filtering for GSM850/EGSM900 and DCS1800/PCS1900 is realized to avoid additional discrete filters. The isolation in TX OFF mode is used to achieve the isolation which is necessary before the active part of the burst. Two lines from the baseband control the band-selection of the TX switches. The three receiver chains include SAW filter for GSM850/EGSM900, DCS1800 and PCS1900 to protect the receivers from strong blocking signals according to the 3GPP specification. To provide symmetrical signals with good balance to the receiver, a discrete balun is used for each band inside the module.

##### Variants

X75 uses either the bands EGSM900/DCS1800/PCS1900 and GSM850/DCS1800/PCS1900 as two variants.

#### Discrete 26MHz VCXO reference oscillator

The 26 MHz signal is created by a discrete VCXO. A Colpitts oscillator is used with a post-connected buffer stage. The active part (cascode circuit of the VCXO and the buffer) is realised within the Bright. For temperature measurements of the VCXO a temperature sensor is used. The frequency of the reference oscillator can be fine tuned by the baseband via a filtered PNM modulated AFC signal. Two active buffer stages are included in Bright 5PL to provide clock signals for the baseband IC and the PMU. An additional external buffer is used to deliver a 26MHz clock signal to the bluetooth circuitry.

#### Functional block diagram

The functional block diagram shows the Renesas RF solution

### EGPRS Modem

For details see link to platform file: EGPRS MODEM TECHNICAL CONCEPT:

M1 enclosed documents Minos\EGPRS Modem Technical Concept.rtf

## Digital Hardware and Electro Acoustic

### Overview of Hardware Structure

The following figure gives an overview about all digital and electro acoustic parts, where a detailed description follows in the chapters below.

- New Parts

| Dig. Hardware / Baseband: |  | 1st supplier | 2nd supplier | 3rd supplier |
| --- | --- | --- | --- | --- |
| Base band solution | S-GOLD2 PMB8876 | Infineon | --- | --- |
| PMU | as R65 | Dialog Mozart | STM Twigo4 | --- |
| Memory | 2 x 256 Flash + 128 SDRAM | Intel PF38F4460LVYGB0 | AMD | Samsung (optional) |
| Bluetooth | BRF6150 | TI | - | - |
| Power supply | As R65 | Astec | Friwo | Delta |
| Accessory MUX | EMIF05MUX | Philips | STM | - |
| SIM-Filter | IP4044CX8_LF | Philips | STM |  |
| Gimmick-Chip | Imageon W2182 | ATI | - | - |
| flash LED | 4 LEDs serial | Everlight | Liteon | Toshiba |
| IrDA | Low profile SIR | Citizen | Agilent |  |
| keypad LED | 0.4mm high; white | Tbd. Potential suppliers are: | Everlight, Liteon, Samsung | Testing ongoing until cw50 |
| Battery pack | Capacity 750 mAh, package size: 50x34x4,6mm³ (taken from Penelope) | NEC | Varta | Samsung |
| LCD-module | 132 x 176 pixels; 256k colours TFT 1,8" (Andromeda) | Sharp | Philips | Alps |
| Camera-module | 1.3 Mpix | Premier | SEMCO | Toshiba |
| Speaker | rework of Fugu speaker | Hosiden | Bujeon |  |
| Microphone | 4mm omnidirectional (taken from Penelope) | Bujeon | Hosiden | BSE |
| Vibra | taken from Marlin S55 | Shicoh | Melco | Matsushita Electric Ind. |
| Converter Gimmickchip | IC ANA CHP/REG LM2770 (1.5V/1.2V) LLP10 | NSC | Fairchild |  |
| Converter Powersupply | IC ANALOG DC/DC BOOST CONVERTER SOT23-5 | NSC / Fairchild (under evaluation) | NSC / Fairchild (under evaluation) | (Possibly only 2nd solution available) |

### Digital Baseband

#### Baseband Processor S-GOLDlite (PMB8875)

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_SGOLD2.doc

#### Memory

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_External_Memory_V0.4_draft.doc

### Camera

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_1.3MPix_Camera_draft20041001.doc

### Display Module

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_Andromeda_V03.doc

### Keyboard

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_Keyboard_V01.doc

### Illumination

#### Keyboard

The illumination of the keypad will be done via high-brightness LEDs (white color top-shooter, driven by 5mA / LED).

#### Display

The 4 serial connected LEDs for the Display are supplied by one constant current source, to ensure the same brightness and color of the white backlight.

### SIM

SIM Cards with supply voltages of 1,8V and 3V will be supported.

### IRDA

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_IRDA.doc

### Bluetooth

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_Bluetooth_040910.doc

### Vibration Motor

The vibration motor is mounted in the lower case over the Lumberg I/O connector. The electrical connection to the PCB is realised with pressure contacts. The S55 Vibration motor will be reused.

### Electro Acoustic

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_Audio_01.doc

### LED-Flash

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_LED_FLASH.doc

### Stepup for LED-Flash

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_Stepup_for_Flash_LED.doc

### Power Supply, Battery, Charging

#### PMU Mozart / Twigo4+

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_PMU.doc

#### Battery

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_Battery_02.doc

#### Charging Concept

##### General

The battery is charged in the phone. The hardware and software is designed for LiIon with 4.2V technology.

Charging is started as soon as the phone is connected to an external charger. If the phone is not switched on, then charging shall take place in the background (the customer can see this via the “Charge” symbol in the display). During normal use the phone is being charged (restrictions: see below).

Charging is enabled via a PMOS switch in the phone. This PMOS switch closes the circuit for the external charger to the battery. The processor takes over the control of this switch depending on the charge level of the battery, whereby a disable function in the PMU hardware can override/interrupt the charging in the case of over voltage of the battery.

For controlling the charging process it is necessary to measure the ambient (phone) temperature and the battery voltage. The temperature sensor will be an NTC resistor with a nominal resistance of 22k at 25°C. The determination of the temperature is achieved via a voltage measurement on a voltage divider in which one component is the NTC. Charging is ongoing as long the temperature is within the range +5°C to 45°C. The maximal charge time will be 2 hours (Imax=750mA).

##### Measurement of Battery voltage, Battery Type and Ambient Temperature

The voltage equivalent of the temperature and battery code on the voltage separator will be calculated as the difference against a reference voltage of the S-GOLD2. Inside the S-GOLD2 are some analog to digital converters. These are used to measure the battery voltage, battery code resistor and the ambient temperature.

##### Timing of the Battery Voltage Measurement

Unless the battery is being charged, the measurement shall be made in the TX time slot. During charging it will be done after the TX time slot.

##### Recognition of the Battery Type

The different batteries will be encoded by different resistors within the battery pack itself.

##### Charging Characteristic of Lithium-Ion Cells

LiIon batteries are charged with a U/I characteristic, i.e. the charging current is regulated in relation to the battery voltage until a minimal charging current has been achieved. The maximum charging current is given by the connected charger. The battery voltage may not exceed 4.2V 50mV average. During the charging pulse current the voltage may reach 4.3V. The temperature range in which charging of the phone may be performed is in the ranges from 0...50°C. Outside this range no charging takes place, the battery only supplies current.

##### Trickle Charging

The PMU is able to charge the battery at voltages below 3.2V without any support from the charge SW. The current will by measured indirectly via the voltage drop over a shunt resistor and linearly regulated inside the PMU by means of the external FET. The current level during trickle charge for voltages <2.8V is in a range of 20-50mA and in a range of 50-100mA for voltages up to 3.2V. To limit the power dissipation of the dual charge FET the trickle charging is stopped in case the output voltage of the charger exceeds 10 Volt. The maximum trickle time is limited to 1 hour. As soon as the battery voltage reaches 3.2 V the PMU will switch on the phone automatically and normal charging will be initiated by software.

##### Normal Charging (Fast charge)

For battery voltages above 3.2 Volt and normal ambient temperature between 0 and 50°C the battery can be charged with a charge current up to 1C. This charging mode is SW controlled and starts if an accessory (charger) is detected with a supply voltage above 6.4 Volt by the PMU ASIC. The level of charge current is only limited by the charger.

##### USB Charging

The PMU can support USB charging when USB charging is integrated in the charging software. If charge voltage is in the range 4.4V to 5.25 V USB charging is ongoing. A switched off mobile phone will not wake up automatically if it is connected to an USB-Interface. During USB charging only limited charging is possible. Charge current is limited to 75, 150, 300 or 400 mA.

### Accessory Interface

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_IO.doc

### Gimmick-Chip

M1 enclosed documents Minos\M1_Product_Description_X75_Minos_Gimmick_Chip_ATI_W2182.doc

## Printed Circuit Board

The X75 Minos has following layer build up, schematic and assembly list of the Printed Circuit Boards (PCB), namely the main board and the MMI board. The Main PCB will be produced with CuOSP surface and will have 8 layers for the main board, but as a design-to-cost measure a 6 layer PCB will be targeted. And 4 layers for the MMI board.

X75 Minos main board layer build up:

X75 Minos main board assembly plan (top and bottom side, Hitachi RF part):

bottom side: 					top side:

X75 Minos MMI board:

The MMI board will be a 4 layer PCB , but as a design-to-cost measure a 2 layer PCB will be targeted in a 2nd step.

## EMC/ESD-Concept

### EMC

To reduce EMC components on the PCB and to increase manufacturing capacity all EMC components which may not be needed are left empty at the beginning. Via correspondingly intensive measurements in the EMC chamber we will attempt to calculate the minimally required number of EMC capacitors. We are assuming that the necessary chambers, equipment, software, time and staff will be available for this in good time and in sufficient quantities.

### ESD

The galvanization of the upper case must be connected on three points to the ground (one north and two south part of the phone). The contact resistance must be less than 1 Ohm for each contact.

Hitachi- and Murata Front End Modules:

Both front end modules fail the Siemens internal requirement (8 kV) during ESD contact discharge with external antenna (GQR test). The front end modules are stable up to 5.5 kV (limit for approval is 4 kV).

### Keypad

The keypad will be covered by a metal frame, which will be connected at least two points to the ground to get sufficient ESD protection. Because of “No-Id” concept, the height of the LED’s must be deeper than the height of the metal frame (Ulysses concept).

### SIM + MMC

SIM-Reader:

The SIM contacts will possibly be open, so that –as in former projects– contact discharging with 8kV (internal limit) could lead to the destruction of the SGOLD2. A limit of 5 kV seems to be reachable and fulfils the R&TTE-requirements. A possible solution to achieve higher ESD-levels is investigated. This solution will need less area on the PCB, but causes higher costs. Because of a low distance to the antenna the SIM reader must be shielded at least like S65. If the shielding is not sufficient SIM card error can occur.

MMC-Reader:

The MMC-reader must be completely shielded to avoid EMC problems.

### Camera

The camera must be shielded and connected to the ground. If camera is active, while a call is established, higher self interferer is possible, maybe we cannot fulfill the Siemens requirements. In camera preview mode there is also the risk to reduce the RX performance caused by the camera.

### Display

With the color display higher self interferer are expected. Furthermore, RF power radiated by the antenna or RF-components may have an impact on the display contrast and can cause display flickering.

### Electromagnetic Compatibility

EMC tests will be carried out according to R&TTE-requirements.

### Antenna performance

GSM-Antenna:

The antenna concept is based on same punch bending part as Penelope (2.7 band PIFA antenna) and will be optimized for European market (only one antenna version; no special tuning required for TMO).

It will be mounted between the plastic of the lower case and the plastic of the rear-cover. Therefore we expect a performance loss in TX and RX directions in comparison to Ulysses.

Because of low distance between speaker and antenna performance loss (Rx/Tx) is expected. During pre measurements it is detected that the speaker absorbs RF power in GSM 900 band and causes a significant deterioration of the antenna performance. To overcome this problem two impeders have to be implemented inside the speaker module (between spring contact and speaker coil).

Bluetooth-Antenna:

For the Bluetooth-Antenna at the moment two concepts are under investigation (final solution will be fixed after B1 run):

- partially printed and partially punch bending part (hybrid PIFA antenna). The punch bending part with only one contact will be mounted in lower case.

- fully printed PIFA structure. The complete antenna is printed on the main PCB. This solution is the most cost effective solution.

Because of low distance to the metal parts (battery, display frame, galvanized upper case etc.) the antenna will not have an omnidirectional radiation characteristic (for both solutions). The tolerances of the battery cell, dielectric constant of the PCB and the housing can cause range variations. The minimum range requirement from BT-spec is 10 m.

### SAR

SAR value of 0.8W/kg (1g) is SIEMENS target. Measurements with Maguro and Penelope handsets has shown that the Siemens target can be reached if the ground connection of the galvanization of the upper case is realized on the symmetry plane of the top and bottom area of the phone. The actual mechanical concept has planned an asymmetric placement of the north contact. Therefore further investigations have to be done with clearly defined mockups. A reliable statement can be made after measurements with B1 samples. Experiences from Penelope has shown that a spring contact has no stable contact resistance. Good results were obtained with Gore gaskets. Therefore we recommend to use for all three contacts the Gore gaskets.

### General

It must be noted in the manual that the device has to be switched off, before the battery cover or the upper case/keypad is disassembled.

# Software

## Features

The detailed feature list for the X75 generation V2.0_040_02 or the finally agreed version at M1 is the binding description for all the software that will be provided within the Minos project

M1 enclosed documents Minos\X75_SW_Feature_List_V2.0_040_02.xls

## Requirements to SW

M1 enclosed documents Minos\Requirements_to_SW.xls

## User Interface

The user interface is described in the UI specification document which is not finalized.

The pre version of the HMI Spec is

Hyperlink to UI-specification

## Availability

M1 enclosed documents Minos\SW_integration_75_platform.xls

## Customization

The Software will be launched initially with the variantes refered in [11]. Based on a variant specification from the customer (Operator), R&D will create the required initialisation files for the different operator variants. This is necessary because of the complexity of these variants. They have to be tested by the system test group and CCQ (field test). Standard customer variants will support only a limited functionality concerning the customization. This does not include the customer specific initialization of the Flex Menu Tree, Skins, Themes and certificates (i.e. not included in customization guide). A customer may request a more complex customization variant which cannot be handled based on the standard customization guide. Those variants can be supported but the time schedule and the availability of resources has to be agreed with R&D and all parties involved for each variant individually

# Accessories

For the X75 Minos accessory devices a separate Performance Description (see chapter 1.1 Project specific references) will be available. The following figure gives a preliminary about the planned accessories.

# Manufacturing Concept

## Overview

The manufacture of the X75 Minos main PCB will be done on the standard SMD production lines for mobile phones with 0201 components and lead free process (new production technology).

## Modules

The differences of components from top to bottom side will be compensated by the use of combinated 3 times top/bottom PCB panel (6 times panel).

## Components Spectrum

### SMD Component Number and Number of Types (as of 15.110.2004)

Main PCB:

### SMD Spectrum (as of 15.11.2003)

The smallest passive design form is Chip 0201, the smallest grid used is 0.5mm.

- SGOLD 2, Pitch 0.5mm

- Flash 256Mbit (Intel)

- SDRAM

- Camera-ASIC, Pitch 0,5mm

- Mozart-ASIC, Pitch 0,5mm

- EMI F07 USB-Filter, Pitch 0,5mm

- IP4044 Filter, Pitch 0,5mm

- LP1986 Regler, Pitch 0,5mm

- LP3985 Regler, Pitch 0,5mm

- Gimmick Chip

Components for special SMD Processing (F-Machines)

- I/O Connector

- SIM-Reader

- RS MMC-Reader

- Battery Connector

- Display connector 20 pol.

- B2B connector MMI / PCB 14pol.

- HF plug in

- Bluetooth

- Prem Can Shielding Frames, and Fish Can

Camera fixing frame

- Display fixing frame

### Manual Soldering

No manual soldering.

### Delivered Form of the Components

All SMD Components have to be delivered in 13” tape and reel. Any deviations from this packaging can be decided on or cleared only by SCM NPI or GO2.

All mechanical parts (Displays / Kamera / ass. Plastic parts/ pastic parts) have to be delivered in Tape & Reel form or in trays acc. the new specifications. This specification will also be part of the TTD´s

and Valid is also DIS6119 Annex 5

## Production Means and Stages

Soldering pasting Side 1	DEK Screen printing machine

SMD placement Side 1	Siplace 80S/HS50/Philips FCM/Siplace 80F

Optical inspection 	Opto-Control AOI

SMD placement Side 1	Siplace 80F (shielding / I/O-connector)

Reflow soldering Side 1	Soltec reflow soldering unit

Soldering pasting Side 2	DEK Screen printing machine

SMD placement Side 2	Siplace 80S/HS50/Philips FCM/Siplace 80F

Optical inspection 	Opto-Control AOI

SMD placement Side 2	Siplace 80F (shielding / I/O-connector)

Re-flow soldering Side 2	Soltec reflow soldering unit

Panel separation  	Pematech Saw Separation

Testing/Pre-Assembly/Assembly/Screwing/Camera-Test/Customer Test/Customer Init/Assembly Rear cover

Baumann automatic line assembly

Packaging (Anica cell automatisation optional)

## Basic Assembly Concept

See chapter 3.6. A separate MMI PCB with SMD components will be assembled with the metaldome foil and will be delivered as one component.

## New production technology

- Leadfree Soldering Process

- 0.5pitch CSP components

- 512MBit Flash Offline / Inline boot process

## Quality Targets GJ03/04

The following quality targets are aimed at:

## See Chapter:   8.2  NPI Quality requirements

The targets will not be achieved during the run-in phase. They can be achieved in a stable production line, only.

## Expected Production Quantities/Production Capacity (as of 14.11.2004)

Source: Anke Jakob 19.11.04

# Test Rig Planning

## General Requirements

It is absolutely essential to comply with the following requirements and to include these into the product definition.

Any deviations from them must be clarified with GO and are defined during a production-testing workshop (A. Zimmer ICM MP SCM NPI31).

Topic:		Requirements of Technical Testing for Mobile Phones

Author:		ICM MP SCM GO 2 KLF Mr. Grein

Link to:  M1 enclosed documents Minos\pv1pta37e.doc

In this set of regulations the most essential requirements are as follows:

- The central module can be switched on and operated via the mixer plug.

- No mechanical match-ups.

- Make sure that the necessary HF measuring points are given 50-Ohm impedance or make available any necessary impedance transformation outside switching.

- Match-ups with higher requirements as regards accuracy (transmitter output etc.) are only possible in the testing stage Match-up / System test.

- The free spaces and the test point diameters for test points must be kept (see design regulation).

- Fulfil PCB requirements as regarding the fixing drillings and carrier placing (see design regulation).

- Make available a Windows based software library for controlling the phone functions.

For more details please refer to “Testing technical standards for mobile phones” (Version 3.7; pv1pta37e .doc;06.10.2004)

## NPI Quality requirements

# Customer Care

## Customer Care strategy

Attached diagram documents contains the CCQ MP Strategy:

The acceptable care standard per region for Phoenix as well as the additional service packages has to be defined together with sales and PM.

## Service Objectives

- Siemens bears responsibility for the products with a Siemens Logo and co-branded products

- Local Service Organisations (LSO’s) will ensure the international service for a country or a region

- Implementation of hotline for customers (end users)

- Supply of small parts to end user via hotline

- Standard repair fee will be charged for out of warranty repairs

- Car mounting and accessory service will be rendered by business partner (retailers)

- SW update over the air (SWUOTA) as a mandatory feature starting with product launch after 06/04 in order to reduce Level 0 returns

- Strategic wireless Services like phone settings over the air (OTA Standard – SyncMLDM) will be continuously enhanced in order to reduce hotline calls

- SW update and download of phone settings should be possible for end customer directly via internet

- Self helf tool via Internet for the end customer based on the CCQ knowledge database

## Repair level definition

In general, 6 repair levels for mobile phones are defined:

- Level 0
Phone is not disassembled: SW Updates, un-blocking variant configuration, logistics, in-warranty check, error reproduction

- Level 1
Change of non soldered components like housing, keypads etc., exceptions: adjustment for display
Level 2
Exchange of complete boards

- Level 2.5
Trouble shooting and repair of defined soldered components without adjustment

- Level 2.5e (extended). 
Trouble shooting and repair of defined soldered components including adjustment

- Level 3
Trouble shooting and repair of all soldered components including complete adjustment and test. Use of automated test equipment.

## World-wide distribution of service level

- Europe/Near East/RSA/America
– level 1/2/2.5/2.5e repairs at the LSO and/or service partners
– swap and/or repair of devices for end users
– level 3 repair at Siemens workshops

- Far East/Australia
– level 1/2/2.5/2.5e repairs at the LSO and/or service partners
– swap and/or repair of devices for end users
– level 3 repairs at the ASC in Singapore

- Republic of China
– level 1/2/2.5/2.5e repairs at service partners
– repair of devices for end users
– level 3 repairs at the Service Centre Shanghai

## Roll out plan for the service concept

- Acceptable care standard and additional service packages are defined together with PM and sales at market launch at the latest.

- Definition of the service parts when the mechanical design is available (or samples) but 4 month before market launch at the latest.

- Service parts will be planned on a basis of the expected return rate and quantities (1 month after the availability of the planned quantities of the sales department)

- The procurement of these planned service parts will be initiated.

- The prices for service parts and repair fees will be available via e-commerce two weeks prior to market launch.

- Quantity of swap units/delivery units for the affected LSO’s are agreed and delivery is initiated at market launch at the latest.

- Investment for repair line will be planned (2 month after M1). This planning is put into action at market launch at the latest.

- Carry out of the training for the service partners (LSO’s) is planned and will be put into action with market launch.

- Service documentation and training documents will be available via Internet on market launch at the latest.

- Crosscheck of the user guide by the hotline staff.

- Training of the hotline 4 weeks before market launch at the latest.

- Definition and procurement/making of test equipment and test software. This will be available on market launch at the latest.

## Service parts

As soon as the mechanical design is available the service parts will be defined. In principle the following parts will be defined:

Swap:		- mobile phone without battery for variants which will be distributed in Germany, China and NAFTA

- control board without SIM lock

- control boards for each SIM lock type as prepared SIM lock

Spare parts*:	display unit/module

board

lower case shell with pre-assembled antenna but without vibra and microphone

vibra motor
		microphone

camera module

acoustic sealing

upper case shell (cover)

battery cover

shielding cover

keypad

accessory components

SAR Frame

MMI Board inclusive metaldome foil

*final decision on spare parts will be made after B1+ prototyping

## Technical Service Requirements

### Test equipment

The test concept will  be deduced by the concept the production applies (level 2.5e/3). All necessary information regarding change of used hard and software for testing must be provided to the service project responsible immediately by the production implementation responsible.

### Technical Service Requirements

The ‘Global Repair Requirements’ are written down in the M0 Document and available in the appropriate project folder to the 2 documents:

“Global Repair Requirements A0 – V3.03 2004 03 23.doc”

“Global Repair Requirements A1 – V3.03 2004 03 23.doc”

All Technical Service Requirements of the M0 Document will be fulfilled. The items are listed as a short form for inhouse products in the X75 feature list, register “Service_repair”.

# Quality

## General Quality Requirements

The general quality requirements for phone and accessories are contained in the document “General Quality Requirements for Cellular Deliverables (Cellular Phones / Pocket PCs and Accessories)” that was agreed between Development, Product Marketing, Purchasing and Quality Management.

Currently in force and valid for this project is Rev. 3.1 from January 29th 2002. In case of new revisions of the document after M1, Product Marketing, Development, Business Administration and Quality Management have to agree if any updated requirements shall be applied in this project.

The remainder of this subsection outlines some basic requirements. For details and additional requirements refer to the General Quality Requirements Rev. 3.1.

###### Temperature Ranges for Mobile Phones

| Warehousing | -40°C to +85°C | Mobile phone without packaging |
| --- | --- | --- |
| Non-deformation (plastics) | -40°C to +85°C | -30°C for LCD display/ camera |
| Solar radiation (outdoor) | 1120 W/m² at +55°C |  |
| Battery operation | -10°C to +55°C | Fully operable according to relevant specifications |
| Operation with external power source | -20°C to +65°C | Additional check for making/ receiving calls (emergency call) |
| Nondestructive range in switched on mode | -30°C to +70°C |  |

###### Ease of Operation

Standards regarding the ease of operation / user-friendliness for different components include, amongst others:

Housing

The casings must keep their shape when pressed in the user's hand. Manual pressure on the casings may not lead to physical damage or impact the function of the test device.

The front and back casings must fit tight; no noticeable play between the casings is allowed. No creaking and grating is accepted. There may be no noticeable projections or protrusions (seam or ridge / overflow). The join must be uniform.

The surface must have a good feel and be dirt resistant. In particular, fingerprints (oil and sweat marks) must not show on the housing surface.

The materials used (housing parts and adhesives) may not be hazardous to the user's health. Also, the housing may not have a noticeable smell.

Keypad

Keys must have a noticeable mechanical pressure point supported by a mechanical resistance or acoustic "click".

The number “5” key must have a nub or other tangible means of orientation (ETSI-Standard; ES201381).

Incorrect pressing of keys (off-center, forceful pressing) may not lead to a mechanical sticking of the keys.

The illumination of the keypad has to conform to the product specification (potential faults are single-edged illuminated keypads or a consistent decrease of the illumination from one side to the other, if a uniform illumination is required).

The printing and/or key markings must be clearly legible.

The individual operating elements must be designed in such a way that they are easy to operate (applies, in particular, to the keypad). The power ON / OFF key must be secured against inadvertent switching on.

Display and Window

The contrast and read-out angle should be optimized so that the display is as free from distortion and reflection as possible. The display must be designed in such a way that the user cannot see the insides of the unit. The lighting must be uniform.

The window must be designed in such a way (form and placement) that it is largely protected against inadvertent scratching (for example, the housing may be designed so that it protects the window). At the same time, the window must be designed in such a way that the underlying display is not destroyed if placed under pressure.

The display window shall be designed in such a way that an accumulation of dirt or grime between the glass and the window due to static charging is avoided.

The clearness of the display window may not change (become opaque) due to exposure to climatic factors (neither before, during or after testing). There may be no permanent changes in the colors shown on the display after the various climatic tests have been performed.

The visibility of the display must be correct when using polarization glasses.

SIM Card Reader

Insertion and removal of the SIM card must be easy to handle. The reader / SIM card must allow smooth movement, with only marginal play. The reader may not tilt or jam. To prevent incorrect operation, there must be a clear control to show that the SIM card has been inserted properly. The system must tolerate a possible incorrect operation. The card (holder and reader) may not be damaged if incorrectly inserted. If incorrectly inserted, the card must be easy to remove.

Removal during operation of the phone may not lead to damage of the SIM card.

Plug System

The plug must be easy to plug in, without noticeable play. The plug must go in straight and may not jam. The patch plug must lock cleanly into place.

An incorrect insertion of the plug (upside down, i.e. 180° flip-over) must be prohibited.

Mechanical stress to the soldering points must be avoided.

I/O connector

All pins of the I/O-connector must withstand a short circuit to 0 V or to any other pin without a remaining impact. This is also valid for the external connectors of the battery pack and the contacts of the whole accessory as well. The inversely polarized feed of external voltages to accessories and / or unit may not lead to a hazard of the customer.

Battery Pack

The battery pack must allow smooth movement, with only marginal play. The pack may not tilt or jam when inserted or removed. The lock may not jam, must be easy to operate and may not show any wear and tear for the life of the unit. All moveable parts must be covered to prevent injury to the user. The battery pack must be designed in such a way that it cannot be inserted incorrectly.

If the housing should be specially designed, the color of all related parts must be uniform to ensure a suitable match.

###### Lifetime and Utilisation

For the complete life of the phone the following functions of the phone must be guaranteed and must remain preserved without any optically visible wear (excerpt from Standard Test Plan):

Number of cycles for operation elements

| Operating element: | Number of repetitions: |
| --- | --- |
| 1. Keypad including all function keys | 150,000 |
| 2. Antenna (flexible) | 25,000 |
| 3. I/O connector | 10,000 |
| 4. DC connector | 10,000 |
| 5. SIM contacts | 2,000 |
| 6. Switch / control | 120,000 |
| 7. Jog dial | 100,000 |
| 8. Pop-up | 100,000 |
| 9. Flexible cables | 80,000 |
| 10. Sliding button | 100,000 |
| 11. Battery lock | 2,000 |
| 12. Battery contact | 2,000 |
| 13. Vibrator (tremblings) | 50,000 |
| 14. MMC | 5,000 |
| 15. coax switch | 10,000 |
| 16. flap/ clamp shell | 80,000 |
| 17. camera flexible/removable | 10,000 |

Other requirements

- The contrast of the LCD display must not reduce by more than 50% within 5 years.

- The phone housing must be distortion-proof for its service life and resistance against fracture from a fall from 1.52m on to a concrete surface.

- No changes of vibrator performance (loudness and vibration) over the lifetime.

- The phone must (with or without a battery) withstand a fall on to a concrete floor from a height of 1.52m in any situation without incurring damage. 
The battery must withstand a fall from 1.00 m on to concrete or steel.

- The life of the battery under GSM conditions is around 500 charge cycles and within this time the battery performance must not fall below 80% of its nominal capacity.

Scratch proofness

Tested with hardness tester - Erichsen, model 318 (engraving stylus type Bosch Ø 0.75 mm, speed 10 mm/sec., length 10 mm)

| a) Display window: | 10N |
| --- | --- |
| b) Dyed granulated material: | 10N |
| c) Dyed granulated material, polished: | 5N |
| d) Painted surface: | 7 N |

Very faint marks that only change the surface shine are not to be considered as scratches.

###### IP Classes

IP classes (dust and water resistance) for the product are defined as follows:

| Cellphone: (M-Phones) | IP 50 (IP 54) |
| --- | --- |
| SIM card reader: | IP 40 |
| Antenna: | IP 62 |

###### Mechanical, Climatic and Ageing Requirements

Mechanical, Climatic and Ageing Requirements for mobile phones and accessories (test specifications and assessment criteria) are contained in detail in the General Quality Requirements Rev. 3.1.

###### Electromagnetic Compatibility, SAR and Others

Standards and additional Siemens requirements regarding EMC, ESD, SAR and other issues are contained in detail in the General Quality Requirements Rev. 3.1.

These requirements include the Siemens requirements for robustness against ESD discharges (+/- 8 kV direct contact discharge and +/- 15 kV air discharge without ground connection, with permissible functional disturbances but no damage).

## Quality Requirements for Visual Appearance of Mobile Phones

The requirements as set forth in the document “Quality Requirements For Visual Appearance Of Mobile Phones” are valid for this project.

## Environmental Protection

Siemens AG places great emphasis on the importance of environmental protection. The law is also making greater demands on the recyclability of products. One of the most significant publications in this respect is the “Electronic Waste Regulation”, which prescribes as obligatory the recycling of used electrical appliances. In order to do full justice to these requirements and to our own ecological aims, the following considerations are paramount when dealing with new products:

- The product must be able to be dismantled easily

- No use of welds and adhesives

- A reduction in the variety of materials used

- Avoidance of compound materials

- Marking of plastic parts

- No pollutants must be used

- Recycling/disposal documentation must be drawn up

###### Requirements of Recycling Process

As we cannot make any safe predictions on the nature of future recycling technology, the following recommendations are based on the trends recognisable today for future recycling technology. In this respect recycling technology requires that old machines can be dismantled, taken to pieces and sorted out into those materials for which reprocessing capacities exist or which have to be taken to depots or incinerators.

Dismantling or taking to pieces of telephones in:

- Electronic/electrical components		(assembled FBG, display, ....)

- Plastics (by type)				(housing, ...)

- Cable and leads				(coax cable, ...)

- Metal					(screws, ...)

- Paper/cardboard				(IMEI plate, ...)

- Waste/other					(Keypad mat, earpiece, ...)

Recycling Concept

Depending on the structure of the components the requirements on materials must satisfy motor car requirements:

- No use of Cadmium

- No CFCs

- As far as possible components made of polymer materials must be marked (depending on size)

- Emission of organic compounds

- Free from Asbestos

- Marking of the battery pack	(Recycling Instructions: don’t throw into fire, etc.)

###### Construction

For a construction sequence, which does justice to recycling factors those operational steps, are important which determine the choice of material and the method of jointing them.

An easy-to-assemble automated construction normally also fulfils the criteria for easy disassembly.

From the point of view of recycling, the rules and basic premises of the standard design and the standard constructional design must be kept and extended.

The following recommendations are a general aid for making sure that when the product is designed and developed, the parts of the newly developed products can be recycled.

For detailed descriptions and information please refer to SN 36350.

General

- As few separate parts as possible

- Avoid material compounding (e.g. sticking together of different materials and laminates)

- Joining and de-jointing wherever possible in one direction

- Guarantee accessibility of disassembly tooling

- Use unified screw heads in respect of type and size

Connections

- Connections should be used which can still be separated easily even after the planned product utilisation life.

- The number of different types of connection should be minimised.

- Standardised connection processes should be selected.

- Self-retaining connection processes (clipping and snapping) should be used.

- In the case of snap connections, one unlocking possibility should always be foreseen. If this is not possible, then the connection should be easy to take apart by knocking.

Materials Selection

- Reduce the number of different materials used

- Only use plastic from the selected or preferred list

- Do not use any materials which are difficult to recycle

- Plastic parts should be marked in line with DIN 54840 or MP-specific regulations (Standard Construction Concept, Sheet 32.1),

- Avoid using material compounding (e.g. injected plugs, sticking of different materials, laminates). If it is not possible to avoid these compound structures, then separating aids should be foreseen, e.g. easy opening points, and these should be included in the User Manual.

## Quality Plan

A project specific quality plan is created and maintained by the QM department. This document contains:

- Checklist of basic failures from former products

- FMEAs (responsible: R&D)

- Environmental Tests (status of device, variants, amount; RD responsible before B2, QM responsible from B2)

- Field trial (status of device, amount)

- Product audit

- Milestone review

- Checklist risk analysis

- Checklist safety instructions in the user manual

## Department PSQA-plans

Project specific quality assurance plans (PSQA-plans) are defined by the relevant departments and responsible persons are named. These departments are, according to MEP Issue 4 from May 31st 2002: Radio, EMC, Logic, Mechanics and Layout/Production Data.

The PSQA-plans should contain, amongst others:

- Planning of failure mode and effect analysis (FMEAs)

- Planning of design reviews

- Test plans for components / sub-systems / assemblies

- For the mechanical PSQA-plan: Test plans for environmental testing (shock, drop, climatic factors, aging, etc.) in the mechanical test plan

- For the Electrical and EMC PSQA-plans: Test plans for electrical and EMC tests to ensure compliance with the requirements set out in the specifications for the device and the interfaces

## Product Safety and Technical Risk Assessment

A technical risk assessment for this project will be created until M1 by the Quality department. The technical risk assessment contains, amongst others, acoustic shock, short circuits, charger, battery.

For reasons of product liability, the user must be protected from electric shock caused by voltages applied to the outside of the device or the accessories.

The product and supplied accessories (in the following called only “product”) must comply with all relevant international and/or national standards in accordance of the country of distribution.

For Europe the product must comply with all relevant EU directives in order to fulfil the requirements for CE marking.

- The Radio and Telecommunications Terminal Equipment Directive

- Low Voltage Directive (LVD)

- EMC Directive

- SAR

- VDA guidelines (Accessories)

- Eye safety of laser components (IEC/EN60 825-1 "Safety of Laser Products")

For each of the directive the applicable international harmonised standard(s) have to be applied. National deviations or standards (in case international standards are not existing) in accordance to the country of distribution have to be considered.

Where harmonised standards do not exist, latest technical publications have to be considered.

Suitable means (e.g. ramping like in L55) to guarantee protection from acoustic shock have to be implemented.

Special Precautions:

- Precautions (mechanical, software engineering) must be taken to prevent danger to or injury of the user through inadvertent switching on of the device.

- Software engineering precautions are to be taken to prevent danger to or injury of the user through an acoustic shock as the ringer function or Handsfree is implemented over the receiver. A separate review must be carried out to ensure that this requirement is met.

- In order to protect the user, ramping has to be activated for all tones and melodies (increase of the sound pressure level from a value below 120 dB SPL to the maximum value in 1.5 seconds or longer).If ramping is not possible (short tones, like signal tones), the sound pressure level shall not exceed 120 dB SPL.
The user protection shall be realized by means of hardware (e.g. second speaker, ramping and limitation of sound pressure level realized by an electronic circuit,…. not controlled by software.)

###### Product Safety and the Software

- Precautions (mechanical, software engineering) must be taken to prevent danger to or injury of the user through inadvertent switching on of the device.

- Software engineering precautions are to be taken to prevent danger to or injury of the user through an acoustic shock as the ringer function or handsfree mode is implemented over the receiver. A separate review must be carried out by R&D-Department to ensure that this requirement is met.

## Software Quality

###### Software QA Plan

A Software Quality Assurance Plan will be set in place until M1 that sets out both the general and project-specific requirements and stipulates the quality assurance measures that are to be taken during the development process. Reviews by Quality Management are done on the basis of the currently valid milestone checklists and the stipulations set out in the Software Quality Assurance Plan.

Software Process Code

Software is to be developed in accordance with the Software Process Code (QMS – VA T010 SW-Pro) set out in the Software Project Management Guidelines (currently in the pipeline) and the Software Quality Assurance Process (QMS – VA Q025-MP SW-QA in projects).

## Field Trials

###### Aim and Focus of the Field Trial

- Confirmation of compliance in home and foreign networks. Tests according to GCF AP (GSM Certification Forum - Application Procedure)

- Confirmation of hardware and software quality (ready to be introduced into the market)

- Detection of weak points in the HW construction of the mobile phone and accessories, SW errors/bugs by functional tests as well the check of user manual

###### Realisation

- Dependent on the product specification, GCF Field Trial tests in different GSM networks will be performed

- End User test with pre-production samples (SIEMENS B2-samples)

- Extended End User with samples from pilot series

- The accessories for the product shall be included in the Field Trial

###### Time Frame

The basic functionality with the accessory shall be tested and confirmed with B1 samples.

The official Field Trial shall start with the provision of B2 pre-production samples at S3 and shall end with S4 (at least four weeks testing time). The extended Field Trial shall start with the production of the first samples from the pilot series (at least four weeks testing time).

The beginning of the Field Trial shall start under the following conditions:

Hardware

- B2 Field Trial samples are available. Those samples shall have the final layout ready for approval. For the Field Trial it will be taken into account that HW - variants (e.g. alternate display manufacturers) exist. Such different variants shall be available for the Field Trial. The number of samples shall be defined according to Six Sigma tools and dependents on the number of different variants.

- For the extended Field Trial, samples from pilot series shall be made available. The number of samples shall be defined according to Six Sigma tools and dependents on the number of different variants.

- Agreed accessories shall be available for Field Trial

- The Field Trial samples shall have a valid test IMEI

- Access to mobile engineering functions shall be possible

To avoid any time delay during the Field Trial phase the following shall be ensured:

- The samples shall have the possibility to record a mobile trace via PC – software (not only in office environment, but also in the field). Necessary HW (e.g. tracing cable, adapted devices with 2nd BfBus - connector) and tracing SW shall be made available at S25.

- The provision of a monitor mode in the mobile shall be ensured

- Field Trial samples shall not have a SIMLOCK

Software

- The SW shall be stable (frozen).

- All agreed features are implemented and tested. (Note: Implementation of features during the Field Trial phase will lead to re-tests)

- At milestone S3 (prior to the Field Trial), all known errors shall be recorded and evaluated in a review between R&D and Software Quality Assurance. Errors, which hinder the realisation of the Field Trial, shall be closed.

To avoid any time delay during the Field Trial phase the following shall be ensured:

- During the Field Trial phase, the SW of the mobile phone shall be upgradeable also outside the factory (e.g. SIEMENS LG´s). The necessary SW tools + HW (upgrade cable) shall be made available to QM PV at S25. QM PV shall have at least limited EEPROM read/write access (SW exit codes etc.)

- Any SW feature implementation, which leads to the need of a SW update which can not be done in the field (e.g. re-writing the IMEI because of implementation of security features) shall be implemented prior to the start of the Field Trial.

Engineering equipment

The following equipment shall be made available to ICM MP CCQ QM PV:

| Description | Number | Date |
| --- | --- | --- |
| Mobile phones (B1) including charger (The voltage range of the charger shall include AC220-240V; 50/60Hz or if applicable AC100-120V; 50/60Hz. A plug adapter for standard European socket outlets – if necessary – shall be available). (Note: Those samples are needed to test the Tracer SW, SW upgrade etc. prior to the Field Trial ) | 2 | S25 |
| Tracer Software | - | S25 |
| Tracing Hardware (cable) | 50 | S25 |
| Tracing Hardware (adapted devices; 2nd BfBus connector) | 6 | S25 |
| Mobile – Boot Configuration (e.g. Upgrade cable if different from tracing cable, service box etc.) | 25 | S25 |
| SW upgrade tools (e.g. SWUP, Initialisation tool ) | - | S25 |
| External Antenna (possibility to connect the mobile to a CMD) | 2 | S25 |
| Access to SW and HW – error tracking database | - | S25 |
| Others: Overview to the planned SW “standard – variant” (Documentation) | - | S3 |

It shall be ensured that those engineering equipment can be used during the whole Field Trial phase (no changes after S25).

Performance aspects covered by Field Trial  (Overview)

The following general functions and aspects of the mobile phone shall be tested during the GCF and End User Field Trial. For the End User a questionnaire shall be prepared and evaluated.

- Basic handling, including set up, clear and in-call functions

- Cell selection and reselection

- Automatic & manual PLMN selection

- Handover

- Operation of each basic service, supplementary service and features within the scope of GCF and which is supported by mobile. (fax / data services / GPRS). For speech it includes a degree of subjective speech testing sufficient.

- SMS (MO – PP, MT – PP and CB)

- Interworking with different SIM cards (using different types of SIM/card / profile)

- Interworking with agreed accessories

- Review of user manual

- HW Construction of the mobile phone

###### Documentation

SW / HW - errors from the Field Trial will be recorded in the relevant error tracking data base (e.g. Clearquest for SW; equivalent HW tracking data base).

At the end of the official Field Trial, a summary report shall be prepared.

The successfully finished Field Trial is the basic for the milestone S4.

###### Quantity Planning GCF

For the GCF Field Trial a quantity of 25pcs of  B2 or newer samples is needed.

###### Quantity Planning Friendly User Test

For the friendly user field trial a quantity of 140pcs-300pcs of B2 or newer samples field trial mobiles is needed, depending on the agreed scope of the trial. Details are contained in the field trial plan.

Example:

| GCF | 25 |
| --- | --- |
| EMEA | 40 |
| APAC | 40 |
| China Variant | 40 |
| Customer Acceptance | 60 |
| US | 20 |
| SWAP | 15 |

Total:			 240pcs

These mobiles have also be available latest 2 cw before the milestone S3.

###### Pre-Validation Field Trial

The Pre-Validation Field Trial is supported from QM PV, if this is requested by the project.

Quantity of mobiles is between 20pcs –30pcs .

###### Other Requirements from Field Trial

- All mobiles need the same boot-kernel

- All mobiles have to be signed with the Field Test key

- Open BF Bus

- Information about booting and tooling

- Delta Description for new software versions

- New software has to be available latest until Friday 09:00h

- Access to developer drives

- Current SW has to be available as *.exe file by start of validation

- Change of blocks, e.g. new NF parameters, have to be available also as *.exe files

- It is not possible to update mapping files global

- Development environment is not global wide available

## Requirements for Product Audit

If no antenna connector is accessible for the Product Audit, an antenna tube must be made available for measuring purposes at the Product Audit.

- The responsible department for  the developing of this antenna tube must be the RD
(the Product Audit has only the possibility to support the R)

- The coupling between mobile antenna and coupling antenna must be < 12dB

- Reproducibility TX +/- 0,3dB

- In case of different RF-chipsets (e.g. Hitachi/ Infineon), it must be possible to use the tube for both variants

- With the antenna tube, it must be possible to test all items of the Product Audit (excluding spurious emissions). These test items are contained in a separate file which is available to RD.

## Requirements for Outgoing Inspection

A Device Check must be implemented in the software based on the implemented feature set and the current requirements for the factory’s outgoing inspection and box opening.

## Robust Development

The phone will be developed using  the Robust Development Process with RPD modules and targets as described in the RPD scorecard. The RPD process will be regularly monitored. The owner of the RDP-Process is the PLT.

## Field Return Rate

- Target Fy  2005/2006 :Return rate <= 7,5 % (75.000 dpm)

## Quality of Suppliers and Components

See separate Quality Assurance Agreements (QAAs) for each supplier and component.

## Quality in Production

Required Quality in Production

The medium and long-term aims of QA Production are a continuous improvement in the through-flow rates and Q numbers in the production processes.

To achieve these targets the essential factors are qualified processes with Cpk values larger than 1.33, batch tolerant switchings, simple assembly concepts, a reduction/standardisation of the parts (phone, accompanying packs). In the early developmental stage the pre-requisites should be planned by the departments concerned in order to achieve quality benchmark figures and in the course of the developmental process these should be checked for their effectiveness.

The quality key figures shown in the table are target values that are defined at the beginning of each business year. They are mean values over all products. In the future, detailed target values for different segments might be set.

Quality Benchmark Figures

Data source ICM MP CCQ QM SC

###### NPI requirements

Requirements Set for Electric Specifications for Mass Production Variability of parameters to be balanced

mena +/- 3

Receiver sensitivity (all channels, without fading):

GSM	< -104dBm

PCN1800/1900	< -102dBm

Transmit output power (all channels)

GSM	>31.5dBm

PCN1800/1900	>29.4dBm

Transmission phase distortion

GSM / PCN 1800/1900	<3.8°

AF signal-to-noise ratio

-S/(N+D)>20dB measured at the receiver capsule at a nominal sound pressure of 94dbspl (1Pa) at 1kHz.

## Deviations from Agreed Quality Level

Any deviations in quality which may occur and the decision as to whether these are to be accepted shall be made by Product Marketing, Business Administration, Development

# System Test

## Introduction

This document describes the system requirements for GSM/GPRS terminals in terms of regulatory and voluntary requirements in different regions and for the different functionalities of the terminals. The requirements on GSM terminals are applicable for 850/900/1800/1900 MHz if supported by the terminal. Additionally the document describes requirements from the network operator TIM, Italy, on GSM terminals.

## Abbreviations and Definitions

| Bluetooth SIG | Bluetooth Special Interests Group |
| --- | --- |
| CTIA | Cellular Telecommunications and Internet Association, US. |
| CTR | Common Technical Regulation |
| EC | The European Community |
| EMC | Electro-Magnetic Compatibility |
| FCC | Federal Communications Commission |
| GCF | Global Certification Forum |
| GSMA | GSM Association. The Association of GSM Network Operators worldwide. |
| GSMNA | GSM North America. The North American Regional Interest Group of the GSM Association |
| ME | Mobile Equipment |
| PTCRB | PCS Type Certification Review Board |
| R&TTE | Directive 1999/5/EEC – Radio Equipment and Telecommunications Terminal Equipment Directive |
| SAR | Specific Absorption Rate |
| SIM | Subscriber Identity Module |
| SDO | Standards Development Organisation |
| TOG | The Open Group. WAP testing & Certification |
| 3GPP | Third Generation Partnership Project. A project of SDOs in charge of elaboration of specifications for a 3. Generation mobile radio system (UMTS) |

## Type Approval (TA)

Following chapter describe the requirements mobiles of the X75 platform have to meet to gain required type approvals and certifications. In this content the string “X75-Products” is placeholder for the major products of the X75 platform which are Minos, Virgo, Taurus and Hydra supporting the frequency 900MHz, 1800MHz and 1900MHz. The String “X76-Products” is a placeholder for the products supporting 850MHz,1800MHz and 1900MHz.

## Regulatory requirements - Overview

### European Community

GSM and UMTS terminals are under the scope of Directive 1999/5/EEC, Radio Equipment & Telecommunications Terminal Equipment(R&TTE). Directive 1999/5/EC is the overall framework for radio equipment and telecommunications terminal equipment for all countries in the European Community (EC) and defines requirements for:

- Health and safety aspects

- EMC aspects

- Spectrum usage aspects

### Outside Europe

For North America and other countries outside the EC, the technical requirements are almost identical to those applied in the EC. Additional requirements are listed in the requirements matrix.

China / APAC approval requirements are under PM responsibility and listed in a separate chapter below.

## Voluntary requirements - Overview

### GCF

Within GCF, manufacturers are required to fill in a list of Core Specs (e.g. 3GPP TS 45.005 for RF) applied to the design. Moreover, the exact version of Core Specs shall be listed for a range of Core Specs. The GCF Steering Group defines minimum requirements to Versions of Standards. By default, the terminal design shall therefore be made based on Core Specs.

The GCF requirements are defined in the current version of GCF-CC.

### PTCRB

The PTCRB requirements are defined in the current version of the NAPRD. PTCRB is required for customers roaming into the networks of operator’s part of the PTCRB group (North America).

### Additional voluntary requirements

- Besides regulatory requirements, the product shall comply with a range of supplementary, “voluntary” certification schemes.

- Voluntary certification schemes are developed by various entities defining requirements for specific functionalities e.g. Bluetooth, IrDA and WAP as described in the requirements matrix.

- An additional requirement for TIM, Italy, regarding Network selection from SIM Card Preferred List for GSM terminals is included.

## Requirements Matrix

### Regulatory requirements

| Aspect | Aspect |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | US (& Canada) |  |
|  |  | Technical requirement | Mandated by | Technical requirement |
| Health and safety | Health and safety | Health aspect: Specific Absorption Rate (SAR) according to EN 50 360 and EN 50 361. | FCC | FCC SAR Regulation OET 65 |
|  |  | Safety aspect: Safety of Laser Products EN60825 |  |  |
|  |  | Safety aspect: Electrical safety according to EN 60 950. | - | - |
| EMC | EMC | EN 301 489-1 EN 301 489-7 EN 55013:2001 EN 55020:2002 EN 55022:1998 IMT2000 UTRA FDD: EN 301 489-24 Bluetooth: EN 301 489-17 (EN 301 489-3 is not used) | FCC | FCC Part 2 FCC Part 15 FCC part 22 FCC Part 24 HAC: ANSI C63.19 |
| Radio Spectrum | Radio Spectrum | GSM: EN 301 511 IMT 2000 UTRA FDD UE: EN 301 908-1 EN 301 908-2 |  |  |
|  |  | Bluetooth: EN 300 328-2 (EN 300 440-1) (EN 300 440-2)* |  |  |
| Special requirements | Special requirements | None specified for Europe at the moment; IMEI security is discussed | FCC | TTY: Cellular text Telephone Modem according to 3GPP TS 26.226 TS 26.230 TS 26.231 |
|  |  |  |  | USA FCC Wireless E911 Rules |
| Inter-operability | Inter-operability | - | GSMNA | PTCRB NATWG.03 |

### “Voluntary” requirements

| Aspect | Certification planned for typical Product | Specified by | Technical requirements |
| --- | --- | --- | --- |
| Mobile <-> Network Inter-operability | Yes | GSM and UMTS: Global Certification Forum (GCF) | GCF-CC Field Trials |
| Bluetooth functionality | Yes Bluetooth certification shall be printed on the box | Bluetooth Special Interest Group | Bluetooth Qualification Program Reference Document V0.9 |
| WAP functionality | No | The Open Group | WAP Certification and Testing Process |
| OpenWave Browser | Yes Mandantory if OpenWave Browser is included. | OpenWave | Browser Compliance Verification |
| IrDA functionality | No IrDa shall be tested but no type approval is necessary | Infrared Data Association | IrDA Interoperability Tests |
| USB functionality | Yes The certification is mandantory if the USB logo shall be used. | USB Implementers Forum | USB Compliance Program |
| Network Selection from SIM Card Preferred List | Yes This test shall be performed. | TIM | TIM Proprietary specification |
| CTIA | No | CTIA | CTIA GSM-1900 Terminal Unit Certification Program Management Document |
| Sync ML (OMA DS&DM) | No (voluntary) | OMA / Data Sync (DS), Device Management (DM) |  |
| Java (Engine) | Yes (mandatory) Sun requires it | Sun |  |
| MMS (OMA) | No (voluntary) | OMA / MMS |  |
| IMPS (Instant Messaging and Presence Services) (OMA) | No (voluntary) Mandatory if we want to use the Wireless Village Logo | OMA / Wireless Village |  |
| I Mode | Yes: for opperator which reqequire I-mode | Docomu | Docomu Approval |

## Regulatory requirements - detailed

### Health aspect

Health is a new aspect introduced by Article 3.1.a of Directive 1999/5/EEC in EC.

Specific Absorption Rate is subject to regulatory approval within EC. Harmonised Standards for SAR; i.e. EN 50 360 and EN 50 361 are available and can be used for Annex III of R&TTE..

| Relevant harmonized standard for article 3.1a) | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 50360 | 2001 | Requirements for mobile phones for human expose to EM fields | --- |

As of today the question remains when the new phantom will be available. Currently testing has still to be done according to  EN50 360 but with the old phantom of ES59005. The new phantom is expected within the timeframe of terminals currently under development.

Actual limits currently given by Council Recommendation 1999/519/EEC will be applicable using the Harmonised Standards.

In addition to demonstrating technical compliance it is required to disclose actual SAR values to the public by means of a company WEB page.

Health aspects are handled by both FCC and CTIA in US.

The FCC and CTIA have continuously further developed SAR limits to include requirements for e.g. Body-worn configurations. Requirements as of today are known, however these may not be valid within the timeframe  of terminals currently under deveopment.

SAR values for US are made public available at FCC Website. In addition, both FCC and CTIA require detailed wordings to be included in user manuals.

### Safety aspect

Safety for stand-alone terminals operating at a few Volts is a new aspect introduced by Article 3.1.a of Directive 1999/5/EEC in EC.

EN 60 950 is the corresponding Harmonised Standard applicable within the EC.

EN 60 950 concerns:

- Protection against electrical shock and hazards

- Protection against thermal rises

- Resistance to heat and fire

| Relevant harmonized standard for article 3.1a) | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 60 950 | 2001 | Safety of information technology equipment |  |

National standards for other regions e.g. AS 3260 for Australia and UL1950 for US are all based on IEC 950, from which also EN 60 950 is derived.

Compliance to safety regulations includes flammability requirements for plastic materials documented by corresponding UL File Listings (Yellow Cards) and Certificates of Conformity with all deliveries. PCBs as well as batteries need to be UL recognised.

IEC/EN 60825-1/2

- This standard must be fulfill for the flash LED

- The handbook should contain at least following hints concerning the Flash LED (proposal from other mobile phone producer). Flash Precautions Class 1 LED Product This product complies with, IEC/EN60 825-1 "Safety of Laser Products". Class 1 products are safe under reasonably foreseeable conditions of operation. Do not fire the flash at drivers of road vehicles, as this may cause the driver to lose control of the vehicle and result in a traffic accident.

### EMC aspect

EMC is an integrated requirement of Directive 1999/5/EEC, identified as an essential requirement in Article 3.1.b.

Under previous European legislation, the EMC standard for GSM terminals ETS 300 342-1 was not harmonised. A range of product specific EMC standards for telecommunications terminal equipment are now restructured into the following relevant harmonized standards for article 3.1 b): (listed in the OJ 10th August 2002)

| Relevant harmonized standard for article 3.1b) | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 301 489-1 | V1.4.1 | common requirements for a large range of product types |  |
| EN 301 489-3 | V1.4.1 | specific requirements for short-range devices; i.e. Bluetooth functionality. |  |
| EN 301 489-7 | V1.2.1 | specific requirements for GSM terminals operating in the 900 MHz and 1800 MHz frequency range |  |
| EN 301 489-17 | V1.2.1 | specific requirements for short-range devices; i.e. Bluetooth functionality |  |
| EN 55020 | 2002 | FM Radio receiver – Immunity aspects |  |
| EN 55013 | 2001 | Fm Radio receiver – Emission aspects |  |
| EN 55022 A1:2000 A2:2003 | 1998 2000 2003 | Information Technology Equipment – radio disturbance characteristics (e.g. for A-GPS) |  |

Updated information’s on Harmonized Standards for EMC can always be found at the European Commissions website:

For US, the FCC, who is in charge of radio spectrum management, handles the EMC aspect. The corresponding technical requirements are defined in FCC Part 2, Part 15, Part 22 and Part 24. Separate EMC testing and certification is mandatory in US.

The FCC EMC requirements only concern emission. No immunity requirements are applied in US.

HAC – “Hearing Aid Compatibility”: With order from August 2003, the FCC requires major handset manufactures to bring at least two phones supporting HAC on the US market. The transition period will exceed in August 2005 for EMC requirements (reduced emissions,  ANSI C63.19 rating “U3”) and one year later for additional telecoil-coupling ("UT3").

The full FCC report and order is available on the FCC website: FCC-03-168A1.DOC

### Radio spectrum usage aspect

Directive 1999/5/EEC identifies in Article 3.2 radio spectrum usage as an essential requirement because radio spectrum is a limited resource.

Under previous European legislation under Directive 91/263/EEC not only radio spectrum usage requirements were defined. At that time, the regulatory requirements for GSM terminals included signalling aspects, acoustics and SIM/ME interface. The corresponding technical requirements were defined in Common Technical Regulations (CTRs) with references to GSM 11.10. Approximately 400+ test cases were included for compliance to the CTRs and/or TBRs for GSM terminals.

Under Directive 1999/5/EEC the only applicable requirements in EC concerns effective radio spectrum usage.

#### Harmonized standards for GSM article 3.2

| Relevant harmonized standard for GSM article 3.2 | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 301 511 | V7.0.1 WKI_ID=9225 | specific requirements for GSM terminals operating in the 900 MHz and 1800 MHz frequency range pointing to GSM 11.10 |  |
| EN 301 511 | V9.0.2 (in the approval process) WKI_ID=13302 | Includes EGPRS and pointing to TS 151 010, is planned to be listed in the OJ 2003-07-30(not in the OJ up to now) |  |

#### Harmonized standards article 3.2 for Bluetooth

| Relevant harmonized standard for Bluetooth for article 3.2 | Version | Purpose | Date of cessation of presumption of conformity |
| --- | --- | --- | --- |
| EN 300 328-2 | V1.2.1 | Short range device ISM 2.4 GHz |  |
| EN 300 440-2 | V1.1.1 | Short Range 1-40GHz Conformity to EN 300 440-1 and EN 300 440-2 is not mandatory for annex III but helps to prevent extra declarations for France ??? |  |

For US, the FCC handles radio spectrum aspects. See EMC section above.

### Special requirements

#### European Community

Directive 1999/5/EEC provides in Article 3.3 several options to include further special requirements concerning:

- Network inter-operability / support for certain network interface connecting points.

- Avoiding harm to the network and degradation of service.

- Safeguards to ensure personal data and privacy of users and subscribers.

- Support of certain features to avoid fraud.

- Support of certain features ensuring access to emergency features.

- Support of certain features to facilitate terminal usage by users with a disability.

By January 2003, no such requirements are defined under Article 3.3 in EC but inclusion of IMEI security is under discussion.

#### USA

##### TTY (if included in product feature list)

For US, there is a special requirement under Telecommunications Act Section 255 regarding terminal usage by users with a disability. Deaf, hard of hearing, and speech-impaired persons have been using specific Text Telephone (referred to as TTY in North America) equipment in the fixed network for many years to transmit text and speech through ordinary speech traffic channels. Modern digital cellular systems, however, do not provide satisfactory character error rates for text transmitted in the speech channel with the traditional modulation developed for the fixed network. The FCC under the US Government has required an urgent solution for all emergency (911) calls for one specific text telephone protocol called Baudot Code.

The standardisation of Cellular Text Telephone Modem takes place within 3GPP. The corresponding technical requirements are defined in

- 3GPP TS 26.226: General description

- 3GPP TS 26.230: Transmitter Bit Exact C-Code

- 3GPP TS 26.231: Minimum Performance Requirements

These special requirements are mandatory as of January 1, 2002. However, a manufacturer fulfils these special requirements if the manufacturer has at least one product supporting TTY on the US market.

##### 5-key nob

To ease blind people’s use of cellular telephones, a special nob shall be located on the 5-key or just below the 5-key. The nob shall be easily detectable and durable.

For US, the FCC has adopted revisions to USA FCC Wireless E911 Rules according to 3GPP TS 22.071 Annex B. Location of Emergency Calls can be provided by different means. This area is subject to further standardisation.

#### IMEI Security

For PTCRB the manufacturer has to confirm IMEI security by “a letter certifying sound engineering practices in securing the IMEI in accordance with 3GPP TS 22.016, section 2. All terminals must comply by June 1st, 2002”.

For GCF IMEI security as specified in the relevant core specification is also required.

### Inter-operability

In EC, inter-operability requirements are removed from the regulatory requirements. See above for special requirements under Article 3.3 of Directive 1999/5/EEC.

By contrast, this is not the case for US. GSMNA have set up the PCS1900 Type Certification Review Board (PCTRB) and entitled PCTRB to undertake type approval matters including inter-operability aspects for US.

Technical and procedural requirements are given in Permanent Reference Document NATWG.03. PTCRB type certification is based on GSM Specifications with modifications per North American Standards and additional requirements from FCC rules, or any other government agency that may have jurisdiction and or competence in the matter.

In addition, also CTIA operates the CTIA GSM-1900 Terminal Unit Certification scheme.

Both CTIA and PCTRB schemes are considered regulatory in US.

## Voluntary requirements

### Global Certification Forum - Inter-operability

Under earlier EU-legislation, GSM terminals were subject to a substantial amount of regulatory testing and certification or type approval before market launch following the provisions of Directive 91/263/EEC.

The amount of tests provided a high degree of confidence that relatively few and small inter-operability problems would occur between handsets and infrastructure of live networks.

Following the implementation of Directive 1999/5/EEC the amount of regulatory testing is reduced to radio spectrum aspects.

Consequently, type approval does not provide the same level of confidence. In particular, inter-operability aspects are not covered by regulatory testing under Directive 1999/5/EEC.

On this background, the GSM Certification Forum (GCF) was established as a voluntary certification scheme for GSM terminals in December 1999 after some years discussions between network operators and handset manufacturers mainly concerning the organisation of GCF, the technical requirements, the relations to authorities and corresponding regulatory requirements etc.

Since January 2002 GCF covers not only GSM terminals, but also UMTS terminals. Consequently GCF was renamed as Global Certification Forum. The scope of GCF is basically to provide a common set of Certification Criterias to re-establish the confidence needed from both sides of the mobile industry, that inter-operability problems can be avoided. During mutual recognition of manufacturer’s declarations, duplicate testing can be avoided.

A more detailed description of the principles and procedures of GCF is given in

- GCF-PD: Principles Document

- GCF-AP: Application Procedures

- GCF-AD: Abbreviations and Definitions

- GCF-OP: Operating Procedures

- GCF-OB: Operating Budget

The corresponding technical requirements are defined in document

- GCF-CC: Certification Criterias

The valid versions of the above mentioned documents is available at

http://gcf.gsm.org

Note that a user login and a password is required to access the GCF web site.

Document CC incorporates basically the same technical requirements as were contained in the now expired CTRs. However, new tests are continuously added as new features and services are introduced in the GSM system the roll-out is expected to happen at the end of 2002. Compliance can be demonstrated by conducting tests in test laboratories using validated test cases running on commercially available test equipment.

In addition there is a new requirement for Field Trials. Field Trials are performed on live networks and intended to cover dynamic scenarios, which can not be performed in laboratory environments. Field Trials shall be performed in at least five network configurations per supported band, representing infrastructure implementations from all major suppliers.

Another new requirement after June 2002 from GCF is a declaration concerning the IMEI security.

Following shall the manufacturer declare:

He has taken necessary and sufficient steps to ensure that any individual or organisation cannot economically change the IMEI after the ME’s  final production process.

And that the IMEI resists tampering, i.e. manipulation and change, by any means (e.g. physical, electrical and software).

Within GCF, manufacturers are required to fill in a list of Core Specs (e.g. 3GPP TS 45.005 for RF) applied to the design. Moreover, the exact version of Core Specs shall be listed for a range of Core Specs. The GCF Steering Group defines minimum requirements to Versions of Standards. By default, the terminal design shall therefore be made based on Core Specs.

### Bluetooth functionality

Bluetooth is standardized by Bluetooth Special Interest Group (SIG). SIG also specified the Bluetooth Qualification Process. Bluetooth functionality of terminals shall be qualified following the provisions of Bluetooth Qualification Program Reference Document (Feb. 2000: Version 0.9).

The PRD specifies the framework of the qualification process. The details, such as the forms, templates, and checklists used to support this process, are provided in companion documents, which are referred to by the PRD, and/or are accessible using hyperlinks imbedded inside the PRD. Apart from these resources, supplemental information is available at the Bluetooth Web Site, including answers to frequently asked questions (FAQ).

A number of Profiles exists for various Bluetooth functionalities. Specific test suites apply to Profiles.

### WAP functionality

WAP is standardised by Wireless Application Protocol Forum. Testing and Certification of WAP devices is undertaken by The Open Group (TOG).

TOG is not a member of WAP Forum, but contracted to develop and maintain application layer test suites and to operate the certification programme on behalf of WAP Forum as Certification Authority.

### IrDA functionality

IrDA is an international organization that creates and promotes interoperable, low cost infrared data interconnection standards that support a walk-up, point-to-point user model. The standards support a broad range of appliances, computing and communications devices.

The IrDA specifications are under restructuring into Profiles as for Bluetooth. Currently (Febr 2001), only one Profile is defined, namely Point and Shoot profile.

More detailed information on IrDA is available at www.irda.org

### USB functionality

The Universal Serial Bus is specified by USB Implementors Forum. Corresponding test requirements are defined in USB Compliance Program. More detailed information on USB is available at www.usb.org

### Network Selection from SIM Card Preferred List

This requirement solely applies to TIM, Italy.

The purpose of the test is to check, that a Ph2 GSM terminal equipped with a non-empty SIM card Preferred List, selects a network in an International Roaming scenario, according to Core specification GSM 03.22.

## Marking and Labeling

A range of mandatory marking and labelling requirements are hereby defined.

Markings shall be visible for inspection without the need for tools.

The size of the IMEI label shall be large enough for all needed signs, numbers, etc. (e.g. IMEI, CE sign, FCC ID).

| Requirement basis | Corresponding marking |
| --- | --- |
| Directive 1999/5/EEC for EC | 99/05/EC article 12 and annex VII: Marking required on mobile: CE 0168 (if opinion of BABT as notified body is used) For Bluetooth with higher power classes, additionally the Alert symbol (!) (exclamation mark in a circle) (e.g. CE 0168 (!)) Requirements from 99/05/EC annex VII: The CE marking must have a height of at least 5 mm except where this is not possible on account of the nature of the apparatus. 3. The CE marking must be affixed to the product or to its data plate. Additionally it must be affixed to the packaging, if any, and to the accompanying documents. 4. The CE marking must be affixed visibly, legibly and indelibly. 5. The equipment class identifier must take a form to be decided by the Commission in accordance with the procedure laid down in Article 14. Where appropriate it must include an element intended to provide information to the user that the apparatus makes use of radio frequency bands where their use is not harmonised throughout the Community. It must have the same height as the initials 'CE'. Notified body numbers and the equipment class identifier, (e.g. !) being part of the CE marking need to be put on the packaging and in the manual as follows:. 99/05/EC Article 6.3 Member States shall ensure that the manufacturer or the person responsible for placing the apparatus on the market provides information for the user on the intended use of the apparatus, together with the declaration of conformity to the essential requirements. Where it concerns radio equipment, such information shall be sufficient to identify on the packaging and the instructions for use of the apparatus the Member States or the geographical area within a Member State where the equipment is intended to be used and shall alert the user by the marking on the apparatus referred to in Annex VII, paragraph 5, to potential restrictions or requirements for authorisation of use of the radio equipment in certain Member States. Where it concerns telecommunications terminal equipment, such information shall be sufficient to identify interfaces of the public telecommunications networks to which the equipment is intended to be connected. For all apparatus such information shall be prominently displayed. |
|  | Siemens Type Designation |
| R&TTE Directive 1999/5/EEC for EC: Article 12 (4) | Apparatus shall be identified by the manufacturer by means of type, batch and/or serial numbers and by the name of the manufacturer or the person responsible for placing the apparatus on the market: Product Name |
|  | Serial Number / IMEI |
|  | Manufacturer Name |
| EN 60 950 and other safety standards | Max. rated voltage and current consumption |
| CTIA | CTIA logo (CTIA Certification not mandatory) |
| FCC | FCC-ID number Country of Origin |
| IC | Canada ID |
| Additional | Siemens stock number IMEI in bar-code format and writing |
| CCC | CCC label is mandantory for China (MII) |

### China / APAC

#### General APAC & China requirements

The APAC variants must comply with or have declaration on:

- Declaration of Conformity (Siemens self-declaration) for

- EN301 511 (“Spectrum Usage” for GSM)

- EMC: EN 301 489- 1 (V1.3.1 valid until 30.11.05 or V1.4.1)

- EN 301-489-7 (V1.1.1 valid until 30.11.2005 or V1.2.1)

- Safety: EN 60950

- BABT Certificate for R&TTE Directive 1999/5/EC (Opinion of Notified Body)

- Test reports for above norms (GSM Report from e.g. Cetecom, 7 layers etc.)

- Declaration of Conformity (Self-declaration document by Siemens) which declares the charger and adapter (3-pin UK plug) for the Siemens product complies with the Safety regulation BS1363

- Commercial Product photographs including front shot, back, PCB board and the “commercial” IMEI number. (Photos has to be taken by a digital camera)

- CB Test Report (by International Electrotechnical Commission (IEC)) or Test Report from other approved authority

- ICNIRP report/certificate meeting International SAR standard the GPRS IOT reports against 5 different Networks. ( Ericsson, Nortel and Nokia infrastructure)

- Australian safety approval certification of charger (i.e. copy of certificate issued by Australian regulatory to manufacturer) - see attached pdf file.

#### Bluetooth Products

- ETSI Standard EN300 328 or EN300 836

#### Additional APAC & China Requirements

Additionally APAC variants must comply with or have declaration on:

- Charger type approval must be done by charger supplier for Singapore - http://www.safety.org.sg

- Australia additional requirement where charger pins must be partially isolated - http://www.aeema.asn.au/docs/insulated_pins.pdf

Additionally China variants must comply with or have declaration on:

- NA test sample drawing:
R&D / PM in HQ send an IMEI list of 500 samples for MII Lab to choose from freely first, then R&D/PM send samples with those with IMEI chosen by the MII Lab to China for EMC testing, RF testing and MT net testing. Samples are not allowed to be replaced and modified during NA testing.

- OEM product:
Must be clearly stated any specified relationship between manufacturer, ODM vendor and the owner of the product in a product investigation form

- Test failure:
If the units failed in testing, there is only one chance to modify, and retest should be conducted after 1 month with new samples re-drawn by MII lab. If the test is still failed, then NA application stop and the product can be re-applied for NA half a year later!

### Variant Overview

| Requirement | X75-Products (900/1800/1900) | X75-Products China | X76-Products (850/1800/1900) |
| --- | --- | --- | --- |
| R&TTE | X | X | X |
| FCC | X | X | X |
| PTCRB (approved by CTIA) | X (1900 band) | X (1900 band) | X (all bands) |
| GCF | X | X |  |
| CCC (MII China) |  | X |  |
| TTY |  |  | X |
| HAC |  |  |  |
| IMEI Security Declaration | X | X | X |
| 5-key nob | X | X | X |

TTY and Hearing Aid Compatibility (HAC) are not supported by this product.

#### Variant and Product Synergy

As the technical realization of X75- and X76-Products allows synergies, with regards to type approval and certification testing, a reduced depth in testing might be applied

#### EMEA Variant (900/1800/1900)

The 1900 test cases are performed in a PTCRB accredited test lab. 900/1800 tests are planned to be performed in parallel as in-house tests to reduce cost and time-to-market.

#### NAM Variant (850/1800/1900)

All test cases in all three bands are planned to be performed externally in a PTCRB certified test lab.

### Additional System Test requirements

#### Interfaces

##### 50 Ohm RF Interface

The phone needs a 50 Ohm RF interface for approval testing. The interface can be realized by a coax test connector, which is accessible and mechanically suitable for approval testing or by an RF cable with same properties (SL55 solution). The related GSM specification is 3GPP TS 51.010-1 Annex A.

##### BFC/RCCP Interface

For remote control during type approval testing (FTA “Auto mode”), the mobile supports BFB, BFC (or equivalent) and AT-C/RCCP commands via serial interface for basic call procedures and GPRS testing.

If the BFB library is replaced by BFC, in principal the same functionality (remote control) for the mobile has to be provided.

For the product these functionality has to be provided in a way which can be implemented in the Test machine setup of System test.

### New type approval and certification requirements

Within the certification schemes GCF and PTCRB new approval/certification requirements came up, which will be applicable for the X75 generation. Some of those are driven by the so called Application enabler group, others are driven by PTCRB/CTIA (OTAP).

Those requirements are:

| Requirements | Scheme | Type of testing | Responsibility | Status |
| --- | --- | --- | --- | --- |
| OTAP (Over The Air Performance) | PTCRB/CTIA | Emission testing | EMV | mandatory |
| MMS | PTCRB / GCF | IOT testing | ST | mandatory |
|  |  | Conformance testing |  | mandatory |
| EMS | PTCRB / GCF | IOT | ST | See Note |
|  |  | Conformance testing |  | See Note |
| POC | PTCRB / GCF | IOT | ST | See Note |
|  |  | Conformance testing |  | See Note |

Note:

Additional requirements might be defined by a so called application enabler group. Those additional requirements might become mandatory for X75-and X76-products, as those products will go through type approval and certification mid of 2005.

# Milestones

X75 Minos /  EMEA & APAC product

|  | Milestone | Date |
| --- | --- | --- |
| M0 | Product(s) within platforms are defined | (20.08.04 / CW34) |
| S0 HW | Requirement Release – Design freeze | (15.09.04 / CW38) |
| S0 SW | Requirement Release – Design freeze | (30.11.04 / CW49) |
| M1 | Implementation Release | (30.11.04 / CW49) |
| S15 | A1 Module Operable in Basic Function | (03.12.04 / CW49) |
| S2 | Tested Functional Sample (B1) | (02.03.05 / CW09) |
| C3 SW components |  | (29.04.05 / CW17) |
| S25 TA SW freeze |  | (13.05.05 / CW19) |
| S25 SW Freeze |  | (02.06.05 / CW22) |
| S25 | Tested Prototype Device B1+ | (24.06.05 / CW25) |
| AS | Approval Start | 27.06.05 / CW26) |
| M2 | Sales Release | (07.07.05 / CW27) |
| S3 HW ( | Pre-Series Release B2 for HW | 08.07.05 / CW27) |
| Start Pilot Run* | Release for Unrestricted Series Delivery | (25.07.05 / CW30)* |
| MKA | Maturity of Key Applications | (05.08.05 / CW31) |
| PS | Production Start | (05.08.05 / CW31) |
| S3 SW | Pre-Series Release B2 | (26.08.05 / CW34) |
| DS SW available |  | (26.08.05 / CW34) |
| DS | Delivery Start | (15.09.05 / CW37) |
| S4 HW | Series Production Baseline HW | (14.10.05 / CW41) |
| S4 SW | Series Production Baseline | (27.01.2006) |
| M3 | Release for Unrestricted Series Delivery | (01.03.2006) |

* The start of the pilot series is planned for CW30 2005 and it is aimed to produce 135 thousand units of the standard variant

# Miscellaneous

## Technical Risks

For major risk assessment see following separate documents within the M1 folder.

M1 enclosed documents Minos\PD_risk_assessment_Minos_Final.xls

M1 enclosed documents Minos\SW-Risk-assessment 001_X75_SRMP.xls

M1 enclosed documents Minos\platform_risks30-11-2004.ppt

### EA

- Antenna optimized only for European market (2.7 bands)  accepted by PM

- Humming problem could appear with new speaker (no shielding).

- The antenna might be interfered by the speaker or the speaker is absorbing power or causes parasitic resonances. Due to material differences this problem could be increased with the 2nd source (so far no samples from Boujon available)).

- Reduced ESD and antenna performance, high SAR values, self interferer could appear if the Gore-cushion contacts (3x) are not reliable.

- MMC-/SIM-Reader  if the shielding is not sufficient self interferer and/or card error can appear.

- Display shielding could be not sufficient  risk of self interferer and display flickering.

- Camera shielding could be not sufficient  risk of self interferer during view find ore video mode

- Soldering problem of RF/BB/BT shielding frames (same problem in the 65 generation)  risk of self interferer, EMI and humming.

- Soldering problem with antenna reference sheet (integrated with display frame)  antenna performance could be decreased.

- Megapixel camera with gimmick chip   self interferer and EMS problems could appear.

- Bluetooth antenna  new antenna concept.

- Nolato as antenna supplier  Nolato has no experiences with antennas. The complete RF measurement set-up must be implemented at Nolato.

### ST2

- 'S25 TA'-SW freeze (13.05.05) planned 6 weeks before AS (27.06.05) will not or not fully support all type approval relevant features which may result to a delay of the AS

- As milestone S3 HW (08.07.05) is later than AS (27.06.05), type approval relevant HW changes after AS will result in a complete re-start of the TA tests

- Failure analysis not possible during IOT as we have actually no commitment by the SW supporting a minimum of 460 kBit/s for tracing on the 2nd interface (serial interface or USB)

### BB

- Gimmick chip from ATI  is a new component: ES samples may come late, feature set may be not secured

- SG2 ES2 is available very late (01/05), additional redesign necessary?

- lead-time of display and camera is 4 months; SW for B1 must be mature enough to release these components

### NPI32

- lead free needs to be qualified, process evaluation is not finalized

- LCD frame size is biggest ever soldered

- front cover glueing and screwing is a new process

### NPI31

- Delivery dates and content of Mob-SW are not fixed now (floating process), based on that fact the testing SW (dates & content are direct linked to the Mob-SW) is also floating. Dates can not committed

- cycle time will be increased from 16s (Target) up to 20s (Target) because of EDGE testing

### MD

- Lead free soldering reliability is not as good as leaded soldering:

Thicker PCB will improve the reliability but increases the costs. Necessity of this measure will be assessed with B1.

- Digital coating of the display lens:

Technology is new and not approved at Siemens. Up to now no series equipment is available at the Supplier (Nolato), B2 samples will be 1st parts produced under series conditions.

Fall back solution is a display lens w/o coating (no additional efforts necessary). IMD-technology rejected by PM due to significant increase of the costs.

- Readability and illumination of artwork (especially for Arabic, Thai and BoPoMoFo) critical due to limited printing area on the small keypad caps.

### SW

- The HMI Spec is not finalized.

- The following major sections are not finalized till M1 and will be finsihed till:

Media player: 			26.11.04

Contacts: 			03.12.04

PoC: 				17.12.04

Instant messaging: 		03.12.04

Messaging: 			14.01.05

Organizer, Call Records: 	03.12.04

## Packaging/User Manual

The packaging will be printed in several colours and consists of the packaging itself and an insert. Packaging will be printed with the CE symbol and for customers in Germany with the “green point“ in the selected colour. It is intended to do the packaging in DIN A6 format (A1 standardlasche). The carton will contain a folding base which is self-erecting.

The supply package shall contain:

- Packaging for the phone

- Siemens CX75 phone including battery cover and signs/labels

- Battery pack

- Standard charging unit

- User Manual (up to 4 per supply unit)

- CD

- AD Leporello (tbd.)

- Stereo headset

- RS-MMC tray

## Economic Product Plan

The EPP (WPP) can be viewed by contacting Carlos Schmidt, ICM MP UC CS

Conformity to this requirement is not a must but helps to prevent extra declarations for France. Approval for Canada (Industry Canada, IC) will be done jointly with FCC approval.

Bluetooth certification mandatory if Bluetooth is activated