C68 Obsidian

M1 statement EMC and Antenna

Revision Information

| Created: | Revision | Name | Comments |
| --- | --- | --- | --- |
| 27.10.05 | 0.1 | Arne Bisgaard Kristensen | Initial revision in PVCS |
| Modified |  |  |  |
| 06.02.2006 | 1.0 | Arne Bisgaard Kristensen | Prepared for Obsidian |

Contents

1	Introduction	1

2	Results and open issues M1	1

3	Conclusion	4

# Introduction

This document is the basis for the M1 milestone declaration. Section 2 contains the M0 checklist according to the MEP 7.01 and section 3 gives a statement to the milestone.

M1 Implementation Release

# Results and open issues M1

| - Test plan for the system coordinated between the affected partners | Same as item 7. |
| --- | --- |
| - EMC concept coordinated for the product/system | System FMEA performed. EMC concept implemented as defined in performance description. |
| - Antenna defined precisely, manufacturer defined | Antenna manufacturer is Lumberg. Antenna is defined precisely. |
| - EMC SW defined with SW | Required EMC SW features defined. |
| - Technical Terms of Delivery (TTD) of customized components and modules created and reviewed. Exceptions latest signed by S3 HW defined | Draft version from C65 created. Antenna drawing will be added when new MD CAD system is up running. Antenna test PCB drawing will be added later when available. Test limits are TBD until antenna statistic is available from supplier. |
| - A1 tape out completed | A1 is taped out and available. |
| - Test plan (Master Actions Plan) according to product functionality accessories available including AD EA work split | Obsidian MAP plan available. AD work split agreed. |
| - Product concept validation has taken place for all areas (electronics, mechanics, optics, acoustics etc.) incl. worst case considerations | Proven concepts are implemented. |
| - Production test requirements and needs for reference samples defined and agreed with production (PT) | Preliminary production test limits provided. Antenna reference samples to be submitted to Andre Splawski (Production Test) until B1+ production run. Latest at B2 run. |
| - Hardware FMEAs done. Measures (e.g. RDP project) agreed with PQA | No EMC and Antenna hardware FMEA required. |
| - Detailed Primavera project plan up to M3/S4 is available, showing dedicated resources by name and on department level for external consultants | Primavera plan completed. |
| - Complete project plan reviewed and agreed | Project plan reviewed and agreed. |
| - Status of product relevant pre-/component developments reviewed. Risk analysis done and measures agreed, if targets are not reached | No pre development or components relevant. |
| - Accessories to be fully supported by product are defined and agreed with Accessory Devices | AD portfolio of January 26 2006 is valid. |
| - Product technical specification reviewed | Product technical specification input given. |
| - Time schedule for provision of devices for conformity assessment and certification agreed with System Test | Included in Primavera time schedule. |
| - PSQA plan built up and provided to PQA | PSQA plan (MAP) is provided to PQA. |

Ad 15)

Target at M1:

Target antenna performance is to meet the FBT requirements.

| The FBT requirements are: (minimum per Channel) | TX | RX |
| --- | --- | --- |
| EGSM | 28dBm | -101dBm |
| PCN | 25dBm | -101dBm |
| PCS | 25dBm | -101dBm |

A head SAR value of 0.8W/kg (1g) is BenQ target.

The International SAR value limits as shown in the table below will be satisfied.

| Limit | Head/ Body worn | Based on Standard | Countries |
| --- | --- | --- | --- |
| 1,6W/kg (1g) | Head | IEEE | Canada, S. Korea, Taiwan, USA |
| 1,6W/kg (1g) | Body worn | IEEE | Canada, USA |
| 2,0W/kg (10g) | Head | ICNIRP | EMEA, APAC, Australia |
| 2,0W/kg (10g) | Body worn | ICNIRP, ARPANSA | EMEA, APAC, LAM, Australia |

Status at M1 milestone:

Preliminary measurements on FDM parts without any SAR reduction measures show following performance following:

Head SAR 900 band: 1.9W/kg (1g) / 1.2 W/kg (10g) @ 28.9dBm radiated power

Head SAR 1800 band: 0.6W/kg (1g) / 0.3 W/kg (10g) @ 26.8dBm radiated power

Head SAR 1900 band: 0.6W/kg (1g) 0.3 W/kg (10g) @ 25.0dBm radiated power

Body worn SAR 0mm distance 1 TX 900 band: 1.8W/kg (1g) / 1.2 W/kg (10g)

Body worn SAR 0mm distance 1 TX 1800 band: 3.0W/kg (1g) / 1.4 W/kg (10g)

Body worn SAR 0mm distance 1 TX 1900 band: 2.6W/kg (1g) / 1.3 W/kg (10g)

Conclusion:

Head SAR in 900 band has to be reduced to below 1,0 W/kg (1g) by various SAR reduction means (power reduction, SAR ring, virtual extension of PCB etc).

Body worn SAR meets the 0 mm requirement for 1 TX in EMEA region (10g value). For the 1g region the body worn requirement is fulfilled by increasing the distance to the body up to 25mm.

# Conclusion

EA can declare M1 based on above results.