95 Obsidian

M0 statement EMC and Antenna

Revision Information

| Created: | Revision | Name | Comments |
| --- | --- | --- | --- |
| 27.10.05 | 0.1 | Arne Bisgaard Kristensen | Initial revision in PVCS |
| Modified |  |  |  |
| 27.10.2005 | 0.2 | Arne Bisgaard Kristensen | MEP7.01 inserted |

Contents

1	Introduction	3

2	Results and open issues M0	3

3	Conclusion	5

# Introduction

This document is the basis for the M0 milestone declaration. Section 2 contains the M0 checklist according to the MEP 7.01 and section 3 gives a statement to the milestone.

M0 Product is defined

# Results and open issues M0

| - Project leader HW (PL HW)/ part project leader (PPL) EMC/Antenna has been designated by the department head. | Arne Bisgaard Kristensen is EA PPL. |
| --- | --- |
| - Detailed Primavera project plan from M0 to M1 showing dedicated resources by name is available | Done |
| - Rough Primavera project plan from M1 to M3/S4 showing dedicated resources on department level is available | Done. Assigned resources in PMC are transferred with hours to Primavera. |
| - Definition of power class (e.g. GPRS, UMTS) for selection of holster / belt clip for SAR reasons done. | EGSM 900 power class 4 (33dBm), GSM1800/1900 power class 1 (30dBm) GPRS class 10b (2Tx/4Rx) 8-PSK 900/1800/1900 power class E2 (27/26/26dBm) |
| - Antenna Mockup 1 (rapid prototype upper and lower casing, Prototype simulating the PCB outline (main dimensions) including preliminary shielding boxes) for EA assessment delivered by MD (M0 – 9 weeks) | Minos sample used. Similar antenna carrier and volume will be used in Obsidian. |
| - Antenna Mockup 2 (rapid prototype upper and lower casing, Prototype simulating the PCB outline (main dimensions) including preliminary shielding boxes and antenna related contact concepts (e.g. slider contacting)) for EA assessment delivered by MD (M0 – 5 weeks) | Minos sample used. Feed pin contact position is common for Minos and Obsidian. |
| - The antenna concept and volume is fixed (90 %). SAR aspects considered. | GSM antenna: Dual Patch Planar Inverted L-Antenna will be implemented due to low building height (6.65 mm) and center off-set of camera. Conductive SAR painting on inside of front cover with contact points to ground will be used for SAR reduction. BT antenna: Inverted F-antenna. Reuse from Polaris or a new will be stamped. |
| - Assembly concept for mechanical design dependent components or modules coordinated and agreed with MD, dHW and RF (e.g. display, camera, acoustics, vibra, battery, card readers, IrDa, LED flash, connectors, etc.) | Done. |
| - Product requirements as specified in the M0 document achievable with available technologies / know-how by the specified S3 HW deadline | Yes – only new issue is a Stereo FM radio headset to be done by “AD” |
| - Product relevant pre-/component developments defined. Time schedules and deliveries aligned to product development schedule | Antenna schedule is aligned with mechanical deliverables. |
| - Platform released for start of product integration available i.e. platform master schematics, porting guide etc. | Yes. |
| - Product related specifications for EMC are defined with the affected specialist departments in respect of layout, screening, SW, construction concept, accessories | Yes and GQR draft version 4_19b is used. |
| - EMC SW defined with SW, necessary test tools and testing concept agreed with SW; availability of tools committed by SW | Standard request issued to SW PL. Request under investigation. A meeting will be held in CW 48 to agree on SW deliveries until DS. |
| - Product related deviations from the platform defined and agreed with involved parties i.e. SW, MD etc.. | No deviation from platform. |
| - Product related Technical Terms of Delivery (TTD) are available (This checkpoint shall apply only for products which are not derived from platforms developed according to the new Platform Process V 2.0 and the corresponding platform checklist MRPlatD) | Draft GSM antenna TDD taken from C65. |

# Conclusion

EA recommend to declare M0 based on above open items.