- Concept Description M0

Product/Version/Country:					GPRS class 10

Polaris (EMEA, APAC)

Document Title:							input.doc

Document Status:						Draft

Document Version:						0.1

Name	Department	Signature, Date

Thomas Neumann	ICM MP PD PGG

Distribution:

| Name | Department |
| --- | --- |
| Alexandra Bernhard | MD CCQ SLI |
| Alfred Hauenstein | MD PBM PG85 |
| Antje Hartung | MD LCM |
| Detlev Manser | MD LCM |
| Dr. Markus Jaeger | MD PD SW |
| Eric Hachmann | MD PBM PG85 |
| Falk Mletzko | MD SM UE |
| Felicitas Reuper | MD PBM PM M |
| Frank Schoger | MD PD MD |
| Guenter Jost | MD PD HW |
| Heiko Kuehling | MD CCQ QM PL PP |
| Juergen Schmitz | MD PD PGG |
| Klaus Fischer | MD AD PM |
| Leonhardt Waigel | MD PD ST |
| Michael Schmitz | MD PD ST |
| Oliver Nam | MD S PS |
| Peter Nolte | MD PD HW |
| Roman Muehlleitner | MD PD HW |
| Rudolfo Lara | MD PBM PM P |
| Sebastian Siebinger | MD PD HW |
| Sebastian Stier | MD PBM PSO |
| Soja Rappl | SD D PE MD |
| Stefan Daniel Fritzel | MD PD HW |
| Thomas Miller | MD CCQ GRM R |
| Thorsten Keller | MD PBM BA |
| Thorsten Langer | MD AD RD PL |
| Zlatko Kirin | MD CCQ QM PL PP |

Document History:

| Version | Date | Name | Amendment |
| --- | --- | --- | --- |
| 0.1 | 17.02.05 | Thomas Neumann | Initial Version |

Table of Contents

Document History:	3

Document History:	3

Table of Contents	4

Table of Figures	6

References	7

Project specific references	7

General References	8

Supplementary Documents for PCS 1900	9

1	General	10

1.1	Description	10

1.2	Time Line	10

1.2.1	Milestones	10

1.2.2	Prototype Availability	10

1.2.3	Reference Samples	10

1.3	Technical Risks	11

1.4	Sourcing	11

2	Mechanics	12

2.1	Unit Description Polaris	12

2.2	Interfaces X75 Virgo to accessories	12

2.3	Key-Data	12

2.4	Finishing	12

2.4.1	Front Cover	12

2.4.2	Battery Cover	12

2.4.3	Keypad	12

2.5	Main PCB	13

2.6	MMI PCB	13

3	Electronic	14

3.1	Radio part	14

3.1.1	Description	14

3.1.2	Compliance	14

3.2	Digital Hardware and Electro Acoustic	14

3.2.1	Overview	14

3.2.2	Digital Base-Band	14

3.2.3	External memory	15

3.2.4	Display Module	15

3.2.5	MMI	16

3.2.6	SIM	17

3.2.7	Bluetooth	17

3.2.8	Vibration Motor	17

3.2.9	Acoustical Transducer	17

3.2.10	Power Supply, Battery, Charging	17

3.2.11	Antenna	18

4	Software	19

4.1	Software Feature List	19

4.2	Requirements to SW	19

5	Accessories	20

5.1	Portfolio	20

5.2	Feature List	20

6	System Test	21

6.1	Abbreviations and Definitions	21

6.2	Approvals	21

Table of Figures

Error! No table of figures entries found.

References

Project specific references

| No. | File | Revision | Issued |
| --- | --- | --- | --- |
| 1 | Polaris Product Brief | V0.6 | 09/02/05 |
| 2 | RMEA M0 |  | 02/16/05 |
| 3 | Polaris Rough Planning M0 |  | 02/07/05 |
| 4 | Polaris Product Profile |  | 02/05/16 |

General References

- GSM Recommendation Phase 2 / Phase2+ (GSM51.010)

- GSM 11.10-1 release 1999 version 8.30 (RF part)

- ETSI GSM Phase 2 / Phase 2+ (TR 101.186 V6.0.0) TR Technical Standards (incl. Multi-band, except .PRS/HSCSD)

- ETS 300 342-1 EMC for Cellular Telephones (GSM 900 MHz and DCS 1800 MHz)

- EN 55022 Limits and Measuring Methods for Radio Disturbances

- CE as per Directive 89/336/EWG

- EN 50361 Basic standard for the measurement of Specific Absorption Rate related to human exposure to electromagnetic fields from mobile telephones (300 MHz - 3 GHz); German version prEN 50361:2000

- SAR as per Recommendation 1999/519/EG

- CISPR Pub. No. I 16-1

- R&TTE 99/5/EC Guidelines for Telecommunications Terminals (valid as of 2000-04-08) Product Safety as per EN 60950

- Product Safety as per 1999 / 5 / EEC

- DIN EN 50361 Basic standard for the measurement of Specific Absorption Rate related to human exposure to electromagnetic fields from mobile telephones (300 MHz - 3 GHz);

- Cellular Telecommunications Systems, (GSM 900 MHz and DCS 1800 MHz)

- ENV 50166 -2 1995 Human Exposure to Electromagnetic Field High Frequency (10 kHz - 300 GHz)

- DIN VDE 0848, Part 2

- CENELC, SC211/B, WGMTE (2nd draft, Rome, Feb. 1996, 3rd draft, Zurich, April 1996)

- ESD as per ETS 300342-1

- ETS 300 019-1-0 / ETS 300 019-2-7 (Environmental Requirements)

- EN 301 489-01 Electromagnetic compatibility and Radio spectrum Matters (ERM); Electromagnetic Compatibility (EMC) standard for radio equipment and services; Part 1: Common technical requirements

- EN 301 489-07 Electromagnetic compatibility and Radio spectrum Matters (ERM); Electromagnetic Compatibility (EMC) standard for radio equipment and services; Part 7: Specific conditions for mobile and portable radio and ancillary equipment of digital cellular radio telecommunications systems (GSM and DCS)

- ETR 051 (Human Factors - Basic Requirements) and ETR 166 (Evaluation Methods)

- IEC 68 ff. (Environmental Requirements)

- DIN 40050 (Climatic Conditions and Protection Classes)

- DIN 40839 (Line-Connected Disturbance Variables Impacting On-Board Networks of Road Vehicles)

- Low-Voltage Guidelines 73/23 EWG (revised in 93/68 EWG)

- VDA Recommendations

- ISO 7637 (1990) Road Vehicles Parts 1 and 2

- DIN/VDE 0879 Part2, Draft 10/88 (for car accessories)

- Internal Auxiliary Specification for VHF Broadcasting Range (-82dBm) (Siemens internal requirement)

- EN 1811 Reference Testing Procedure for Nickel releasing Products in Prolonged Direct Contact with Human Skin

- Laser Guidelines IEC 60825

- DS/ EN 71-1 Safety of toys - Part 1: Mechanical and physical properties

- DIN EN 22248 Packaging; complete, filled transport packages; vertical impact test by dropping

- SN 47030-1 Molded plastics; thermoplastic molding materials; Siemens-item-numbers

- SN 27650 Testing of Paint Coatings and Similar Coatings; Test for Abrasive Strength

- EN ISO 2233 Packaging. Complete, filled transport packages. Conditioning for testing

- SN 36350-1 Environment Compatible Products; Part 1: Guide for product development

- USB – Specification Revision 1.1

- Testing technical standards for mobile phones (Version 3.1+; 24.04.2003)

- General Quality Requirements (Rev. 3.1, 2002-01-23)

- Hardware Interface Specification R65 V0.1.pdf

- Clip_on_Flash_Specification0V5.pdf (plus modification: photodiode for light intensity measurement)

- DIN VDE 0470 (IP classes - 53/54; German version of EN 60 529)

Supplementary Documents for PCS 1900

- IEEE C95.1-1991 Standard for Safety Values for Electromagnetic Fields

- EIA / TIA 571 Environmental Aspects for Telecommunications Equipment

- EIA / TIA 136, 136A, 136-270B (TDMA)

- Safety Requirements as per CSA 22.2 No.950-95 (UL 1950)

# General

## Description

For more information regarding ID, Feature-Description etc. pls. refer to [1].

## Time Line

The complete Project has been planned with Primavera Project Manager (Rough Planning). For more Details refer to [3].

Following information have been withdrawn from that.

### Milestones

|  | Milestone | Date EMEA | Date APAC |
| --- | --- | --- | --- |
| M0 | Preliminary Project Plan / Start of Packaging Formation | 31.03.05 | 31.03.05 |
| S0 | Requirement Release / Design Decision | 22.04.05 | 22.04.05 |
| M1 | Implementation Release / Product Contract | 13.05.05 | 13.05.05 |
| S15 | A1 Module Operable in Basic Function, 100 % Mechanical Data Released | 29.06.05 | 29.06.05 |
| S2 | Tested Functional Sample (B1) | 24.08.05 | 24.08.05 |
| S25 | Tested Prototype Device (B1+) | 31.10.05 | Tbc |
| S3 HW | Pre-series Release HW based on B2 tested devices / NO SW-S3 | 20.01.06 | Tbc |
| AS | Approval Start | 20.01.06 | Tbc |
| S3 | Pre-series Release | 20.01.06 | Tbc |
| PS | Production Start | 27.01.06 | Tbc |
| DS | Delivery Start | 20.02.06 | Tbc |
| S4 | Series Production Baseline | tbc | Tbc |
| M3 | Release for Unrestricted Series Delivery | tbc | tbc |

### Prototype Availability

|  | Prototype Run | First Devices Date |
| --- | --- | --- |
| A1 | SLA Devices | 07.06.05 |
| B1 | Soft Tool Devices | 25.07.05 |
| B1+ | 1st Hard Tool Devices | 07.10.05 |
| B2 | 2nd Hard Tool Devices | 14.12.05 |

### Reference Samples

|  | Prototype Run | Qty. | Date |
| --- | --- | --- | --- |
| B2 | 2nd Hard Tool Devices for Operator Approval only | 500 | 20.01.06 |

## Technical Risks

Please refer to the [2]

## Sourcing

The List [4] gives an overview about all A-Components and about number of suppliers.

# Mechanics

## Unit Description Polaris

Polaris is an ultra slim brick phone device with enhanced TFT color display.

The front cover is made of full metal, Aluminum.

The Battery cover is designed cover the whole rear cover.

## Interfaces X75 Virgo to accessories

Polaris has mechanical compatible interface to X85 accessories. The I/O-connector (Nano Lumberg) is centered on the bottom side. The RF connection is made via inductive coupling.

## Key-Data

Volume:	60ccm

Weight:	tbc

Length:	107.5mm

Width:	44mm

Thickness:	12.7mm

## Finishing

### Front Cover

The front cover is made of Aluminum. The surface is being brushed and anodized.

### Battery Cover

The Battery Cover is lacquered high glossy or soft touch.

### Keypad

Metal dome foil with Aluminum hard caps.

## Main PCB

Technology: 	6-Layer HDI with FA4 core, thickness 1.1mm, OSP.

Population: 	Double Sided

## MMI PCB

Technology: 	2- or 4- Layer, thickness 0.6mm, Gold Plated + OSP

Population:	Double sided

# Electronic

## Radio part

### Description

The RF Design is close aligned with the X85 Platform RF Design. Key Components are the Transceiver Chip (Renesas Bright 6E), the Power Amplifier (Renesas PF08143B) and the RF Front End (Renesas HWXQ521.

### Compliance

#### Receiver Sensitivity:

The Receiver Sensitivity must comply with the corresponding GSM recommendations in all operating conditions (temperature, battery level…..)

GSM 900:	GMSK: -102dBm (Specification, with fading; static@25°C GMSK:-108.0dBm)

GSM1800: 	GMSK: -102dBm (Specification, with fading; static@25°C GMSK:-107.0dBm)

GSM1900: 	GMSK: -102dBm (Specification, with fading; static@25°C GMSK:-106.5dBm)

#### Transmitter Power:

The RF part is compliant to GSM 3GPP TS51.010-1 V4.9.0 (2002-07). The transmitter output power (reference point is 50 Ohm test connector) is compliant to following power classes:

GSM 900: 	GMSK: power class 4;

GSM1800/GSM1900: 	GMSK: power class 1;

The following values are valid for normal conditions:

GSM 900	GMSK: >31,1dBm (31,6dBm nominal)

GSM1800	GMSK: >29,0dBm (29,5dBm nominal)

GSM1900	GMSK: >29,0dBm (29,5dBm nominal)

## Digital Hardware and Electro Acoustic

### Overview

The HW Design is close aligned with the X75/85 SGold2 Platform HW Design, see. All device specific parts, dedicated to the Polaris, are specified below.

### Digital Base-Band

#### Baseband Processor S-GOLD2 (PMB8876)

Please refer [] for more detailed information.

The S-GOLD2 is the successor of the SGOLDlite and will be used as the baseband solution for low to mid range phones as well as wireless modules.

It will use the ARM 926EJ-S core at 156MHz (208MHz are feasible but will for the moment not be specified for ICM), which is expected to give sufficient performance to the X75 series. A MOVE Companion Processor will be attached. Furthermore the Teaklite DSP will run with up to 138,67 MHz.

### External memory

For more details, please refer to [].

The external memory is a three die stacked device with one Flash for code, one Flash for data and one LP-SDRAM as main memory. The densities are 256Mbit + 256Mbit + 128Mbit.

All memories are sharing the EBU_AHB- BUS to SGold2.

#### Code Flash

It is a non-volatile-, re-programmable- memory (SW-updateable), with a high performance interface. The mobile-SW can be executed directly. The Flash has an unchangeable serial number.

#### Data Flash

Non-volatile re-programmable- memory for saving user data (pictures, sounds, videos etc.)

#### Low Power SDRAM

The SDRAM (Synchronic Dynamic Random Access Memory) is for volatile data.

### Display Module

#### General description

In the mobile phone a display module with an intelligent graphic Liquid Crystal Display (LCD) is used (Hydra Panel with modified plastic and FPC). The display module consists of the following parts and features

- Active Matrix Liquid Crystal Display Panel, transflective, 1.8”, 132x176 dots, 262k colors, negative mode with wide viewing angle technology (ASV)

- display controller mounted on the display glass (COG) which is connected to a FPC inside the module

- Light guide with 4 white LEDs

- Illumination foil stack is implemented in the module on both sides of the single light-guide to hint the specified brightness and homogeneity

- A PCB/FPC with all passive components.

- A plastic frame

- Full contrast adjusted

- Parallel 8-bit interface

- Mechanical design and delivery concept supports automatic and half automatic display assembly at Siemens production line

#### Expected optical performance

| Items | Reflective Mode |  | Transmissive Mode |  | Unit |
| --- | --- | --- | --- | --- | --- |
| Main-Display | Min | Typ | Min | Typ |  |
| Contrast Ratio |  |  |  |  | - |
| Luminance |  |  |  |  | cd/m² |
| Color Gamut Ratio Related to NSTC |  |  |  |  | % |
| Sub-Display |  |  |  |  |  |
| Contrast Ratio |  |  |  |  | - |
| Luminance |  |  |  |  | cd/m² |
| Color Gamut Ratio Related to NSTC |  |  |  |  | % |

#### Available Display modules

| Supplier | Supplier Part Nr. | Controller / Sub controller | Siemens SAP |
| --- | --- | --- | --- |
| Epson |  |  |  |

#### Realization Principles / Boundary Conditions

The typical operation temperatures are: -20°C to +55°C. The reduced functional temperature range is -25°C to +65°C (the contrast will not be according to the specifications, but will still be readable).

#### Contrast and Color Adjustment

For all display modules no contrast adjustment is necessary. The contrast is fixed in the module. It is not possible to change the contrast by the mobile phone software. Also the color adjustment of the display panel is fixed by the supplier.

#### Illumination

The 4 serial LEDs for the display are supplied by one constant current source, to ensure the same brightness and color of the white backlight.

### MMI

#### Keyboard

##### General description

The Keypad matrix consists of the keys "0" to "9", "*", "#", two soft keys, a 5-way-navy-key, a "SEND"-, an "END

In addition, four side keys are connected to the matrix.

#### Keypad Illumination

The illumination of the keypad is made by a “White” EL-Foil

### SIM

SIM cards with supply voltages of 1.8V and 3V will be supported.

### Bluetooth

Please refer [].

### Vibration Motor

All details are specified in [].

The vibration motor is mounted on the lower case. The electrical connection to the PCB is realized with spring contacts.

The modified SL65 Vibration motor will be reused. The vibration motor uses a silicon grommet motor housing to improve drop shock performance.

### Acoustical Transducer

All details are specified in [].

A dedicated receiver is used for telephony (no hands free) only. A second speaker on the rear side is used for all hands-free operations, ringing and playback.

### Power Supply, Battery, Charging

#### Overview

Most of the important functions for the power supply of the phone are carried out by the power supply ASIC Mozart/Twigo4. There are some additional power supply regulators. The POWER-pin of the I/O-Connector is for charging the battery with an external power supply. The standard X75 platform power supply is an unregulated open loop switched-mode charger.

#### Battery

For more details, refer to [].

A Li-Ion Battery with a nominal capacity of 550mAh (at relevant time frame) will be used.

Under GSM – discharge conditions the battery will provide a discharge capacity of.

An internal safety circuit prevents from over-charging, over-discharging and over-current.

### Antenna

#### Performance

In the Virgo antenna is integrated in the mobile. It is mounted on the lower mounting frame. It is coved by the battery cover and will be visible and touchable if the battery cover is disassembled. The Antenna will be a triple band antenna.

Two Bands GSM900 and GSM1800 will fulfill FBT requirements. The third Band PCS1900 only fulfills FBT requirements over 70% of all radio channels.

| Band | TX Low | TX Mid | TX high | RX low | RX mid | RX high |
| --- | --- | --- | --- | --- | --- | --- |
| EGSM | ≥28 | ≥28 | ≥28 | <-101 | <-101 | <-101 |
| PCN | ≥25 | ≥25 | ≥25 | <-101 | <-101 | <-101 |
| PCS | ≥TBC | ≥25 | ≥TBC | <-101 | <-101 | <-101 |

Please note:

| The FBT requirements are: (minimum per channel): | TX | RX |
| --- | --- | --- |
| EGSM | 28dBm | -101dBm |
| PCN | 25dBm | -101dBm |
| PCS | 25dBm | -101dBm |

#### SAR

SAR value of 0.8W/kg (1g) is SIEMENS target. First measurements indicated a value lower than 0.8 W/kg by today.

But for Data Transfer Mode the Handset Device may be held on close to the head with 2 TX radiating. A value less than 0.8 W/kg is not possible in this certain case.

The International SAR value limits as shown in the table below will be satisfied anyway.

| Limit | Head/ Body worn | Based on Standard | Countries |
| --- | --- | --- | --- |
| 1,6W/kg (1g) | Head | IEEE | Australia, Canada, S. Korea, Taiwan, USA |
| 1,6W/kg (1g) | Body worn | IEEE | Canada, Taiwan, USA |
| 2,0W/kg (10g) | Head | ICNIRP | EMEA, APAC |

# Software

## Software Feature List

Refer to [].

## Requirements to SW

Refer to [].

# Accessories

## Portfolio

Please refer to []:

## Feature List

All Features are specified in the global X75 Feature List, refer [].

# System Test

## Abbreviations and Definitions

| Bluetooth SIG | Bluetooth Special Interests Group |
| --- | --- |
| CTIA | Cellular Telecommunications and Internet Association, US. |
| CTR | Common Technical Regulation |
| EC | The European Community |
| EMC | Electro-Magnetic Compatibility |
| FCC | Federal Communications Commission |
| GCF | Global Certification Forum |
| GSMA | GSM Association. The Association of GSM Network Operators worldwide. |
| GSMNA | GSM North America. The North American Regional Interest Group of the GSM Association |
| ME | Mobile Equipment |
| PTCRB | PCS Type Certification Review Board |
| R&TTE | Directive 1999/5/EEC – Radio Equipment and Telecommunications Terminal Equipment Directive |
| SAR | Specific Absorption Rate |
| SIM | Subscriber Identity Module |
| SDO | Standards Development Organization |
| TOG | The Open Group. WAP testing & Certification |
| 3GPP | Third Generation Partnership Project. A project of SDOs in charge of elaboration of specifications for a 3. Generation mobile radio system (UMTS) |

## Approvals

| Requirement | Polaris EMEA (900/1800/1900) | Polaris APAC (900/1800/1900) |
| --- | --- | --- |
| R&TTE | X | X |
| FCC | X | X |
| PTCRB (approved by CTIA) | Not required | Not required |
| GCF | X | X |
| CCC (MII China) |  | X |
| TTY | Not required | Not required |
| HAC | Not required | Not required |
| IMEI Security Declaration | X | X |
| 5-key nob | X | X |

The X75 Products that are related to the R65 Platform still have a SGOLD2 and the Memory configurations are 256Mbit Code Flash stacked with 128Mbit LP-SDRAM

Battery will be discharged with 140mA, down to a voltage of 2,75 V

Battery will be discharged with 2A(0.6ms)+0.25A(0.4ms) down to 3.2V