- SL45

- Level 2.5e

Repair Documentation

- V 1.0

- Table of Contents:

1.	List of available level 2,5e parts  SL45	4

2.	Required Equipment for Level 2,5e SL45	5

3.	Required Software for Level 2,5e SL45	5

4.	Radio Part	6

4.1.	Power Supply RF-Part	6

4.2.	Frequency generation	7

4.2.1.	Synthesizer: The discrete VCXO (13MHz)	7

4.2.2.	Synthesizer: LO1	8

4.2.3.	Synthesizer: LO2	9

4.2.4.	Synthesizer: PLL	10

4.3.	Antenna switch (electrical/mechenical)	11

4.4.	Receivers	13

4.4.1.	Receiver: GSM900/1800 –Filter and LNA	13

4.4.2.	Receiver : Mixer, IF Amplifier and Demodulator	14

4.4.3.	Smarti IC Overview	15

4.5.	Transmitter	16

4.5.1.	Transmitter: Modulator and Up-conversion Loop	16

4.5.2.	Transmitter: Power Amplifier and Antenna Switch	17

5.	Power Supply	19

5.1.	Overview and Voltages	19

5.2.	STV-Schalke-ASIC	20

5.3.	Power Supply MP3 (LARA)	24

6.Logic Part	25

6.1.	Overview  Logic	25

6.2.	Overview  EGOLD+	26

6.3.	Overview EGAIM	28

6.4.	Overview  MMG ASIC	31

6.5.	MP3 (LARA) HW components	32

6.6.	FLASH memory	33

6.7.	Real time clock inside the EGOLD+	33

6.8.	IrDA	33

6.9.	SIM	34

7.	Acoustics	34

7.1.	General	34

7.2.	Vibra	34

7.3.	Microphone	35

7.4.	Loudspeaker	36

8.	Illumination and keyboard:	39

8.1.	Illumination	39

8.2.	Keyboard	40

9.	Connectors	41

9.1	I/O connector	41

9.2	Battery connector	42

9.3	Display connector	42

9.4	MMI connector	43

# List of available level 2,5e parts  SL45

ID-No		Type		Name(function)/Location	Rep-Code	Order  No.

| D100 | IC | EGOLD+ | 4EGO | L36810-G6103-D670 |
| --- | --- | --- | --- | --- |
| D250 | IC | SCHALKE_ASIC/Pow.Supply | 4PSA | L36145-J4682-Y29 |
| D400 | IC | MP3 (LARA) 1 | 4LA1 | L36820-C6118-D670 |
| D401 | IC | MP3 (LARA) 2 | 4LA2 | L36820-U6068-D670 |
| D470 | IC | MMG_SCHALKE_ASIC | 4MMG | L36145-J4681-Y54 |
| D490 | IC | Volt.Reg/RF | 4REG | L36820-C6148-D670 |
| D500 | IC | SMARTI | 4SMA |  |
| D550 | IC | LNA/RF | 4LNA | L36820-L6047-D670 |
| N600 | IC | Op.Amp/RF | 4OPA | L36810-C6046-D670 |
| N630 | IC | Volt.Reg/RF | 4REG | L36820-C6070-D670 |
| N631 | IC | Volt.Reg/RF | 4REG | L36820-C6147-D670 |
| V251 | Transistor | Charge/Logic | 4CHT | L36830-C1064-D670 |
| V530 | Transistor | VCO_SW/RF | 4SWT | L36820-C6047-D670 |
| V601 | Transistor | PA_Cont./RF | 4SWT | L36840-C4014-D670 |
| V651 | Transistor | PA_Cont./RF | 4SWT | L36840-C4009-D670 |
| V653 | Transistor | PA_Cont./RF | 4SWT | L36840-C2074-D670 |
| V810 | Transistor | 13_AMP/RF | 4SWT | L36840-C4039-D670 |
| Z100 | Quarz | EGOLD+/Logic | 4OSC | L36145-F102-Y14 |
| Z400 | Quarz | Quarz/MP3 (LARA) | 4OSL | L36145-F260-Y13 |
| Z500 | Filter | IF_360/RF | 4IFF | L36145-K280-Y127 |
| Z530 | VCO | 1LO/RF | 4VC1 | L36145-G100-Y32 |
| Z555 | Filter | RX-PCN/RF | 4FI1 | L36145-K280-Y167 |
| Z556 | Filter | RX-GSM/RF | 4FI3 | L36145-K280-Y160 |
| Z570 | VCO | TX/RF | 4VCT | L36145-G100-Y32 |
| Z650 | IC | Power_Amplifier | 4PAM | L36851-Z2002-A45 |
| Z800 | Quarz | 13MHz/RF | 4VCX | L36145-F220-Y4 |

# Required Equipment for Level 2,5e SL45

- GSM-Tester		(CMU200 or 4400S incl. Options)

- PC-incl. Monitor, Keyboard and Mouse

- Bootadapter 2000  (L36880-N9241-A200)

- Troubleshooting Frame P35 (F30032-A82-A1)

- Power Supply

- Spectrum Analyser  (Advantest 3162)

- RF-Probe incl. Power Supply  (e.g. from Agilent)

- Oscilloscope incl. Probe

- RF-Connector  (N<>SMA(f))

- Power Supply Cables

- Dongle (F30032-P28-A1)

- BGA Soldering equipment

Reference: Equipment recommendation Level 2,5e

# Required Software for Level 2,5e SL45

- Windows NT Version4

- Winsui version1.22 or higher

- Winswup

- Windows software for GSM-Tester ( Cats or CMU-GO)

- Software for 13MHz adjustment

- Internet unblocking solution

# Radio Part

The radio part converts the I/Q base band signals supplied by the logic 		Notes

(EGOLD+) into RF-signals with characteristics as per the GSM recommendation

(transmission) which are radiated by the antenna.

Or the radio part converts the received GMSK signal supplied by the antenna

into IQ base band signals which can then be further processed by the logic

(EGOLD+). The radio part is designed for Dual Band operation and can

therefore serve the frequency bands EGSM900 and GSM1800. The radio part

can never transmit and receive in both bands simultaneously. However, the

monitor time slot can be selected independently of the frequency band.

Transmitter and receiver are of course never operated simultaneously.

- The radio part consists of the following blocks:

- Power supply (RF-Voltage regulators)

- Synthesizer (partly located in SMARTI)

- Receiver (partly located in SMARTI)

- Transmitter (Up conversion loop partly located in SMARTI)

- Transmitter (Power amplifier)

- Antenna Switch

## Power Supply RF-Part

- Two voltage regulators (N630/631) with a nominal output voltage of 2.8V are

used, to perform the required “RF-Voltages”.

The voltage regulator N630 is activated via SLEEPQ provided by the

EGOLD+ (TDMA-Timer H13).

The voltage regulator N631 is activated via VCXOEN provided by

the EGOLD+  (Functional P7).

- The name of the voltages are: a) VCC_SYN activated by VCXOEN

and  b) VCC2_8    activated by SLEEPQ

- For both voltages BATT+ is required.

## Frequency generation

### Synthesizer: The discrete VCXO (13MHz)

The generation of the 13MHz signal is done in the SL45 via a discrete VCXO.		Notes

A Colpitts oscillator with a crystal Z800 and a post-switched buffer stage is used

as oscillator switch.

The subsequent oscillating circuit (C807,C817, L800) and the resistor R806

create a de-coupling of the synthesiser from interference signals coming from the

logic (SIN13M (functional M14)).

The oscillator frequency is controlled by the (AFC_PNM) signal which is generated

from the EGOLD+  (D100 (functional R3)) and the capacity diode V800.

To reduce the charging time of the low pass (R816, C816) the resistor R816 is

bridged by the diode V802

For the temperature control a temperature-dependent resistance R802 is placed

near the VCXO.

- The required voltage VCC_SYN is provided by the N631

- The picture 4211 shows the SIN 13M measured at TP120.

4211

E-Gold+

### Synthesizer: LO1

The first local oscillator (LO1) consists of the PLL part of the Smarti (D500), a loop	Notes

filter

and a VCO (Z530) module. This LO1 circuit generates frequencies from

1445MHz to 1520MHz for GSM 1800 RX-operation and from 1285MHz to 1361MHz

for the other operations. It is switched to select the channels in stages of 200kHz.

The VCO module is switched on by the EGOLD+  signal PLLON (TDMA-Timer J12)

via (V530). The switching between GSM900 and GSM1800 is done via the signal

OSW signal from the Smarti (D400 pin 21),

The channel programming of the PLL part of the Smarti happens via the EGOLD+

signals SYGCCL, SYGCDT, SYNSTR (RF Control  K14, K15, M15).

The VCO output signal enables the Smarti IC to mix the IF-Frequency (360 MHz)

The VCO output is also guided to the Smarti PLL part (D400) to ensure frequency

stability (output Smarti D500 pin 20). To do so the 13MHz frequency is used as the

reference signal for the PLL circuit.

The required voltage are: VCC2_8 for D400 is provided by N630.

VCC_SYN for the VCO is provided by N631

- The picture 4221 shows the VCO output signal

The picture 4222 shows Control voltage

The picture 4223 shows the programming signals for the PLL

4221

4223

4222

### Synthesizer: LO2

The second local oscillator (LO2) consists of the PLL part of the Smarti (D400),		Notes

2 internal VCO´s (1440 MHz for RX-Mode and 1696 MHz for TX-Mode) and an

external filter (C509,C510,R501),

Both frequencies are divided by 4 to get the 360MHz demodulator IF frequency and

the 424MHz modulator frequency.

The RX/TX switching is done internally and is initiated by the EGOLD+ through the

- SYGCCL, SYGCDT,SYNSTR (RF Control  K14, K15, M15) signals.

Responsible for switching “On/Off” the second local oscillator in the EGOLD+ signal

- PUPLO2 (TDMA-Timer L11) at SMARTI pin 16.

- The required voltage VCC2_8 for D400 is provided by N630

The picture 4231 shows the VCO output signal

4231

External filter LO2

VCO LO2

PUPLO2

### Synthesizer: PLL

- PLL as a part of the PMB6250 (Smarti) IC

- Blockdiagramm

## Antenna switch (electrical/mechenical)

Internal/External <> GSM900/1800 <> Receiver/Transmitter 			Notes

- The SL45 mobile consists of two antenna switches.

a:	The mechanical antenna switch (connector X675) for the differentiation

between the internal and external antenna

from/to

diplexer

b:	The electrical antenna switch (Diplexer Z670) for the differentiation

between the receiving and transmitting signals, just like the differentiation

between GSM900 and GSM1800.

To do so the Transistors V672,V673 are used to switch the

diplexer input signals VC1–VC4 as required.

The required voltage VCC2_8 for D400 is provided by N630

GSM900

from PA

GSM1800

to receiver

GSM900

to receiver

GSM1800

from/to

antenna

- Notes

- The matrix below shows the different conditions at the Diplexer and the

accompanying signals.

## Receivers

### Receiver: GSM900/1800 –Filter and LNA

From the antenna switch, up to the IF-Mixer the GSM1800 receiver circuit 		Notes

consists of a ceramic front end filter (Z555),a LNA (Low Noise Amplifier D550) and

a discrete distortion LC-high-pass-filter after LNA amplification.

For GSM900 the signal flow is as follows: From the antenna switch via a SAW filter

(Z556) through the LNA (D550) and a discrete distortion LC-low-pass-filter to the

IF-Mixer.

The amplification of both LNA´s is approx. 18dB with a matched 50 ohm output. To

switch on the LNA, the signal LNAON (Smarti pin 36) is used.

LNAG (Smarti pin 33) is in use if the received signal level is too high. With the

high signal LNAG the LNA reduces the incoming signal in one step by 20dB

For activating the GSM1800 or the GSM900 LNA part a signal BSW is generated

from the (Smarti pin 37)

The non-symmetrical output of the LNA (D550) is connected to the IF mixer

via a discrete balancing and adaptation circuit This circuit converts the asymmetrical

input signal into a symmetrical signal.

- The required voltage VCC2_8 is provided by the N630

4412

4415

4411

4414

4413

### Receiver : Mixer, IF Amplifier and Demodulator

The Smarti IC (D400) has two separate input mixers one for EGSM900 and 		Notes

one for GSM1800.

The mixing result for both mixers is an intermediate frequency from 360 MHz.

The GSM900 mixer works in inverted sideband mode (LO1 above RX frequency),

the GSM1800 mixer in step mode (LO1 below RX frequency).

The outputs of both mixer are connected to the same IF Filter.

After passing an external IF Filter (Z500) the signal arrives at the programmable IF

amplifier in the Smarti. This amplifier has a dynamic of 80dB (-20dB to 60dB) and

can be set (PGCSTR;SYGCDT;SYGCCL) in 2dB steps.

Finally the signal is mixed down by the IQ demodulator to differential

I and Q (RX_I,RX_IX-RX_Q,RX_QX) signals off 50mVpp.

This signals are guided to the EGold+  A/D converters in the base band path.

4422

4421

4423

4424

from LNA

to

EGold+

### Smarti IC Overview

Not active

## Transmitter

### Transmitter: Modulator and Up-conversion Loop

The SL45 modulation is based on the principle of the up-conversion modulation		Notes

phase locked loop and is accomplished via the Smarti IC(D400).

The Smarti IC provides the quadratic modulator working with the TX IF frequencies

GSM/PCN 424 MHz. Whereby these frequencies are supplied from the second local

oscillator signals (1694MHz/4).

This so generated GMSK RF signal is compared in a phase detector with the down

mixed GMSK RF output.

To get the comparison signal the TXVCO_OUT signal is mixed with LO1 signal.

With the help of the 1.LO the GMSK-RF signal appearing at

the output of the TXVCO (Z570) is mixed to a TX IF below the TX signal and is

led on to the phase detector. The I-Q modulated signal in the IF position is also led

to the phase detector.

The output signal of the phase detector passes a discrete loop filter realized by

capacitors and resistors and force the TXVCO to work on the right frequency.

This large loop band width guarantees that the regulating process is considerably

quicker than the changes in the modulation signal.

The TXVCO is a so-called two-in-one VCO, this means the VCO module contains

the GSM900-VCO and the GSM1800-VCO in one housing.

The TXVCO is switched from GSM to PCN by using the signals GSM_TX_VCOENQ

and PCN_TX_VCOENQ from the EGold+ (TDMA Timer J13, K12)

- The required voltage VCC2_8 is provided by N630

The required voltage VCC_SYN is provided by N631

4511

: 4

Phase

Detector

I Q

Mod

ulator

1.LO

1285-1361 MHz

424 MHz

TX-VCO

TP

Mod Signal

2.LO

1696 MHz

880-915 MHz

424 MHz

1710-1785 MHz

424 MHz

4512

4513

### Transmitter: Power Amplifier and Antenna Switch

Splited by a discrete circuit into GSM900 (low pass R651/L650) and GSM1800 		Notes

(high pass R652/C651) the TXVCO output signal arrives at the power amplifier.

The dual band power amplifier module (Z650) is assembled on a ceramic substrate

in one housing. The module amplifies the output signal of the TXVCO to the

required PCL. Controlled by the feedback circuit according to settings from the

logic.

- A part of the TX output signal is decoupled via a directional coupler (realised by

conductive tracks) and is equalised at a detector diode (V620).

- This so gained voltage is compared by an operation amplifier (N600) with the

PA_RAMP signal provided by the EGold+ (GAIM/BASEBAND H2), to ensure that

the PA is working within the required PCL´s.

- The different amplifiers are selected on by the PCN_TX_VXCOENQ signal from

the EGold+ (TDMA Timer K12). The power amplifier is feeded directly from the

battery (BATT+).

After amplification the signal passes on the way to the antenna the diplexer (Z670)

and antenna connector (X675).

The required voltage BATT+ is provided by the battery.

The required voltage VCC2_8SW is provided by transistor V600.

4522

4521

4523

4524

TXVCO_OUT

PA_RAMP

to diplexer

PF08112B

VCC2.8SW

Batt+

# Power Supply

## Overview and Voltages

Notes

Ri

Linear-

Regler

[SCHALKE_ASIC]

Linear-

Regler

[SCHALKE_ASIC]

IrDA

LED

1,6 A

im TX

47 µF

(in Logik)

110 mA max.

2,9V +-2%

2,65V +-3%

140mA max.

220mA max.

2,8V +- 3,5%

2,8V +- 3,5%

75 mA max.

45 mA max.

60mA max.

50mA max. 3)

EGOLD+ Core

MMG SCHALKE_ASIC

4Mbit SRAM

Flash

Beleuchtung Display

+ Tastatur

Flash 2) + SIM

EGOLD+ Portpin

Vibrator (evtl. an Batt+)

RX, TX

RF Synth / VCXO

Leerlaufspannung 1) :

4,3V..3,2V

Li Thin-Ion

540mAh

PA

+Treiber

+Regelung

Linear-

Regler

[SCHALKE_ASIC]

2,07V +-3%

VDDRTC

Linear-

Regler

[SCHALKE_ASIC]

2,0V +-5%

140mA max.

10mA max.

EGOLD+ Ringing

(16Ohm HK+GAIM)

50mA

150mA + 20mA.

MMC

55mA+6mA

5mA

MMG-SCHALKE_ASIC I/O

22 µF

(an PA)

Display

20mA max.

1mA max.

2) Intel 55mA max.

ST 20mA max.

1) bei Akku ziehen:

6,5V max. für ?s

LARA (MP3)

Linear-

Regler

extern

2,9V +-4%

100mA max.

Linear-

Regler

extern

Linear-

Regler

extern

100mA max.

100mA max.

100mA max.

3) 3201: 32mA typ.

The following restrictions must be observed:

- The phone cannot be operated without battery.

- The phone will be damaged if the battery is inserted the wrong way

round (the mechanics of the phone prevent the battery from being put

in the wrong way round). The electric system assumes that the battery

as been inserted correct.

## STV-Schalke-ASIC

- The power supply Schalke-ASIC contains the following functions:			Notes

- Control of “switching on” the mobile phone via

- 1.The On/Off Key. (Schalke-ASIC F4)

2.The bottom connector with the signal VDD_CHARGE (Schalke-ASIC A4)

3.The Real Time Clock RTC_Int (Schalke-ASIC F5)

- Watchdog monitoring

1.Control of “switching off” the mobile phone via WATCHDOG_P.

- 2. Watchdog observation

- Switch off of mobile phone in the case of overvoltage at battery connection.

- Generation of RESET signal for EGOLD+, Flash, MMG_ASIC and

Display

- Generation of 2.90 V via linear controller (REG_1)

- Generation of 2.00 V via linear controller (REG_2)

- Generation of 2.65 V via linear controller (REG_3)

- Generation of 2.00 V via linear controller (LPREG)

- Analog switches

- SIM Switch

- LIGHT Switch

- IRDA Switch

- VSPG_Teiler Switch

- Battery charge support:

- Normal charging

- Trickle charging, if the battery voltage is below 3,2 V.

- Timer generation

All internal timers and pulses are derived from a 900 kHz  10% internal

oscillator. Responsible for the frequency stability is an external resistor

(R257) (1%) at the RREF pin.

- Audio

- Audio amplifier

- Mono audio driver for internal loudspeaker

- Driver to generate ringer tones

- Key-click generator

- I2C bus for communication between EGOLD+ and Schalke-ASIC

- Switch on timing overview

- Switch off timing overview

#### Battery

A Li-Ion battery with a nominal capacity of 540mAh is used for SL45.			Notes.

A temperature sensor (NTC) is integrated to monitor the battery temperature.

#### Charging Concept

- General

- To control the charging process a charging control unit consists of a Fast Charge

- Controller and a Trickle Charger is implemented. Existing of a internal current

source, a temperature monitoring, a external charge-FET, a charge detector

( voltage level and/or charge current of an external charger supplied trickle charge

circuit) and a detection of the charge current in 5 steps.

The charging control unit communication with the EGOLD+ is done via the I2C bus.

- CHARGE_C: charge request input of the EGOLD+

- CHARGE: Open Drain, -300...-600A

- TBAT: comparator input for temperature sensor of the battery

- REF_EXE: Reference voltage for temperature sensor

- VDD_CHARGE: Charger typ detector input, power supply for trickle charge

- SENSE_IN: Voltage input of external current sensor (with reference to AVDD)

- With rising slope on CHARGE_C the current source is switched on via a LOW

- active CHARGE signal and the Charge-FET is conductive.

Condition:

- CHARGE_C is HIGH for min. 20s

- temperature sensor detects no overhead

- No overvoltage on VDD.

- With falling slope on CHARGE_C the current source is switched off and the

Charge-FET is locked.

Temperature sensor works by T>60°C.

- Fast charging

- After connecting a charger and a battery voltage higher then 3,2 V the controller

switches via a HIGH level of CHARGE_C the charge-FET conductive, if the level

of TBAT is > then the level of REF_EXE.

The charging current (and battery voltage) is controlled with the pulse duty factor of

CHARGE_C. The current flow can detected with the signal SENCE_IN, controlled

via the I2C bus from the EGOLD+.

- Trickle charge

If the phone has not been used for a longish time (longer than approx. 1 month), the

battery could be totally self-discharged. (battery voltage less then 3,2V), so that

it is not possible to charge the battery via the normal charging circuit. In this case

only trickle charge is possible.

The Schalke-ASIC controlles the charging circuit himself.

Battery voltage below 2,8 Volt charging current 20mA.

Battery voltage below 3,2 Volt charging current 50mA.

Battery voltage over 3,2 Volt “Normal charging”.

Power supply for the Schalke-ASIC in this mode is the external charger.

(VDD_CHARGE) This charging mode is not visible for the customer. IF the battery

voltage reaches the 3,2V level the Schalke-ASIC switch into normal charge.

#### Measurement of Battery and Ambient Temperature				Notes

- The voltage equivalent of the temperature on the voltage divider is measured

as the difference against a reference voltage of the EGOLD+. For this, the

integrated  converter of the EGOLD+ of the RX-I base band branch is used.

Via an analog multiplexer, either the RX-I base band signal,(the battery temperature

- Voltage) or the ambient temperature voltage can be switched to the input of the

converter. The 1-Bit current of the converter will be subjected to a data reduction via

the DSP circuit so that the measured voltage (for battery and ambient temperature)

will be available at the end as a 10-bit data word.

#### Measurement of the Battery Voltage

Analog to the I-branch either the RX-Q base band signal or the battery voltage can

be measured in the Q-branch. The processing in the DSP circuit is done similar to

the I-branch. The EGOLD+ is specified for voltages measurements at the input

pin N1 (VBAT) from  3V...5.5V.

#### Timing of the Battery Voltage Measurement

- Unless the battery is going to be charged, the measurements are made in the

TX time slot. While charging the measurement is done after the TX time slot.

- At the same time, either the battery temperature (in the I-branch) and the

battery voltage (in the Q-branch) or the ambient temperature in the I-branch can be

measured

- Other combinations are not possible. For the time of the measurement

the multiplexer in the EGOLD+ must be programmed to the corresponding

measurement.

## Power Supply MP3 (LARA)

- A voltage regulator (N490) with a nominal output voltage of 2.9V is

used, to perform the required MP3 (LARA)  voltage.

The voltage regulator N490 is activated via MP3 (LARA) _EN provided by the

EGOLD+ (RF-Control L12).

- The name of the voltages is  2V9_MP3 (LARA)  activated by MP3 (LARA) _EN

- For the voltage BATT+ is required.

# 6.Logic Part

## Overview  Logic

MICP1,

MICN1

EPP1,

EPP2

VSPG_TEILER

Display

80x101

Flash

32Mbit

MMG

inkl.

3Mbit SRAM

MMC

LARA (MP3)-

Dek.

DAC

EGOLD+

Keyboard

Radio-

part

IrDa

SIM

3V

1,8V

SIM-Switch

6 Register  I2C               LDO  1,92V

LDO 2,65V

GAIMLDO  2,9V

Schalter                                                                            RTC-LDO 2,0V

Schalke

Laden,                     Licht-PWM

Trickle Charge

Dimmen

I/O

Battery

540mAh

Li-Ion

LEDs

Key/

Display-

illumination

Vibra

GAIM-

Voltage-

divider

A(0:22), D(0:16), RD, WR

CS_FLASH

CS_SRAM, BHE

CS_MMG

KB0:KB9

ON_OFF=KB9

CS_LCD, RS_LCD,

SCL, SI

I2CI

LS_LEFT,

LS_RIGHT

I²C EPP2/EPN2

I²S

LARA (MP3)

CLI,

WSI,

DAI

SIC,

SID,

DAT_REQ

MMCCLK,

MMCDAT,

MMCCMD

Power

AKKU

CCVCC

CCRST,

CCCLK,

CCIO

IR_RX,

IR_TX

IRDA_OFF

s. Schnittstelle HF-Logik

LIGHT

CCVZQ

RESET_2.65V

CHARGE_uC

WA_DOG,

RTC_INT,

SLEEP, LOW_BATTERY

VCXO_EN

LDO

3V

I²S

3V

3V

MMCVCC

MMCSW

LARA (MP3)_EN

BATT+

2,9V

ON_OFF

IRQ_MMG, SYSCLK

RESET_2.0V

s. I/O-Schnittstelle

Vertraulich - Confidential

VIBRA_ON

EPP1_FIL/EPP2_FIL

## Overview  EGOLD+

The µC-part components								Notes

- Mikrocontroller

- Systeminterfaces for internal and external peripherals

- Onchip peripherals and memory chips

- The Controller-Firmware:

- Controlling of the Man Machine Interface 
(Keypad, LCD, Illumination, ... )

- GSM Layer 1-3

- Controlling Radio part (Synthesizer, AGC, AFC, ...),

- Controlling of the Baseband part (EGAIM)

- The SP-Part components

- DSP Signalprozessor

- Die DSP-Firmware

- Equaliser

- Channel Coder

- Channel Decoder,

- Voice Encoder

- Voice Decoder

## Overview EGAIM

- Notes

- EGAIM inside the EGOLD+

This IC is the interface between digital and analogue signals.

EGAIM components

- 2 Sigma Delta A/D-converter for RX/Battery

- 2 D/A- converter for GMSK TX Inphase- und Quadratursignals,

- 1 D/A- converter for the Power Ramping signal,

- 1 Sigma Delta A/D- und D/A-converter for the voiceband part

- Measurement of Battery and Ambient Temperature

The temperature is measured as a voltage equivalent of the temperature on the	Notes

voltage dividers R115,R116,R117 for the battery temperature and R118,R119,

R120 for the ambient temperature from the EGAIM.

For this, the integrated  converter of the EGAIM of the RX-I base band

branch is used. This  converter compares the voltage of TBAT and TENV

internally with a reference voltage BREF.

Via an analog multiplexer, either the RX-I base band signal, or the TBAT signal

and the TENV signal can be switched to the input of the converter.

The signal MEAS_ON from the EGOLD+(GSM TDMA-TIMER G11)  activates the

measurement and is used to generate to BREF by the help of R121,R122

b)	Measurement of the Battery Voltage

The measurement of the battery voltage is done in the Q-branch of the EGAIM.

For this BATT+ is connected via a voltage divider R113, R114 to the EGOLD+

(GAIM N2) (Input limitation 1.33V to 5.91V) .An analog multiplexer does the

switching between the baseband signal processing and the voltage

measurement.

- A/D conversion of MIC-Path signals incl. coding

The Microphone signals (HF_MICRO, MICN2, MICP, MICN) arrive at	Notes

the voiceband part of the EGAIM. For further operations the signals will

be converted into digital information, filtered, coded and finally formed

into the GMSK-Signal by the internal GMSK-Modulator.

This so generated signals (IT, ITX, QT, QTX) are given to the SMARI IC

(pins 9-12) in the transmitter path.

- D/A conversion of EP-Path signals incl. decoding

Arriving at the Baseband-Part the demodulated signals (RX_I, RX_IX, RX_Q,

RX_QX) will be filtered and A/D converted. In the voiceband part after decoding

(with help of the uC part) and filtering the signals will be D/A converted amplified

and given as (EPP1, EPN1, EPP2, EPN2) to the internal earpiece or the external

loudspeaker.

- Generation of the PA Control Signal (PA_RAMP)

The RF output power amplifier needs an analog ramp up/down control voltage.

For this the system interface on EGOLD+ generates 10 bit digital values which

have to be transferred serially to the power ramping path. After loading into an

10 bit latch the control value will be converted into the corresponding analog

voltage with a maximum of ~2V

Key-Value

## Overview  MMG ASIC

MMG-ASIC components								Notes

- Frequency generation

The PLL generates a 52MHz signal based on the 13MHz-

(output CLKSXM EGOLD+). The signal is  the reference of all  Clock and Timing

signals of the ASIC.

- I²S-Interface:

The interface modulate the data of the DAI-Interfaces (EGOLD+) into a

I2S-data format, used for the input of the stereo-D/A-converter in the DAC3550.

- MP3 (LARA) - Interface:

Generate a MP3 (LARA) -data current. Conversation of parallel-/serial data of

the 16bit-parallelbus

- MMC-Interface:

Access to the MMC via serial HW-Interface. Control commands and

data transmission from and to the EGOLD+.

- 3MBit SRAM:

SRAM implemented for write and read access.

## MP3 (LARA) HW components

#### DAC3550

Stereo-D/A-converter analogue - multiplexer are inside, for speech and 		Notes

MP3 (LARA)  output. (not simultaneously). Loudness level adjusted via I²C.

- MAS3507

Decoding off MP3 (LARA) data

## FLASH memory

Non volatile, but erasable and re-programmable (Software-Update) memory chip 	Notes

of the EGOLD+, also to save user data (menu settings), Voice Memo,

adjustment values

## Real time clock inside the EGOLD+

Real time clock, supplied from a voltage regulator inside the Schalke-ASIC.

Also active when mobile phone is switched off. During battery change a

capacitor (C258) as power supply is used for minimum 30 sec to save

the stored data.

## IrDA

Infrared diode for data transfer to standard IrDa devices.

## SIM

SIM cards support of 1.8V and 3V cards.						Notes

# Acoustics

## General

- The Electro-Acoustic components are:	a) Vibra

b) Microphone

c) Loudspeaker

d)Ringer

## Vibra

The Vibra is driven and controlled from the EGOLD+ (Miscellaneous M6)via the

signal Vibra_ON.

The vibrator is activated by the transistor V290 via the Signal Vibra_on from		Notes

EGOLD+. Batt+ is required to provide the VIBRA. The diode V292 is used

to protect the circuit against over voltage and switching spikes.

## Microphone

Both Microphones are direct connected to the EGOLD+.(Voiceband F1-F4) via the

signals MICN1, MICP1 (Internal Microphone )and MICN2, MICP2

(External Microphone/Headset). Power supply for the Microphone is

VMIC (Voiceband E1)

- Internal Microphone

External Microphone									Notes

## Loudspeaker

The internal Loudspeaker (Earpiece) is connected to the voiceband part of the

EGOLD+ (VOICEBAND C1,D2) via the mono audio amplifier inside the Schalke-ASIC.

Also the ringing tones are generated with the loudspeaker.

Signal names:	 EGOLD+ - Schalke-ASIC		EPN1_FIL - EPP1_FIL

Signal names:	 Schalke-ASIC - Speaker		EPP1 - EPN2			Notes

- The external Loudspeaker (Headset) is connected to the voiceband part of the

EGOLD+ (VOICEBAND D1,E2) (EPN/EPP2)via the Stereo-D/A-converter

analogue - multiplexer inside the DAC3550.(HS_LEFT/HD_RIGHT)

The SL45 has no ringer component. Responsible for the audio output of the ringer 	Notes

tones is the loudspeaker. The ringer tones are generated by a signal RINGIN from

the EGOLD+ (Miscellaneous M11). This signal goes to the mono audio amplifier inside

the Schalke-ASIC.

# Illumination and keyboard:

## Illumination

The Light is switched via an analogue switch inside the Schalke-ASIC. It is controlled	Notes

from the EGOLD+ with the signal LIGHT_OFF. Output is the signal LIGHT which is

connected via the MMI connector the LEDs.

## Keyboard

The keyboard lines are via MMI connector linked to the EGOLD+			Notes

- (Keybad R7/R8/P8/N8/M8/N9/R10/P10/N10/R11). KB (keyboard line ) 0 to 5 are outputs,

KB 6 to 9 inputs. To make a short cut from an output to an input line with the keypad

the EGOLD+ registered with key is pressed.

# Connectors

## I/O connector

Notes

| Pin | Name | IN/OUT | Notes |
| --- | --- | --- | --- |
| 1 | GND |  |  |
| 2 | HS_LEFT | I/O | Output left channel for stereo-headset. |
| 3 | POWER | I | Charging Current |
| 4 | FBatt+ | O | Power supply for the accessories. |
| 5 | TX | O | Serial interface |
| 6 | RX | I | Serial interface |
| 7 | ZUB_CLK | I/O | Clock line for accessory bus Use as DTC In data operation |
| 8 | ZUB_DATA | I/O | Data line for accessory bus. Use as CTS in data operation |
| 9 | GND_MIC |  | For external microphone |
| 10 | MICP2 | I | External microphone |
| 11 | HS_RIGHT | O | Output right channel for stereo-headset |
| 12 | LGND |  | For external loudspeaker |

## Battery connector

| Pin | Name | IN/OUT | Notes |
| --- | --- | --- | --- |
| 1 | GND |  |  |
| 2 | Akku_Temp | O | Temperature control of the battery pack. |
| 3 | POWER + | I/O | Battery voltage |

## Display connector

Connection Egold+ to

Display-Controller

## MMI connector

| Function | Names |
| --- | --- |
| ILLUMINATION | Light |
| KEYBOARD | KB0 – KB9 / ON_OFF |
| IrDA INTERACE | IR_RX / IR/TX_BUFFER |
| BATTERY VOLTAGE | BATT+ |