S25(88)

Level 2.5

Repair Documentation

V 1.0

# Introduction

The S25 is the first dualband handset (GSM-900 and GSM-1800) in the S-class. 
The S2588 is a special version for the asian market with the same hardware but a different Software.

The repairs for S25 and S2588 are identical unless otherwise noted.

This manual is intended to help you carry out repairs on level 2.5, meaning limited component repairs. Failure highlights are documented and should be repaired in the local workshops.

It must be noted that all repairs have to be carried out in an environment set up according to the ESD (Electrostatic Discharge Sensitive Devices) regulations defined in international standards.

If you have any questions regarding the repair procedures or spare parts do not hesitate to contact our technical support team in Kamp-Lintfort, Germany:

Tel.: +49 2842 95 4666

Fax: +49 2842 95 4302

e-mail: dominik.schnoor@klf.siemens.de

Table of Contents:

1	Introduction	2

2	Antenna Connector	4

3	Ringer Connector	8

4	Bottom Connector (Lumberg)	11

5	18µH Coil	14

6	Antenna Spring	18

7	1,5 A Fuse	21

8	Cardreader	24

9	Vibraconnector	27

10	Sideswitch MMI	30

11	Connector MMI	34

# Antenna Connector

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3ANC

## Fault Description

### Fault Symptoms for customers:

Network Search when using the external antenna (carkit)

No location update possible on external antenna (carkit)

### Fault Symptom on GSM-Tester:

Output power problems on the external antenna

No location update possible

### Component Information

The Antenna Connector is a mechanical switch operated by the RF plug of a carkit or, for testing purposes, of an RF clip.

Normally the RF signal goes to and comes from the internal antenna. Whenever an RF plug is plugged into the antenna connector the connection to the internal antenna is openend and the connection to the external antenna socket is made. See drawing below.

From Power Amplifier/

To Receiver

To / From Internal Antenna

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Check the output power of the handset with the LSO testprogram. Especially watch the external antenna power!

#### Repair by component change

Use hot air blower to remove defective connector

Avoid excessive heat!

Watch surrounding components!

Resolder new connector afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair as described above.

### List of needed material

#### Components

S25 antenna connector

Part-Number:	L36334-Z93-C261

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 2: C25 Antenna Connector Placement (X400)  (Top View)

Figure 1: S25 Board Antenna Connector Side (Top View)

Figure 2: S25 Antenna Connector Side (Top View)

# Ringer Connector

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3RIC

## Fault Description

### Fault Symptoms for customers:

Problems with the handset ringer. No ringer tone audible.

### Fault Symptom on GSM-Tester:

Handset fails ringer test.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

The connector X5 is connecting the main board of the C25 with the piezo ringer through a two pin cable.

#### Diagnosis

Visually check the connector. Watch for bent contacts and dry joints.

#### Repair by component change

Resolder dry soldering joints.

If the connector is physically damaged use hot air blower or wick to remove defective connector.

Avoid excessive heat!

Watch surrounding components!

Resolder new connector afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

Ringer Connector S25:

Part-Number:	L36334-Z97-C43

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

Flux

### Drawings

Figure 1: S25 Board Ringer Connector Side

Figure 2: S25 Ringer Connector (X5) Placement (Top View)

# Bottom Connector (Lumberg)

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3LUC

## Fault Description

### Fault Symptoms for customers:

Charging problems.

Problems with external loudspeaker or microphone when using a car kit.

Problems with accessories connected at the bottom connector.

### Fault Symptom on GSM-Tester:

Testequipment cannot communicate with the handset.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Visually check the bottom connector. Watch for dry joints!

#### Repair by component change

Use hot air blower remove defective bottom connector.

Avoid excessive heat!

Watch surrounding components!

Resolder new bottom connector afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

Bottom Connector S25

Part-Number:	 L36334-Z93-C262

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 1: S25 Board Bottom Connector Side

Figure 2: S25 Bottom Connector (X4) Placement (Top View)

# 18µH Coil

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3COI

## Fault Description

### Fault Symptoms for customers:

Loud humming noise in loudspeaker.

### Fault Symptom on GSM-Tester:

Handset fails with loud humming noise in echo loop.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

The 18µH coil is used in the step up converter which is generating a

5.4 V supply voltage for the power amplifier out of the 2.8V battery voltage.
If the coil is mechanically damaged (broken) it produces heavy interference with the acoustical elements of the S25 resulting in a loud humming noise in the earpiece.

A broken coil can easily be diagnosed by trying to move it with two fingers. If it moves, the core is broken and the coil has to be

replaced.

#### Repair by component change

Use hot air to remove defective coil.

Avoid excessive heat!

Watch surrounding components!!

Resolder new coil afterwards

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair by checking the audio quality with the echo loop of the testprogram.

### List of needed material

#### Components	18µH Coil

Part-Number:	L36151-F5183-M

#### Jigs and Tools

Soldering Iron

Hot Air Blower

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 1: S25 Board 18µH Coil (L201) Side

Figure 2: S25 18µH Coil (L1) Placement (Top View)

# Antenna Spring

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3ANS

## Fault Description

### Fault Symptoms for customers:

Network Search.
Handset drops calls.

### Fault Symptom on GSM-Tester:

Power problems on the internal antenna of the handset 
					only.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

The antennaspring connects the main board with the internal antenna of the handset.

#### Diagnosis

Visually check the status of the spring. Bent or oxidated springs have to be replaced.

#### Repair by component change

Use soldering iron to remove defective spring.

Avoid excessive heat!

Watch surrounding components!

Resolder new spring afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

Antenna Spring S25

Part-Number:	 L36158-A25-C9

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 1: S25 Board Antenna Spring Side

Figure 2: S25 Antenna Spring (X401) Placement (Top View)

# 1,5 A Fuse

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3FU1

## Fault Description

### Fault Symptoms for customers:

Battery charging doesn’t work.

### Fault Symptom on GSM-Tester:

This fault cannot be detected with a GSM-Tester.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Simply measure the resistance of the fuse with a multimeter. Open connection means the fuse is defective. Otherwise it should be close to zero ohms.

#### Repair by component change

Use soldering iron to resolder dry joints or use hot air blower to 				remove defective fuse.

Avoid excessive heat!

Watch surrounding components!

Resolder new fuse afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

1,5 A fuse S25:

Part-Number:	 L36145-A820-Y12

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 1: S25 Board 1,5 A Fuse Side

Figure 2: S25 Fuse (F1) Placement (Top View)

# Cardreader

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3REA

## Fault Description

### Fault Symptoms for customers:

Handset does not accept Simcard

### Fault Symptom on GSM-Tester:

This fault cannot be detected with a GSM-Tester.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Visually check the SIM-Card reader. Look for dry joints or mechanical problems.

#### Repair by component change

Use soldering iron to resolder dry joints or use hot air blower to 				remove defective reader.

Avoid excessive heat!

Watch surrounding components!

Resolder new reader afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

SIM-Card reader S25:

Part-Number:	 L36334-Z97-C51

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 1: S25 Board Cardreader Side

Figure 2: S25 Cardreader Placement (Top View)

# Vibraconnector

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3VIC

## Fault Description

### Fault Symptoms for customers:

Vibrator function does not work

### Fault Symptom on GSM-Tester:

This fault cannot be detected with a GSM-Tester.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Visually check the Vibramotor connector.

#### Repair by component change

Use soldering iron to resolder dry joints or use hot air blower to 				remove defective connector.

Avoid excessive heat!

Watch surrounding components!

Resolder new connector afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

Vibraconnector S25:

Part-Number:	 L36334-Z93-C268

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 1: S25 Board Vibraconnector Side

Figure 2: S25 Vibraconnector Placement (Top View)

# Sideswitch MMI

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3SSW

## Fault Description

### Fault Symptoms for customers:

Voice memo or up/down button does not work

### Fault Symptom on GSM-Tester:

The keyboard test fails on the defective button.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Visually check the switches on the MMI board.

#### Repair by component change

Use soldering iron to resolder dry joints or use hot air blower to 				remove defective switch.

Avoid excessive heat!

Watch surrounding components!

Resolder new switch afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

Sideswitch S25:

Part-Number:	 L36315-.Z77-C194

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 1: S25 MMI Board Sideswitch Side

Figure 2: S25 Up/Down Switch Placement (Top View)

Figure 3: S25 Voice Memo Switch Placement (Top View)

# Connector MMI

## Affected Units

### Type:					S25

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3MMC

## Fault Description

### Fault Symptoms for customers:

Problems with MMI functions like:

- Display malfunction

- Keyboard malfunction

- Illumination problems

### Fault Symptom on GSM-Tester:

The keyboard / display test fails.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Visually check the connector on the MMI board. Look for mechanical damages or dry joints.

#### Repair by component change

Use soldering iron to resolder dry joints or use hot air blower to 				remove defective connector.

Avoid excessive heat!

Watch surrounding components!

Resolder new connector afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

MMI Connctor S25:

Part-Number:	 L36195-Z26-C626

#### Jigs and Tools

Hot Air Blower

Soldering Iron

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

### Drawings

Figure 1: S25 MMI Board Sideswitch Side

Figure 2: S25 MMI Connector Placement (Top View)